3GPP TSG-RAN WG1 Meeting #122-bis	R1-2506946

Prague, Czech Republic, October 13th–17th, 2025

Agenda Item:	10.5.1

Source:	Huawei, HiSilicon

Title:	Performance metric, methodologies, and initial evaluation results for ISAC

Document for:	Discussion and Decision

Introduction

In RAN1#122, a set of agreements were reached on the aspects of performance metrics, methodologies and simulation assumptions for 5G-A ISAC. According to the SID [1] and the feature lead summary [2], some remaining issues are also suggested to be fully studied.

Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized.

In this contribution, we investigate the FFS pertaining to the agreements and provide our input to simulation assumptions, sensing measurements and quantization as well as the initial evaluation results.

Remaining issues on sensing metrics and methodologies

In this section, we will discuss the sensing metrics and related methodologies along with the FFS for false alarm probability Type2.

Agreement - False alarm probability is agreed as performance metric for NR ISAC. - For cases without true target dropped in simulation area, False alarm probability Type 1 is computed and reported - For cases with true targets dropped in simulation area, False alarm probability Type 2 is computed and reported - Note: both False alarm probability Types are mandatory - KPI values for False alarm probability Type 1 and 2 can be discussed separately. Definitions: - False alarm probability Type 1 (no target dropped in simulation area): An object is detected when there is no target present in simulation area is considered a false alarm. Pf1=n=0N-1QnN Where, - Qn equal to 1 if at least one object is detected when there is no target dropped in the simulation area in the drop n, otherwise Qn equal to 0. - N is the total number of drops without targets in the simulation area. - False alarm probability Type 2 (targets dropped in simulation area): An object is detected but not associated with any true targets in the simulation area is considered as a false alarm. Pf2=0≤n<NMn'≠0Dn'Mn'K Where, - Dn' is the number of detected objects but not associated with any true targets in the drop n. - Mn' is the total number of detected objects in the drop n. - FFS: - Option 1: K is number of drops (N) - Option 2: K is number of drops with at least one detected object Note: the number of targets should be reported by companies when providing False alarm probability Type 2

For the FFS on the false alarm probability Type 2, the denominator is defined differently: Option 1 uses the total number of drops, while Option 2 uses the number of drops that contain at least one detected object.

In our view, firstly the derivation of Type 2 should be based on the ratio of the number of false alarms to the number of detected objects whereas the indetermined form of 0/0 should not be taken as 0% false alarm probability for the drop.

Moreover, according to the FLS in [2], one candidate definition for false alarm probability Type 2 is expressed as Pf=n=0N-1Dn'n=0N-1Mn', where there is no contribution to the calculation of Pf for the drops with 0/0, which is quite aligned with option 2.

With the above reasons, option 2 for false alarm probability Type 2 is preferred.

Proposal 1: Option 2 that K is number of drops with at least one detected object is selected for the false alarm probability Type 2.

Simulation assumptions

In RAN1#122 meeting, some basic simulation assumptions had reached the agreements, and we will further discuss the remaining assumptions.

As baseline, our evaluation is based on the existing OFDM waveform and PRS reference signal. The PRS resource configuration can be provided via the existing higher layer parameters [3] NR-DL-PRS-PositioningFrequencyLayer, NR-DL-PRS-ResourceSet and NR-DL-PRS-Resource.

A DL PRS positioning frequency layer consists of one or more DL PRS resource sets, which have the same carrier frequency and numerology. A DL PRS resource set consists of one or more DL PRS resources. Specifically, the PRS in frequency domain can be configured with different comb sizes {2, 4, 6, 12} and in the time domain, its symbol number is configured by dl-PRS-NumSymbols with candidate values {1, 2, 4, 6, 12}. The periodicity and repetition of each PRS resource also provide high flexibility in the time domain. For instance, there is an example of PRS configuration in Table 1, and the specific time and frequency patterns are shown in Figure 2.

Table 1 Example of PRS configuration via higher layer parameters.

|  | Higher layer configuration IE |  | Resource-0 |
| --- | --- | --- | --- |
| Frequency domain | NR-DL-PRS-PositioningFrequencyLayer | dl-PRS-SubcarrierSpacing-r16 | 30kHz |
|  |  | dl-PRS-ResourceBandwidth-r16 | 63 |
|  |  | … |  |
|  | NR-DL-PRS-Resource-r16 | dl-PRS-CombSizeN | comb-2 |
|  |  | … |  |
| Time domain | NR-DL-PRS-ResourceSet-r16 | dl-PRS-NumSymbols-r16 | 1 symbol |
|  |  | dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 | 5 slots |
|  |  | … |  |
|  | NR-DL-PRS-Resource-r16 | dl-PRS-ResourceSlotOffset-r16 | 0 |
|  |  | dl-PRS-ResourceSymbolOffset-r16 | 2 |
|  |  | … |  |

Figure 2 Example of PRS-based sensing signal pattern.

Observation 1: The sensing signal for gNB-based mono-static sensing can be realized by legacy signals, especially for PRS, which offers high flexibility.

For antenna configuration, Table 2 provides details on the transmitted and received antenna profile in 4.9GHz and 6GHz, where 128TxRU and 256TxRU for total antenna array are assumed, respectively.

Table 2 Antenna configurations for FR1.

| Parameters | Value |
| --- | --- |
| BS antenna configurations | For 4.9GHz: - Tx: (M, N, P, Mg, Ng) = (12,16,2,1,1) with one port transmission virtualized by the Tx array - Rx: (M, N, P, Mg, Ng; Mp, Np) = (12,16,2,1,1;2,16) - (dH, dV) = (0.5, 0.8)λ |
|  | For 6GHz: - Tx: (M, N, P, Mg, Ng) = (16,16,2,1,1) with one port transmission virtualized by the Tx array - Rx: (M, N, P, Mg, Ng; Mp, Np) = (16,16,2,1,1;4,16) - (dH, dV) = (0.5, 0.8)λ |
| BS antenna downtilting | Mechanical downtilt of 90 degrees |
| Antenna pattern | Table 9 in Report ITU-R M.2412 |
| Polarized antenna model | Model-2 in clause 7.3.2 in TR 38.901 |

Proposal 2: Using the antenna configurations in the following table for simulation.

| Parameters | Value |
| --- | --- |
| BS antenna configurations | For 4.9GHz: - Tx: (M, N, P, Mg, Ng) = (12,16,2,1,1) with one port transmission virtualized by the Tx array - Rx: (M, N, P, Mg, Ng; Mp, Np) = (12,16,2,1,1;2,16) - (dH, dV) = (0.5, 0.8)λ |
|  | For 6GHz: - Tx: (M, N, P, Mg, Ng) = (16,16,2,1,1) with one port transmission virtualized by the Tx array - Rx: (M, N, P, Mg, Ng; Mp, Np) = (16,16,2,1,1;4,16) - (dH, dV) = (0.5, 0.8)λ |
| BS antenna downtilting | Mechanical downtilt of 90 degrees |
| Antenna pattern | Table 9 in Report ITU-R M.2412 |
| Polarized antenna model | Model-2 in clause 7.3.2 in TR 38.901 |

For the BS Tx power, it can be modelled as the sum of Tx-Rx isolation and Rx saturation power. Two levels of isolation, i.e., 65 and 80 dBc are considered under the circumstance of -28dBm saturation power. Therefore, 37dBm and 52dBm are two options for Tx power.

Regarding the interference model, a general principle is that communication interference is excluded by TDM between communication and sensing on the same frequency, while two types of sensing interference sources are considered: inter-site and intra-site interference.

For inter-site interference, we think that multi-domain division multiplexing methods (such as TDM, SDM, FDM, and CDM) are effective in mitigating these effects.

For intra-site interference, including the leakage from the Tx to Rx antenna in the same panel and the interference within the panels of different sectors, we recommend not modeling them in the evaluation. The reasons are listed as follows:

- The impact of intra-panel interference occurs within close proximity and can be mitigated by ignoring detected peaks with small delays. Additionally, UAVs are typically deployed at a distance of over 10 meters, which aligns well with this approach.

- The characteristics of leakage differ significantly from those defined in TR 38.901 [5], where electromagnetic propagation is highly complex. Moreover, few rigorous and mature modeling methods are available for reference.

In summary, interference does not need to be modeled for gNB-based monostatic sensing evaluations.

Proposal 3: Interference from sensing and communication is not modelled and Tx power takes 37dBm and 52dBm as two options in the simulation for gNB-based mono-static sensing.

Furthermore, in the simulation, certain specific parameters or algorithms may significantly influence the outcomes of sensing. For example, increased utilization of sensing resources generally leads to improved performance, while the limitation of the association algorithm or the adoption of specialized processing techniques such as MUSIC/ESPIRIT can also affect the results. To ensure the credibility and reproducibility of findings, each contributing company should report detailed parameter configurations, enabling the sharing of convincing and verifiable results.

Proposal 4: Companies should report sensing resources and details of any specialized algorithms.

Initial evaluation results for UAV

In this section, we present results for two frequency bands, namely 4.9 GHz and 6 GHz, including horizontal and vertical position accuracy, radial velocity accuracy, missed detection probability, and false alarm probability (Type1 and Type2).

In our simulation, the sensing resources configuration is detailed in Table 3 with an associated overhead of approximately 1.4%. Within the receiver processing chain, the OS-CFAR algorithm is employed for candidate point detection, while the FFT algorithm is used for angle estimation.

Table 3 Configuration of sensing resource

| Parameters | Assumptions |
| --- | --- |
| Coherent processing interval | 160ms |
| Time resource allocation | 1 symbol per 5 slots |
| Frequency resource allocation | Comb-2 |

Furthermore, to compute metrics such as sensing accuracy, missed detection probability, and false alarm probability, it is essential to associate detected objects with their corresponding ground truth targets. In our simulation, this association is established based on a geometric distance criterion, where a detected object is matched with a ground truth target if the distance between their locations is below a predefined threshold, which is defined as follows:

Locobject-Loctarget≤ϵ

Where x is the Euclidean distance of x, and ϵ are set to 10m.

Evaluation results for 4.9GHz

| - CDF of horizontal localization error. | - CDF of vertical localization error. |
| --- | --- |
| - CDF of radial velocity error. |  |

Figure 3 Simulation results for 4.9GHz with Tx=37dBm.

| - CDF of horizontal localization error. | - CDF of vertical localization error. |
| --- | --- |
| - CDF of radial velocity error. |  |

Figure 4 Simulation results for 4.9GHz with Tx=52dBm.

The simulation results are also summarized in Table 4.

Table 4 Simulation results for 4.9GHz.

| Fc = 4.9GHz |  | Tx Power = 37dBm | Tx Power = 52dBm |
| --- | --- | --- | --- |
| Horizontal localization error [m] | @90% | 0.87 | 0.26 |
|  | @95% | 1.44 | 0.42 |
| Vertical localization error [m] | @90% | 0.64 | 0.13 |
|  | @95% | 1.03 | 0.2 |
| Radial velocity error [m/s] | @90% | 0.039 | 0.018 |
|  | @95% | 0.058 | 0.031 |
| Missed detection probability [%] |  | 2.6 | 2.4 |
| False alarm probability Type1 [%] |  | 0.016 | 0.03 |
| False alarm probability Type2 [%] |  | 0.4 | 0.5 |

Evaluation results for 6GHz

| - CDF of horizontal localization error. | - CDF of vertical localization error. |
| --- | --- |
| - CDF of radial velocity error. |  |

Figure 5 Simulation results for 6GHz with Tx=37dBm.

| - CDF of horizontal localization error. | - CDF of vertical localization error. |
| --- | --- |
| - CDF of radial velocity error. |  |

Figure 6 Simulation results for 6GHz with Tx=52dBm.

Table 5 Simulation results for 6GHz.

| Fc = 6GHz |  | Tx Power = 37dBm | Tx Power = 52dBm |
| --- | --- | --- | --- |
| Horizontal localization error [m] | @90% | 0.84 | 0.31 |
|  | @95% | 1.44 | 0.56 |
| Vertical localization error [m] | @90% | 0.51 | 0.12 |
|  | @95% | 0.8 | 0.2 |
| Radial velocity error [m/s] | @90% | 0.031 | 0.023 |
|  | @95% | 0.047 | 0.047 |
| Missed detection probability [%] |  | 2.5 | 0.8 |
| False alarm probability Type1 [%] |  | 0.02 | 0.03 |
| False alarm probability Type2 [%] |  | 0.4 | 0.6 |

Simulation observation

SA1 has currently proposed different service requirements for the scenario of drone detection and tracking [4]. In our view, those requirements are for end-to-end performance, which includes the trajectory processing. We suggest to analyse the end-to-end performance with trajectory processing based on the result from each individual one-shot evaluation. We show a simplified analysis for the evaluation results before trajectory processing.

Trajectory-level localization and velocity accuracy

For localization and velocity accuracy, the non-coherent gain can be easily obtained through the utilization of historical trajectory information. Thus, the requirements of one-shot evaluation for localization and velocity accuracy should be loose or the same even in the worst case that just takes the one-shot evaluation results as the results after trajectory processing.

Trajectory-level missed detection probability and false alarm probability

For missed detection probability and false alarm probability, the trajectory processing impact highly relies on the algorithms for filtering with various parameters. For example, it is necessary to consider how long the trajectory filtering should be performed, as well as the criteria for trajectory interruption and trajectory generation.

To simplify the analysis of this impact, we assume that every one-shot evaluation is independent, and a trajectory is claimed to be detected when at least one positive detection happens during N CPIs using the “or-operation”. With this assumption, the missed detected probability and false alarm probability can be derived as:

Pmisstrack=Pmissone shotN

Pfatrack=1-1-Pfaone shotN

It is worth noting that it naturally increases the detection probability of the trajectory, and in the same time it also increases the false alarm probability of the trajectory since one false alarm during N CPIs results in the false alarm of the trajectory.

Based on the comprehensive results, the missed detection probability and false alarm probability with trajectory processing can be calculated as followed.

Table 6 Tracking/trajectory-level results derived from one-shot evaluation with N=3.

| Frequency | 4.9GHz |  | 6GHz |  |
| --- | --- | --- | --- | --- |
| Tx Power | 37dBm | 52dBm | 37dBm | 52dBm |
| Missed detection probability [%] | 1.8E-3 | 1.4E-3 | 1.6E-3 | 1E-4 |
| False alarm probability Type1 [%] | 0.047 | 0.089 | 0.059 | 0.089 |
| False alarm probability Type2 [%] | 1.2 | 1.49 | 1.2 | 1.79 |

It can be seen that the sensing performance of a single base station can meet the requirements of category 1 specified in SA1 as below.

Table 7 Sensing requirements for object detection and tracking specified in TS22.137.

| Scenario | Sensing service category | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] |  |  |
| Object detection and tracking | 1 | 95 | 10 | 10 | N/A | N/A | 5 | 2 |
|  | 2 | 95 | 2 | 5 | 1 | N/A | 0.1 to 5 | 5 |
|  | 3 | 95 | 1 | 1 | 1 | 1 | 2 | 2 |
|  | 4 | 99 for public safety, otherwise, 95 | 0.5 | 0.5 | 1.5 for pedestrian, 15 for vehicle, otherwise, 0.1 | 1.5 for pedestrian | 1 | 3 |

Observation 2: The gNB-based mono-static sensing results of UAV could satisfy the sensing service category 1&2 of object detection and tracking.

Measurement and measurement quantization

FLS in [2] listed some potential measurement for NR ISAC, as shown below:

Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing - The following measurements can be further studied for NR ISAC. - Spectrum of delay, and/or Doppler, and/or angle, i.e., the powers or the amplitudes/phases are reported for at least a range of samples of delay, and/or Doppler, and/or angle - Intermediate sensing results per detected path - Delay, Doppler, angle, and power per path - Intermediate sensing results per detected point - Option 1: Position, velocity, and power per point - Option 2: Range, velocity, angle, and power per point - Final sensing results per detected object/target - Option 1: Position and velocity - Option 2: Range, velocity, and angle - Note: Trajectory report is separate discussion - SINR, RSRP from perspective of sensing

As shown in Figure 7, the sensing receiver can achieve various stages of measurement results (including spectrum, intermediate results, and final results) through different processing steps applied to the sensing echoes. These results can be analyzed from four perspectives: overhead, individual-link sensing accuracy, cooperative-link sensing accuracy, and receiver complexity, as detailed in Table 8.

Figure 7 Process flow diagram of sensing processing chain.

Table 8 Comparison of different sensing measurements.

|  | Spectrum | Intermediate results | Final results |
| --- | --- | --- | --- |
| Feedback overhead | Very high | Low | Low |
| Sensing accuracy for individual link based on the feedback | High | Middle | Low |
| Sensing accuracy for cooperative link based on the feedback | High | Middle | Low |
| Receiver complexity | Low | Middle | High |

First of all, the data sizes are calculated based on the general assumptions. For spectrum, we have following configurations as:

- Number of sub-carriers used for sensing: NF=1638 (comb-2)

- Number of range bins used for sensing: NR=288 or 2048

- Number of symbols used for sensing: NT=64

- Number of receiving antenna ports: NS=64

- Number of TRPs: Ntrp=3

- Value quantization for each bin: Float (32 bits) format for each real and imaginary of complex value

With regards to the number of range bins NR, to the best of our knowledge, the sensing coverage is limited, which necessitates flexibility in adjusting the detected range. In this context, the range bin number can be set as low as

- NR=288 that corresponds to the CP length (~350m)

- NR=2048 that corresponds to maximum ambiguity range resulted from comb-2 structure (~2500m)

Then, the data size of spectrum is about 0.2-1.6Gbits subject to different values on the number of range bins (NR=288 or 2048), with the calculation as:

NR×NT×NS×Ntrp×32×2≈0.2-1.6Gbits

As for intermediate results, it includes those paths or points being detected from initial range/angle/doppler spectrum and most of them locate in the sparse domain of time, spatial (azimuth and zenith direction), and Doppler. Here are the configurations and assumptions:

- Number of points or paths is Npoints=160

- Number of parameters per path or point: Npara=4

- Number of TRPs: Ntrp=3

- Value quantization for each parameter: Float format

then the data size of intermediate results is about 60Kbits, with the calculation as:

Npoints×Npara×Ntrp×32=60Kbits

Regarding the final results, the number of targets is almost the same with the intermediate points due to the sparse distribution of UAV and single point target model of small size UAV. Then, the data size of final results is around 60Kbits, calculated by:

Nobject×Npara×Ntrp×32=60Kbits

In terms of sensing accuracy for an individual link and cooperative links, it follows a general principle that useful information from the sensing target is inevitably lost after each processing algorithm. Therefore, feedback of the spectrum achieves the highest target accuracy at the cost of extremely high overhead. Conversely, the final results employ the most complex algorithms among the three measurement types.

In our view, intermediate results—such as parameters for per path or per point—provide an optimal trade-off, delivering acceptable sensing accuracy while maintaining reasonable overhead. Furthermore, measurements per detected point are preferable, as they enable the pre-compensation of non-ideal factors in the receiver through specific models or algorithms, up to the implementation of the company.

Proposal 5: Intermediate sensing results per detected point should be studied in the sensing measurements.

We also consider the content of intermediate results, which shall include both basic parameters related to location and velocity, and a measurement quality indicator for each detected point. The following basic parameter option should be supported, which are computationally efficient and simplified within the relative coordinate system:

  - Intermediate sensing results per detected point

    - Range, velocity, angle

For the quality indicator, it is preferable to adopt the uncertainty defined in TS 23.032, TS 38.355, and TS 24.080 as already used in existing location description.

| Existing specification | ## Description |
| --- | --- |
| TS 23.032 | ## 5.10 Range and Direction The "range and direction" from a point A to a point B is characterised by three components comprising a range from point A to point B, an azimuth direction from point A to point B and an elevation direction from point A to point B as shown in Figure 3d. The range provides a straight-line distance from point A to point B. The azimuth provides a direction to point B from point A in a horizontal plane through point A and as measured clockwise from North. The elevation provides a direction to point B from point A in a vertical plane through the points A and B and as measured upwards or downwards from a horizontal plane through point A. The range, azimuth and elevation can be each independently included or excluded in a "range and direction" and each has an uncertainty and a confidence. Figure 3d: Description of a Range and Direction ## 8.4a Relative Velocity with Uncertainty The relative velocity with uncertainty of a device B relative to a device A is characterised by a radial velocity component (measured in meters / second) and a perpendicular transverse velocity component (measured in degrees / second). The radial velocity component is characterized by a rate of change of a range between the device A and device B. The transverse velocity component is characterized by a rate of change of a direction to the device B from the device A, where the change rate of the direction includes the change rate of an angle of azimuth (measured in degrees / second) and change rate of an angle of elevation (measured in degrees / second). The angle of azimuth is measured clockwise from North in a horizontal plane through the device A and the angle of elevation measured upwards or downwards in a vertical plane through the devices A and B from a horizontal plane through the device A. The rates of change of the range and the angles of azimuth and elevation can be each independently included or excluded in the relative velocity and each has an uncertainty and a confidence, Figure 12a: Description of a Relative Velocity with Uncertainty |
| TS 38.355 | RangeAndOrDirection ::= SEQUENCE { range Range OPTIONAL, azimuth Azimuth OPTIONAL, elevation Elevation OPTIONAL } Range ::= SEQUENCE { rangeResult INTEGER (0..134217727), uncertainty INTEGER (0..255), confidence INTEGER (0..100) OPTIONAL } Azimuth ::= SEQUENCE { azimuthResult INTEGER (0..3599), uncertainty INTEGER (0..127), confidence INTEGER (0..100) OPTIONAL } Elevation ::= SEQUENCE { elevationResult INTEGER (0..1800), uncertainty INTEGER (0..63), confidence INTEGER (0..100) OPTIONAL } RelativeVelocityWithUncertainty ::= SEQUENCE { radialVelocityComponent SEQUENCE { unitsRadialVelocity ENUMERATED { mPerS, cmPerS, ... }, radialVelocity INTEGER (-2048..2047), uncertaintyRadialVelocity INTEGER (0..255), confidenceUncertaintyRadialVelocity INTEGER (0..100) } OPTIONAL, transverseVelocityComponent SEQUENCE { unitsTransverseVelocity ENUMERATED { degPerSec1, degPerSec0-1, ... }, azimuth SEQUENCE { azimuthRateOfChange INTEGER (0..1023), uncertaintyAzimuthRateOfChange INTEGER (0..255), confidenceUncertaintyAzimuthRateOfChange INTEGER (0..100) } OPTIONAL, elevation SEQUENCE { elevationRateOfChange INTEGER (0..1023), uncertaintyElevationRateOfChange INTEGER (0..255), confidenceUncertaintyElevationRateOfChange INTEGER (0..100) } OPTIONAL } OPTIONAL, ..., [[ azimuthRateOfChangeDirection ENUMERATED {anticlockwise} OPTIONAL, elevationRateOfChangeDirection ENUMERATED {downward} OPTIONAL ]] } |
| TS 24.080 | RangeDirection ::= SEQUENCE { range [0] Range OPTIONAL, azimuth [1] Azimuth OPTIONAL, elevation [2] Elevation OPTIONAL, ...} Range ::= SEQUENCE { rangeResult [0] RangeResult, uncertainty [1] Uncertainty, confidence [2] Confidence OPTIONAL, ...} Azimuth::= SEQUENCE { azimuthResult [0] AzimuthResult, uncertainty [1] Uncertainty, confidence [2] Confidence OPTIONAL, ...} Elevation ::= SEQUENCE { elevationResult [0] ElevationResult, uncertainty [1] Uncertainty, confidence [2] Confidence OPTIONAL, ...} RelVelocityEstimate ::= SEQUENCE { rVelocity [0] RadialVelocity OPTIONAL, rUncertainty [1] Uncertainty OPTIONAL, aTransverseVelocity [2] AngularVelocity OPTIONAL, aVelocityUncertainty [3] AngularVelocityUncertainty OPTIONAL, eTransverseVelocity [4] AngularVelocity OPTIONAL, eVelocityUncertainty [5] AngularVelocityUncertainty OPTIONAL, unitsRVelocity [6] UnitsLinearVelocity OPTIONAL, unitsAVelocity [7] UnitsAngularVelocity OPTIONAL, unitsEVelocity [8] UnitsAngularVelocity OPTIONAL, ...} |

Other measurements, such as target ID, are unjustified as there is no evidence showing that they will significantly improve overall sensing performance in the processing chain.

Proposal 6:  Intermediate sensing results includes range, velocity, angle and uncertainty for each detected point.

As for measurement quantization, TS 23.032 [6] has already defined various quantization shapes and coding for measurements. These established formats can serve as a baseline for quantizing the location and velocity. Typically, the following formats in TS 23.032 can be treated as starting point:

- 5.10	Range and Direction

- 8.4a	Relative Velocity with Uncertainty

Proposal 7: TS 23.032 shall be used as a starting point for measurement quantization.

Conclusions

The observations and proposals made in this contribution are summarized below:

Observation 1: The sensing signal for gNB-based mono-static sensing can be realized by legacy signals, especially for PRS, which offers high flexibility.

Observation 2: The gNB-based mono-static sensing results of UAV could satisfy the sensing service category 1&2 of object detection and tracking.

Proposal 1: Option 2 that K is number of drops with at least one detected object is selected for the false alarm probability Type 2.

Proposal 2: Using the antenna configurations in the following table for simulation.

| Parameters | Value |
| --- | --- |
| BS antenna configurations | For 4.9GHz: - Tx: (M, N, P, Mg, Ng) = (12,16,2,1,1) with one port transmission virtualized by the Tx array - Rx: (M, N, P, Mg, Ng; Mp, Np) = (12,16,2,1,1;2,16) - (dH, dV) = (0.5, 0.8)λ |
|  | For 6GHz: - Tx: (M, N, P, Mg, Ng) = (16,16,2,1,1) with one port transmission virtualized by the Tx array - Rx: (M, N, P, Mg, Ng; Mp, Np) = (16,16,2,1,1;4,16) - (dH, dV) = (0.5, 0.8)λ |
| BS antenna downtilting | Mechanical downtilt of 90 degrees |
| Antenna pattern | Table 9 in Report ITU-R M.2412 |
| Polarized antenna model | Model-2 in clause 7.3.2 in TR 38.901 |

Proposal 3: Interference from sensing and communication is not modelled and Tx power takes 37dBm and 52dBm as two options in the simulation for gNB-based mono-static sensing.

Proposal 4: Companies should report sensing resources and details of any specialized algorithms.

Proposal 5: Intermediate sensing results per detected point should be studied in the sensing measurements.

Proposal 6:  Intermediate sensing results includes range, velocity, angle and uncertainty for each detected point.

Proposal 7: TS 23.032 shall be used as a starting point for measurement quantization.

- References

R1-251861, “New SID: Study on Integrated Sensing And Communication (ISAC) for NR” 3GPP TSG RAN#108, June 9th – June 13th, 2025.

R1-2506480, “Summary #5 on evaluations for NR ISAC” 3GPP TSG RAN WG1 #122, Aug 25th-29th, 2025.

3GPP TS37.355, “LTE Positioning Protocol (LPP)”, June, 2025.

3GPP TS22.137, “Service requirements for Integrated Sensing and Communication”, March, 2024.

3GPP TR38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”, June, 2025.

3GPP TS23.032, “Universal Geographical Area Description (GAD)”, Dec. 2024.