3GPP TSG RAN WG1 #122-bis			R1-2506952

Prague, CZ, October 13-17, 2025

## Agenda item: 11.3.1

Source: Kyocera

Title: 6GR DL Waveform

Document for:	Discussion

# Introduction

According to the 3GPP 6G SID in [1], the following objectives related to 6G Waveform are given in Section 4.1:

(2)	Physical Layer structure for 6GR,

a)	Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4]

b)	Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1]

In RAN1#122 the following agreements were made:

Agreement

CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink

- Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions

- Other OFDM based waveforms are not precluded.

Agreement

CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink

- Enhancements/modifications on CP-OFDM will be studied as potential additions

- DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink

Note: proponents identify at least the target use cases, signals/channels to use the waveform, and how the proposal is intended (if applicable) to support multiplexing with CP-OFDM, including MRSS, and how multi-user multiplexing is supported, etc.

In addition to the agreements the proponents for the new waveforms were encouraged to provide more detailed information at this meeting. Some companies have suggested studying DFT-s-OFDM for DL at least in NTN and FR2. In this contribution, we will discuss the advantages and the disadvantages of DFT-S-OFDM for 6GR DL.

# Benefits of introducing DFT-S-OFDM for downlink

Introducing the DFT-S-OFDM waveform for 6GR DL offers potential benefits in coverage, and energy efficiency, especially for scenarios prioritizing low PAPR and power amplifier (PA) efficiency compared to the CP-OFDM waveform.

Coverage and Energy Efficiency

DFT-S-OFDM exhibits a lower PAPR, which allows transmitters to operate PAs more efficiently and at higher output powers without distortion. This can significantly enhance downlink coverage— some studies [3] indicate up to 20% improvement in coverage in single-user, rank-1 transmission scenarios, especially when using low modulation orders. Additionally, due to the low PAPR characteristics of DFT-S-OFDM, further energy consumption savings per transferred bit are possible while maintaining signal integrity.

Deploying DFT-S-OFDM aligns with rising industry interest in network sustainability, as reducing energy consumption directly lowers operational costs and the environmental footprint. Since energy efficiency traditionally received less focus in earlier wireless generations, the integration of DFT-S-OFDM could help address efficiency targets without sacrificing essential performance metrics like throughput and reliability.

Supporting Scenarios

DFT-S-OFDM is especially advantageous in power-limited or coverage-critical environments, such as rural macro cells, edge areas, NTN, or IoT deployments requiring broad reach and lower device power consumption. The waveform’s known robustness to RF impairments also ensures reliable communication under challenging channel conditions.

In general, while CP-OFDM remains preferred for extremely high-throughput, dense urban scenarios due to its superior spectral efficiency, DFT-S-OFDM could complement CP-OFDM by offering optional downlink support where coverage and energy savings are prioritized.

# Drawbacks of introducing DFT-S-OFDM for downlink

DFT-S-OFDM generally exhibits multiple disadvantages compared to CP-OFDM. So far in LTE and in 5G NR, DFT-S-OFDM has been optimized for uplink use, particularly for power-limited devices, but its application on the downlink introduces several technical and performance drawbacks.

Power Constraints Are Irrelevant

DFT-S-OFDM's primary uplink advantage is low PAPR, which protects UE from PA distortion. Downlink transmission, from powerful base stations, doesn't suffer from these constraints, so this benefit is redundant. Additionally, the NTN work [2] showed that satellite connectivity does not require a dedicated satellite waveform since 5G NR based on CP-OFDM (for DL) and DFT-s-OFDM (for UL) can be sufficient for satellite communications.

Spectral Efficiency Loss

In DFT-S-OFDM, data symbols are mapped onto groups of subcarriers defined by the DFT size, which is smaller than the FFT size. When the DFT-S-OFDM signal occupies only a portion of the available bandwidth, the subcarriers outside this allocation within the FFT remain unused, thereby resulting in suboptimal spectral efficiency.

Since downlink transmission isn't limited by mobile PAs, maximizing spectral efficiency is prioritized, making this inefficiency less acceptable.

Reduced Flexibility and Scheduling

Downlink transmissions require high flexibility for multi-user and resource allocation. DFT-S-OFDM, being closer to a single-carrier system, is less flexible and more complex to implement for dynamic resource management especially for MRSS deployments.

MIMO Complexity

5G NR downlink frequently employs MIMO techniques in conjunction with high-order modulation schemes. While CP-OFDM provides full flexibility by enabling data streams to be mapped at the subcarrier level, DFT-S-OFDM is inherently constrained by its design. Originally tailored for single-stream transmission, DFT-S-OFDM offers only limited support for multi-layer (rank > 1) MIMO configurations. The subcarrier aggregation effect introduced by DFT precoding diminishes the mapping granularity available in CP-OFDM, thereby restricting the extent to which DFT-S-OFDM can realize the spatial multiplexing gains of MIMO.

# Conclusions

We have briefly looked at the advantages and disadvantages of introducing DFT-S-OFDM in 6GR DL. In our view, the disadvantages of spectral efficiency loss, reduced flexibility in scheduling and MIMO are the most critical aspects for downlink that outweigh the advantages.

Observation 1: DFT-S-OFDM does not offer a substantial advantage as a candidate waveform for 6GR downlink.

# References

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025.

[2] 3GPP, Solutions for NR to support non-terrestrial networks, TR 38.821, v16.2.0, April 2023.

[3] Anttila, L., et al, Feasibility of DFT-S-OFDM in 6G Downlink: Coverage and Energy-Efficiency Assessment, 2023