3GPP TSG RAN WG1 #122bis		R1-2507141

Prague, Czech Republic, October 13th – 17th, 2025

Source:	OPPO

Title:	Discussion on LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC

Agenda Item:	5

Document for:	Discussion and Decision

# Introduction

In SA2#130 meeting, SA2 has discussed the potential issues when using RoHC to support IMS voice over NB-IoT NTN connected to EPC, and an LS was sent to other WGs to evaluate the viability of using RoHC [1]. Specially, SA2 asked the following question to RAN1 regarding the probability that 16 or 64 consecutive packets are lost or erroneously decompressed.

1. Overall Description: Under the study item FS_5GSAT_Ph4_ARC SA2 is studying the support for IMS voice over NB-IoT NTN connected to EPC. In the context of this study item SA2 is investigating solutions in TR 23.700-019 that could potentially use IP or non-IP type (i.e. with and without IP/UDP header) for the (voice) traffic and would like to ask some questions in order to help SA2 to evaluate the alternative solutions. When IP PDN connection is used, in order to achieve the target KPIs for support of IMS voice over NB-IoT NTN connected to EPC, it is expected that the support for Robust Header Compression (RoHC) will likely be required. RoHC is supported between UE and eNB when "user plane" DRBs are used to transport the data or between the UE and MME when "control plane CIoT" and SRBs are used to transport the data. In order for SA2 to be able to evaluate the viability of using RoHC for support of IMS voice over NB-IoT NTN connected to EPC, SA2 has the following questions: During the voice call, if enough consecutive packets are lost or erroneously decompressed, the compressor can find itself leave Second Order (SO) state and enter First Order (FO) state, and then if another enough consecutive packets are lost or erroneously decompressed, the compressor can find itself leave First Order state (FO) and enter the Initialization and Refresh (IR) state, causing gaps in the transmission of the voice packets.Some companies indicated in SA2 that the number of consecutive lost packets that will trigger the compressor to fall back from SO state to FO state is around 16 (e.g., when UO-0 header with 4 bits SN is used in SO state), and the number of consecutive lost packets that will trigger the compressor to fall back from FO state to IR state is around 64 (e.g., when UOR-2 header with 6 bits SN is used in FO state). Question 1 (To RAN2): Does RAN2 have any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC? Question 2 (To RAN1): Can RAN1 provide any data regarding the probability that such number of consecutive packets e.g. 16 or 64 can be lost or erroneously decompressed? How often such event can occur? 2. Actions: To RAN2, SA4, SA3, CT1, SA1, RAN1: ACTION: SA2 requests the respective WGs in “TO” to answer the questions addressed to them as per above and provide any other feedback they think is necessary. 3. Date of Next TSG SA WG2 Meetings: TSG-SA2 Meeting #171 13-17 October 2025 Wuhan, PRC TSG-SA2 Meeting #172 17-21 November 2025 Dallas, USA

In this contribution, we provide our simulation results and analysis on the probability and frequency of the consecutively lost packets, and the draft reply LS is provided in our companion contribution [2].

# Discussion

When evaluating the viability of using RoHC for support of IMS voice over NB-IoT NTN connected to EPC, SA2 identifies that if enough consecutive packets are lost or erroneously decompressed, the compressor can fall back from Second Order (SO) state to First Order (FO) state or fall back from FO state to Initialization and Refresh (IR) state, where the state transition causes gaps in the transmission of the voice packets. Specifically, SA provides two reference value for evaluation, including 16 consecutive lost packets will trigger the compressor to fall back from SO state to FO state and 64 consecutive lost packets will trigger the compressor to fall back from FO state to IR state. Thus, RAN1 can take 16 and 64 consecutively lost packets as the evaluation metric. Nevertheless, decompressing touches on higher layer operation and RAN1 cannot provides the probability or frequency that consecutive packets are erroneously decompressed.

Proposal 1: Decompressing touches on higher layer operation and RAN1 cannot provides the probability or frequency that consecutive packets are erroneously decompressed.

Regarding the evaluation of consecutively lost packets, the typical simulation parameters provided in the LS from SA4 are used in our simulation [3]. For UL, since the UE Tx power is limited, single-tone NPUSCH with 3.75kHz SCS is considered to achieve the UL SNR = 2.6dB. for DL, 12-tones NPDSCH with 15kHz SCS is considered to reduce the transmission time, and DL SNR = -3.3dB.

Table 1 and Table 2 provide the probability and frequency of 16 and 64 consecutively lost packets for UL and DL respectively, where 160000 packets are generated/observed in our link-level simulation. For UL SNR = 2.6dB, it can be observed that the probability of 16 consecutively lost UL packets is below 0.1% and the event occurs with the average time interval of 1055s. In addition, 64 consecutively lost UL packets are not observed in our evaluation results. For DL SNR = -3.3dB, it can be observed that the probability of 16 consecutively DL lost packets is around 0.1% and the event occurs with the average time interval of 81s. For 64 consecutively DL lost packets, it occurs with a probability below 0.1% and the average time interval of 1370s. Based on the LLS simulation results, it can be concluded that the probability that 16 and 64 consecutive packets are lost is very low, whether for DL or UL transmission. Therefore, the impact of consecutively lost packets on using RoHC for support of IMS voice over NB-IoT NTN can be negligible.

Table 1. Statistic of 16 and 64 consecutively lost UL packets w.r.t. various SNR.

|  | 16 consecutively lost packets |  | 64 consecutively lost packets |  |
| --- | --- | --- | --- | --- |
| SNR (dB) | Probability | Time interval (s) | Probability | Frequency |
| -1 dB | 0.1% (154/160000) | 82s | 0.019% (3/160000) | 2043s |
| 0 dB | 0.02% (30/160000) | 387s | 0 | NA |
| 1 dB | 0.004% (7/160000) | 976s | 0 | NA |
| 2 dB | 0.003% (5/160000) | 1055s | 0 | NA |
| 3 dB | 0 | NA | 0 | NA |
| 4 dB | 0 | NA | 0 | NA |

Table 2. Statistic of 16 and 64 consecutively lost DL packets w.r.t. various SNR.

|  | 16 consecutively lost packets |  | 64 consecutively lost packets |  |
| --- | --- | --- | --- | --- |
| SNR (dB) | Probability | Time interval (s) | Probability | Frequency |
| -5 dB | 0.68% (1088/160000) | 12s | 0.06% (96/160000) | 126s |
| -4 dB | 0.29% (457/160000) | 28s | 0.01% (15/160000) | 804s |
| -3 dB | 0.1% (157/160000) | 81s | 0.003% (6/160000) | 1370s |
| -2 dB | 0.03% (53/160000) | 236s | 0 | NA |
| -1 dB | 0.02% (25/160000) | 437s | 0 | NA |
| 0 dB | 0.01% (13/160000) | 805s | 0 | NA |

Observation 1: For UL SNR = 2 dB, during the observation time of 160000 packets:

- The probability of 16 consecutively lost UL packets is below 0.1%, and the event occurs on average every 1055s.

- 64 consecutively lost UL packets are not observed.

Observation 2: For DL SNR = -3 dB, during the observation time of 160000 packets:

- The probability of 16 consecutively lost DL packets is around 0.1%, and the event occurs on average every 81s.

- The probability of 64 consecutively lost DL packets is below 0.1%, and the event occurs on average every 1379s.

Proposal 2: The probability that 16 and 64 consecutive packets are lost is very low, and the impact of consecutively lost packets on using RoHC for support of IMS voice over NB-IoT NTN can be negligible.

# Conclusion

In this contribution, we discussed the SA2 LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC. The following observations and proposals are made:

Observation 1: For UL SNR = 2 dB, during the observation time of 160000 packets:

- The probability of 16 consecutively lost UL packets is below 0.1%, and the event occurs on average every 1055s.

- 64 consecutively lost UL packets are not observed.

Observation 2: For DL SNR = -3 dB, during the observation time of 160000 packets:

- The probability of 16 consecutively lost DL packets is around 0.1%, and the event occurs on average every 81s.

- The probability of 64 consecutively lost DL packets is below 0.1%, and the event occurs on average every 1379s.

Proposal 1: Decompressing touches on higher layer operation and RAN1 cannot provides the probability or frequency that consecutive packets are erroneously decompressed.

Proposal 2: The probability that 16 and 64 consecutive packets are lost is very low, and the impact of consecutively lost packets on using RoHC for support of IMS voice over NB-IoT NTN can be negligible.

# Reference

[1]       S2-2507636, LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC, SA2.

[2]      R1-2507142, Draft reply LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC, OPPO.

[3]       S4-251584, LS on the RAN simulation assumptions for ULBC, SA4.

# Appendix

## Simulation assumptions

Table 1. LLS parameters for NPUSCH format 1.

| orbit | GEO |  |
| --- | --- | --- |
| Elevation angle | 12.5 degree |  |
| channel | NPUSCH | NPDSCH |
| carrier frequency | 2GHz |  |
| Channel model | NTN-TDL-C |  |
| Frequency error | Uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs Variation of frequency error is negligible. |  |
| Timing error | Uniform random selection from [-29Ts, +29Ts] for all UEs Timing drift 0 for GEO. |  |
| SCS | 3.75KHz | 15kHz |
| Number of tones | Single tone | 12 tones |
| Waveform | DFT-s-OFDM | CP-OFDM |
| Frequency hopping | w/o frequency hopping |  |
| MIMO scheme | SISO |  |
| Number of resource unit (NRU) / subframes (NSF) | NRU=1 | NSF=1 |
| Modulation order (Qm) | QPSK | QPSK |
| TBS (ITBS) | ITBS = 8 (120bits) | ITBS = 8 (120bits) |
| Number of repetitions (Nrep) | 2 | 4 |
| Packet time interval | Ptime = 80ms |  |
| Velocity of UE | 3km/h |  |
| Receiver algorithm | MMSE |  |
| Channel estimation | Real channel estimation |  |