3GPP TSG RAN WG1 #122bis   	                              R1-2507396

Prague, Czech, Oct 13th – 17th, 2025

Agenda item:		10.1.1.1

Source:		LG Electronics

Title:			Discussion on inference related aspects for CSI compression

Document for:	Discussion and Decision

Introduction

In RAN#108, the following was made to provide the normative support for the CSI compression [1]:

CSI feedback enhancement, encompassing (two-sided model) [RAN1, RAN2]: - CSI spatial/frequency compression without temporal aspects (“Case 0”), - Specify necessary signalling/mechanism(s) as per the identified potential specification impacts in FS_NR_AIML_Air , including: - Model pairing procedure including ID and applicability reporting - Inference aspects including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports - NW and UE side data collection for training, - Target CSI format - Note The framework defined in BM and CSI prediction use cases could be reused - Specify performance monitoring

In this contribution, we discuss the CSI compression use case especially with respect to the inference-related aspects including target CSI type, CQI/RI determination, reporting mechanism, quantization configuration codebook, and CSI processing criteria.

Discussion

Target CSI type

Regarding target CSI type, precoding matrix in spatial-frequency domain has a strong basis in prior study results. During the Rel-18/19 study, most companies adopted spatial-frequency domain precoding matrix as the target CSI type. Assuming precoding matrix with spatial-frequency domain, sufficient evaluations show performance gains over benchmark. From our perspective, beam-delay domain or beam-frequency domain has a limited number of evaluations. Also, in order to specify beam-delay domain matrix, conversion method also needs to be specified. Therefore, precoding matrix in spatial-frequency domain should be supported.

Support spatial-frequency domain precoding matrix for target CSI type.

Inference report

Payload-related configuration

Since AI/ML-based CSI compression adopts the two-sided model, the encoder output (i.e., latent message) has to be delivered from UE to NW via CSI report. In general, the latent message is given as real-valued vector, which has to be quantized before reporting. In fact, pairing ID can be used to indicate mapping from target CSI into latent message, which is to be quantized. Meanwhile, the payload size of CSI report is subject to how the latent message is quantized. In this sense, CSI payload is inherently affected by pairing ID. Therefore, association between pairing ID and payload size configuration should be taken into account. For instance, applicability report procedure can be considered for the association.

- For AI/ML-based CSI compression, CSI report payload size inherently depends on pairing ID, which indicates mapping from target CSI into latent message.

Consider the methods for association between pairing ID and payload configuration (e.g., via signaling procedure for applicable functionality reporting).

In the previous meeting, the following were agreed.

Regardless of whether SQ (scalar quantization) or VQ (vector quantization) is used, the payload size is proportional to dz×Q, where dz denotes the latent message size, and Q denotes the number of quantization bits. Since the payload size varies directly with the product of dz and Q, the number of possible payload sizes increases with the degrees of freedom in dz and Q. Moreover, there may be multiple parameter combinations even for a single payload size as the combination of at least two parameters determines payload size. It is desirable to fix the value of either dz or Q, in order to limit the total number of parameter combinations at least for the same payload size. Considering that dz has more direct relationship with model design, introducing common/fixed Q values is more preferable. With flexibility of dz, payload size scalability is effectively realized. In particular, the rank-specific latent dimension dzν, enables effective payload adaptation to prevent becoming huge payload size for higher rank (e.g., rank ν=3 or 4) as in legacy.

For scalability of payload size and restriction on potential parameter combinations of (d, Q), consider to introduce common/fixed values of Q over rank/layer, where Q is the number of quantization bits and d is the latent message size.

Support rank-specific latent dimension (i.e., dzν) to ensure flexibility of payload size, enabling payload adaptation for different rank, where ν denotes rank.

CQI/RI determination

In current non-AI/ML based CSI feedback based on legacy codebook, CQI determination at UE is based on PMI of the codebook, and the gNB can transmit PDSCH by taking the UE side CSI feedback on CQI, RI, and PMI into account. Meanwhile, there is an issue on the CQI determination for CSI compression using two-sided model since the CSI reconstruction part is not available at UE-side. Hence, in Rel-18 study, the following options were discussed to determine the CQI at UE-side (at RAN1#112)

Agreement In CSI compression using two-sided model use case, further study the following options for CQI determination in CSI report, if CQI in CSI report is configured. - Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 1a: CQI is calculated based on target CSI with realistic channel measurement - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment - Option 1c: CQI is calculated based on legacy codebook - Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment - Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW. - Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder. - Other options are not precluded - Note1: feasibility of different options should be evaluated - Note2: Gap analyses between the UE side CQI calculation results and the NW side results, as well as the impact on the scheduling performance should be evaluated - Note3: Complexity of CQI calculation needs to be evaluated, including the computing complexity and potential RS/signaling overhead

Option 1 does not depend on the CSI reconstruction part and is based on using realistic channel measurement or legacy codebook, but this method can cause a CQI mismatch compared to CQI using AI/ML-based model. As a potential approach to alleviate the issue, Option 1b with some offset signaling from NW-side in order to adjust the CQI gap can be considered. Also, in Option 1c, non-AI/ML-based CSI report using legacy codebook can be triggered in addition to AI/ML-based CSI report. Option 2 is based on the CSI reconstruction part but there are several problems on the signaling overhead to deliver the CSI reconstruction part at NW-side and UE computational complexity. Using proxy model(s) to calculate the CQI can alleviate the above issues but still has a problem on reliability compared to the actual CSI reconstruction part. Option 2b relies on two-step signaling which means that more latency is required compared to other options for CQI calculation. So, it could cause CQI mismatch due to channel aging problem which is not preferable.

The details of approaches that can belong to Option 1 are as follows:

For Option 1a, CQI is based on the realistic channel measurement, but it may cause CQI mismatch issue. In Option 1b, NW can configure the CQI offset value, based on which the UE can calculate the CQI. As a result, CQI mismatch issue can be resolved.

In Option 1c, for the acquisition of CQI and RI at NW side, NW can trigger the additional legacy CSI report linked or paired with the AI/ML-based compressed CSI report. The legacy CSI report means the codebook-based CSI report including Type I CSI, Type II CSI (e.g., Rel-15/16/17/18). Via the additional legacy CSI report linked/paired to AI/ML CSI report, the NW can acquire the CQI and/or RI. AI/ML-based CSI report and legacy CSI report may or may not be the separate reports. Different CSI reporting/resource configurations can be set, which may be associated/linked/paired. Alternatively, legacy codebook can be configured in the CSI report configuration for CSI compression. Then, UE can calculate CQI based on the configured legacy codebook. As another alternative, instead of CQI, UE can report the other CSI contents (e.g., SINR), based on which the NW can schedule PDSCH with proper MCS level for the UE.

For CQI determination in CSI compression using two-sided model, consider to prioritize Option 1b (via realistic channel measurement + potential adjustment) or Option 1c (via legacy codebook).

With respect to the RI determination, UE may report a preferred RI value as legacy. Alternatively, UE may not report the preferred RI, as the NW can be aware of the RI implicitly based on the CSI payload size with multiple blind detections. To resolve the issue, some parameters regarding payload determination can be reported to the NW, which will be discussed in the following section.

- Legacy RI determination and reporting can be reused in inference report of CSI compression.

Reporting mechanism

In the Rel-18 study, regarding UCI format, following was agreed.

Agreement In CSI compression using two-sided model use case, for the study of UCI format, consider the legacy CSI reporting principle with CSI Part 1 and Part 2 as a starting point, where Part 1 has a network configured fixed size and Part 2 size is dynamic, determined by information in Part 1.

For the inference report, report contents and UCI mapping can be discussed. First of all, to report compressed CSI generated by UE-side model, legacy two-part encoding can be reused where Part 1 CSI has fixed payload and consists of CRI, RI, 1st CQI, and Part 2 CSI has dynamic payload and consists of LI, PMI, if precoding matrix-like model input is used. Therefore, RI and CQI can be determined as discussed in the section above, PMI part in Part 2 CSI can be replaced by compressed CSI. Additionally, payload indication information such as compression ratio, payload indicator can be added in Part 1 CSI.

We now turn our attention to considering the raw channel matrix as model input, if supported. In this case, the CSI reporting content would be all-in-one CSI which incorporate the legacy RI/CQI/PMI. Note that the concept of all-in-one CSI is more likely to explicit feedback rather than legacy implicit feedback. The all-in-one CSI may be composed of multiple information blocks that can be accumulated when decoding. For example, reconstruction quality may depend on the number of information blocks the decoder model take as the input, when the different information blocks correspond to different levels in terms of compression ratio. For this all-in-one explicit CSI feedback, CSI encoding scheme can be described as follows:

- 1st UCI part (fixed payload): the number (N) of information blocks and the 1st information block

- 2nd UCI part (dynamic payload): N-1 information blocks (i.e., 2nd, …, N-th)

With respect to the all-in-one CSI report, the multi-user decoder (CSI reconstruction part) model can also be utilized as a method for NW to select MCS, taking the inter-UE inference into account without UE’s CQI determination/report.

Study on the CSI encoding method with payload size indication, where part 1 of the CSI report can include either of the following: compression ratio, payload indicator, or the number of multiple information blocks (if applicable).

Another issue can be whether configuration of inference report and monitoring report can be separated or not. Actually, inference measurement resource and monitoring resource can be the same, so configuring dedicated resource for monitoring report seems to be flexible. Also, if dedicated monitoring report is allowed, there is case that one of reports can be dropped due to collision. Thus, it would be good to consider reporting both results of inference and monitoring together when monitoring report occasion is occurred. This approach may be beneficial for UE to manage PU as well as buffer.

Consider report of both inference result and monitoring result together when monitoring occasion is occurred.

Quantization configuration codebook

In RAN1#120bis meeting, the following agreements were reached with respect to the quantization codebook for the compressed CSI feedback and the configuration of the codebook.

Agreement For inter-vendor collaboration Direction C, - Use standardized quantization codebook For inter-vendor collaboration Direction A Options 4-1, 3a-1 (with or without NW-side target CSI sharing), - Standardize configuration(s) of quantization codebook, e.g., scalar or vector quantization, segment size of VQ, codebook size. - FFS: applicability of the above for Case 2 - Exchange quantization codebook of (the selected) standardized configuration(s) from NW-side to UE-side along with each exchanged dataset or model parameters. - FFS: whether quantization codebook may be different across different payload size configurations Agreement It is clarified that, for previous agreement on standardize configuration(s) of quantization for inter-vendor collaboration Direction A Options 4-1, 3a-1 (with or without NW-side target CSI sharing), resolve the first FFS with the following clarifications: - The standardized quantization configuration(s) refers to the configuration(s) related to the dimensionality related to the quantization operation (e.g., e.g., scalar or vector quantization, segment size of VQ, codebook size) and does not preclude the use of more advanced quantization algorithms (e.g., using past samples). - Exchanged quantization codebook does not preclude the use of codebook parameterized by parameters, e.g. quantization range and step size determined by some parameterized formula.

The common understanding of the quantization rule for the compressed CSI feedback between UE and NW is essential; therefore specification effort for aligning quantization is needed. The optimal quantization rule basically depends on the distribution of the latent vector (i.e., compressed CSI feedback). There can be various factors for the distribution of the latent such as learning scheme, training phase, model structure, active/loss functions, and collected data distribution (e.g., different users, different channel characteristics). It is expected that the optimization of the quantization rules for various latent distributions leads to considerable signaling overhead and/or specification effort. For quantization codebook alignment, following two potential approaches can be considered:

- New quantization rule defined and exchanged between NW and UE, whenever the latent distribution shifts.

- Standardized quantization codebooks specifying the pre-defined quantization rules for every possible latent distribution.

The former may result in huge amount of signaling overhead, whereas the latter may be inefficient in terms of specification effort and device storage since it needs to cover a number of cases. The related agreement made in the previous meeting is captured below.

Agreement: For quantization codebook used for quantizing the feedback, - For inter-vendor collaboration Direction A, select one of the following: - Alt1: Exchange quantization codebook from NW-side to UE-side along with each exchanged dataset or model parameters - Alt2: Use a standardized quantization codebook decided by RAN1.

If Alt2 is supported, NW should train models that align with the fixed codewords of CSI feedback. In this case, the advantage of inter-vendor training collaboration Direction A over Direction C diminishes, as the flexibility of model development for NW becomes limited. Therefore, Alt1 (exchanging codebook from NW-side to UE-side along with dataset) is preferable.

Regarding quantization codebook for CSI feedback, support Alt1 (NW trained and exchanged).

Meanwhile, the quantization-aware training can be adopted for the two-sided model use case in order to acquire AI/ML models which are robust to quantization errors. In this case, the encoder (i.e., CSI generation part) model can be viewed as including the quantization function, and the dequantization task can be interpreted as part of the CSI reconstruction part (decoder) model.

- When it comes to quantization-aware training, AI/ML model itself includes the quantization/dequantization block/layers.

Therefore, the model identification procedure can include the quantization configuration. For instance, the model (pairing) ID or dataset ID may implicitly indicate the quantization configuration for the compressed CSI feedback. As the model identification procedure serves as the indication of the quantization rule, the separate procedure for the quantization codebook exchange and/or quantization configuration indication from NW to UE may not be required. Thus, the signaling overhead and/or specification effort for the compressed CSI feedback quantization could be reduced.

Quantization alignment may also be performed by means of the dataset exchange in case of inter-vendor training collaboration sub-option 4-1 (Direction A). In order to lower the size of the dataset {target CSI, CSI feedback}, the format of CSI feedback could be specified as codeword index of the codebook rather than CSI feedback itself, assuming the NW determines the codebook with which the proxy/nominal encoder and decoder at the NW is learned, and then delivers the determined codebook to UE. In case the NW utilizes the standardized codebook for training the models, by contrast, data format of CSI feedback may be unquantized to facilitate the UE learning of the encoder with the dataset. Along with the dataset, which codebook is selected and employed for training among multiple standardized quantization codebook also should be indicated to UE by the NW.

Further study on following quantization alignment methods.

  - Via model identification procedure

  - Via dataset {target CSI, CSI feedback} exchange, where data format for CSI feedback may be specified as

    - codeword index of NW-determined codebook with additional signaling for quantization configuration (e.g., codebook exchange)

    - unquantized version with indication of NW’s selection from standardized (3GPP-specified) quantization codebooks

  - CSI processing criteria

For the CSI compression based on the two-sided model, there exists the CSI generation part (encoder model) at UE-side to perform the inference by default. However, UE may contain not only encoder model for inference, but also multiple different types of models such as nominal/proxy decoder model (CSI reconstruction part), and/or intermediate KPI estimator (e.g., SGCS estimator) as well, for the purpose of UE-side (UE-assisted) monitoring. Thus, ambiguity of how to count the processing unit (e.g., APU and/or CPU) for various types of reports/models may arise. In this regard, it may be required to define a clear PU counting rule to efficiently manage UE’s processing unit.

For CSI compression, study on PU counting rule considering

  - Different types of CSI reports, e.g., inference or monitoring

  - Different types of AI/ML models, e.g., encoder, proxy/nominal decoder, intermediate KPI estimator

The concept of PU has been adopted for resolving/managing UE-side computing capability. Since the CSI compression is enabled by two-sided model, it is natural to consider a method to address the excessive computing capability issue of the NW for supporting multiple UEs and various AI/ML models for different use cases (e.g., beam management). For example, AI/ML engine at NW may not be available in a certain timing duration, as NW runs AI/ML models with different purposes other than CSI decoding. In this case, NW may delay decoding the compressed information; then the reconstructed CSI may be aged and thus less useful information for NW. In this case, it will be better for UE not to do inference operation in the UE power saving perspective. This issue can frequently occur in case of semi-persistent inference reporting.

Study on methods to manage the NW-side computing capability for two-sided model, e.g., NW indication of inference skipping.

Conclusion

In this contribution, we provided our views on the inference-related aspects of the CSI compression. The summary of our views is listed below.

Observation #1: For AI/ML-based CSI compression, CSI report payload size inherently depends on pairing ID, which indicates mapping from target CSI into latent message.

Observation #2: Legacy RI determination and reporting can be reused in inference report of CSI compression.

Observation #3: When it comes to quantization-aware training, AI/ML model itself includes the quantization/dequantization block/layers.

- Proposal #1: Support spatial-frequency domain precoding matrix for target CSI type.

- Proposal #2: Consider the methods for association between pairing ID and payload configuration (e.g., via signaling procedure for applicable functionality reporting).

- Proposal #3: For scalability of payload size and restriction on potential parameter combinations of (d, Q), consider to introduce common/fixed values of Q over rank/layer, where Q is the number of quantization bits and d is the latent message size.

- Proposal #4: Support rank-specific latent dimension (i.e., dzν) to ensure flexibility of payload size, enabling payload adaptation for different rank, where ν denotes rank.

- Proposal #5: For CQI determination in CSI compression using two-sided model, consider to prioritize Option 1b (via realistic channel measurement + potential adjustment) or Option 1c (via legacy codebook).

- Proposal #6: Study on the CSI encoding method with payload size indication, where part 1 of the CSI report can include either of the following: compression ratio, payload indicator, or the number of multiple information blocks (if applicable).

- Proposal #7: Consider report of both inference result and monitoring result together when monitoring occasion is occurred.

- Proposal #8: Regarding quantization codebook for CSI feedback, support Alt1 (NW trained and exchanged).

- Proposal #9: Further study on following quantization alignment methods.

  - Via model identification procedure

  - Via dataset {target CSI, CSI feedback} exchange, where data format for CSI feedback may be specified as

    - codeword index of NW-determined codebook with additional signaling for quantization configuration (e.g., codebook exchange)

    - unquantized version with indication of NW’s selection from standardized (3GPP-specified) quantization codebooks

- Proposal #10: For CSI compression, study on PU counting rule considering

  - Different types of CSI reports, e.g., inference or monitoring

  - Different types of AI/ML models, e.g., encoder, proxy/nominal decoder, intermediate KPI estimator

- Proposal #11: Study on methods to manage the NW-side computing capability for two-sided model, e.g., NW indication of inference skipping.

- Reference

[1] RP-251870, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements”, Qualcomm