3GPP TSG RAN WG1 #122bis                                                  R1-2507634

Prague, Czech, Oct 13th – 17th, 2025

Source: Google

Title:	Discussion on Performance Evaluations for Rel-20 ISAC

Document for: Discussion and Decision

# Introduction

In RAN #108, the SID on Study on Integrated Sensing And Communication (ISAC) for NR was agreed [1]. The objective of the Rel-20 SI on ISAC for NR is to evaluate the performance of gNB-based mono-static sensing for the UAV use case. Several key metrics and baseline assumptions have been established. This contribution focuses on providing proposals to finalize the remaining key metrics and open issues.

# Evaluation Metrics

In RAN1#122, the following agreements have been reached. RAN1 has agreed on Positioning Accuracy, Velocity Accuracy, Missed Detection Probability, and False Alarm Probability (Types 1 and 2) as core metrics.

Agreement

Horizontal/vertical positioning accuracy are agreed as performance metrics for NR ISAC.

  - It is defined as the absolute value of the difference between the estimated horizontal/vertical position and the corresponding true position of a sensing target.

    - Note: in RAN1 evaluations, there should be only one estimated horizontal/vertical position corresponding to the true position of a sensing target

Agreement

- Velocity accuracy is agreed as performance metric for NR ISAC.

  - Velocity accuracy is defined as the absolute value of the difference between the estimated velocity and the corresponding true velocity of a sensing target.

  - For single TRP monostatic sensing,

    - The radial velocity accuracy can be estimated

      - The true radial velocity is the projection of true velocity on the direction from TRP to target for TRP monostatic.

    - The true velocity accuracy can be estimated.

Agreement

- Missed detection probability is agreed as performance metric for NR ISAC.

  - It is defined as the conditional probability of not detecting the presence of a target when the target is actually present in the simulation area.

Pmd=n=0N-1DnMnN

Where,

      - Dn is the number of missed targets in the drop n, i.e., the true target not associated with any detected object

      - Mn is the number of true targets in the drop n.

      - N is total number of drops with at least one target per drop

Agreement

- False alarm probability is agreed as performance metric for NR ISAC.

  - For cases without true target dropped in simulation area, False alarm probability Type 1 is computed and reported

  - For cases with true targets dropped in simulation area, False alarm probability Type 2 is computed and reported

  - Note: both False alarm probability Types are mandatory

  - KPI values for False alarm probability Type 1 and 2 can be discussed separately.

Definitions:

  - False alarm probability Type 1 (no target dropped in simulation area): An object is detected when there is no target present in simulation area is considered a false alarm.

Pf1=n=0N-1QnN

Where,

    - Qn equal to 1 if at least one object is detected when there is no target dropped in the simulation area in the drop n, otherwise Qn equal to 0.

    - N is the total number of drops without targets in the simulation area.

  - False alarm probability Type 2 (targets dropped in simulation area): An object is detected but not associated with any true targets in the simulation area is considered as a false alarm.

Pf2=0≤n<NMn'≠0Dn'Mn'K

Where,

      - Dn' is the number of detected objects but not associated with any true targets in the drop n.

      - Mn' is the total number of detected objects in the drop n.

      - FFS:

        - Option 1: K is number of drops (N)

        - Option 2: K is number of drops with at least one detected object

Note: the number of targets should be reported by companies when providing False alarm probability Type 2

- False Alarm Type 2 Calculation:

Regarding the remaining FFS for the false alarm Type 2 Pf2 calculation, we support Option 1. In Option 2, only drops where at least one object is detected are considered in the total number of drops. This metric will be biased by the quality of the detection. It ignores drops where the ISAC system fails to detect anything entirely and would get an artificially low Pf2.

- For the false alarm Type 2 Pf2 calculation, support Option 1 for the number of drops:

- Option 1: K is number of drops (N)

- CDF of positioning accuracy

In RAN1#122, the following proposal has been discussed:

Proposal

For Horizontal/vertical positioning accuracy,

  - The required value of positioning accuracy shall be obtained based on the [X] percentile point of the cumulative distribution function (CDF) of the positioning estimation errors, for a given maximum false alarm and miss detection rate.

    - X=[90%, 95%, other values]

We support this proposal for the CDF percentile points at 90% and 95% for the horizontal/vertical positioning accuracy. The 95% percentile represents a higher confidence threshold, which is essential for use cases with stricter reliability requirements, such as those related to public safety or critical infrastructure, and is also consistent with requirements specified in relevant SA1 specifications. The 90% percentile provides a balance between meeting stringent accuracy demands and accommodating practical implementation constraints, aligning with established confidence levels utilized in the positioning use case. Therefore, by evaluating performance at both 90% and 95% the framework can assess the robustness of the system and provides the necessary flexibility to satisfy the diverse range of ISAC service requirements across various use cases and deployment scenarios.

- Support 90%, 95% percentiles for the determination of the value of positioning accuracy.

- Sensing Resolution

Sensing resolution is the ability of the ISAC system to distinguish and separate two closely spaced targets in a specific domain (e.g., range, velocity, or angle). When we specifiy system requirements, we will often demand, for example, a range resolution of ΔR ≤ 0.5m  or an angular resolution of Δθ ≤ 1∘ .  The final delivered value is a measure of the system performance. It will depend on the sensing algorithm but also on the configurable system parameters. For example the range resolution is inversely proportional to the bandwidth and doppler resolution is inversely proportional to the total observation time. Therefore, it depends on both the system settings (e.g., bandwidth, CPI, antenna size, …) but also on the sensing algorithm.

We are fine to down-prioritize the sensing resolution as a performance metric for evaluation of Rel-20 NR ISAC to ease the progress and simplify the evaluations but it should be included in 6G evaluations.

- Sensing resolution is not considered as a performance metric for evaluation of Rel-20 NR ISAC

- Sensing service latency and refreshing rate

We agree with the majority view that Sensing Service Latency and Refreshing Rate should not be considered as fundamental NR ISAC performance metrics for simulation and evaluation at the RAN1 physical layer.

Making a precise RAN1-level simulation of these metrics is impractical. Furthermore, the refreshing rate is highly coupled with the CPI which is the true hard limitation imposed by the physical layer. However, simply excluding them is insufficient. We strongly support that the constraint imposed by latency must be incorporated into the simulation assumptions. Therefore, we advocate that instead of evaluating latency and refreshing rate as metrics, a clear limitation or constraint on the CPI value must be defined as a simulation assumption. This implicitly ensures that the evaluated performance metrics (like detection and accuracy) are achieved under realistic time constraints, thereby preventing systems from achieving arbitrarily good performance via unrealistically long observation times, and keeping the design compliant with practical latency budgets as required by SA1.

- Sensing service latency and refreshing rate are not considered as performance metrics for Rel-20 NR ISAC  .

- Constraint on the CPI value should be defined as a simulation assumption for Rel-20 NR ISAC.

# Association Method

In RAN1#122, the following agreement has been reached for the association of a detected object and the true target.

Agreement

For the purpose of performance metric calculation, association of the detected object(s) and the true target(s) should fulfil at least the following conditions:

  - One true target is associated with at most one detected object

  - One detected object is associated with at most one true target

  - The same association applies to miss detection, false alarm probability Type 2 (if defined) and positioning/velocity accuracy

  - Companies to report the method used for association of the detected object(s) and the true target(s)

It was agreed that object association should be left to individual companies implementations, with each company reporting the specific method used for the association. The association threshold may be based on both position and velocity. Furthermore, the association algorithm can be more sophisticated, incorporating information such as micro-Doppler or target characteristics. We support the requirement that the association method used, and particularly the information utilized in the association, be reported alongside the submitted evaluation results.

- Association method used, and particularly the information utilized in the association (e.g., position, velocity,  micor-doppler, target characteritics, …) be reported alongside the submitted evaluation results.

# Waveform and Sensing RS

In RAN1#122, the following two options have been discussed for the sensing RS overhead:

  - Option 1:

| Parameters | Assumptions |
| --- | --- |
| Sensing RS overhead | Around 10% overhead Exact T/F RE mapping and CPI are up to company choice |
| Waveform | OFDM |

  - Option 2:

| Parameters | Assumptions |
| --- | --- |
| Sensing RS overhead | - Comb 2 in frequency domain - Symbol 0 & 7 in each slot |
| CPI | 80ms for one sensing process |
| Waveform | OFDM |

There is a divergence between achieving comparable simulation results and allowing maximal evaluation fflexibility. We support that the DL NR waveform (CP-OFDM) should be the baseline.

Also, we support abstracting the exact RS and focus on the T/F RE mapping. However, simply enforcing a ∼10% overhead (in Option 1) while leaving the RE mapping open will lead to incomparable results, fundamentally undermining the study.

Therefore, we support Option 1 (Flexible T/F Mapping) combined with the CPI Constraint. Specifically, the baseline should define an upper bound on CPI to ensure practical latency is addressed. Also, the exact sensing RS overhead must be clearly reported alongside the achieved sensing performance as the 10% overhead should be treated as a reporting parameter/target range.

- Support DL NR waveform CP-OFDM as be the baseline for NR ISAC evaluation.

- Support Option 1 for the sensing RS overhead combined with the CPI Constraint

# Conclusion

In this contribution, we have presented several proposals for the evaluation of the performance of gNB-based mono-static sensing for UAV use case:

- For the false alarm Type 2 Pf2 calculation, support Option 1 for the number of drops:

- Option 1: K is number of drops (N)

- Support 90%, 95% percentiles for the determination of the value of positioning accuracy.

- Sensing resolution is not considered as a performance metric for evaluation of Rel-20 NR ISAC

- Sensing service latency and refreshing rate are not considered as performance metrics for Rel-20 NR ISAC  .

- Constraint on the CPI value should be defined as a simulation assumption for Rel-20 NR ISAC.

- Association method used, and particularly the information utilized in the association (e.g., position, velocity,  micor-doppler, target characteritics, …) be reported alongside the submitted evaluation results.

- Support DL NR waveform CP-OFDM as be the baseline for NR ISAC evaluation.

- Support Option 1 for the sensing RS overhead combined with the CPI Constraint

# References

- RP-242348, “Revised SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR”, Xiaomi, AT&T

- R1-2505777	Work plan for study on Integrated Sensing And Communication (ISAC) for NR	Xiaomi, China Telecom

- R1-2505778	TR skeleton for study on Integrated Sensing And Communication (ISAC) for NR	Xiaomi, China Telecom