3GPP TSG RAN WG1 #122bis		    							             				             R1-2507822

Prague, Czech, Oct 13th – 17th, 2025

Source: 	Sharp

Title:	Discussions on further coverage enhancement for NR

Agenda item:	10.4.1

Document for:	Discussion and Decision

Introduction

In Further NR coverage enhancements WI, the following objectives have been identified [1]:

- Specify following PRACH coverage enhancements [RAN1, RAN2] - Multiple PRACH transmissions with different Tx beams for 4-step RACH procedure. - UE may receive UL beam information after transmission of MSG1 - UL beam information is to assist the UE decision on Msg3 beam selection. - Note 1: “different Tx beams” is for the purpose of future RAN1 discussions - Note 2: The enhancements of PRACH are targeting for FR2 and can also apply to FR1 when applicable. - Note 3: The enhancements of PRACH are targeting short PRACH formats and can also apply to other formats when applicable. - Note 4: The PRACH repetitions are transmitted over ROs associated with the same SSB. - Note 5: The procedure for repetitions of a PRACH transmission is as in Rel-18 - Specify enhancements to support PUSCH repetition scheduled by DCI 0_0 with C-RNTI [RAN1, RAN2] - Specify enhancements to improve PUSCH coverage for higher uplink data rate by extending pi/2-BPSK to more MCS entries in MCS tables [RAN1]

In the contribution, we present our views on further NR coverage enhancement, including PRACH coverage enhancement and PUSCH repetition scheduled by DCI 0_0 with C-RNTI.

Discussions on PRACH coverage enhancement

Multiple PRACH transmissions with different Tx beams

For multiple PRACH transmission with different Tx beams, the following two scenarios can be considered:

- Scenario 1: SSB is transmitted with a wider beamwidth, while multiple PRACH can be transmitted with narrower beam sweeping.

- Scenario 2: during initial access, UE that is not capable of full beam correspondence may perform PRACH with beam sweeping.

For Scenario 1, a hierarchical beam structure can be employed at gNB for system operating at mmWave bands, where the SSB can be transmitted using a wider beamwidth to ensure broader coverage, and CSI-RS can be transmitted using narrower beams as part of a beam refinement procedure to support higher data rates. When the UE attempts to access the network, it selects an SSB with RSRP above a configured threshold and transmits the PRACH using the Tx beam based on association between SSB and PRACH occasion.

According to the current NR specification, the UE transmits one or more PRACH using a single Tx beam and attempts to monitor RAR within a RAR monitoring window. If the PRACH is not detected by the gNB, the UE needs to wait for next PRACH opportunity to retransmit the PRACH, potentially using the same or a different Tx beam after the RAR monitoring window. To reduce the access latency and improve the reliability of PRACH detection during 4-step RACH procedure, the UE may transmit multiple PRACH repetitions with narrower beam sweeping.

Figure 1 illustrates PRACH transmissions with narrow beam sweeping during the 4-step RACH procedure. It should be noted that all these narrower beams are linked to the same SSB with wider beamwidth, i.e., PRACH repetitions are transmitted over ROs associated with the same SSB. Further, as indicated in the WID, the same procedure defined for multiple PRACH transmissions using the same Tx beam can be reused for multiple PRACH transmissions with different Tx beams [1].

Figure 1. PRACH transmissions with narrow beam sweeping

Observation 1

- When SSB is transmitted with wide beam, PRACH transmission with narrow beam sweeping may help reduce access latency and improve reliability of PRACH detection, compared to single PRACH transmission.

For the Scenario 2, as defined in the UE feature list, UE that fulfils the beam correspondence requirement with the uplink beam sweeping shall set the bit to 0. This indicates that during initial access, the UE may not support full beam correspondence. In such cases, the Tx and Rx beam are not fully aligned, which indicates that the selected Rx beam for the corresponding SSB may not be the most suitable Tx beam for PRACH transmission.

If the UE utilizes the single Tx beam for PRACH transmission, gNB may fail to detect the PRACH preamble due to misalignment between Tx and Rx beams. To address this, it may be beneficial for the UE to perform PRACH beam sweeping prior to the RAR monitoring window. This can improve PRACH detection performance and reduce access latency, especially when considering the scenarios where narrow beams are used for both SSB transmission and PRACH detection.

Observation 2

- When UE is not capable of full beam correspondence, it may be beneficial to employ the PRACH beam sweeping at the UE side to improve the detection performance and reduce the access latency, compared to single PRACH transmission.

Procedure for multiple PRACH transmissions with different beams

Figure 2 illustrates multiple PRACH transmissions with same and different Tx beams during 4-step PRACH procedure. For multiple PRACH transmissions with different Tx beams, upon successful detection of the PRACH with repetitions, the gNB may determine the optimal Tx beams for Msg3 transmission based on received power measurements across different PRACH occasions. Subsequently, the gNB can indicate the selected Tx beams to the UE for use in Msg3 transmission. In this regard, the UE may apply the indicated Tx beams for Msg3, thereby improving coverage performance.

Figure 2. Multiple PRACH transmissions with same and different Tx beams

As defined in Rel-18 coverage enhancement, PRACH resource partitioning is utilized to differentiate a single PRACH transmission and multiple PRACH repetitions with same Tx beam. This design principle can be further extended to support multiple PRACH transmissions using different Tx beams. In such cases, the gNB can simply measure the received power of each detected PRACH to identify the best Tx beam for a given UE. This approach differs from the case of multiple PRACH transmissions with same Tx beam, where the gNB may need to perform non-coherent combining in order to improve the PRACH detection performance.

Based on the discussions above, PRACH resource partitioning can be employed to differentiate among the following scenarios:

- Single PRACH transmission,

- multiple PRACH transmission with same Tx beam and

- multiple PRACH transmission with different Tx beams

To enable this differentiation, either separate PRACH occasions or shared PRACH occasions with separate PRACH preambles may be configured. Further, similar to multiple PRACH transmissions with same Tx beam, at least repetition level of 2 and 4 may be applied to the multiple PRACH transmissions with different Tx beams. In this regard, PRACH resource partitioning may be used to differentiate the multiple PRACH transmissions with different PRACH repetition levels.

It should be noted that repetition level of 8 may need further investigation for multiple PRACH transmissions with different Tx beams. In particular, at least for scenarios where a wider beam is employed for SSB transmissions and narrower beam sweeping is applied for multiple PRACH transmissions, 4 distinct narrow beams for PRACH transmission would be sufficient to cover the wide beam used for the associated SSB transmission.

Proposal 1

- For multiple PRACH transmissions with different Tx beams

  - Separate PRACH occasions or shared PRACH occasions with separate PRACH preambles can be configured to differentiate 1) single PRACH transmissions, 2) multiple PRACH transmissions with same Tx beam, 3) multiple PRACH transmission with different Tx beams, using different PRACH repetition levels.

  - At least repetition level of 2 and 4 is supported. FFS: repetition level of 8.

Further, the same PRACH preamble can be used across multiple PRACH transmissions with different Tx beams. This enables the gNB to identify the optimal Tx beams based on simple received power measurements of the PRACH signals. As noted in the WID [1], PRACH repetitions are transmitted over ROs associated with the same SSB. In such a scenario, the same SSB-to-RO group mapping for multiple PRACH transmission with same Tx beams can be reused for multiple PRACH transmissions with different Tx beams.

Since the gNB may need to measure the received power after PRACH detection across multiple PRACH transmissions with different Tx beams in order to identify the optimal Tx beam for Msg3 transmission, it is more appropriate to apply the same Tx power across all PRACH transmissions. This approach is essential to ensure accurate power measurements at the gNB side.

Proposal 2

- For multiple PRACH transmissions with different Tx beams

  - The same PRACH preamble is applied during multiple PRACH transmissions.

  - The same SSB to RO group mapping for multiple PRACH transmission with same Tx beams is reused.

  - The same Tx power is applied during multiple PRACH transmissions.

Indication of Tx beam for Msg3 transmission

To indicate the identified Tx beams for Msg3 transmission, several options can be considered as follows:

- Option 1: explicit indication of the selected Tx beams for Msg3 transmission in the RAR

- Option 2: implicit indication of the selected Tx beams for Msg3 transmission by extending Type1-PDCCH CSS set

For Option 1 with explicit indication of the selected Tx beams for Msg3 transmission, this is similar to the indication of Msg3 repetitions as defined in Rel-17 coverage enhancement WI, where the MCS field in the UL grant is repurposed to signal the Msg3 repetition level. While this solution introduces minimal impact to the specifications, it poses practical challenge in identifying available code points or fields within the RAR message for indication of the selected Tx beam for Msg3 transmission, especially when considering 8 repetitions for multiple PRACH transmissions with different Tx beams. In such scenarios, 3-bit indication may be needed, which is likely infeasible given the limited space available in the UL grant in the RAR.

For Option 2 with implicit indication of the selected Tx beams for Msg3 transmission by extending Type1-PDCCH CSS set, multiple Type1-PDCCH CSS sets may be configured by SIB1, where each Type1-PDCCH CSS set may correspond to a particular PRACH repetition. In this approach, after transmitting a PRACH repetition during multiple PRACH transmissions, the UE may monitor the RAR using the Type1-PDCCH CSS set that is associated with the PRACH repetition.

Figure 3 illustrates an example of two Type1-PDCCH CSS sets in case of 2 repetitions for multiple PRACH transmissions with different Tx beams. It should be noted that the UE is expected to detect only one RAR across the multiple Type1-PDCCH CSS sets within the RAR monitoring window. Based on the Type1-PDCCH CSS set where the RAR is detected, the UE may determine the corresponding PRACH repetition index and apply the associated Tx beam for Msg3 transmission.

Figure 3. Multiple Type1-PDCCH CSS sets for multiple PRACH transmissions with different Tx beams

While this approach avoids the need for explicit indication of the selected Tx beam for Msg3 transmission, it may introduce additional latency in the 4-step RACH procedure when multiple PRACH transmissions with different Tx beams are used. For instance, if 4 or 8 repetitions are applied for multiple PRACH transmissions, the UE may need to wait for the last Type1-PDCCH CSS set to successfully decode the RAR, thereby delaying the Msg3 transmission. It should be noted that for this option, different RA-RNTIs may be used for PDCCH monitoring associated with different PRACH repetitions.

Based on the discussions above, careful investigation is needed on the solution for the indication of the selected Tx beams for Msg3 transmission for multiple PRACH transmissions with different Tx beams. To strike a reasonable balance between the limited signaling space in the UL grant and the potential latency introduced in the 4-step RACH procedure, a hybrid approach combining both explicit and implicit beam indication methods may be considered for the Tx beam indication for Msg3 transmission.

Proposal 3

- RAN1 to study the solution to indicate the selected Tx beam for Msg3 transmission.

Discussions on PUSCH repetition scheduled by DCI 0_0 with C-RNTI

Procedure on PUSCH repetitions scheduled by DCI 0_0

As defined in NR, semi-static configuration and dynamic indication of the number of repetitions for PUSCH repetition Type A scheduled by non-fallback DCI format including DCI format 0_1/0_2/0_3 is supported. In particular, the number of repetitions can be configured either as part of the PUSCH configuration or via a row in the TDRA table. Further, in Rel-17 coverage enhancement WI, Msg3 repetition was supported, where MCS field in the RAR UL grant and DCI format 0_0 with TC-RNTI is repurposed to indicate the number of repetitions for Msg3 transmission and retransmission, respectively.

For PUSCH repetitions, the UE may be scheduled by DCI format 0_0 with CRC scrambled by the C-RNTI, even before capability reporting and RRC configuration. In such cases, the gNB may not be aware of whether the UE supports this feature and therefore may not be able to schedule PUSCH repetitions appropriately. To address this issue, UE may need to report its capability to support this feature during 4-RACH procedure so as to allow gNB to schedule the PUSCH repetitions.

To indicate support for this feature, the UE may report its capability using either PRACH resource partitioning or Msg3 transmission. In NR, PRACH resource partitioning has been defined across multiple releases for various purposes, including 2-step RACH, Msg3 repetition, multiple PRACH transmissions with same Tx beams, small data transmission, etc. Given the limited resources for PRACH transmission during 4-step RACH procedure, it is more desirable not to allocate dedicated PRACH resource only for indicating support of this feature, which can also help minimize the impact on legacy UEs in the network.

Based on the discussion above, UE may indicate support of this feature, i.e., PUSCH repetitions scheduled by DCI format 0_0 with CRC scrambled by C-RNTI via Msg3 transmission. This is analogous to the capability indication for PUCCH repetition carrying HARQ-ACK feedback in response to Msg4 transmission, as define in Rel-18 NTN.

Further, if the UE supports both Msg3 repetition and PUSCH repetition scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, and if the UE requests Msg3 repetition based on a measured RSRP that is less than a configured threshold, such a request may serve as an indication for PUSCH repetition scheduled by DCI format 0_0. Upon receiving the capability indication and request of Msg3 repetition, the gNB may determine that the UE is in poor coverage and accordingly schedule the PUSCH repetition using DCI format 0_0. Further, if UE does not request Msg3 repetition, the UE may follow legacy behavior and assume that single PUSCH transmission is scheduled by DCI format 0_0 with C-RNTI.

Proposal 4

- UE indicates support of PUSCH repetition scheduled by DCI format 0_0 with C-RNTI via Msg3 transmission.

- If UE requests the Msg3 repetition, the UE determines the number of repetitions for PUSCH transmission scheduled by DCI format 0_0 with C-RNTI.

As mentioned above, PUSCH repetition may be scheduled by DCI format 0_0 with CRC scrambled by C-RNTI prior to RRC configuration. In this scenario, considering the mandatory support of PUSCH repetition Type A, it is reasonable to assume that Type A PUSCH repetition is applied for PUSCH scheduled by DCI format 0_0 with C-RNTI.

As introduced in Rel-17 coverage enhancement WI, available slot counting can be enabled for the PUSCH repetitions. More specifically, a two-step approach was introduced, where in the first step, the UE determines available slots for K PUSCH repetitions; in the second step the UE determines whether to drop each of the K PUSCH repetition or not according to PUSCH dropping rules, where the PUSCH repetition is still counted in the K repetitions even if it is dropped. In this first step, UE determines a slot as available slots if the corresponding PUSCH repetition does not overlap with SSB symbols or DL symbols that are semi-statically configured by TDD UL/DL configuration.

For PUSCH repetition scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, it is more desirable to employ available slot counting for the PUSCH repetition, which can help improve the coverage for PUSCH transmission.

Proposal 5

- Type A PUSCH repetition is applied for PUSCH scheduled by DCI format 0_0 with C-RNTI.

- Available slot counting is applied for PUSCH repetition scheduled by DCI format 0_0 with C-RNTI.

Indication of the number of PUSCH repetitions

The number of repetitions for PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI may be determined based on whether the UE is configured by the repetition parameter by higher layers. In cases where the number of repetitions is configured by higher layers either as part of the PUSCH configuration or via a row in the TDRA table, same mechanism as defined for PUSCH repetition Type A scheduled by DCI format 0_1/0_2/0_3 can be reused, which can help minimize the specification impact.

Further, if the number of repetitions is not configured by higher layers either as part of the PUSCH configuration or via a row in the TDRA table, certain fields in the DCI format 0_0 with CRC scrambled by C-RNTI may need to be repurposed to indicate the number of repetitions. Following the design principle adopted for Msg3 repetition during 4-step RACH procedure, it is more appropriate to utilize the MCS field to indicate the number of repetitions for PUSCH transmission. This is primarily due to the fact that lower MCS values are typically expected for UEs in poor coverage conditions where PUSCH repetition is employed.

When the MCS field in the DCI format 0_0 is utilized to indicate the number of repetitions for PUSCH transmission, the 2 MSBs of the MCS fields can be reinterpreted to identify one of the 4 repetition levels that is configured by higher layers. In addition, the remaining 3 LSBs of the MCS fields can be used to indicate the MCS index selected from a predefined or configured subset of the MCS table.

Proposal 6

- When numberOfRepetitions or pusch-AggregationFactor is configured by higher layers, same mechanism for PUSCH repetition scheduled by DCI format 0_1/0_2/0_3 is reused for that scheduled by DCI format 0_0.

- When numberOfRepetitions and pusch-AggregationFactor is not configured by higher layers, MCS field in the DCI format 0_0 with CRC scrambled by C-RNTI is repurposed to indicate the number of repetitions for PUSCH transmission.

  - 4 number of repetitions can be configured by higher layers.

  - 2 MSBs of MCS field are used to indicate the number of repetitions.

Conclusions

In this contribution, we presented our views on further NR coverage enhancement, including PRACH coverage enhancement and PUSCH repetition scheduled by DCI 0_0 with C-RNTI. Further, we summarize the observations and proposals as follows:

Observation 1

- When SSB is transmitted with wide beam, PRACH transmission with narrow beam sweeping may help reduce access latency and improve reliability of PRACH detection, compared to single PRACH transmission.

Observation 2

- When UE is not capable of full beam correspondence, it may be beneficial to employ the PRACH beam sweeping at the UE side to improve the detection performance and reduce the access latency, compared to single PRACH transmission.

Proposal 1

- For multiple PRACH transmissions with different Tx beams

  - Separate PRACH occasions or shared PRACH occasions with separate PRACH preambles can be configured to differentiate 1) single PRACH transmissions, 2) multiple PRACH transmissions with same Tx beam, 3) multiple PRACH transmission with different Tx beams, using different PRACH repetition levels.

  - At least repetition level of 2 and 4 is supported. FFS: repetition level of 8.

Proposal 2

- For multiple PRACH transmissions with different Tx beams

  - The same PRACH preamble is applied during multiple PRACH transmissions.

  - The same SSB to RO group mapping for multiple PRACH transmission with same Tx beams is reused.

  - The same Tx power is applied during multiple PRACH transmissions.

Proposal 3

- RAN1 to study the solution to indicate the selected Tx beam for Msg3 transmission.

Proposal 4

- UE indicates support of PUSCH repetition scheduled by DCI format 0_0 with C-RNTI via Msg3 transmission.

- If UE requests the Msg3 repetition, the UE determines the number of repetitions for PUSCH transmission scheduled by DCI format 0_0 with C-RNTI.

Proposal 5

- Type A PUSCH repetition is applied for PUSCH scheduled by DCI format 0_0 with C-RNTI.

- Available slot counting is applied for PUSCH repetition scheduled by DCI format 0_0 with C-RNTI.

Proposal 6

- When numberOfRepetitions or pusch-AggregationFactor is configured by higher layers, same mechanism for PUSCH repetition scheduled by DCI format 0_1/0_2/0_3 is reused for that scheduled by DCI format 0_0.

- When numberOfRepetitions and pusch-AggregationFactor is not configured by higher layers, MCS field in the DCI format 0_0 with CRC scrambled by C-RNTI is repurposed to indicate the number of repetitions for PUSCH transmission.

  - 4 number of repetitions can be configured by higher layers.

  - 2 MSBs of MCS field are used to indicate the number of repetitions.

- References

- RP-251862, “New WID: Coverage enhancements for NR Phase 3”, June 2025