# 3GPP TSG RAN WG1 #122-bis								 R1-2507840

# Prague, Czech, Oct 13th – 17th, 2025

# Source:	Hanbat National University

# Title:		Discussion on 6GR modulation schemes

# Agenda item:	11.4.2

# Document for:	Discussion/Decision

Introduction

The study item on “Study on 6G Radio” was approved in RAN#108 meeting and it contains several physical layer structure design aspects in its scope [1]. as follows:

- Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2] - MIMO operation [RAN1, RAN4] - Duplexing [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1]. RAN4 can be involved, if necessary, based on the LS from RAN1

In this regard, as a starting point of the 6GR modulation schemes, the following agreements on the evaluation assumptions were made in the 3GPP RAN1#122 meeting [2]:

Agreement - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes

In this contribution, we discuss some considerations for identifying the further details of 6GR modulation schemes.

Discussions

Legacy uniform QAM constellation

RAN1 agreed to use 5G NR uniform QAM constellation as the study baseline while giving room for further enhancements on 4096QAM (DL) and 1024QAM (UL). 4096QAM transmits 12 bits/symbol while 256QAM transmits 8 bits, and 1024QAM, 10 bits. Channel environments with high signal-to-noise-ratio (SNR) for example line-of-sight, short range, no mobility, or very high-gain beamformed links, the bits per symbol can directly increase throughput without needing more bandwidth. This is the main motivation for considering 4096QAM on the DL. 4096QAM however demands extremely low EVM or high linearity. Even if the Base Station (BS) RF can be improved, the channel (and UE receiver linearity/noise figure) still limits end-to-end performance. Hence the need to apply advanced receiver impairment algorithms for added complexities.

For the uplink, DFTs-OFDM (used in UL to reduce PAPR) and User Equipment (UE) Power Amplifier (PA) linearity constrain the ability to transmit very high-order uniform QAM. The common evaluation framework requires explicit PAPR/PA linearity assumptions; that is, unless UE PAs and linearization are improved, 1024QAM UL will be usable only by a small subset of UEs.

Proposal 1: RAN1 to support 4096QAM for DL data channels, but only as an optional modulation scheme which is restricted to very high SNR environments.

Proposal 2: Considering stringent power conditions for UEs, RAN1 to support up to 256QAM for UL data channels.

Shaped constellation

It was agreed to use 5G NR uniform QAMs as the baseline for 6GR study. To address the "FFS: Enhancements and other modulation schemes" and meet 6G's goals for superior spectral efficiency, energy efficiency, and coverage, it is proposed that shaped constellations should be considered as an additional candidate for 6GR modulation.

Shaped constellations are a proven technique to reduce the performance gap to the Shannon limit, becoming increasingly crucial as we move to higher-order modulations like 1024QAM and 4096QAM where the gap to capacity is more significant.

Geometric Constellation Shaping (GCS) can be prioritized with 64QAM to 1024QAM modulation schemes. The fixed ~0.5 dB gain provided by GCS is expected to offer an immediate coverage improvement with minimal implementation complexity, while the novel approach of selecting optimized subsets from larger uniform QAM constellations maintains low-complexity demodulation suitable for all UE categories across both CP-OFDM and DFT-s-OFDM waveforms.

Probabilistic Constellation Shaping (PCS) is optimally suited for Fixed Wireless Access and backhaul applications utilizing 256QAM to 4096QAM modulation schemes. The fine-grained rate adaptation capability of PCS maximizes spectral efficiency under stable channel conditions, though the distribution matcher complexity and sensitivity to power amplifier nonlinearities require careful system design, particularly when operating with higher-order modulation schemes where the shaping gain is most pronounced.

Proposal 3: For 6GR, RAN1 is encouraged to continue to study GCS and PCS as additional constellation techniques.

Conclusion

In this contribution, we discussed 6GR modulation schemes. Our proposals are reproduced below:

Proposal 1: RAN1 to support 4096QAM for DL data channels, but only as an optional modulation scheme which is restricted to very high SNR environments.

Proposal 2: Considering stringent power conditions for UEs, RAN1 to support up to 256QAM for UL data channels.

Proposal 3: For 6GR, RAN1 is encouraged to continue to study GCS and PCS as additional constellation techniques.

References

- RP-251881 “New SID: Study on 6G Radio,” 3GPP RAN#108, Jun. 2025.

- 3GPP RAN1 Chair’s notes RAN1#122, Aug. 2025.