3GPP TSG RAN WG1 #122bis			R1- 2508060

Prague, Czech, Oct 13th – 17th, 2025

Agenda Item:	8.1

Source: 	Moderator (LG Electronics)

Title: 	Summary #1 of maintenance on CSI prediction

Document for:	Discussion and decision

Introduction

This is the summary document for maintenance on CSI prediction, agenda item 8.1 (maintenance on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface).

Contact information

Please provide your contact information or any updates.

| Company | Name | Email |
| --- | --- | --- |
| Moderator (LG Electronics) | Haewook Park | haewook.park@lge.com |
| New H3C | Lei Zhou | Zhou.leih@h3c.com |
| Samsung | Ameha Tsegaye Abebe | amehat.abebe@samsung.com |
| Indian Institute of Technology Madras (IIT Madras) | Sai Prasad Pirati | venkatasiva@5gtbiitm.in |
| SK telecom | Yunesung Kim | yunesung.kim@sktelecom.com |
| MediaTek | Pedram Kheirkhah Sangdeh Yuching Huang | Pedram.kheirkhah@mediatek.com yuching.huang@mediatek.com |
| Panasonic | Tetsuya Yamamoto Xuan Tuong Tran (Henry) | yamamoto.tetsuya001@jp.panasonic.com xuantuong.tran@sg.panasonic.com |
| vivo | Hao Wu | hao.wu@vivo.com |
| Huawei, HiSilicon | Keyvan Zarifi, Yuan Li | Keyvan.Zarifi@huawei.com liyuan3@huawei.com |
| NEC | Peng Guan, Yi Jiang | guan_peng@nec.cn y-jiang_ct@nec.com |
| Fujitsu | WANG Guotong (David) | wangguotong@fujitsu.com |
| Qualcomm | Chenxi Hao | chenxih@qti.qualcomm.com |
| OPPO | Wendong Liu | liuwendong1@oppo.com |
| NTT DOCOMO | Xin Wang Haruhi Echigo | wangx@DOCOMOlabs-beijing.com.cn haruhi.echigo.fw@nttDOCOMO.com |
| Spreadtrum | Shijia Shao Hualei Wang | Shijia.shao@unisoc.com Hualei.wang@unisoc.com |
| Ericsson | Xinlin Zhang Jingya Li | Xinlin.zhang@ericsson.com Jingya.li@ericsson.com |
| ZTE | Xingguang WEI Yunqi Sun | wei.xingguang@zte.com.cn sun.yunqi@zte.com.cn |
| IIT Kanpur | Shyam Vijay Gadhai Abhishek Kumar Singh | svgadhai@iitk.ac.in abhishekks@iitk.ac.in |
| AT&T | Isfar Tariq | Isfar.tariq@att.com |
| Xiaomi | Liuzhengxuan Liumin | liuzhengxuan@xiaomi.com liumin10@xiaomi.com |
| Intel | Victor Sergeev | Victor.sergeev@intel.com |
| CMCC | Yuhua Cao Yi Zheng Dan Song | caoyuhua@chinamobile.com zhengyi@chinamobile.com songdan@chinamobile.com |
| Lenovo, MotM | Ahmed Hindy Vahid Pourahmadi | ahmedhindy@motorola.com vpourahmadi@lenovo.com |
| CEWiT | Shiv Shankar | shivshankar@cewit.org.in |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| CATT | Qianrui Li | liqianrui@catt.cn |
| TCL | Didi Zhang | didi.zhang@tcl.com |
| LGE | Junhyuk Yoo | Junhyuk.yoo@lge.com |
| Sony | Sam Atungsiri | Sam.atungsiri@sony.com |
| InterDigital | Moon-il Lee | moonil.lee@interdigital.com |
| ETRI | Youngjoon Yoon Anseok Lee | youngjoon.yoon@etri.re.kr alee@etri.re.kr |
| Mavenir | Fan Yang Yuanlong Yang | fan.yang@mavenir.com yuanlong.yang@mavenir.com |
| Apple | Huaning Niu | huaning_niu@apple.com |
| Tejas Networks | Pavan Kalyan D | pavankalyand@tejasnetworks.com |
| Sharp | Makoto Kitahara Liqing Liu | kitahara.makoto@sharp.co.jp liu.liqing@sharp.co.jp |
| ETRI | Youngjoon Yoon | youngjoon.yoon@etri.re.kr |
| ASUSTeK | Denny Huang | Denny_Huang@asus.com |
| Futurewei | Chunhui (Allan) Zhu | czhu@futurewei.com |
| Nokia | Filippo Tosato Keeth Jayasinghe | filipppo.tosato@nokia.com keeth.jayasinghe@nokia.com |

Maintenance on CSI prediction

Applicability report

In this meeting, proposals/observations related to applicability report are copied below:

| CATT | Proposal 4: For CSI prediction, support both Option A and Option B for applicability reporting procedure, and the following RRC parameters are to be used for Option B: - resourcesForChannelMeasurement - vectorLengthDD (N4) - predictionDelay (delta) - unitDurationDD (d) |
| --- | --- |

[FL note]

In my understanding, in RAN2#131, RAN2 agreed that Option B is not supported for CSI prediction. So, we don’t need to discuss proposal from CATT.

Agreements on Applicability Report - The applicability reporting procedures for CSI prediction are the same as for beam management. RAN2 confirms that option B is not supported for CSI prediction, given no parameters were provided by RAN1. - RAN2 understand that when network releases inference configurations of poor-performed applicable functionalities, network may also provide either non-AI/ML configuration in CSI-ReportConfig or may provide full inference configuration of other applicable functionalities, if previously not configured to UE. There is no spec impact and feedback from the NW to the UE to adjust the applicability determination procedure is not supported in Rel-19. - No enhancements are pursued for reporting applicability in RRCReestablishmentComplete. - The UE can report applicability via RRCResumeComplete for SCG inference configurations received in RRCResume, without specification impact beyond already agreed applicability reporting procedure. - Applicability reporting is added in RRCResumeComplete for inference configurations that exist at the UE based on legacy procedures (restored or received in RRCResume). - RAN2 assumes for no NR DC enhancements are considered and will not ask RAN3 work in Rel-19.

Please provide your input/suggestion on this issue.

| Company | Views |
| --- | --- |
| CATT | As is mentioned in the highlighted part, the reason RAN2 confirms that option B is not supported for CSI prediction is merely because RAN1 has not provided parameters. But applicability Reporting has never been discussed in AI CSI prediction in Rel-19, so it’s obvious we have not provided parameters. In our understanding, whether or not to support applicability Reporting option B in AI CSI prediction should be first discussed in RAN1. |
| Mod | I can understand your comments. Let me check the temperature regarding this issue during offline. |

Data collection related aspects

In this meeting, proposals/observations related to data collection for training are copied below:

| Ericsson | Observation 1 Without separate configuration of CSI-RS resources for training data collection for model input and ground-truth CSI, it is not always possible to train a model that corresponds to a supported CSI report configuration for inference, e.g., model inference using aperiodic CSI-RSs for CMR. Proposal 3 For UE-sided CSI prediction use case, for training data collection, support two separate CSI-RS resources for UE-sided training data collection, one for model input and the other one for ground-truth. |
| --- | --- |
| Huawei | Proposal 12: For UE side data collection, gNB indicates the preferred inference related information (which will be potentially configured at the inference phase) as guidance to facilitate UE side training, including: - Prediction window related information, including the time gap between the earliest of N4 predicted instances and the reference time (e.g., CSI-RS occasion), the time unit of predicted instance (i.e., d), and the number of predicted time instances (i.e., N4). - Observation window related information, including the number of A-CSI-RS for channel measurement (i.e., K). |
| Google | Proposal 3-4: Support to configure the data collection window - CSI-RS for data collection is transmitted in the data collection window - CSI-RS for data collection is not transmitted outside the data collection window |

[FL note]

As shown above, for data collection for training, several issues were raised by some companies

- Issue 2.2-1: RS configuration (single vs. two-separate RS configuration)

- Issue 2.2-2: Necessity of assistant information by NW or UE

- Issue 2.2-3: Configuration of data collection window (from Google’s proposal)

Regarding issue 2.2-1, this issue was discussed in the previous meetings, but there is no further progress due to diverged views from different companies. Moderator thinks that the situation remains the same which means that most of companies think that this is not a essential issue. For issue 2.2-2, please discuss in section 3 (reply LS). For issue 2.2-3, moderator think it is upto NW implementation.

Please provide your input/suggestion on Issue 2.2-1/2.2-2/2.2-3.

| Company | Views |
| --- | --- |
| Qualcomm | Regarding 2.2-1, we don’t see any technical reason for objecting two-separate RS configurations. |
| Google | For issue 2.2-3, could moderator clarify a bit how this can be handled by NW implementation? The data collection should not be quite frequent. |
| Ericsson | On 2.2-1, it has been agreed to use a single resource setting, while whether having a single or two separate resources in the resource setting is not clear. Clarifying this is also helpful for replying the LS. |
| Mod | @ Qualcomm and Ericsson: I will check the temperature during the offline @ Google: Sorry I misunderstood your proposal. But, I want to know the motivation of having data collection window. And other than Google, which company support having data collection window? |

Time relaxation for SP CSI report

In this meeting, proposals/observations related to timeline are copied below:

| LG Electronics | Proposal #5: Do not introduce timing relaxation for SP inference report. |
| --- | --- |
| Nokia | Proposal 5: For AI/ML CSI prediction, it is unnecessary to add any specification clarification for the time relaxation of SP CSI reporting. |
| Xiaomi | Proposal 6: Whether needs additional time to load AI model for SP CSI report depends on UE capability. |

In RAN1#122, Qualcomm proposed to introduce time relaxation for SP CSI report. Regarding this issue, in this meeting, only LG, Nokia and Xiaomi provide their view. LG and Nokia think that there is no need to add any specification clarification for the time relaxation for SP CSI report. Meanwhile, Xiaomi propose that whether to have additional time for SP CSI report is up to UE capability. Based on inputs from previous meeting and this meeting, majority companies think it is not needed. If you still have strong preference to introduce time relaxation for SP CSI report, please

If you still have strong preference to introduce time relaxation for SP CSI report, please provide your input on this issue.

| Company | Views |
| --- | --- |
| Qualcomm | If companies agreed to relax the timeline for A-CSI, we don’t understand why it does not apply to SP-CSI. If the argument is model loading time, we think it is generic. For SP-CSI, there could be another A-CSI report triggered prior to any SP-CSI occasion, or there can be any proprietary AI operations prior to any SP-CSI occasion, then UE has to load the model back so as to facilitate the SP-CSI reporting. In legacy, the SP-CSI timeline is more relaxed than A-CSI. We don’t see the rational that SP-CSI timeline for AIML becomes even more tight than A-CSI. |
| Google | Support to relax the time for SP CSI report |
| Xiaomi | In our understanding, whether extending the time line for SP CSI report depends on relation between T∙Kp and AI model load time . If T∙Kp is larger than the AI model load time, it does not need to the additional time, where T and Kp denote the periodicity of CSI-RS resource and the number of active CSI-RS resource, respectively. Otherwise, additional time is needed to load AI model. |
| Mod | I think Qualcomm, Google, Xiaomi want to relax the timeline for SP inference report. Other than those companies, which companies wants to relax the timeline for SP inference report? |

Performance monitoring

In this meeting, proposals/observations related to performance monitoring are copied below:

| ZTE | Reason for change: In current specification, there is no linkage between monitoring occasion and the predicted CSI in inference occasion. Summary of change: Linkage between monitoring occasion and the predicted CSI in inference occasion is added by specifying the CSI-RS resource transmission occasion for channel measurement for monitoring. Consequence if not approved: Incomplete capture of the agreement. Text Proposal 3: Adopt the following changes in section 5.2.1.4.1, TS 38.214. TS38.214 ##### 5.2.1.4.6 CSI-PAI reporting << Unchanged parts are omitted >> - to report CSI-PAI for the second Reporting Setting, the UE is expected to be configured in the second Report Setting with codebookType set to ‘etypeII-r16’ and it shall: - determine predicted PMI (see Clause 5.2.2.2.10 or Clause 5.2.2.2.11) for each of the subband(s) configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband for the S-th time instance configured by timeinstanceforCSImonitoring-r19 in the second Reporting Setting (S≤N4 , with N4>1 provided in the first Reporting Setting), based on the CSI prediction performed by the UE using the channel measurement corresponding to the first Reporting Setting, - determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, which is corresponding to the S-th time instance of the report of the first Reporting Setting, based on the CSI-RS resource(s) whose transmission occasion is closest to the S-th time instance configured by timeinstanceforCSImonitoring-r19 in the second Reporting Setting in the resource set for channel measurement for the report corresponding to the second Reporting Setting. << Unchanged parts are omitted >>. |
| --- | --- |
| Vivo | - For CSI-PAI report, each monitoring RS occasion is linked to an inference occasion. - The measurement result of nth (n = 1,..,N) latest transmission occasion of the CSI-RS resource for monitoring is linked with the monitored time instance for prediction, where the monitored time instance has the minimal slot offset to the nth transmission occasion of the CSI-RS resource for monitoring. - Wherein, the corresponding inference report, and the transmission occasions of the CSI-RS resource for monitoring, are no later than the CSI reference resource corresponding to the CSI report for monitoring - Predefines a threshold X =64, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance. - Adopt the follow TP Reason for change: The first version for TS38.214 [1] lacks necessary technical component of how to link each monitoring occasion with interference occasion. In AI based beam management, we have agreed to use a dedicated CSI report configuration (including report configuration and RS configuration) for monitoring. Further, each monitoring RS occasion is linked to an inference occasion to calculated the metric. A same approach shall be supported for AI based CSI prediction. Specifically, from mechanism perspective, the monitoring of CSI prediction and BM Case 2 are almost same except metric definition and calculation. Hence we have the following TP: A dedicated CSI report configuration is configured for monitoring which is associated to the inference configuration. Each monitoring RS occasion is linked to an inference occasion. Summary of change: In TS38.214 section 5.2.1.4.6, add the temporal relationship between monitoring resources and inference reports and the correlation between inference results and monitoring resources. Consequences if not approved: It may lead to excessive misalignment of monitoring resources and prediction results in the time domain. TS38.214-5.2.1.4.6 CSI-PAI reporting When the UE is configured with a first CSI Reporting Setting for reporting CSI prediction, based on a CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19] and a second CSI Reporting Setting for reporting CSI-PAI, based on a CSI-ReportConfig with reportQuantity-r19 set to 'csi-pai-r19', and the second Reporting Setting is linked to the first Reporting Setting by inferenceReportConfigId-r19, the reporting of CSI-PAI corresponding to the second CSI Reporting Setting shall consider the following: - for the transmission occasion of the second CSI Reporting Setting, and the latest report of the first CSI Reporting Setting considered for linking, are no later than the CSI reference resource corresponding to the CSI report of the second CSI Reporting setting. - when aperiodic Reporting Setting is configured for both first and second Reporting Settings: { Unchanged parts are omitted } - to report CSI-PAI for the second Reporting Setting, the UE shall: { Unchanged parts are omitted } - When timeinstanceformonitoring-r19 configured in the second CSI Reporting Setting indicates the timeinstanceformonitoring-r19-th time instance among N4 time instance(s), and the linking is determined if the slot corresponding to the time instance indicated by timeinstanceformonitoring-r19 of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting. |
| Huawei | Figure 9 Ambiguity on the UE behavior due to inference DCI missing Proposal 13: For the monitoring CSI report with UCI padding, when there is no linked inference CSI report with valid ‘v’ and predicted PMI values (e.g., due to DCI missing), ‘v’ is assumed as 0 for the UCI padding. Proposal 14: For CSI-PAI report, UE expects to be configured with the same typeII-RI-Restriction and subbandsize with that of the inference report. Proposal 15: The codebookConfig in the monitoring CSI-reportConfig is applied for the PMI of ground-truth CSI and the PMI of non-predicted CSI for the calculation of SGCS1 and SGCS2, respectively. Proposal 16: For the CSI report configuration of monitoring Type 3, measurement result of a transmission occasion of the CSI-RS resources for monitoring is linked with the corresponding f-th doppler domain unit for prediction, where the linkage rule can consider either of the following two options: - Option 1: the f-th doppler domain unit has the minimal time offset to the transmission occasion of the CSI-RS resources for monitoring. - Option 2: the f-th doppler domain unit is overlapped with the transmission occasion of the CSI-RS resources for monitoring. - Wherein, the corresponding inference report, and the transmission occasions of the CSI-RS resources for monitoring, are no later than the CSI reference resource corresponding to the CSI report for monitoring. Table 3 Text proposal for additional active resource counting for monitoring #### 5.2.1.6 CSI processing criteria …… In any slot, the UE is not expected to have more active CSI-RS ports or active CSI-RS resources in active BWPs than reported as capability. NZP CSI-RS resource is active in a duration of time defined as follows. For aperiodic CSI-RS, (excluding the case in which the corresponding aperiodic CSI Reporting Setting is configured with the higher layer parameter [RRC_name-r19] and is linked to another aperiodic CSI Reporting Setting with reportQuantity-r19 set to 'csi-pai-r19'), starting from the end of the PDCCH containing the request and ending at the end of the scheduled PUSCH containing the report associated with this aperiodic CSI-RS. When the PDCCH candidates are associated with a search space set configured with searchSpaceLinkingId, for the purpose of determining the NZP CSI-RS resource active duration, the PDCCH candidate that ends later in time among the two linked PDCCH candidates is used. For semi-persistent CSI-RS, starting from the end of when the activation command is applied, and ending at the end of when the deactivation command is applied. For periodic CSI-RS, starting when the periodic CSI-RS is configured by higher layer signalling, and ending when the periodic CSI-RS configuration is released. For aperiodic CSI-RS, when the corresponding aperiodic CSI Reporting Setting is configured with the higher layer parameter [RRC_name-r19] and is linked to another aperiodic CSI Reporting Setting with reportQuantity-r19 set to 'csi-pai-r19', NZP CSI-RS resource is active in a duration starting from the end of the PDCCH containing the request for PUSCH containing the report of the corresponding CSI Reporting Setting and ending at the end of the scheduled PUSCH containing the report associated with the report of the linked CSI Reporting Setting. For the corresponding aperiodic CSI Reporting Setting, two aperiodic CSI-RS resources are counted as active regardless of the number of configured aperiodic CSI-RS resources.. Proposal 17: Regarding the active resource counting for monitoring an inference CSI with A-CSI-RS inference measurement resource, the additional active resource should be counted per inference CSI report rather than per A-CSI-RS. - The value of the additional active resource should be 2 (for storing non-predicted CSI and predicted PMI). - Consider the text proposal in Table 3 for the PU occupation and active resource counting for monitoring in 38.214. Proposal 18: Regarding the active resource counting for monitoring, if there are multiple monitoring CSI reports associated to the inference CSI report (for monitoring different predicted time instances), the additional active resource should last till the last one of the multiple monitoring CSI reports. |
| Xiaomi | Proposal 3: Suggest Rel-16 eType II codebook with codebook parameter paramCombination-r16 setting to 6 to calculate the precoder wk corresponding to ground-truth channel and the precoder wk correspond to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance. Proposal 4: For aperiodic CSI-PAI report, the monitoring information calculation delay is equal to δd+Z2, where δd and Z2 respectively denote the time delay from DCI to monitoring measurement resource and the delay of calculating non-predicted CSI. Proposal 5: For semi-persistent CSI-PAI report, nCSI_ref is the smallest value greater than or equal to 4⋅2µDL although two measurement resources are configured for calculating monitoring information. Proposal 7: Linkage between monitoring occasion and predicted CSI in inference occasion is not needed since f-th doppler domain unit includes the transmission occasion of the CSI-RS resource for monitoring. Proposal 8: For the inference occasion if it is associated with a monitoring report occasion, active resource and ports are counted twice for latest CSI-RS resource of K aperiodic CSI-RS resources or latest CSI-RS transmission for periodic/semi-persistent CSI-RS resource, which is no later than CSI reference resource associated with inference report. |
| CATT | Reason for change: The following agreement on PMI assumption of SGCS calculation was achieved [1]. Agreement@RAN1#122 For PMI assumption on SGCS calculation of wk and wk, support Rel-16 eType II CSI.. According to the agreement, Rel-16 eType II codebook is used for calculating both the precoder wk represented by PMI based on the channel measurement corresponding to the monitoring report and the precoder wk represented by PMI based on the latest CSI-RS resource transmission occasion, no later than CSI reference resource, corresponding to the inference report. However, the codebook for calculating wkwas not clearly captured by the specification. Summary of change: We propose to add the corresponding description of the codebook for calculating wk in TS 38.214. Consequence if not approved: How to select the codebook for calculating wk is not clearly defined in TS 38.214, which is not aligned with the agreement. Proposal 2: For CSI-PAI reporting, adopt the following TP: ---------------------------------------------------------Start of TP for TS38.214--------------------------------------------- 5.2.1.4.6 CSI-PAI reporting <Unrelated part omitted> - to report CSI-PAI for the second Reporting Setting, the UE is expected to be configured in the second Report Setting with codebookType set to ‘etypeII-r16’ and it shall: <Unrelated part omitted> - for k ∈1,2,…,v, v is the RI reported for the CSI report corresponding to the first Reporting Setting, calculate SGCS value(s) as, SGCSk1=n=1NwknHwknwkn.wkn2N, SGCSk2=n=1NwknHwknwkn.wkn2N, where wkn is the predicted precoder represented by predicted PMI for k-th layer, n-th subband and for S-th time instance corresponding to the report of the first Reporting Setting, wkn is the precoder represented by non-predicted PMI for k-th layer, n-th subband based on the channel measurement corresponding to the second Reporting Setting, and wk wkn is the precoder represented by PMI for k-th layer and n-th subband, n∈1,2,…,N, corresponds to the subbands configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, based on the latest CSI-RS resource transmission occasion, no later than CSI reference resource, corresponding to the report of the first Reporting Setting. Both wkn and wkn are calculated based on the codebookConfig within the second Reporting Setting. The UE is expected to be configured with same csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband configurations for the first CSI Reporting Setting and the second CSI Reporting Setting. <Unrelated part omitted> ----------------------------------------------------------End of TP for TS38.214----------------------------------------------- Proposal 3: For CSI prediction using UE-side model, to calculate the inference report using Doppler codebook, for occupancy duration of CPU and APU and active resource/port counting, use the same rule whether or not the inference report is associated with a monitoring report. |
| OPPO | Proposal 13: For PMI assumption on SGCS calculation of wk and wk, support parameter combination configured by NW through other higher layer parameter instead of paramCombination-r16, e.g., paramCombination-r16-Csi-pai, in CSIReportConfig for monitoring. |
| NEC | The UE is expected to receive a CSI-RS resource configuration for monitoring which has a slot offset, smaller than [FFS] slots, to the f-th doppler domain unit. Adopt the following TP to TS 38.214 V19.1.0. - Reason for change: No slot offset restriction is defined between the slot of monitoring CSI-RS and slot of CSI prediction. - Summary of change: Add UE's expectation about the slot offset restriction between the slot of monitoring CSI-RS and slot of CSI prediction. - Consequences if not approved: CSI-PAI will be not reliable because slot offset between the slot of monitoring CSI-RS and slot of CSI prediction is too large. ##### 5.2.1.4.6 CSI-PAI reporting ##### <Unchanged part omitted> When the UE is configured with a first CSI Reporting Setting for reporting CSI prediction, based on a CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19] and a second CSI Reporting Setting for reporting CSI-PAI, based on a CSI-ReportConfig with reportQuantity-r19 set to 'csi-pai-r19', and the second Reporting Setting is linked to the first Reporting Setting by inferenceReportConfigId-r19, the reporting of CSI-PAI corresponding to the second CSI Reporting Setting shall consider the following: - The UE is expected that the slot corresponding to the CSI-RS resource(s) in the resource set for channel measurement for the report corresponding to the second Reporting Setting has a slot offset, smaller than [FFS] slots, to the S-th time instance configured by timeinstanceforCSImonitoring-r19 in the second Reporting Setting of the report of the first CSI reporting Setting. <Unchanged part omitted>. |
| Nokia | Proposal 6: For AI/ML CSI prediction, it is unnecessary to add any specification clarification to extend the CSI-PAI report timeline. Proposal 7: For AI/ML CSI prediction, it is unnecessary to add any specification clarification on resource /port counting for a monitoring resource. Proposal 8: For AI/ML CSI prediction, for the combination of an SP-CSI-inference report and an SP-CSI-monitoring report, it is up to the gNB to ensure time alignment between inference prediction time(s) and monitoring CSI-RSs. |
| LG Electronics | Proposal #6: It is up to gNB implementation that monitoring RS is overlapped with f-th doppler domain unit. Proposal #7: Define new minimum required timeline (e.g., Z’’) where Z’’ is CSI processing time after receiving the latest CSI-RS transmission occasion not later than the CSI reference resource of inference report until corresponding monitoring report. Proposal #8: For CSI-PAI report, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted twice. Proposal #9: Adopt following TP for TS38.214 Section 5.2.1.6. Reason for change; In current specification, active resource and port for CSI-PAI report has not been specified, which means that it is counted as 1 since only a single monitoring occasion is supported for CSI-PAI report. However, for the calculation of CSI-PAI, it is required measurement from monitoring RS, measurement from inference RS (i.e, the latest CSI-RS transmission occasion not later than the CSI reference resource of inference report) and predicted CSI. For Predicted CSI, it can be obtained from corresponding inference report, since the active resource and ports counting time of inference report is extended to CSI-PAI report. Therefore, active resource and ports for CSI-PAI report should take measurements from monitoring RS and inference RS into account.. Summary of change; Add relevant descriptions to clarify the CSI-RS resource and the CSI-RS ports within the CSI-RS resource configured for CSI-PAI report are counted twice.. Consequences if not approved; Incomplete UE behaviour on active resources and ports counting for CSI-PAI report.. TS38.214 ##### 5.2.1.6 CSI processing criteria ##### << Unchanged parts are omitted >> For a periodic or semi-persistent CSI-RS resource in a CSI-RS resource set for channel measurement linked to a CSI-ReportConfig configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18', the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted KP times, where the value of KP∈{1,2,4} is indicated by UE capability. For aperiodic CSI-RS, when the corresponding aperiodic CSI Reporting Setting is configured with the higher layer parameter [RRC_name-r19] and is linked to another aperiodic CSI Reporting Setting with reportQuantity-r19 set to 'csi-pai-r19', NZP CSI-RS resource is active in a duration starting from the end of the PDCCH containing the request for the report of the corresponding CSI Reporting Setting and ending at the end of the scheduled PUSCH containing the report associated with the report of the linked CSI Reporting Setting. For periodic or semi-persistent or aperiodic CSI-RS resource in a CSI-RS resource set for channel measurement linked to a CSI-ReportConfig configured with the higher layer parameter reportQuantity-r19 set to 'csi-pai-r19', the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted twice. For a CSI-RS Resource Set for channel measurement with K CSI-RS resources linked to a CSI-ReportConfig configured with the higher layer parameter codebookType set to 'typeI-SinglePanel-r19', 'typeI-MultiPanel-r19', 'eTypeII-r19' or 'typeII-FePortSelection-r19', the K CSI-RS resources are counted as one for both capability 1 and 2 reported by UE capability indication. For an aperiodic CSI-RS Resource Set for channel measurement configured with KDOPP resource groups with K CSI-RS resources each, linked to a CSI-ReportConfig configured with the higher layer parameter codebookType set to 'typeII-Doppler-r19', the KDOPP⋅K CSI-RS resources are counted as KDOPP for both capability 1 and 2 reported by UE capability indication. For a periodic or semi-persistent CSI-RS Resource Set for channel measurement with K CSI-RS resources and P ports across the K resources, linked to a CSI-ReportConfig configured with the higher layer parameter codebookType set to 'typeII-Doppler-r19', the K CSI-RS resources are counted as Kp and the P CSI-RS ports are counted as KP⋅P, where the value of KP∈{1,2,4} is indicated by UE capability, for both capability 1 and 2. For a CSI-RS Resource Set for channel measurement with K CSI-RS resources linked to a CSI-ReportConfig configured with codebookType set to 'typeI-SinglePanel-r19' and with a list of sub-configurations, if M is the number of sub-configurations that refer to any of the K CSI-RS resources, the K CSI-RS resources are counted as M and the CSI-RS ports aggregated across the K resources are counted as maxs=1MPs, P, where P is the total number of CSI-RS ports across the K resources and Ps is the number of CSI-RS ports in the s-th sub-configuration derived from the corresponding antenna port subset indicator, portSubsetIndicator-r19. << Unchanged parts are omitted >>. |
| Google | Proposal 3-1: Clarify the CMRs for CSI-PAI report and its linked CSI report should be configured with the same RBs, TCI state, number of ports, frequency domain density and power offset, and endorse the following TP for 38.214. - 5.2.1.4.2 Report quantity configurations <omitted text> If the UE is configured with a CSI-ReportConfig with reportQuantity-r19 set to 'csi-pai-r19', - the UE shall be configured with inferenceReportConfigId-r19 to link another CSI-ReportConfig configured with a higher layer parameter [RRC_name-r19], - when semi-persistent Reporting Setting is configured for the CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19], the UE is not expected to be configured with a periodic Reporting Setting for the CSI-ReportConfig with reportQuantity-r19 set to 'csi-pai-r19'. - when aperiodic Reporting Setting is configured for the CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19], the UE is not expected to be configured with a periodic or semi-persistent Reporting Setting for the CSI-ReportConfig with reportQuantity-r19 set to reportQuantity-r19 set to 'csi-pai-r19'. - the UE shall expect the CSI-RS for channel measurement in the CSI-ReportConfig with reportQuantity-r19 set to 'csi-pai-r19' and the CSI-RS for channel measurement in the linked CSI-ReportConfig are configured with the same density, nrofPorts, starting RB, number of RBs, cdm-type, qcl-info, powerControlOffset and powerControlOffsetSS.. Proposal 3-2: Clarify the transmission occasion of the CMR for CSI-PAI report as the one within 2 consecutive slots as the predicted CSI, and endorse the following TP. - 5.2.1.4.6 CSI-PAI Reporting <omitted text> - to report CSI-PAI for the second Reporting Setting, the UE is expected to be configured in the second Report Setting with codebookType set to 'etypeII-r16' and it shall: - determine predicted PMI (see Clause 5.2.2.2.10 or Clause 5.2.2.2.11) for each of the subband(s) configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband for the S-th time instance configured by timeinstanceforCSImonitoring-r19 in the second Reporting Setting (S≤N4 , with N4>1 provided in the first Reporting Setting), based on the CSI prediction performed by the UE using the channel measurement corresponding to the first Reporting Setting, - determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, which is corresponding to the S-th time instance of the report of the first Reporting Setting, based on the transmission occasion of the CSI-RS resource(s) in the resource set for channel measurement for the report corresponding to the second Reporting Setting, where the transmission occasion is within 2 slots based on the slot for the S-th time instance.. Proposal 3-3: Endorse the following TP for 38.214 to clarify the UE does not receive the CSI-RS for CSI-PAI report during cell DTX inactive period. 5.1.6.1 CSI-RS reception procedure <omitted text> During non-active periods of cell DTX if cell DTX is activated for a serving cell, the UE is not expected to receive the periodic CSI-RS and semi-persistent CSI-RS on the serving cell configured in CSI report configuration in CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI' or 'cjtc-P' or ‘rs-pai’. If the cell DTX is activated for a serving cell [10, TS 38.321], the most recent CSI measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on the serving cell occurs in active periods of cell DTX for CSI report configured by CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI' or 'cjtc-P' or ‘csi-pai’.. |
| Apple | Proposal 3-1: For CPU processing timeline for CSI-PAI, to accommodate the computation required for the two SGCS for performance monitoring, additional times of 5x Z2/Z2’ is needed on top of inference computation. 5.4 UE CSI computation time ……. - (Z2+14(K-1)m+t,Z2'+t), with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4=1, codebookType is set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is aperiodic with K CSI-RS resources, and t is according to UE reported capability as defined in [13, TS 38.306] when higher layer parameter [RRC_name-r19] is configured in CSI-ReportConfig, otherwise t=0, or - (Z2+w+t,Z2'+t), with (Z2,Z2') of table 5.4-2, where w=56⋅(KP-1) or 56⋅KP symbols, according to the reported UE capability, where the value of KP ∈{1,2,4} is indicated by UE capability, if the CSI report is configured with N4=1, codebookType is set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is periodic or semi-persistent with a single CSI-RS resource, and t is according to UE reported capability as defined in [13, TS 38.306] when higher layer parameter [RRC_name-r19] is configured in CSI-ReportConfig, otherwise t=0, or - (Z2+14(K-1)m+t,Z2'+t) or (Z2+14(K-1)m+Z2'+t,2Z2'+t), according to UE reported capability, with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4>1, codebookType is set to 'typeII-Doppler-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is aperiodic with K CSI-RS resources, and t is according to UE reported capability as defined in [13, TS 38.306] when higher layer parameter [RRC_name-r19] is configured in CSI-ReportConfig, otherwise t=0, or - (Z2+w+t,Z2'+t) or (Z2+w+Z2'+t,2Z2'+t), according to UE reported capability, with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4>1, codebookType is set to 'typeII-Doppler-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is periodic or semi-persistent with a single CSI-RS resource, and t is according to UE reported capability as defined in [13, TS 38.306] when higher layer parameter [RRC_name-r19] is configured in CSI-ReportConfig, otherwise t=0, or - (Z2+14(K-1)m+t+5Z2,Z2'+t+ 5Z2'), with (Z2,Z2') of table 5.4-2, if reportQuantity-r19 is set to 'csi-pai-r19' in the first CSI report configuration, with a linked CSI reporting with codebookType is set to 'typeII-Doppler-r18' with N4=1, and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is aperiodic with K CSI-RS resources, and t is according to UE reported capability as defined in [13, TS 38.306] when higher layer parameter [RRC_name-r19] is configured in CSI-ReportConfig, otherwise t=0, or - (Z2+w+t+5Z2,Z2'+t+5Z2'), with (Z2,Z2') of table 5.4-2, where w=56⋅(KP-1) or 56⋅KP symbols, according to the reported UE capability, where the value of KP ∈{1,2,4} is indicated by UE capability, if reportQuantity-r19 is set to 'csi-pai-r19' in the first CSI report configuration, with a linked CSI reporting with codebookType is set to 'typeII-Doppler-r18' with N4=1, and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is periodic or semi-persistent with a single CSI-RS resource, and t is according to UE reported capability as defined in [13, TS 38.306] when higher layer parameter [RRC_name-r19] is configured in CSI-ReportConfig, otherwise t=0, or - (Z2+14(K-1)m+t+5Z2,Z2'+t+5Z2') or (Z2+14(K-1)m+Z2'+t+5Z2,2Z2'+t+5Z2'), according to UE reported capability, with (Z2,Z2') of table 5.4-2, if reportQuantity-r19 is set to 'csi-pai-r19' in the first CSI report configuration, with a linked CSI reporting with codebookType is set to 'typeII-Doppler-r18' with N4>1, and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is aperiodic with K CSI-RS resources, and t is according to UE reported capability as defined in [13, TS 38.306] when higher layer parameter [RRC_name-r19] is configured in CSI-ReportConfig, otherwise t=0, or - (Z2+w+t +5Z2,Z2'+t +5Z2') or (Z2+w+Z2'+t+5Z2,2Z2'+t +5Z2'), according to UE reported capability, with (Z2,Z2') of table 5.4-2, if reportQuantity-r19 is set to 'csi-pai-r19' in the first CSI report configuration, with a linked CSI reporting with codebookType is set to 'typeII-Doppler-r18' with N4>1 and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is periodic or semi-persistent with a single CSI-RS resource, and t is according to UE reported capability as defined in [13, TS 38.306] when higher layer parameter [RRC_name-r19] is configured in CSI-ReportConfig, otherwise t=0, or Proposal 3-2: For activate resource counting for CSI-PAI, both the CSI-RS resource and the CSI-RS ports within the CSI-RS resource for CSI-PAI report configuration, and the linked CSI-RS resource/CSI-RS ports within the resource for inference is counted as active resource. |
| Quectel | Summary of change: We propose to add the description of the corresponding measurement reference for the calculating wkn in TS 38.214. Consequence if not approved: The selection of a measurement reference signal for wkn is not clearly defined in TS 38.214, which is not aligned for implementation at UEs and gNBs. - For CSI-PAI reporting, adopt the following TP: |
| ASUSTeK | Proposal 2: An inference report for determining the S-th time instance corresponds to the latest inference report no later than the CSI reference resource of a monitoring report, and apply following TP: Text proposal: ##### 5.2.1.4.6 CSI-PAI reporting When the UE is configured with a first CSI Reporting Setting for reporting CSI prediction, based on a CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19] and a second CSI Reporting Setting for reporting CSI-PAI, based on a CSI-ReportConfig with reportQuantity-r19 set to 'csi-pai-r19', and the second Reporting Setting is linked to the first Reporting Setting by inferenceReportConfigId-r19, the reporting of CSI-PAI corresponding to the second CSI Reporting Setting shall consider the following: … - to report CSI-PAI for the second Reporting Setting, the UE is expected to be configured in the second Report Setting with codebookType set to 'etypeII-r16' and it shall: - determine predicted PMI (see Clause 5.2.2.2.10 or Clause 5.2.2.2.11) for each of the subband(s) configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband for the S-th time instance of the latest report of the first Reporting Setting no later than the CSI reference resource of the CSI report, corresponding to the second Reporting Setting, which value of the S is configured by timeinstanceforCSImonitoring-r19 in the second Reporting Setting (S≤N4 , with N4>1 provided in the first Reporting Setting), based on the CSI prediction performed by the UE using the channel measurement corresponding to the first Reporting Setting, - determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, which is corresponding to the S-th time instance of the latest report of the first Reporting Setting no later than the CSI reference resource of the CSI report, corresponding to the second Reporting Setting, based on the CSI-RS resource(s) in the resource set for channel measurement for the report corresponding to the second Reporting Setting. - determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, based on the latest CSI-RS resource transmission occasion, no later than the CSI reference resource of the CSI report, corresponding to the first Reporting Setting.. |

- Issue 2.5-1. Linkage between monitoring occasion and predicted CSI in inference occasion

For this issue, ZTE, Huawei, vivo, NEC, Google, Quectel and ASUSTek still think linkage between monitoring occasion and predicted CSI in inference occasion is needed. Meanwhile, Nokia, Xiaomi, LG think this issue could be resolved by NW implementation. From the proposal from companies, the solution can be summarized as below.

- Option 1: the f-th doppler domain unit has the minimal time offset to the transmission occasion of the CSI-RS resources for monitoring.

  - Wherein, the corresponding inference report, and the transmission occasions of the CSI-RS resources for monitoring, are no later than the CSI reference resource corresponding to the CSI report for monitoring.

  - Fujitsu, CATT, Xiaomi(?)

- Option 2: the f-th doppler domain unit is overlapped contain with the transmission occasion of the CSI-RS resources for monitoring.

  - Wherein, the corresponding inference report, and the transmission occasions of the CSI-RS resources for monitoring, are no later than the CSI reference resource corresponding to the CSI report for monitoring.

  - Huawei, Qualcomm, ZTE,

- Option 3: it is up to NW implementation and no specification impact is expected.

  - Nokia, LGE

- Option 4:  The UE is expected to receive a CSI-RS resource configuration for monitoring which has a slot offset, smaller than [FFS] slots, to the f-th doppler domain unit.

  - NEC

- Question 2.5-1. Which option do you prefer?

Please provide your preference on issue 2.5-1.

| Company | Views |
| --- | --- |
| Huawei, HiSilicon | Firstly, there should be a clear definition of time domain linkage, otherwise, it may cause trouble for UE to understand the monitoring report trigger is no later than which associated inference report under the following agreement. Second, between Option1 and Option2, Option 2 may be more appropriate for CSI prediction. |
| Qualcomm | In the last meeting, we discussed this issue and option 2 should be the common understanding. Better to clarify in the spec. |
| Fujitsu | Prefer with Option 1 |
| Google | Both option 1 and option 2 are ok to us. But the formulation for option 2, we should it should be defined as the f-th Doppler domain unit and the transmission occasion are in the same slot. The Doppler domain unit is defined per unit, the transmission occasion is defined per symbol. We do not know how option 3 can work. |
| CATT | We are OK to introduce such linkage. Option 1 is preferred. |
| NEC | We prefer a combination solution of option 1 and option 2. For option 1, we agree to define a time offset between f-th doppler domain unit and the transmission occasion of the CSI-RS resources, but it may not be proper to say "a minimal time offset" because we believe NW will only provide one transmission occasion of the CSI-RS resources for monitoring. For option 2, on the other hand, we also believe it's not proper to force NW to always transmit the CSI-RS resources which is overlapped with the f-th doppler domain unit. Therefore, we prefer a combined solution, which is: Option 4: The UE is expected to receive a CSI-RS resource configuration for monitoring which has a slot offset, smaller than [FFS] slots, to the f-th doppler domain unit. |
| ZTE | Support Option 2. For option 1, it reads like the f-th doppler unit is determined by the transmission occasion of the monitoring resource. |
| Xiaomi | For Option2, if the f-th Doppler domain unit includes multiple slots, how to understand that the f-th doppler domain unit is overlapped with the transmission occasion of the CSI-RS resources, as the transmission occasion of the CSI-RS resources is sent in one slot. Mod: Your position is option 1?? Right? |
| Qualcomm2 | Option 1 is lack of justification, why NW would fail to configure the resource for monitoring in the duration of the f-th prediction TD unit? For option 2, maybe “contain” is more accurate than “overlap” because a time domain unit is a time span of multiple slots. |

- Issue 2.5-2. UE behavior due to inference DCI missing

As shown above, Huawei proposes the solution to resolve the ambiguity when inference triggering DCI is missed. In such case, UE cannot properly calculate the performance metric and this case may happen in case of AP inference report and AP monitoring report. So, the moderator thinks that this issue needs to be resolved. For the potential solutions, there can be two ways.

- Option 1: all-one padding is applied for whole UCI sequence for monitoring report

    - Huawei/HiSi

- Option 2: UE may ignore scheduling DCI for monitoring report.

    - Fujitsu(?), Google, NEC, LGE

- Option 3: Up to UE implementation, e.g., drop

    - QC, OPPO, Google, ZTE, Xiaomi, LGE

For option 1, the UE behavior can be simple, but UE needs to report redundant information to gNB. For option 2, it may increase gNB complexity (e.g., # of blind detection). So, I would like to hear more view on this issue. Based on the input, I will provide proposal.

- Question 2.5-2. Which option do you prefer?

Please provide your input/suggestion on Question 2.5-2. If you have another solution, please provide it.

| Company | Views |
| --- | --- |
| Huawei, HiSilicon | We prefer Option 1 for simplicity. Option 2 may cause problem if the CSI-PAI is multiplexed with UL-SCH. |
| Qualcomm | How to define the event “DCI is missing”? No need to further discuss this issue, UE does not know there is a A-CSI triggering or not, so the monitoring report is to be handled by UE implementation. Since NW knows UE not transmitting the A-CSI inference, the NW can simply ignore the monitoring report. |
| Fujitsu | Why not just drop the monitoring report in this case? |
| Google | We think this should be handled in a general way, if UE cannot identify a linked report or transmission occasion of CSI-RS for monitoring, UE should drop the report. |
| NEC | In our view, UE can drop or ignore the monitoring report if no linked inference report. |
| OPPO | No need to discuss this issue. It can be resolved by dropping the monitoring report. |
| ZTE | Support dropping monitoring report. In such cases, the UE does not understand the associated inference report configuration for the monitoring report and nothing essential is reported because UE does not calculate a correct SGCS. |
| LGE | If DCI for inference report is missing, inference report is not triggered. Then UE cannot calculate SGCS for monitoring without inference, so monitoring report can’t be done. UE should drop the monitoring report. |
| Xiaomi | We think UE could drop monitoring report, since UE could not calculate SGCS if UE does not receive DCI trigger for inference. |
| Mod | I think slight majority company think that there is no need for spec changes regarding DCI missing case. |

- Issue 2.5-3. Alignment of parameters between monitoring report and inference report

Subband configuration

In the last meeting, following is agreed.

Agreement For CSI-PAI report, UE is expected to be configured with the same subband configuration with that of inference report.

However, current spec is not properly captured above agreements as subband configuration can include subband size, as Huawei pointed out. Therefore, I propose following TPs.

- Proposal 2.5-3A. Adopt the following TP for TS 38.214.

| Reason for change | Based on the agreement, for CSI-PAI report, UE is expected to be configured with the same subband configuration with that of inference report, and subband configuration can include RRC parameters of csi-ReportingBand, subbandSize and numberOfPMI-SubbandsPerCQI-Subband. However, in current specification, subbandSize is missing. |
| --- | --- |
| Summary of change | RRC parameter subbandSize is added. |
| Consequences if not approved | Incorrect capture of the agreement. |
| TS38.214 5.2.1.4.6 CSI-PAI reporting << Unchanged parts are omitted >> - to report CSI-PAI for the second Reporting Setting, the UE is expected to be configured in the second Report Setting with codebookType set to 'etypeII-r16' and it shall: - determine predicted PMI (see Clause 5.2.2.2.10 or Clause 5.2.2.2.11) for each of the subband(s) configured by csi-ReportingBand, subbandSize and numberOfPMI-SubbandsPerCQI-Subband for the S-th time instance configured by timeinstanceforCSImonitoring-r19 in the second Reporting Setting (S≤N4 , with N4>1 provided in the first Reporting Setting), based on the CSI prediction performed by the UE using the channel measurement corresponding to the first Reporting Setting, - determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand, subbandSize and numberOfPMI-SubbandsPerCQI-Subband, which is corresponding to the S-th time instance of the report of the first Reporting Setting, based on the CSI-RS resource(s) in the resource set for channel measurement for the report corresponding to the second Reporting Setting. - determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand, subbandSize and numberOfPMI-SubbandsPerCQI-Subband, based on the latest CSI-RS resource transmission occasion, no later than the CSI reference resource of the CSI report, corresponding to the first Reporting Setting. - for k ∈1,2,…,v, v is the RI reported for the CSI report corresponding to the first Reporting Setting, calculate SGCS value(s) as, SGCSk1=n=1NwknHwknwkn.wkn2N, SGCSk2=n=1NwknHwknwkn.wkn2N, where wkn is the predicted precoder represented by predicted PMI for k-th layer, n-th subband and for S-th time instance corresponding to the report of the first Reporting Setting, wkn is the precoder represented by non-predicted PMI for k-th layer, n-th subband based on the channel measurement corresponding to the second Reporting Setting, and wk is the precoder represented by PMI for k-th layer and n-th subband, n∈1,2,…,N, corresponds to the subbands configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, based on the latest CSI-RS resource transmission occasion, no later than CSI reference resource, corresponding to the report of the first Reporting Setting. The UE is expected to be configured with same csi-ReportingBand, subbandSize and numberOfPMI-SubbandsPerCQI-Subband configurations for the first CSI Reporting Setting and the second CSI Reporting Setting. << Unchanged parts are omitted >> |  |

Please provide your input/suggestion on Proposal 2.5-3A

| Company | Views |
| --- | --- |
| Huawei, HiSilicon | OK with subband size. Also should add typeII-RI-Restriction to be aligned between inference CSI report and monitoring CSI report. Otherwise, the current spec (38.212) implies that the RI-Restriction for UCI padding is obtained from the monitoring CSI report, rather than inference CSI report, which conflicts with the agreement that the number of layers for SGCS follows the reported RI of inference CSI report. |
| Qualcomm | Isn’t it subband size is part of the csi-ReportingBand configuration? |
| Fujitsu | Ok with the proposal |
| Vivo | OK |
| Google | Several parameters should be configured with the same value, e.g., TCI, number of ports, power offset, and some configurations for the two codebooks and so on. We think we can discuss such issue in general. |
| CATT | OK |
| Xiaomi | Support |

Codebook config for wk and wk

In the last meeting, regarding codebook assumption for calculation of monitoring metric, following agreement were made.

Agreement: For PMI assumption on SGCS calculation of wk and wk ,support Rel-16 eType II CSI.

Regarding this, CATT and HW still think there is some ambiguity on calculation of monitoring metric, so they want to clarify by explicitly adding codebook config in the monitoring report configuration is applied for SGCS calculation. Actually, this issue was discussed in the last meeting, but some companies think further clarification is not needed. So let’s hear more views on this issue.

---------------------------------------------------------Start of TP for TS38.214---------------------------------------------

5.2.1.4.6	CSI-PAI reporting

<Unrelated part omitted>

-	to report CSI-PAI for the second Reporting Setting, the UE is expected to be configured in the second Report Setting with codebookType set to ‘etypeII-r16’ and it shall:

<Unrelated part omitted>

- for k ∈1,2,…,v, v is the RI reported for the CSI report corresponding to the first Reporting Setting, calculate SGCS value(s) as,

SGCSk1=n=1NwknHwknwkn.wkn2N,

SGCSk2=n=1NwknHwknwkn.wkn2N,

where wkn is the predicted precoder represented by predicted PMI for k-th layer, n-th subband and for S-th time instance corresponding to the report of the first Reporting Setting, wkn is the precoder represented by non-predicted PMI for k-th layer, n-th subband based on the channel measurement corresponding to the second Reporting Setting, and wk wkn is the precoder represented by PMI for k-th layer and n-th subband, n∈1,2,…,N, corresponds to the subbands configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, based on the latest CSI-RS resource transmission occasion, no later than CSI reference resource, corresponding to the report of the first Reporting Setting. Both wkn and wkn are calculated based on the codebookConfig within the second Reporting Setting. The UE is expected to be configured with same csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband configurations for the first CSI Reporting Setting and the second CSI Reporting Setting.

<Unrelated part omitted>

----------------------------------------------------------End of TP for TS38.214-----------------------------------------------

- Question 2.5-3B Is above TP essential?

Please provide your input/suggestion on Question 2.5-3B.

| Company | Views |
| --- | --- |
| Huawei, HiSilicon | Yes. The PC between ground-truth CSI and non-predicted PMI should be aligned. Otherwise, the different compression ratio will impact the fair comparison between SGCS1 and SGCS2. |
| Fujitsu | Open for discussion |
| Vivo | OK |
| Google | OK |
| CATT | Support. wk (should be wkn to be consistent with other notation in the spec) should be calculated based on codebookConfig within the second Reporting Setting, i.e., codebook type is Rel-16 eType II codebook. This is because the resource for wk is in the first reporting setting and the codebook type in the first reporting setting is Rel-18 eType II Doppler codebook. However, we have agreed in the #122 that Rel-16 eType II codebook should be used. Agreement: For PMI assumption on SGCS calculation of wk and wk ,support Rel-16 eType II CSI.. |
| OPPO | Support. |
| ZTE | Open to discuss. |

Parameter combination for  wk and wk  and RI restriction

Regarding this issue, Xiaomi proposed to restrict paramCombination-r16 setting to 6, however, this issue was discussed during the offline in the last meeting, and the common understanding is such restriction is not needed, and it is upto gNB how to configure parameter combination to ensure monitoring performance.

Also, Oppo proposed to support parameter combination configured by NW through other higher layer parameter instead of paramCombination-r16, e.g., paramCombination-r16-Csi-pai, in CSIReportConfig for monitoring. For this proposal, I fail to see the motivation of introduce different paramCombination, since monitoring report can be differentiated by report quantity.

For RI restriction, Huawei propose that the same restriction needs to be applied for both inference and monitoring report. This issue was discussed in the last meeting, but majority companies think it is up to NW implementation. So, I think further change may not be needed.

Please provide your input/suggestion on this issue, if any.

| Company | Views |
| --- | --- |

- Issue 2.5-4. Alignment of CMR configuration between monitoring report and inference report.

Google propose to clarify the CMRs for CSI-PAI report and its linked CSI report should be configured with the same RBs, TCI state, number of ports, frequency domain density and power offset. Moderator thinks that alignment of these configurations can be handled by gNB. But, I would like to hear more views.

- 5.2.1.4.2 Report quantity configurations <omitted text> If the UE is configured with a CSI-ReportConfig with reportQuantity-r19 set to 'csi-pai-r19', - the UE shall be configured with inferenceReportConfigId-r19 to link another CSI-ReportConfig configured with a higher layer parameter [RRC_name-r19], - when semi-persistent Reporting Setting is configured for the CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19], the UE is not expected to be configured with a periodic Reporting Setting for the CSI-ReportConfig with reportQuantity-r19 set to 'csi-pai-r19'. - when aperiodic Reporting Setting is configured for the CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19], the UE is not expected to be configured with a periodic or semi-persistent Reporting Setting for the CSI-ReportConfig with reportQuantity-r19 set to reportQuantity-r19 set to 'csi-pai-r19'. - the UE shall expect the CSI-RS for channel measurement in the CSI-ReportConfig with reportQuantity-r19 set to 'csi-pai-r19' and the CSI-RS for channel measurement in the linked CSI-ReportConfig are configured with the same density, nrofPorts, starting RB, number of RBs, cdm-type, qcl-info, powerControlOffset and powerControlOffsetSS.

- Question 2.5-4 Is above TP essential?

Yes: Fujitsu, Google, CATT, Xiaomi

No: QC, OPPO, LGE

Please provide your input/suggestion on Question 2.5-4.

| Company | Views |
| --- | --- |
| Qualcomm | Not needed. |
| Fujitsu | Support the TP |
| Google | Support, this is a typical restriction in most of the CSI related configuration |
| CATT | OK to have |
| OPPO | Not needed. |
| LGE | We don’t understand why that configuration is essential. It needs more discussion for clarify. |
| Xiaomi | We are fine. |

- Issue 2.5-5.  CSI-RS for CSI-PAI report during non-active periods of cell DTX.

Google propose to clarify that the UE does not receive the P/SP CSI-RS for CSI-PAI report during cell DTX inactive period. Actually, this restriction applied for inference report. However, whether it is applied for monitoring report is not clear. Moderator think this is a valid issue, so following is proposed.

- Proposal 2.5-5. Adopt the following TP for TS 38.214.

| Reason for change | During non-active periods of cell DTX, UE is not expected to receive P/SP CSI-RS for CSI report associated with report quantity comprising at least RI which means this can be applied to SP inference report. However, whether it is applied for monitoring report is not clear. |
| --- | --- |
| Summary of change | Report quantity ‘csi-pai-r19’ is added. |
| Consequences if not approved | Inconsistent UE behaviour during non-active periods of cell DTX. |
| TS38.214 5.1.6.1 CSI-RS reception procedure << Unchanged parts are omitted >> During non-active periods of cell DTX if cell DTX is activated for a serving cell, the UE is not expected to receive the periodic CSI-RS and semi-persistent CSI-RS on the serving cell configured in CSI report configuration in CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI' or 'cjtc-P' or ‘csi-pai-r19’. If the cell DTX is activated for a serving cell [10, TS 38.321], the most recent CSI measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on the serving cell occurs in active periods of cell DTX for CSI report configured by CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI' or 'cjtc-P' or ‘csi-pai-r19’. << Unchanged parts are omitted >> |  |

Please provide your input/suggestion on Proposal 2.5-5

| Company | Views |
| --- | --- |
| Huawei, HiSilicon | OK |
| Fujitsu | Support. |
| Google | Support |

- Issue 2.5-6. Timeline

AP CSI-PAI report

This issue seems not attractive for many companies. For this issue, Xiaomi, LG, Apple, Nokia provide their view. Xiaomi proposed to have unified timeline for CSI-PAI report by adding some calculation delay. LG proposed to define new timeline (Z’’) starting from latest inference measurement to CSI-PAI report. Apple proposed to consider additional times of 5x Z2/Z2’ is needed on top of inference computation. And Nokia think it is unnecessary to add any changes in specification since it could be handled by UE implementation. However, in my understanding, without any clarification, (Z,Z’) for CSI-PAI report is (𝑍2,𝑍2′) of table 5.4-2 instead of (Z, Z’) of AP inference report. So, to move forward, please share your preference on options for (Z/Z’) for CSI-PAI report.

- Option 1: in addition to Z/Z’ of inference report, UE may report the value of t’

  - t’ is upto UE capability

  - Google, LGE,

- Option 2: the same Z/Z’ of inference report is applied for AP CSI-PAI report

- Option 3: in addition to (Z2, Z2’) of table 5.4-2, UE may report the value of t’

  - t’ is upto UE capability

  - Google

- Option 4: (Z2, Z2’) of table 5.4-2, no spec change is needed.

  - Huawei/HiSi, Qualcomm,

- Question 2.5-6. Which option do you prefer?

Please provide your input/suggestion on Question 2.5-6.

| Company | Views |
| --- | --- |
| Huawei, HiSilicon | Portion of the calculation is eaten in the inference CSI report such as PMI conversion of non-predicted CSI. For monitoring report, it only includes the codebook generation of ground-truth CSI, as well as two SGCS values, not too much increase on top of legacy. Thus, we think Option 4 is fine. Open to discuss Option 2, but 5x is too much. |
| Qualcomm | Not needed. The monitoring report is equivalent to 2~3 CSI reports using legacy codebooks. The computation cost has been covered by reporting X CPUs, and the memory / buffering cost is covered by extending the active resource counting and restricting the triggering DCI. |
| Google | Support option 1 and option 3 |
| LGE | Monitoring report is linked with inference report and triggered before measurement of inference report. So, we can consider new timeline for CSI-PAI report from latest measurement of inference to CSI-PAI report. But option 1 is OK. CSI-PAI report includes calculation for 2 SGCSs, it needs more time than inference. |
| Xiaomi | Legacy (Z2, Z2’) is appropriate to calculate ground truth CSI. However, if (Z2, Z2’) is still used for AP CSI-PAI report, the time delay is two short as both non-predicted CSI and ground truth need to be calculated. How to define appropriate delay value for AP CSI-PAI report needs to discuss. |

SP CSI-PAI report

Xiaomi proposes that for SP CSI-PAI report, nCSI_ref is the smallest value greater than or equal to 4⋅2µDL although two measurement resources are configured for calculating monitoring information. In my understanding, for CSI-PAI report, single measurement resource is configured, so nCSI_ref is the smallest value greater than or equal to 4⋅2µDL. However, we can clarify this issue, if group wants to have clarification or spec changes.

Please provide your input/suggestion on timeline for SP CSI-PAI report, if any.

| Company | Views |
| --- | --- |
| Huawei, HiSilicon | The issue is not fully clear to us: what is the impact to spec? |
| Qualcomm | We should extend the SP timeline for inference report, not the monitoring. |
| Xiaomi | In current spec, the values of CSI reference resource nCSI_ref are introduced to determine the CSI processing delay according to the number of measurement resource. If a single CSI-RS/SSB resource is configured for channel measurement, nCSI_ref is the smallest value greater than or equal to 4⋅2µDL, such that it corresponds to a valid downlink slot. If multiple CSI-RS/SSB resources are configured for channel measurement, nCSI_ref is the smallest value greater than or equal to 5⋅2μDL, such that it corresponds to a valid downlink slot. For SP CSI-PA report, although two measurement resources are configured to respectively calculate non-predicted CSI and ground-truth CSI, the question is whether nCSI_ref is the smallest value greater than or equal to 5⋅2μDL or 4⋅2µDL. In our view, the delay of non-predicted CSI could be included model inference. From this perspective, it still be regarded as configuring one resources for calculate monitoring information. Thus, nCSI_ref is the smallest value greater than or equal to 4⋅2µDL. |
| Mod | I want to hear more view on this issue. |

- Issue 2.5-7. Active resource and port counting

Regarding active resource and port counting for CSI-PAI report, Huawei, Xiaomi, LG, Nokia provide their view. The proposal from companies are diverging, so I summarize as below.

- Option 1: For the inference occasion if it is associated with a monitoring report occasion, active resource and ports are counted twice for latest CSI-RS resource of K aperiodic CSI-RS resources or latest CSI-RS transmission for periodic/semi-persistent CSI-RS resource, which is no later than CSI reference resource associated with inference report.

  - QC

- Option 2: Additional two active resources should be counted per AP inference CSI report rather than per AP CSI-RS.

  - Huawei/HiSi

- Option 3: For A/SP/P CSI-RS for CSI-PAI report, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted twice.

  - Google, LGE,

- Option 4: No spec change is needed.

  - Vivo, Fujitsu, Nokia

- Question 2.5-7. Which option do you prefer?

Please provide your input on Question 2.5-7?

| Company | Views |
| --- | --- |
| Huawei, HiSilicon | Option 1 and Option 2 are similar sense, i.e., only one CSI-RS resource is counted twice to the active resources/ports. Option 3 and Option 4 will cause over counting of active resources. E.g., as shown in the figure above, assuming K=4, then for Option 3, 2K=8 active resources are counted between inference CSI report and monitoring CSI report; for Option 4, K=4 active resources are counted. However, to store the inference CSI and non-predicted PMI, only 2 additional buffers are needed. |
| Qualcomm | Should we double the resource counting for inference or monitoring? In our understanding, it should be the inference. The resource is used twice, 1) for measurement and prediction, and 2) for non-predicted CSI calculation and buffering. The monitoring resource is only used once, i.e., target CSI measurement and calculation. In general, we think this issue should be discussed. |
| Fujitsu | Option 4. |
| Vivo | Option 4 |
| Google | Support Option 3 |
| LGE | Support option 3. For Option 2, it is counted per report. Then we need to define new UE ehaviour which requires more specification efforts. |
| Mod | Please share your view on this issue. |
| Qualcomm | Option 1 is more accurate, but we should also clarify that AP-CSI-RS is also counted twice if the associated AP report is being monitored. |

- Issue 2.5-8. Others

Please let me know if I miss any essential issue.

| Company | Views |
| --- | --- |

Editorial TPs

ZTE proposes editorial TPs to align RRC parameter of report quantity for inference of CSI prediction as below. In other word, current RAN2 running CR adopts the inference parameter name of ‘csi-InferencePrediction-r19’ for explicit indication of inference of CSI prediction.

- Proposal 2.5. Following TP is endorsed as editorial corrections for TS 38.214 for Clauses 5.2.1.4.2, 5.2.1.4.6, 5.2.1.6 and 5.4.

TS38.214 ##### 5.2.1.4.2 Report quantity configurations << Unchanged parts are omitted >> If the UE is configured with a CSI-ReportConfig with reportQuantity-r19 set to ‘csi-pai-r19’, - the UE shall be configured with inferenceReportConfigId-r19 to link another CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19] reportQuantity-r19 set to ‘csi-InferencePrediction-r19’, - when semi-persistent Reporting Setting is configured for the CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19] reportQuantity-r19 set to ‘csi-InferencePrediction-r19’, the UE is not expected to be configured with a periodic Reporting Setting for the CSI-ReportConfig with reportQuantity-r19 set to ‘csi-pai-r19’. - when aperiodic Reporting Setting is configured for the CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19] reportQuantity-r19 set to ‘csi-InferencePrediction-r19’, the UE is not expected to be configured with a periodic or semi-persistent Reporting Setting for the CSI-ReportConfig with reportQuantity-r19 set to ‘csi-pai-r19’. ##### << Unchanged parts are omitted >> ##### 5.2.1.4.6 CSI-PAI reporting When the UE is configured with a first CSI Reporting Setting for reporting CSI prediction, based on a CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19] reportQuantity-r19 set to ‘csi-InferencePrediction-r19’ and a second CSI Reporting Setting for reporting CSI-PAI, based on a CSI-ReportConfig with reportQuantity-r19 set to ‘csi-pai-r19’, and the second Reporting Setting is linked to the first Reporting Setting by inferenceReportConfigId-r19, the reporting of CSI-PAI corresponding to the second CSI Reporting Setting shall consider the following: << Unchanged parts are omitted >> ##### 5.2.1.6 CSI processing criteria The UE indicates the number of supported simultaneous CSI calculations NCPU with parameter simultaneousCSI-ReportsPerCC or simultaneousCSI-SubReportsPerCC-r18 in a component carrier, and simultaneousCSI-ReportsAllCC or simultaneousCSI-SubReportsAllCC-r18 across all component carriers. If UE is configured with at least one CSI report setting with sub-configuration in a component carrier, UE shall use parameter simultaneousCSI-SubReportsPerCC-r18 in the component carrier; otherwise, UE shall use simultaneousCSI-ReportsPerCC in the component carrier. If UE is configured with at least one CSI reporting setting with sub-configuration in any component carrier, UE shall use simultaneousCSI-SubReportsAllCC-r18; otherwise, UE shall use simultaneousCSI-ReportsAllCC. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports. If L CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU-L unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU-L CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU(n), the UE is not required to update the N-M requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤M≤N is the largest value such that n=0M-1OCPU(n)≤NCPU-L holds. For CSI reports with reportQuantity set to ‘p-cri-r19’, ‘p-cri-RSRP-r19’, ‘p-ssb-index-r19’, or ‘p-ssb-index-RSRP-r19’, or CSI reports configured with the higher layer parameter [RRC_name-r19] reportQuantity-r19 set to ‘csi-InferencePrediction-r19’, OCPU= OCPU,1 is considered. For CSI reports with reportQuantity set to ‘p-cri-r19’, ‘p-cri-RSRP-r19’, ‘p-ssb-index-r19’, or ‘p-ssb-index-RSRP-r19’, and CSI reports configured with the higher layer parameter [RRC_name-r19] reportQuantity-r19 set to ‘csi-InferencePrediction-r19’, the UE may indicate a second value for the number of supported simultaneous CSI calculations NCPU,2 with parameter SecondValuesSimultaneousCSI-ReportsPerCC in a component carrier, and SecondValuesSimultaneousCSI-ReportsAllCC across all component carriers, and if applicable, a third value for the number of supported simultaneous CSI calculations NCPU,3 with parameter ThirdValuesSimultaneousCSI-ReportsPerCC in a component carrier, and ThirdValuesSimultaneousCSI-ReportsAllCC across all component carriers, in addition to NCPU. If a UE supports NCPU,x simultaneous CSI calculations it is said to have NCPU,x CSI processing units for processing CSI reports. If Lx CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU,x-Lx unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU,x-Lx CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU,x(n), the UE is not required to update the N-Mx requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤Mx≤N is the largest value such that n=0Mx-1OCPU,x(n)≤NCPU,x-Lx holds. If only the second value is indicated, x = 2. If both of the second value and the third value are indicated, x = 2, 3 where the CSI reports with reportQuantity set to ‘p-cri-r19’, ‘p-cri-RSRP-r19’, ‘p-ssb-index-r19’, or ‘p-ssb-index-RSRP-r19’ corresponds to either x = 2 or x = 3 subject to UE capability and the CSI reports configured with the higher layer parameter [RRC_name-r19] reportQuantity-r19 set to ‘csi-InferencePrediction-r19’ corresponds to either x = 2 or x = 3 subject to UE capability. For CSI reports with reportQuantity set to ‘p-cri-r19’, ‘p-cri-RSRP-r19’, ‘p-ssb-index-r19’, or ‘p-ssb-index-RSRP-r19’, and CSI reports configured with the higher layer parameter [RRC_name-r19] reportQuantity-r19 set to ‘csi-InferencePrediction-r19’ if a CSI report is not considered within any of M and Mx, the values for OCPU,1 and OCPU,x are considered to be 0, for the procedure previously described in this clause and the UE is not required to update the CSI report. A UE is not expected to be configured with an aperiodic CSI trigger state containing more than NCPU Reporting Settings. Processing of a CSI report occupies a number of CPUs for a number of symbols as follows: << Unchanged parts are omitted >> - OCPU,1=X1 and OCPU,x=Xx, for a CSI report configured with the higher layer parameter [RRC_name-r19] reportQuantity-r19 set to ‘csi-InferencePrediction-r19’, where the values of X1 and Xx are reported by UE capability. For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to ‘none’, or a CSI report with LTM-CSI-ReportConfig, or reportQuantity not set to ‘none-bm-r19’ or ‘none-csi-r19’, the CPU(s) (including OCPU,1and/or OCPU,x, for CSI reports with reportQuantity set to ‘p-cri-r19’, ‘p-cri-RSRP-r19’, ‘p-ssb-index-r19’, or ‘p-ssb-index-RSRP-r19’, or ‘csi-pai-r19’, or ‘rs-pai-r19’, and CSI reports configured with the higher layer parameter [RRC_name-r19] reportQuantity-r19 set to ‘csi-InferencePrediction-r19’ are occupied for a number of OFDM symbols as follows: << Unchanged parts are omitted >> In any slot, the UE is not expected to have more active CSI-RS ports or active CSI-RS resources in active BWPs than reported as capability. NZP CSI-RS resource is active in a duration of time defined as follows. For aperiodic CSI-RS, (excluding the case in which the corresponding aperiodic CSI Reporting Setting is configured with the higher layer parameter [RRC_name-r19] reportQuantity-r19 set to ‘csi-InferencePrediction-r19’ and is linked to another aperiodic CSI Reporting Setting with reportQuantity-r19 set to ‘csi-pai-r19’), starting from the end of the PDCCH containing the request and ending at the end of the scheduled PUSCH containing the report associated with this aperiodic CSI-RS. When the PDCCH candidates are associated with a search space set configured with searchSpaceLinkingId, for the purpose of determining the NZP CSI-RS resource active duration, the PDCCH candidate that ends later in time among the two linked PDCCH candidates is used. For semi-persistent CSI-RS, starting from the end of when the activation command is applied, and ending at the end of when the deactivation command is applied. For periodic CSI-RS, starting when the periodic CSI-RS is configured by higher layer signalling, and ending when the periodic CSI-RS configuration is released. << Unchanged parts are omitted >> For aperiodic CSI-RS, when the corresponding aperiodic CSI Reporting Setting is configured with the higher layer parameter [RRC_name-r19] reportQuantity-r19 set to ‘csi-InferencePrediction-r19’ and is linked to another aperiodic CSI Reporting Setting with reportQuantity-r19 set to ‘csi-pai-r19’, NZP CSI-RS resource is active in a duration starting from the end of the PDCCH containing the request for the report of the corresponding CSI Reporting Setting and ending at the end of the scheduled PUSCH containing the report associated with the report of the linked CSI Reporting Setting. ##### 5.4 UE CSI computation time << Unchanged parts are omitted >> (Z2+14K-1m+t,Z2'+t), with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4=1, codebookType is set to ‘typeII-Doppler-r18’ or ‘typeII-Doppler-PortSelection-r18’ and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is aperiodic with K CSI-RS resources, and t is according to UE reported capability as defined in [13, TS 38.306] when the higher layer parameter [RRC_name-r19] reportQuantity-r19 set to ‘csi-InferencePrediction-r19’ is configured in CSI-ReportConfig, otherwise t=0, or - (Z2+w+t,Z2'+t), with (Z2,Z2') of table 5.4-2, where w=56⋅(KP -1) or 56⋅KP symbols, according to the reported UE capability, where the value of KP ∈{1,2,4} is indicated by UE capability, if the CSI report is configured with N4=1, codebookType is set to ‘typeII-Doppler-r18’ or ‘typeII-Doppler-PortSelection-r18’ and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is periodic or semi-persistent with a single CSI-RS resource, and t is according to UE reported capability as defined in [13, TS 38.306] when the higher layer parameter [RRC_name-r19] reportQuantity-r19 set to ‘csi-InferencePrediction-r19’ is configured in CSI-ReportConfig, otherwise t=0, or - (Z2+14K-1m+t,Z2'+t) or (Z2+14K-1m+Z2'+t,2Z2'+t), according to UE reported capability, with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4>1, codebookType is set to ‘typeII-Doppler-r18’ and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is aperiodic with K CSI-RS resources, and t is according to UE reported capability as defined in [13, TS 38.306] when the higher layer parameter [RRC_name-r19] reportQuantity-r19 set to ‘csi-InferencePrediction-r19’ is configured in CSI-ReportConfig, otherwise t=0, or - (Z2+w+t,Z2'+t) or (Z2+w+Z2'+t,2Z2'+t), according to UE reported capability, with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4>1, codebookType is set to ‘typeII-Doppler-r18’ and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is periodic or semi-persistent with a single CSI-RS resource, and t is according to UE reported capability as defined in [13, TS 38.306] when the higher layer parameter [RRC_name-r19] reportQuantity-r19 set to ‘csi-InferencePrediction-r19’ is configured in CSI-ReportConfig, otherwise t=0, or << Unchanged parts are omitted >>

Please provide your input on proposal 2.5.

| Company | Views |
| --- | --- |
| Huawei, HiSilicon | ok |
| Qualcomm | This is not the right way. We didn’t change the codebook, so the air-interface is not changed, how it makes sense to add a new report quantity? What we report is nothing is exactly same as R18 doppler CSI. We should send LS to RAN2 regarding this. |
| Fujitsu | Ok |
| Google | OK |
| CATT | OK |
| ZTE | Thanks FL for capturing this editorial updates. We think it’s convenient to have a new report quantity for inference report to make the spec more readable. |
| Mod | Maybe this discussion depends on the RAN2’s progress, so I will postpone this until RAN2 has stable RRC parameters. |

Discussion on LS on candidate data collection (R1-2506715)

- Overall description For AI/ML based beam management and CSI prediction UE-side data collection, before UE is configured with CSI-ReportConfig for UE-side data collection measurement, RAN2 agreed to allow UE to indicate its preferred UE-side data collection configuration(s) based on the candidate UE-side data collection configuration(s) provided by network. RAN2 also agreed that UE is NOT expected to perform measurement(s) solely based on those candidate UE-side data collection configuration(s) and this UE behavior will be captured in RRC specification, i.e., no RAN1 specification impact. For AI/ML based beam management, according to RAN1 agreement on UE-side data collection, for each candidate UE-side data configuration, RAN2 agreed at least the following IEs are included: - An identifier of the candidate configuration - CSI-ResourceConfigId for Set A - CSI-ResourceConfigId for Set B - Related associated IDs In addition, RAN2 agreed the same framework of candidate/preferred UE-side data collection configuration is also applicable to CSI prediction. However, RAN2 wonders what IEs are needed for CSI prediction as part of candidate UE-side data collection configuration. - Action To RAN1 ACTION: RAN2 respectfully asks RAN1 if there’s any concern on the above agreements and provide the necessary higher layer parameters for candidate UE-side data collection configuration for CSI prediction and for AI/ML beam management if any additional IE is missing.

Per chairman’s guidance, reply LS is handled in CSI prediction while beam management related issue/question will be discussed in beam management and the discussion output will be a part of reply LS.

Based on the submitted Tdocs, I summarize the companies’ view as follows:

Most of companies (vivo, CMCC, CATT, OPPO, Samsung, Nokia, LG, Ericsson, Apple, Sharp, Google) think at least following IE is needed for candidate UE data collection.

- An identifier of the candidate configuration

- CSI-ResourceConfigId

Even, vivo, CMCC, OPPO, Nokia, LG, Ericsson, Apple, Sharp think above two IE are sufficient.

Meanwhile ZTE propose to distinguish CSI-ResourceConfigId for training and inference, and Lenovo propose to distinguish CSI-ResourceConfigId for CMR and IMR. From the moderator’s perspective, there is no clear motivation to distinguish CSI-RS resources for training and inference, since this procedure is for data collection for training. In my understanding, RAN2 guys use the term of UE-side data collection only for data collection for training. In this sense, ReportQuantity set to ‘none-CSI-r19’ seems not needed. Also, there is no clear motivation to include CSI-ResourceConfigId for IMR.

For other IEs, there are several proposals. Google thinks # of port information (n1-n2 which is indicated by codebook configuration in CSI report config) is needed for non-precoded CSI-RS case, and PRG information (e.g., pdsch-BundleSizeForCSI) for pre-coded CSI-RS case. Since typical CSI report considers non-precoded CSI-RS cases, non-pre-coded case should be prioritized. For n1-n2 configuration, moderator think it is valid argument. (Sorry for the confusion, I will take back my word). Since resource configuration contain “nrofPorts”, I don’t see the necessity of include n1-n2 configuration.

Similar to BM-Case 2, CATT, Samsung, Lenovo, Huawei think some IE, e.g., N4, delta, d or whole codebook config, to represent preferred predicted CSI configuration is needed. Since codebook config is more inclusive, my suggestion is to include codebook config for candidate UE-side data collection.

Meanwhile, Qualcomm think whole CSI-report config id is needed. However, moderator think there are many redundant IEs used for reporting not for data collection purpose. So, we need to focus on necessary IE for candidate UE data collection.

Agreement @RAN1 #122 Support the configuration of following parameters in a CSI-ReportConfig with the higher layer parameter reportQuantity set to'none-bm-r19': - TimeGap-r19, i.e., the time gap between two consecutive predicted time instances and between the reference time and the earliest predicted time instance. - nroftimeinstance-r19, i.e., number of predicted time instances for BM-Case 2.

Based on the above discussion, I propose following

- Proposal 3. (Candidate UE-side data collection)

For the UE-side candidate data collection configuration for CSI prediction, include the following IEs:

- An identifier of the candidate configuration

- CSI-ResourceConfigId

- Codebook config

- Include the above information as part of LS reply to RAN2

Please provide your input on proposal 3.

| Company | Views |
| --- | --- |
| Huawei, HiSilicon | 1, predictionDelay-r18 in CodebookConfig is not applicable for UE side data collection, since it is defined as the time gap between scheduled CSI report to the first predicated instance, but the UE side data collection has no CSI report. It should be changed to the time gap between latest CSI-RS to the first predicted instance, similar to AI BM. 2, the definition of the parameters in CodebookConfig should be updated to N4 (vectorLengthDD) - “the expected Doppler-/time-domain (DD/TD) basis vector length of prediction” d value (unitDurationDD) - “the expected DD/TD unit duration (in slots) of prediction, for AP-CSI-RS only” 3, The UE side data collection only supports P/SP-CSI-RS, but the inference could be A-CSI-RS which has K A-CSI-RS resources for prediction as model input. Thus, the expected number of A-CSI-RS for channel measurement (i.e., K) should be indicated to UE at the UE side data collection phase, so that the UE side can train with appropriate input dimension. For the UE-side candidate data collection configuration for CSI prediction, include the following IEs: - An identifier of the candidate configuration - CSI-ResourceConfigId - Codebook config, with following changes on definition: - predictionDelay-r18 (delta): the time gap between reference time to the first predicted instance, where the reference time is the latest CSI-RS time occasion no later than the corresponding CSI reference resource - vectorLengthDD (N4): the expected Doppler-/time-domain (DD/TD) basis vector length of prediction - unitDurationDD (d): the expected DD/TD unit duration (in slots) of prediction, for AP-CSI-RS only - NumberOfAperiodicCSI-RS-Resource: The expected number of configured A-CSI-RS for channel measurement of A-CSI-RS based prediction - Include the above information as part of LS reply to RAN2 |
| Qualcomm | We don’t understand how would the codebook config relevant to data collection. The last bullet can be absorbed by resource ID list also. In our view, NW should send a list of candidate data collection configurations (similar to applicability enquiry option A), and UE response which configuration is preferred. If NW only asks the preference on resource ID, and if the resource ID already exists in the CSI measurement configuration, NW may not need to even explicitly configure data collection configurations because NW would assume UE is able to collect data with the configured resource. However, since UE data collection requires CPU counting, a dedicated data collection configuration is essentially needed. Thus, we believe sending list of data collection configurations directly is the simple and clean way. |
| Fujitsu | Generally fine with this direction but more discussion may be needed. |
| vivo | Support first and second information |
| Google | We think it is also necessary to configure whether the CSI-RS is precoded or not. If it is precoded, the PRG size information is necessary. |
| CATT | Support proposal 3 |
| ZTE | We believe the intention for candidate data collection is to provide from the gNB to the UE only with candidate CSI-RS resource IDs and UE can select their desired IDs for data collection. Therefore, we think resource IDs are sufficient since the UE supported configurations for CSI prediction are reported to gNB before the candidate data collection signaling. |
| LGE | First and second IE is sufficient. |
| Mod | Thanks for the inputs. The discussion point is other than 1st and 2nd bullet, whether we need more IEs for training. @QC: I’m little bit confused on your comments. So, your proposal is full set of report configuration is needed? @Google: Again, I fail to see the motivation of distinguish precoded or non-precoded CSI-RS. This was mostly related to associated id, but that associated id is not supported for CSI prediction. @Huawei: I think your proposal includes new RRC parameters, so I’m hesitant to include your proposal. Let’s hear more views. @ All: please check the proposal below. And provide further input regarding this proposal. - Proposal 3. (Candidate UE-side data collection) For the UE-side candidate data collection configuration for CSI prediction, include the following IEs: - An identifier of the candidate configuration - CSI-ResourceConfigId - Codebook config - n1-n2-codebookSubsetRestriction-r18 - Include the above information as part of LS reply to RAN2 |

(Place holder) Draft reply LS on candidate data collection

Summary and proposals for offline session

Discussions for Monday.

- Proposal 2.5-3A. Adopt the following TP for TS 38.214.

| Reason for change | Based on the agreement, for CSI-PAI report, UE is expected to be configured with the same subband configuration with that of inference report, and subband configuration can include RRC parameters of csi-ReportingBand, subbandSize and numberOfPMI-SubbandsPerCQI-Subband. However, in current specification, subbandSize is missing. |
| --- | --- |
| Summary of change | RRC parameter subbandSize is added. |
| Consequences if not approved | Incorrect capture of the agreement. |
| TS38.214 5.2.1.4.6 CSI-PAI reporting << Unchanged parts are omitted >> - to report CSI-PAI for the second Reporting Setting, the UE is expected to be configured in the second Report Setting with codebookType set to 'etypeII-r16' and it shall: - determine predicted PMI (see Clause 5.2.2.2.10 or Clause 5.2.2.2.11) for each of the subband(s) configured by csi-ReportingBand, subbandSize and numberOfPMI-SubbandsPerCQI-Subband for the S-th time instance configured by timeinstanceforCSImonitoring-r19 in the second Reporting Setting (S≤N4 , with N4>1 provided in the first Reporting Setting), based on the CSI prediction performed by the UE using the channel measurement corresponding to the first Reporting Setting, - determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand, subbandSize and numberOfPMI-SubbandsPerCQI-Subband, which is corresponding to the S-th time instance of the report of the first Reporting Setting, based on the CSI-RS resource(s) in the resource set for channel measurement for the report corresponding to the second Reporting Setting. - determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand, subbandSize and numberOfPMI-SubbandsPerCQI-Subband, based on the latest CSI-RS resource transmission occasion, no later than the CSI reference resource of the CSI report, corresponding to the first Reporting Setting. - for k ∈1,2,…,v, v is the RI reported for the CSI report corresponding to the first Reporting Setting, calculate SGCS value(s) as, SGCSk1=n=1NwknHwknwkn.wkn2N, SGCSk2=n=1NwknHwknwkn.wkn2N, where wkn is the predicted precoder represented by predicted PMI for k-th layer, n-th subband and for S-th time instance corresponding to the report of the first Reporting Setting, wkn is the precoder represented by non-predicted PMI for k-th layer, n-th subband based on the channel measurement corresponding to the second Reporting Setting, and wk is the precoder represented by PMI for k-th layer and n-th subband, n∈1,2,…,N, corresponds to the subbands configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, based on the latest CSI-RS resource transmission occasion, no later than CSI reference resource, corresponding to the report of the first Reporting Setting. The UE is expected to be configured with same csi-ReportingBand, subbandSize and numberOfPMI-SubbandsPerCQI-Subband configurations for the first CSI Reporting Setting and the second CSI Reporting Setting. << Unchanged parts are omitted >> |  |

- Offline agreement.

Adopt the following TP for TS 38.214.

| Reason for change | During non-active periods of cell DTX, UE is not expected to receive P/SP CSI-RS for CSI report associated with report quantity comprising at least RI which means this can be applied to SP inference report. However, whether it is applied for monitoring report is not clear. |
| --- | --- |
| Summary of change | Report quantity ‘csi-pai-r19’ is added. |
| Consequences if not approved | Inconsistent UE behaviour during non-active periods of cell DTX. |
| TS38.214 5.1.6.1 CSI-RS reception procedure << Unchanged parts are omitted >> During non-active periods of cell DTX if cell DTX is activated for a serving cell, the UE is not expected to receive the periodic CSI-RS and semi-persistent CSI-RS on the serving cell configured in CSI report configuration in CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI' or 'cjtc-P' or ‘csi-pai-r19’. If the cell DTX is activated for a serving cell [10, TS 38.321], the most recent CSI measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on the serving cell occurs in active periods of cell DTX for CSI report configured by CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI' or 'cjtc-P' or ‘csi-pai-r19’. << Unchanged parts are omitted >> |  |

- Proposal 3. (Candidate UE-side data collection)

For the UE-side candidate data collection configuration for CSI prediction, include the following IEs:

- An identifier of the candidate configuration

- CSI-ResourceConfigId

- Codebook config

- n1-n2-codebookSubsetRestriction-r18

- Include the above information as part of LS reply to RAN2

- Codebook config, with following changes on definition:

  - predictionDelay-r18 (delta): the time gap between reference time to the first predicted instance, where the reference time is the latest CSI-RS time occasion no later than the corresponding CSI reference resource

  - vectorLengthDD (N4): the expected Doppler-/time-domain (DD/TD) basis vector length of prediction

  - unitDurationDD (d): the expected DD/TD unit duration (in slots) of prediction, for AP-CSI-RS only

- NumberOfAperiodicCSI-RS-Resource: The expected number of configured A-CSI-RS for channel measurement of A-CSI-RS based prediction

Proposals for online session

Proposals for Tuesday.

Offline agreement

Adopt the following TP for TS 38.214.

| Reason for change | During non-active periods of cell DTX, UE is not expected to receive P/SP CSI-RS for CSI report associated with report quantity comprising at least RI which means this can be applied to SP inference report. However, whether it is applied for monitoring report is not clear. |
| --- | --- |
| Summary of change | Report quantity ‘csi-pai-r19’ is added. |
| Consequences if not approved | Inconsistent UE behaviour during non-active periods of cell DTX. |
| TS38.214 5.1.6.1 CSI-RS reception procedure << Unchanged parts are omitted >> During non-active periods of cell DTX if cell DTX is activated for a serving cell, the UE is not expected to receive the periodic CSI-RS and semi-persistent CSI-RS on the serving cell configured in CSI report configuration in CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI' or 'cjtc-P' or ‘csi-pai-r19’. If the cell DTX is activated for a serving cell [10, TS 38.321], the most recent CSI measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on the serving cell occurs in active periods of cell DTX for CSI report configured by CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI' or 'cjtc-P' or ‘csi-pai-r19’. << Unchanged parts are omitted >> |  |

- Proposal 3. (Candidate UE-side data collection)

For the UE-side candidate data collection configuration for CSI prediction, include the following IEs:

- An identifier of the candidate configuration

- CSI-ResourceConfigId

- Include the above information as part of LS reply to RAN2

Agreements

Agreements in RAN1#122

Agreement

Adopt following TPs for TS38.214 Section 5.2.1.4.6.

| Reason for change | In current specification, there is no SGCS quantization mapping table. |
| --- | --- |
| Summary of change | SGCS quantization mapping table is added. |
| Consequences if not approved | Incomplete capture of the agreement. |
| TS38.214 ##### 5.2.1.4.6 CSI-PAI reporting << Unchanged parts are omitted >> - each calculated SGCS value is quantized to a 4-bit field, where the first codepoint ‘0000’ is used to indicate (0 0.3] value range, and the value range [0.3, 1] is quantized in a linear scale with 0.05 step size. Detailed mapping is specified in [xx, TS 38.abc]Table 5.2.1.4.6-1. Table 5.2.1.4.6-1: SGCS Quantization Mapping Code Point; Range. 0; [0.0000, 0.3000]. 1; (0.3000, 0.3500]. 2; (0.3500, 0.4000]. 3; (0.4000, 0.4500]. 4; (0.4500, 0.5000]. 5; (0.5000, 0.5500]. 6; (0.5500, 0.6000]. 7; (0.6000, 0.6500]. 8; (0.6500, 0.7000]. 9; (0.7000, 0.7500]. 10; (0.7500, 0.8000]. 11; (0.8000, 0.8500]. 12; (0.8500, 0.9000]. 13; (0.9000, 0.9500]. 14; (0.9500, 1.0000]. 15; Reserved. << Unchanged parts are omitted >> |

Agreement:

Adopt following TPs for TS38.214 Section 5.2.1.4.6.

| Reason for change | Based on the agreement, the reported wideband SGCS is the average of SGCS values across all subbands, not directly derived from the average precoder. |
| --- | --- |
| Summary of change | The definition of SGCS needs to be changed to the average of SGCS values across all subbands. |
| Consequences if not approved | Incorrect capture of the agreement. |
| TS38.214 5.2.1.4.6 CSI-PAI reporting << Unchanged parts are omitted >> to report CSI-PAI for the second Reporting Setting, the UE shall: - determine predicted PMI (see Clause 5.2.2.2.10 or Clause 5.2.2.2.11) for each of the subband(s) configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband for the S-th time instance configured by timeinstanceforCSImonitoring-r19 in the second Reporting Setting ( , with N4 provided in the first Reporting Setting), based on the CSI prediction performed by the UE using the channel measurement corresponding to the first Reporting Setting, - determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, which is corresponding to the S-th time instance of the report of the first Reporting Setting, based on the CSI-RS resource(s) in the resource set for channel measurement for the report corresponding to the second Reporting Setting. - determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, based on the latest CSI-RS resource transmission occasion, no later than the CSI reference resource of the CSI report, corresponding to the first Reporting Setting - for , v is the RI reported for the CSI report corresponding to the first Reporting Setting, calculate SGCS value(s) as, , , , , where wkn is the predicted precoder represented by predicted PMI for k-th layer, averaged over all the subbands configured by csi-ReportingBand, n-th subband, and for S-th time instance corresponding to the report of the first Reporting Setting, wkn is the precoder represented by non-predicted PMI for k-th layer, averaged over all the subbands configured by csi-ReportingBand, n-th subband, based on the channel measurement corresponding to the second Reporting Setting, and wkn is the precoder represented by PMI for k-th layer and averaged over all the subbands configured by csi-ReportingBand n-th subband, corresponds to the subbands configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, based on the latest CSI-RS resource transmission occasion, no later than CSI reference resource, corresponding to the report of the first Reporting Setting. << Unchanged parts are omitted >> |  |

Agreement

For CSI-PAI report, UE is expected to be configured with the same subband configuration with that of inference report.

Agreement

For CSI prediction, for monitoring report, SP CSI report is only supported on PUSCH.

Agreement

The following TP2.6-1 and TP2.6-2 are endorsed as editorial corrections for TS 38.214 for Clauses 5.2.1.4.6 and 5.2.1.4.2, respectively.

TP 2.6-1 < Unchanged parts are omitted > 5.2.1.4.6 CSI-PAI reporting When the UE is configured with a first CSI Reporting Setting for reporting CSI prediction, based on a CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19] and a second CSI Reporting Setting for reporting CSI-PAI, based on a CSI-ReportConfig with reportQuantity-r19 set to 'csi-pai-r19', and the second Reporting Setting is linked to the first Reporting Setting by inferenceReportConfigId-r19, the reporting of CSI-PAI corresponding to the second CSI Reporting Setting shall consider the following: - when aperiodic Reporting Setting is configured for both first and second Reporting Settings: - if aperiodic CSI-RS resources are configured for channel measurement for the first Reporting Setting, the UE is expected to receive the PDCCH triggering a CSI report of the second Reporting Setting, if any, no later than the first symbol of the earliest occasion of, Ka??{4,8,12} , aperiodic CSI-RS resources in the resource set for channel measurement for the first Reporting Setting. - if a single periodic or semi-persistent CSI-RS resource is configured for channel measurement for the first Reporting Setting, the UE is expected to receive the PDCCH triggering a CSI report of second Reporting Setting, if any, no later than the first symbol of the earliest transmission occasion of the most recent, Kpa??{1,2,4}, periodic or semi-persistent CSI-RS resource, no later than the CSI reference resource of the CSI report, corresponding to the first Reporting Setting. - to report CSI-PAI for the second Reporting Setting, the UE shall: - determine predicted PMI (see Clause 5.2.2.2.10 or Clause 5.2.2.2.11) for each of the subband(s) configured by csi-ReportingBand for the S-th time instance configured by timeinstanceforCSImonitoring-r19 in the second Reporting Setting ( , with N4>1 provided in the first Reporting Setting), based on the CSI prediction performed by the UE using the channel measurement corresponding to the first Reporting Setting, - determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand, which is corresponding to the S-th time instance of the report of the first Reporting Setting, based on the CSI-RS resource(s) in the resource set for channel measurement for the report corresponding to the second Reporting Setting. - determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand, based on the latest CSI-RS resource transmission occasion, no later than the CSI reference resource of the CSI report, corresponding to the first Reporting Setting < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

TP 2.6-2 < Unchanged parts are omitted > 5.2.1.4.2 Report quantity configurations If the UE is configured with a CSI-ReportConfig with reportQuantity-r19 set to 'csi-pai-r19', - the UE shall be configured with inferenceReportConfigId-r19 to link another CSI-ReportConfig configured with a higher layer parameter [RRC_name-r19], - when semi-persistent Reporting Setting is configured for the CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19], the UE is not expected to be configured with a periodic Reporting Setting for the CSI-ReportConfig with reportQuantity-r19 set to 'csi-pai-r19'. - when aperiodic Reporting Setting is configured for the CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19], the UE is not expected to be configured with a periodic or semi-persistent Reporting Setting for the CSI-ReportConfig with reportQuantity-r19 set to 'csi-pai-r19'. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Agreement

For CSI-PAI report, when the UE is configured with two linked CSI report configurations, adopt UCI padding to fixed size

- based on the maximum rank determined from configuration of RI restriction (for simplicity denoted by max_rank here), the size of the CSI-PAI report is 2×max_rank×4 irrespective of the reported rank ‘v’ in the inference report.

- When the linked inference report indicates rank ‘v’, all ones is padded to LSB of the UCI sequence for each of max_rank-v fields of SGCS#1 or SGCS#2

Agreement:

For PMI assumption on SGCS calculation of wk and wk ,support Rel-16 eType II CSI.

Agreement:

For CSI-PAI report, support

- OCPU =X, X is reported by UE capability

- For AP monitoring report, CPU occupancy starts from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the CSI-PAI report

- For SP monitoring report, CPU occupancy starts from the first symbol of the latest periodic/semi-persistent CSI-RS occasion of the inference report not later than CSI reference resource of the inference report, until the last symbol of the configured PUSCH carrying the CSI-PAI report

Agreement:

For CSI prediction using UE-side model, for UE assisted performance monitoring,

- If the inference report is semi-persistent and the monitoring report is aperiodic or semi-persistent, UE is expected to receive the DCI triggering the monitoring report if any, no later than the first symbol of the earliest occasion of the most recent periodic or semi-persistent inference measurement resource transmission occasions that are no later than the CSI reference resource of the associated inference report.

Agreements in RAN1#121

Agreement

For CSI prediction using UE-side model, for data collection for training, reportQuantity is set to ‘none-CSI-r19’.

Agreement

For CSI prediction using UE-side model, to calculate the inference report using Doppler codebook,

- For PU occupancy, support

  - Dedicated AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied,

    - OAPU= 0 or N is reported by UE

    - OCPU=0 or M is reported by UE

    - Note: Detailed values of N and M can be further discussed in UE feature.

    - Note: Combination of OAPU= 0 and OCPU=0 is not allowed

    - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behaviour of exceeding the CPU limit, neither of the PUs are occupied

- For occupancy duration of CPU and APU,

  - the same occupancy duration is used if both CPU and APU are reported non-zero value

  - if associated monitoring report is not configured, reuse following legacy occupancy duration

    - For semi-persistent CSI report on PUSCH with P/SP CSIRS CMR, occupation starts from the first symbol of KP-th latest consecutive P/SP-CSI-RS occasions no later than CSI reference resource, until the last symbol of the PUSCH carrying the report, where KP is indicated by UE capability.

      - Note: Detailed values of KP can be further discussed in UE feature.

    - Aperiodic CSI report occupies PU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report.

- The total number of dedicated AI/ML PU for AI/ML is reported by UE capability

Agreement

For CSI prediction using UE-side model, for inference, regarding active resource/port counting,

- Reuse legacy active resource/port counting at least if associated monitoring report is not configured

Agreement

For CSI prediction using UE-side model, for inference, in addition to legacy Z/Z’ for doppler codebook, UE may report the value of t per SCS

- Detailed value of t can be discussed in UE feature

Agreement

For CSI prediction using UE-side model, for UE assisted performance monitoring,

- Support to reuse CSI framework for the configuration for monitoring result report in L1 signaling

  - Dedicated resource set for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

    - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - For monitoring report type, semi-persistent and aperiodic are supported

    - The following combination for inference report type and monitoring report type are supported

| Monitoring report type Inference report type | SP report | AP report |
| --- | --- | --- |
| AP report | Not support | Support |
| SP report | Support | Support |

  - For monitoring resource type for measurement, periodic, semi-persistent and aperiodic are supported.

Agreement

- For CSI prediction using UE-side model, for UE assisted performance monitoring,

- If the inference measurement resource is aperiodic CSI-RS, UE is expected to receive the monitoring report trigger, if any, no later than the first symbol of the earliest occasion of K inference measurement resources that are no later than the CSI reference resource of the associated inference report;

  - Additional active resource/port counting time for the aperiodic resource of the inference report is from the end of the inference report to the end of the triggered monitoring report

- If inference report is aperiodic and the inference measurement resource is periodic or semi-persistent, UE is expected to receive the monitoring report trigger, if any, no later than the first symbol of the earliest occasion of the most recent Kp inference measurement resource transmission occasions that are no later than the CSI reference resource of the associated inference report

Agreement

For CSI prediction using UE-side model, for data collection for training,

- OCPU=1

- the CPU occupancy starts from the first symbol of each P/SP-CSI-RS occasion till Z3’ symbols after the P/SP-CSI-RS occasion.

Agreement

For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring,

- one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance

- SGCS is reported

  - wideband frequency granularity

  - only for one prediction instance configured by NW

  - per layer, and the total number of layers is the same as the reported RI in the associated inference report

- Introduce new RRC parameter for reportQuantity, e.g., ‘SGCS-r19’

- Each SGCS value is quantized with 4-bit

  - 15 codepoints are used to uniformly quantize in range [0.3 1] in linear scale.

  - One codepoint is used to indicate the value range of (0 0.3]

- Only one monitoring resource set is configured in the CSI-ReportConfig

- The one configured time instance (i.e. f-th doppler domain unit in one inference report) for SGCS calculation is configured in the CSI-ReportConfig for monitoring, for N4>1

Agreement

For CSI prediction using UE-side model, support single UCI part with the following report format (i.e., CSI field mapping order) among a set of SGCSs.

SGCS1 #1

SGCS1 #2

…

SGCS1 #v

SGCS2 #1

SGCS2 #2

…

SGCS2 #v

- SGCS1 is calculated based on predicted CSI for one inference reporting, and ground truth CSI,

- SGCS2 is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance

- SGCSi #k is the SGCS of k-th layer of i-th SGCS where k ={1, …., v}, and i ={1, 2}.

- v is the value of the reported RI in the associated inference report.

Agreements in RAN1#120bis

Agreement

Introduce a dedicated AI/ML PU for AI/ML features for UE,

- The AI/ML PU is used at least for quantifying the simultaneous processing of multiple CSI reports subject to CSI-related AI/ML use case(s), e.g., CSI compression (if supported), CSI prediction, BM spatial prediction, BM temporal prediction.

Conclusion

For CSI prediction using UE-side model, for data collection for training, aperiodic CSI-RS resource for CMR is not supported.

Agreement

For CSI prediction using UE-side model, for training data collection, support

- CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report

  - FFS on how to indicate without CSI report in CSI-ReportConfig

Agreement

For CSI prediction using UE-side model, for performance monitoring, support UE assisted performance monitoring subject to an additional UE capability, and UE assisted performance monitoring is based on Type 3 performance monitoring

Agreement

For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric.

Agreement

For the definition of SGCS,

Note: How to handle layer mapping mismatch, if any, is up to UE implementation

Agreement

For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, down-select one alternative by RAN1#121.

- Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI

- Alt 2: one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance.

- Alt 3: statistic of reporting contents (e.g., mean, x% CDF of SGCS values) of inference reporting instances within configured monitoring window

  - Monitoring window can be configured by NW

    - FFS on signalling details

- FFS on whether to report per prediction instance, selected prediction instance(s), averaged over prediction instances

- FFS on report frequency granularity (e.g., per wideband or per subband or averaged over subband or selected subband)

- FFS on whether to report per layer, the first layer, averaged over layer

- FFS on how to quantize and report mechanism

Agreement

For CSI prediction using UE side model, for inference, consider following options for potential down selection

- Option 1: only dedicated AI/ML PU (OAPU) is occupied

- Option 2: only legacy CPU (OCPU) is occupied

- Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied

- FFS whether OAPU and OCPU are based on defined rule and/or reported by UE

- Note: The supported option(s) by UE is reported by UE capability, if multiple options are supported.

The total number of dedicated AI/ML PU for AI/ML is reported by UE capability.

- Note: The total number of Use case specific dedicated AI/ML PU could be discussed separately.

Agreement

For CSI prediction using UE-side model, at least for inference, introduce new RRC parameter for CSI report configuration to distinguish CSI report of AI-CSI prediction and non-AI CSI prediction.

- Note: terminology of “AI-CSI prediction” and “non-AI CSI prediction” is separate discussion

- Detailed parameter name is upto RAN2

Agreements in RAN1#120

Agreement

For CSI prediction using UE-side model, at least for inference, Rel-18 CSI framework is reused.

- For CSI-RS resource type for CMR, periodic, semi-persistent, and aperiodic CSI-RS are supported.

- For inference report,

  - for N4>=1, support Rel-18 codebook ('typeII-Doppler-r18')

Agreement

For CSI prediction using UE-side model, if performance monitoring type 1 or 3 is supported, for calculation of monitoring metric, support

- Based on intermediate KPI

  - Down select between SGCS and NMSE by RAN1#120bis

  - FFS on the definition of SGCS/NMSE and how to calculate monitoring metric

Agreement

For CSI prediction using UE-side model, for CSI processing criteria and timeline, at least for inference further study on

- Whether the CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting

- Whether the Processing Unit should be shared or separately counted among AI/ML related features/functionalities.

- Whether new timeline is needed/updated for inference, and whether a different timeline is needed when functionality switches/activates.

- Whether legacy framework for active CSI-RS resource and port counting can be reused

Note: Strive to study CSI processing criteria considering both BM and CSI case, and take the existing solutions as starting point.

Agreement

For CSI prediction using UE-side model, for data collection for training,

- For resource configuration,

  - For CSI-RS resource type for CMR, periodic and semi-persistent are supported.

    - FFS on support of aperiodic CSI-RS resource

  - FFS on whether to separately configure CSI-RS resource(s) used for measurements for model input and CSI-RS resource(s) used for measurements for ground-truth CSI

  - FFS whether/how to enhance CSI-RS resource configuration to reduce CSI-RS signalling overhead.

- FFS on CSI report configuration

Reference

- R1-2506760	Remaining Issues of Rel-19 AI/ML for NR Air Interface	Ericsson

- R1-2506770	Discussion on maintenance of AI for Air Interface	ZTE Corporation, Sanechips

- R1-2506871	Maintenance on AI/ML for NR Air Interface	vivo

- R1-2506934	Maintenance of Rel-19 AI/ML for air interface	Huawei, HiSilicon

- R1-2506961	Maintenance on AI/ML for NR Air Interface	Xiaomi

- R1-2507000	Maintenance of specification support for AI/ML for NR Air Interface	CMCC

- R1-2507094	Remaining issues on AI/ML for NR air interface	CATT

- R1-2507155	Maintenance on Rel-19 AI/ML for NR air interface	OPPO

- R1-2507227	Remaining issue on AI/ML for NR Air Interface	Samsung

- R1-2507277	Remaining issues on AI/ML for air interface in Rel-19	Fujitsu

- R1-2507302	Remaining Issues on AIML for NR Air Interface	NEC

- R1-2507374	Discussion on AI/ML maintenance in 5G NR interface	Panasonic

- R1-2507383	Maintenance on AI/ML for NR Air Interface	Nokia

- R1-2507394	Maintenance on AI/ML for NR Air interface	LG Electronics

- R1-2507442	Maintenance on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface	Lenovo

- R1-2507452	Maintenance of AI beam management	Ofinno

- R1-2507515	Maintenance for AI/ML for NR Air Interface	Google

- R1-2507582	Maintenance on Rel-19 AI/ML	InterDigital, Inc.

- R1-2507648	Remaining issues for R19 AI/ML for NR air interface	Apple

- R1-2507694	Maintenance on AI/ML for NR air interface	Qualcomm Incorporated

- R1-2507774	Maintenance on AI/ML for NR Air Interface	Sharp

- R1-2507832	Remaining issues on AI/ML for air interface in Rel-19	Quectel

- R1-2507865	Remaining issues on AI PHY	ASUSTeK

- R1-2506715	LS on candidate data collection	RAN2, Xiaomi, Ericsson

- R1-2506767	Discussion on candidate data collection	ZTE Corporation, Sanechips

- R1-2506768	Draft LS reply on candidate data collection	ZTE Corporation, Sanechips

- R1-2506851	Draft reply LS on candidate data collection	vivo

- R1-2506852	Discussion on candidate data collection	vivo

- R1-2506999	Discussion on LS on candidate data collection	CMCC

- R1-2507081	Draft reply LS on candidate data collection	CATT

- R1-2507157	Reply on LS on candidate data collection	OPPO

- R1-2507220	Discussion on RAN2 LS on candidate data collection		Samsung

- R1-2507306	Discussion on LS on candidate data collection	NEC

- R1-2507384	Discussion for LS on candidate data collection	Nokia

- R1-2507385	Draft LS reply on candidate data collection	Nokia

- R1-2507393	Draft reply LS on candidate data collection	LG Electronics

- R1-2507431	Discussion on the RAN2 LS on candidate data collection	Ericsson

- R1-2507439	Draft Reply LS on candidate data collection	Lenovo

- R1-2507523	Draft Reply LS on Candidate Data Collection	Google

- R1-2507647	Discussion on RAN2 LS on candidate data collection	Apple

- R1-2507691	Discussion for LS reply on candidate data collection	Qualcomm Incorporated

- R1-2507773	Discussion on RAN2 LS on candidate data collection		Sharp

- R1-2507928	Discussion on the LS reply to RAN2 on candidate data collection	Huawei, HiSilicon