3GPP TSG-RAN WG1 Meeting # 122bis	R1-250xxxx

# Prague, Czech Republic, Oct 13th – 17th, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.901 | CR |  | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Handheld UT Polarized Antenna Model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_7_24GHz_CHmod |  |  |  |  |  | Date: |  |  | 2025-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN1#122 as part of R19 7-24 GHz channel model study, additional discussion on polarization modeling and antenna field pattern rotation resulted in a CR which explicitly instructs to use equations (7.3-7) and (7.3-8) for calculating cosψu, sinψu to be used in (7.3-6). Fu,θ''θ',ϕ'Fu, ϕ''θ',ϕ'=+cosψu-sinψu+sinψu+cosψuFθ''''θ'',ϕ''Fϕ''''θ'',ϕ'', (7.3-6) cosψu=cosβucosγusinθ'-sinβucosγucosϕ'-αu-sinγusinϕ'-αucosθ'1-cosβucosγucosθ'+sinβucosγucosϕ'-αu-sinγusinϕ'-αusinθ'2 (7.3-7) sinψu=sinβucosγusinϕ'-αu+sinγucosϕ'-αu1-cosβucosγucosθ'+sinβucosγucosϕ'-αu-sinγusinϕ'-αusinθ'2 (7.3-8) It can be shown that the denominator of (7.3-7), (7.3-8) may be zero in different combinations of the involved angles. The most notable are, e.g.: Example 1: βu=0,γu=0,θ'=0, Example 2: βu=0, ϕ'-αu=π2,γu=π-θ', In Example 1, equations (7.3-7), (7.3-8) would results in sinψu=NaN,cosψu=NaN, however true result should be sinψu=0,cosψu=1. There are more combinations beyond this. If the procedure written in section 7.3.2 for the Handheld UT Model is strictly followed, there may be cases when the resulting field pattern values Fu,θ''θ',ϕ',Fu, ϕ''θ',ϕ' are NaN. To fix that, more general equations such as (7.1-10) through (7.1-15) in Clause 7.1.3 should be used. The CR adds a note to these equations so that more general and reliable ways for calculating ψu can be used when there are issues with (7.3-7) and (7.3-8). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a note to clarify that there are other ways to calculate ψu based on Clause 7.1.3 and equation (7.1-10) when denominator of cosψu, sinψu is equal to 0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approved, the channel generated when following procedure in Clause 7.3.2 may contain Not a Number (NaN) values that is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact analysis: This CR has isolated impact. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 7.3.2	Polarized antenna modelling

< Unchanged parts are omitted >

Each polarized field component of the reference radiation pattern Fθ″″θ″,ϕ″ and Fϕ″″θ″,ϕ″ should be rotated according to the orientation and polarization direction of the each of UT antennae to get Fu,θ''θ',ϕ', Fu, ϕ''θ',ϕ' using equation

Fu,θ''θ',ϕ'Fu, ϕ''θ',ϕ'=+cosψu-sinψu+sinψu+cosψuFθ''''θ'',ϕ''Fϕ''''θ'',ϕ'',	(7.3-6)

cosψu=cosβucosγusinθ'-sinβucosγucosϕ'-αu-sinγusinϕ'-αucosθ'1-cosβucosγucosθ'+sinβucosγucosϕ'-αu-sinγusinϕ'-αusinθ'2,	(7.3-7)

sinψu=sinβucosγusinϕ'-αu+sinγucosϕ'-αu1-cosβucosγucosθ'+sinβucosγucosϕ'-αu-sinγusinϕ'-αusinθ'2,	(7.3-8)

where the 3D-rotation angles, αu, βu and γu, are obtained according to the orientation and polarization direction of each UT antenna u, the zenith and azimuth angles mapping between θ',ϕ' and θ'',ϕ'' are based on equations (7.1-7) and (7.1-8) using the same 3D-rotation angles, αu, βu and γu, and rotated based on the orientation of the UT in the global coordinate system to get Fu,θθ,ϕ and Fu,ϕθ,ϕ using Clause 7.1.3 equation (7.1-11).

Note:	The angle ψu can be computed in numerous ways following equation (7.1-10) in Clause 7.1.3, also to avoid possible division by 0 in equations (7.3-7) and (7.3-8).

< Unchanged parts are omitted >