3GPP TSG-RAN WG1 Meeting #123	Tdoc R1-2508364

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	8.1

Source:	Ericsson

Title:	Remaining Issues of Rel-19 AI/ML for NR Air Interface

Document for:	Discussion, Decision

# Introduction

In RAN1#121, the work on Rel-19 AI/ML use cases was completed. In RAN1#122 and RAN1#122bis, maintenance issues have been discussed and resolved for Rel-19 AI/ML, and response to LS has been sent. In this contribution, the remaining issues are discussed.

# Specification support for beam management

An open issue mentioned in [3] is the following:

### Issue 2.1.1# The determination of updated CSI reports based on CPU and APU

For procedure for the determination of updated CSI reports based on CPU and APU, down-select the following:

Alt 1: Handle the procedure for CPU and APU separately. If any condition for CPU and APU is not met, the CSI report is not required to be updated.

Alt 3: Handle the procedure for CPU and APU together.

  - Step 1: Determinate M, where M is the set of CSI reports that require to occupy CPU (i.e., require CPU only or CPU+APU), and the condition for CPU is met;

    - other CSI reports requiring to occupy CPU are not updated

  - Step 2: Determinate Mx, where Mx is the set of CSI reports that require to occupy APU (i.e., require APU only or CPU+APU), and the condition for APU is met.

    - other CSI reports requiring to occupy APU are not updated

  - Step 3: Determine a set of CSI reports that require to occupy CPU and APU, and are not within M (i.e., the condition for CPU is not met) or not within Mx (i.e., the condition for APU is not met)

    - The corresponding CPU and APU are considered to be zero for the set of CSI reports

In our view, Alt 1 is preferred as the procedures for CPU and APU are handled separately.  Particularly, with Alt 3, a UE may update CSI report(s) with lower priority and skip CSI report(s) with higher priority which deviates from legacy rule of dropping N-M CSI reports with lower priority.  Hence, we propose Alt 1:

For Rel-19 AI/ML based BM, support Alt 1 for the procedure of determining updated CSI reports based on CPU and APU, wherein:

- Alt 1: Handle the procedure for CPU and APU separately. If any condition for CPU and APU is not met, the CSI report is not required to be updated.

# Specification support for positioning accuracy enhancements

In RAN1#122bis, two issues were handled in the maintenance session:

- a reply LS was endorsed in response to RAN2 LS on the associated ID for Info #7;

- a TP to 38.214 v19.1.0 to describe: (i) the UE behavior when receiving the associated ID, and (ii) the associated ID may be provided together with the data collected from PRU.

In RAN1#122bis there were also some proposals for further enhancement (e.g., training data collection). Most of the proposals address topics that RAN1 had discussed but couldn’t converge. In our view, such proposals are considered optimizations and not essential corrections. Since the maintenance phase should address essential corrections only, optimization proposals should be treated as out of scope.

Therefore, in our view, the RAN1 Rel-19 specification for positioning accuracy is complete, and the maintenance phase can be closed.

# Specification support for CSI prediction

## CSI-RS configuration for training data collection

In the current version of 38.214, the following text is for data collection for UE-side data collection for the CSI prediction use case.

##### 5.2.1.4.1 Resource Setting configuration <text omitted> For a UE configured with a CSI-ReportConfig with the higher layer parameter reportQuantity-r19 set to 'none-csi-r19', the CSI-ReportConfig is linked to a periodic or a semi-persistent Resource Setting for channel measurement. <text omitted>

According to the specification, a single resource setting is used for channel measurement. However, it is not clear whether there is one CSI-RS resource or two CSI-RS resources in the resource setting. This relates to the unsolved FFS from RAN1#120, see below.

RAN1#120 Agreement For CSI prediction using UE-side model, for data collection for training, - For resource configuration, - For CSI-RS resource type for CMR, periodic and semi-persistent are supported. - FFS on support of aperiodic CSI-RS resource - FFS on whether to separately configure CSI-RS resource(s) used for measurements for model input and CSI-RS resource(s) used for measurements for ground-truth CSI - FFS whether/how to enhance CSI-RS resource configuration to reduce CSI-RS signalling overhead. - FFS on CSI report configuration

Specification needs to make sure that AI/ML models can be trained for all the supported CSI report configurations for inference, by having a CSI-RS resource configuration for training data collection to train a corresponding model. This necessitates separate configuration of the CSI-RS resources for model input and ground-truth. For example, it has been agreed in RAN1#120 that AP CSI-RS is supported for inference. If the CSI-RS resources in Figure 1 is considered as aperiodic CSI-RS burst, then the value of m and d can be different according to the 'typeII-Doppler-r18’ codebook configuration (e.g., m=2,d=1). In addition, the distance between the first prediction time instance in the prediction window and the last CSI-RS measurement in the measurement window does not necessarily to be an integer multiple of m. Hence, it is not always possible to have a single configuration of CSI-RS resource for collecting both the model input and ground-truth for training data collection.

Figure 1. An illustration of CSI prediction using UE-sided AI/ML model(s), where an aperiodic CSI-RS burst containing K aperiodic CSI-RS resources is configured for channel measurement to create input for model inference.

Some companies mentioned that this issue can be resolved through UE implementation, e.g., the UE can predict the same interval CSI-RS and then interpolate the channel responses that have different CSI-RS intervals. However, such implementation can result in prediction performance degradation, and this scheme has not been evaluated in the study phase.

Without separate configuration of CSI-RS resources for training data collection for model input and ground-truth CSI, it is not always possible to train a model that corresponds to a supported CSI report configuration for inference, e.g., model inference using aperiodic CSI-RSs for CMR.

For UE-sided CSI prediction use case, for training data collection, support two separate CSI-RS resources for UE-sided training data collection, one for model input and the other one for ground-truth.

# Conclusion

Based on the discussion above, the following observation is made:

Observation 1	Without separate configuration of CSI-RS resources for training data collection for model input and ground-truth CSI, it is not always possible to train a model that corresponds to a supported CSI report configuration for inference, e.g., model inference using aperiodic CSI-RSs for CMR.

The following are proposed based on our analysis:

Proposal 1	For Rel-19 AI/ML based BM, support Alt 1 for the procedure of determining updated CSI reports based on CPU and APU, wherein:

Alt 1: Handle the procedure for CPU and APU separately. If any condition for CPU and APU is not met, the CSI report is not required to be updated.

Proposal 2	For UE-sided CSI prediction use case, for training data collection, support two separate CSI-RS resources for UE-sided training data collection, one for model input and the other one for ground-truth.

# References

R1-2506622, “Consolidated Rel-19 higher layers parameters list Post RAN1#122,” Moderator (Ericsson), RAN1#122.

R1-2506626, “LS on Rel-19 higher layers parameters list Post RAN1#122,” RAN1, Ericsson, RAN1#122.

R1-2507999, “FL summary #1 for AI/ML in beam management,” Samsung (Moderator), RAN1#122bis.

R1-2506438, “Summary #3: maintenance of specification support for positioning accuracy enhancement,” Moderator (Ericsson), RAN1#122.

R1-2506573, “Summary #5 of maintenance on CSI prediction,” Moderator (LG Electronics), RAN1#122.