3GPP TSG RAN WG1 #123		R1- 2508709

Dallas, USA, Nov 17th – 21st, 2025

Source:	OPPO

Title:	Maintenance on Rel-19 Ambient IoT

Agenda Item:	8.4

Document for:	Discussion and Decision

Introduction

In this paper we discuss the maintenance issues of Rel-19 A-IoT.

Discussion

The following agreement was made in the last meeting [1]:

Agreement A device is not required to monitor R2D during TR→D for preparing D2R transmission - FFS in RAN1#123: how to capture this behaviour in the specifications

We suggest to capture this behavior in specification as Text Proposal 1 below.

Proposal 1: Adopt Text Proposal 1 in R1-2508709.

===========================Text Proposal 1 ===============================

7.2.3	Monitoring of R2D

A device is not required to monitor R2D:

-	After receiving a PRDCH intended for the device in an R2D transmission ending in chip χendR2D=MchipR2D-1, until the device performs a corresponding D2R transmission with chip χD2R=0 starting an amount of time TR→D after the end of chip χR2D= χendR2D according to the configuration received from higher layers.

-	when performing D2R transmission

-	for a duration TD→Rmin, as specified in TS 38.191 [5], after the end of chip χD2R=χendD2R when the device performs a D2R transmission ending in chip χendD2R=MchipD2R-1.

==============================End of Text Proposal 1=======================

The following conclusion was made in last meeting [1]:

Conclusion When X = 2, for the potential overlap of the 1st and 2nd Msg1 time resource due to the time drift during postamble and padding chips: - RAN1#123 will make a decision on whether to specify a solution for this issue, or to leave it to be avoided or handled by reader implementation.

The issue stems from 1) the appending of padding and 2) uncertainty of padding content as it is up to reader implementation. In the current specification [2] the offset for the 2nd D2R transmission (if X=2) is defined as   TR→D=1.25Toffset+MchipD2RTchipD2R, to address the issue completely the value of TR→D for the 2nd D2R transmission should be increased in case of long padding is appended.

But at maintenance phase we prefer not to open the discussion on when/how to increase TR→D for the 2nd D2R transmission. The reader can set X=1 when long padding is appended and potential overlapping between the 1st and 2nd D2R transmissions that it cannot handle may happen.

Proposal 2: Not to address the issue of potential overlapping between 2 D2R time resources caused by the appending of padding bits in the corresponding PRDCH.  It is up to reader to set X=1 when any potential issue may happen.

Conclusion

In this contribution we discussed the maintenance issues of Rel-19 A-IoT, we have following proposals:

Proposal 1: Adopt Text Proposal 1 in R1-2508709.

Proposal 2: Not to address the issue of potential overlapping between 2 D2R time resources caused by the appending of padding bits in the corresponding PRDCH.  It is up to reader to set X=1 when any potential issue may happen.

References

- Chairman Notes, 3GPP RAN1#122bis, Prague, Czech, Oct 13th – 17th, 2025.

- 3GPP TS 38.291 V19.0.0 (2025-06); 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Ambient IoT Physical layer (Release 19)