3GPP TSG RAN WG1 #123 			R1-2508905

Dallas, USA, Nov 17th – 21st, 2025

Source: 	Moderator (LG Electronics)

Title:	FL summary #3 for 10.3.2.3 “Other procedures”

Document for:	Discussion and decision

Introduction

This feature lead summary (FLS) concerns the sub-agenda 10.3.2.3 of Rel-20 Ambient IoT SI [1].

Based on the contributions submitted to the sub-agenda 10.3.2.3 ([4] – [41]) and some contributions in sub-agenda 10.3.2 and 10.3.2.1 as well, draft proposals/questions are prepared for offline and online discussions, and the proposals/questions will be updated during the meeting based on follow-on discussions.

In this documents, the following tags will be used:

- [Open]/[Closed] to mark open/closed issues

- [H]/[L] to mark priorities of each of the issues. The issue tagged [H] will be interpreted as an issue that needs to be resolved first at least in this meeting, and the issue tagged [L] or without any tag can also be discussed offline and online after the high priority issues are resolved.

With that understanding, please provide your views on the proposals tagged [H] first as they can be discussed first.

Please add (if absent) your e-mail address below for the points of contact for this sub-agenda.

| Company | Point(s) of contact | Email address(es) |
| --- | --- | --- |
| LGE | Jay KIM (FL) | jaehyung.kim@lge.com |
| LGE | Seokmin SHIN | seokmin.shin@lge.com |
| Panasonic | Hamidreza SHARIATMADARI | hamidreza.shariatmadari@eu.panasonic.com |
| Panasonic | HORIIKE Naoto | naoto.horiike@eu.panasonic.com |
| Panasonic | KANG Yang | yang.kang@sg.panasonic.com |
| Panasonic | SUZUKI Hidetoshi | suzuki.hidetoshi@jp.panasonic.com |
| TCL | Wenwen Huang | wenwen5.huang@tcl.com |
| TCL | Rongling Jian | rongling.jian@tcl.com |
| TCL | Yuanqing Yang | yuanqing4.yang@tcl.com |
| Samsung | Hyemin choe | hams.choe@samsung.com |
| Samsung | Miao Zhou | miao.zhou@samsung.com |
| Samsung | JJ | jeongho.j@samsung.com |
| Nokia | Torsten Wildschek | torsten.wildschek@nokia.com |
| Nokia | Hyun-Su Cha | Hyun-su.cha@nokia.com |
| Xiaomi | Teng Ma | mateng5@xiaomi.com |
| Xiaomi | Huan Sun | sunhuan7@xiaomi.com |
| Huawei, HiSilicon | Sarun Selvanesan | sarun.selvanesan@huawei.com |
| Huawei, HiSilicon | Mixiang | shawn.mixiang@huawei.com |
| Huawei, HiSilicon | Jin Zhe | jinzhe.jinzhe@huawei.com |
| Apple | Ankit Bhamri | a.bhamri@apple.com |
| OPPO | Kevin Lin | kevin.lin@oppo.com |
| OPPO | Shichang Zhang | shichangzhang@oppo.com |
| FUTUREWEI | Vip Desai | vdesai@futurewei.com |
| FUTUREWEI | Ruikang Yang | ryang@futurewei.com |
| NTT DOCOMO | Mayuko Okano | mayuko.okano.ca@nttdocomo.com |
| NTT DOCOMO | Sun Weiqi | sunwq@docomolabs-beijing.com.cn |
| NTT DOCOMO | Hiroki Harada | hiroki.harada.sv@nttdocomo.com |
| CMCC | Jingwen Zhang | zhangjingwen@chinamobile.com |
| CMCC | Xiaodong Shen | shenxiaodong@chinamobile.com |
| NEC | Pengyu Ji | ji_pengyu@nec.cn |
| azNEC | Tingting Fan | Tingting.Fan@EMEA.NEC.COM |
| NEC | Yun Liu | liu_yun@nec.cn |
| Lenovo | Xin GUO | guoxin9@lenovo.com |
| HONOR | JIAN ZHANG | zhangjian23@honor.com |
| Qualcomm | Yuchul Kim | yuchulk@qti.qualcomm.com |
| Qualcomm | Le Liu | leliu@qti.qualcomm.com |
| IDCC | Erdem Bala | Erdem.bala@interdigital.com |
| IDCC | Kevin Wanuga | Kevin.wanuga@interdigital.com |

Heads-up for the next meeting

Online/offline proposals

This section intends to provide online/offline proposals for each specific online session (e.g. proposals for Tuesday morning online) with a list of ordered proposals, which saves time for everyone especially online where the Chairman just needs to look at this section to find all the proposals for the session.

Proposals for Tuesday afternoon online session

4.1	Initial frequency/broadcast information acquisition

[Initial frequency acquisition]

FL would like to check if the following proposal is agreeable. If it is controversial, we will have to capture contradicting views on the necessity of frequency/cell/reader search from companies in TR 38.769.

[H] Proposal 4.1.1a

Initial frequency search procedure is necessary for Device 2b and for Device C for outdoor scenarios, at least for flexible deployment of A-IoT frequency location in NR band.

- FFS how to capture observations on the necessity of cell/reader search procedure.

Cell/reader search procedure is necessary for Device 2b and for Device C for outdoor scenarios, if multiple cell/reader scenario is applicable.

- FFS how to capture observations on the necessity of cell/reader search procedure.

[Information included in the broadcast information]

FL suggests that we agree in principle to capture the observations on the broadcast information and continue working on the text proposals based on the above list with some potential down-selection, if possible.

[L] Proposal 4.1.2a

Capture the observations on the broadcast information based on the following list and the corresponding sources in the TR 38.769.

- Cell/reader ID related info

- Power control related information

- L1 control related info (e.g., chip duration, FEC code rate, repetition number)

- Periodic D2R resource configuration for DO-A

- Timing info (e.g., SFN-like information)

- Paging/DRX related information

- Reader/cell selection information

- Information related to frequency resource

- Resource for R2D signals

- SIB1-scheduling information

- RA trigger message

- UL frequency info

[Whether to use PRDCH to carry the broadcast information]

FL suggests to capture Case B1 and Case B2 above, and related observations in TR 38.769.

[H] Proposal 4.1.3a

On how to carry the broadcast information, capture in TR 38.769 the following observations with the following two cases regarding whether to divide the broadcast information into two parts.

- Case B1: Single-part broadcast information (i.e., no division of the broadcast information)

- Case B2: Two-part broadcast information (e.g., MIB-like and the SIB1-like broadcast information)

For Case B1, PRDCH is used to carry the broadcast information.

For Case B2, PRDCH is used to carry the SIB1-like broadcast information.

- FFS for MIB-like broadcast information

[Transmission of broadcast information (periodic vs. aperiodic)]

As all the companies mentioning periodicity of the broadcast information in their contributions consider periodic transmission, FL would like to try with the following agreement to make a progress.

[H] Proposal 4.1.4a

Support periodic transmission of broadcast information for Device 2b and for Device C for outdoor scenarios, and capture observations on the periodic and aperiodic transmissions of the broadcast information.

4.2	DO-A

[H] Proposal 4.2.1a

Adopt the following table format for capturing in TR 38.769 the pros and cons of the for the content of the first D2R transmission for DO-A for Device 2b and for Device C.

|  | Recommended scenarios | Pros | Cons |
| --- | --- | --- | --- |
| Option 1 Msg1-like content | - low probability of DO-A traffic triggering [ZTE] | - Lower latency [LGE] - Reuse existing RA procedure | - Limited access capability [LGE] - High signaling overhead [DCM] |
| Option 2 DO-A data payload | - low probability of DO-A traffic triggering [ZTE] - single-device with period DO-A traffic [ZTE] | - The most efficient approach for short DO-A messages typical in ambient IoT, minimizing latency and signaling overhead [Apple] - Low latency [LGE] - Low signaling overhead [DCM] - Potential risk of large resource waste | - Contention resolution methods should be studied considering contention-based resource allocation. [DCM] |
| Option 3 DO-A scheduling request |  | - Minimum resource overhead by using a common access request sequence/resource for multiple devices [ZTE] - Low power [ZTE] - Low resource waste [ZTE] | - Contention resolution methods should be studied considering contention-based resource allocation. [DCM] - High signaling overhead [DCM] |

[Resource allocation/determination for the first D2R transmission for DO-A]

FL would like to check if the following proposal is agreeable based on the majority view from submitted contributions.  Otherwise, text proposals for capturing the contradicting views on the periodicity of resource allocation for the first D2R transmission for DO-A will be prepared to be captured in TR 38.769.

[H] Proposal 4.2.3a

For resource allocation method for the first D2R transmission for DO-A for Device 2b and for Device C, Option 1 (Periodic D2R resource allocation) is feasible and necessary.

Proposals for Wednesday morning offline session

[Whether to use PRDCH to carry the broadcast information]

[H] Proposal 4.1.3a1

On how A-IoT devices acquire the broadcast information, capture in TR 38.769 the following two cases:

- Case B1: Single-step acquisition of the broadcast information

- Case B2: Two-step acquisition of the broadcast information (e.g., MIB-like and then SIB1-like broadcast information)

On whether to use PRDCH to carry the broadcast information,

- For Case B1, PRDCH is used to carry the broadcast information.

- For Case B2,

  - PRDCH is used to carry the SIB1-like broadcast information.

  - PRDCH or a new physical channel is used to carry the MIB-like broadcast information

[Transmission of broadcast information (periodic vs. aperiodic)]

[H] Proposal 4.1.4a1

Support periodic transmission of broadcast information for Device 2b and for Device C for outdoor scenarios.

- FFS (in SI or WI): whether the transmission of broadcast information is strictly periodic at reader

4.2	DO-A

[H] Proposal 4.2.1b

Adopt the following table format for capturing in TR 38.769 observations on the 3 options based on the content of the first D2R transmission for DO-A for Device 2b and for Device C.

Direction

Try to clarify first two columns for each option

| First D2R transmission for DO-A | Recommended DO-A traffic scenarios (event-triggered or periodic) | Subsequent procedures after the first D2R transmission | Observations (e.g., in terms of signaling overhead, resource efficiency, latency, specification impact/effort, other scenarios such as single vs. multiple devices, …) |
| --- | --- | --- | --- |
| Option 1 Msg1-like content | - To be clarified - Event-triggered (Source 1, …) - Periodic (Source 1, …) | - To be clarified - CBRA |  |
| Option 2 DO-A data payload | - To be clarified - Periodic (Source 1, …) | - To be clarified - CFRA |  |
| Option 3 DO-A scheduling request | - To be clarified - Event-triggered (Source 1, …) - Periodic (Source 1, …) | - To be clarified - CBRA |  |

[Resource allocation/determination for the first D2R transmission for DO-A]

FL would like to check if the following proposal is agreeable based on the majority view from submitted contributions.  Otherwise, text proposals for capturing the contradicting views on the periodicity of resource allocation for the first D2R transmission for DO-A will be prepared to be captured in TR 38.769.

[H] Proposal 4.2.3a

For resource allocation method for the first D2R transmission for DO-A for Device 2b and for Device C, Option 1 (Periodic D2R resource allocation) is feasible and necessary.

[Provisioning of the necessary information for the first D2R transmission for DO-A]

[H] Proposal 4.2.5a

For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, following options are considered feasible.

- Option 1: necessary information provided by broadcast information

- Option 2: necessary information provided by a paging-like R2D message

- Option 3: necessary information derived from a synchronization signal and provided by broadcast information

Note: TR will capture those three options and down-selection will be done in the WI phase.

Proposals for Wednesday afternoon online session

4.1	Initial frequency/broadcast information acquisition

[Initial frequency acquisition]

Agreement

Initial frequency acquisition procedure is necessary for Device 2b and for Device C, at least for flexible deployment of A-IoT frequency location(s) for outdoor scenario.

Agreement

Reader identification/differentiation is necessary for Device 2b and for Device C, at least in outdoor scenario to differentiate R2D among different readers.

[H] Proposal 4.1.1b

For initial frequency acquisition for Device 2b and for Device C, a device is expected to determine a frequency point for reception of the periodic synchronization signal by one of the following options:

- Option 1: searching across pre-defined sync raster points.

  - Details on the sync raster design is handled by RAN4.

- [Option 2: from a pre-defined frequency range.]

- [Option 3: from a single pre-defined frequency point.]

FL’s assessment: • Option 2 needs some clarification (e.g., if it is different from Option 1) • There were some concerns on Option 3 in that it couldn’t provide the flexible deployment which was the main motivation of the initial frequency acquisition procedure.

[H] Proposal 4.1.1c

For reader identification/differentiation, a device is expected to acquire the reader ID-related information by at least one of the following options:

- Option 1: Reader ID-related information is contained in the broadcast information.

  - [whether the broadcast information is from higher layer or from L1]

- Option 2: Reader ID-related information is derived from the periodic sync signal.

- Option 3: Reader ID-related information is carried by [device-specific] L1 R2D control information.

FL’s assessment: • The parts with [ ] was added after offline discussion for further clarification.

[Whether to use PRDCH to carry the broadcast information]

[H] Proposal 4.1.3a1

On how A-IoT devices acquire the broadcast information, capture in TR 38.769 the following two cases:

- Case B1: Single-step acquisition of the broadcast information

- Case B2: Two-step acquisition of the broadcast information (e.g., MIB-like and then SIB1-like broadcast information)

On whether to use PRDCH to carry the broadcast information,

- For Case B1, PRDCH is used to carry the broadcast information.

- For Case B2,

  - PRDCH is used to carry the SIB1-like broadcast information.

  - PRDCH or a new physical channel is used to carry the MIB-like broadcast information

FL’s assessment: • Updated the proposal based on the feedback during the online yesterday. • How the scheduling information for the broadcast information is obtained will be separately discussed.

[Transmission of broadcast information (periodic vs. aperiodic)]

[H] Proposal 4.1.4a1

Support periodic transmission of broadcast information for Device 2b and for Device C for outdoor scenarios.

- FFS (in SI or WI): whether the transmission of broadcast information is strictly periodic at reader

FL’s assessment: • The sub-bullet is to address concerns on the “strictly” periodic transmission of broadcast information. Same approach as we did with periodic sync signals.

4.2	DO-A

[H] Proposal 4.2.1b

Adopt the following table format for capturing in TR 38.769 observations on the 3 options based on the content of the first D2R transmission for DO-A for Device 2b and for Device C.

Direction

Try to clarify first two columns for each option

| First D2R transmission for DO-A | Recommended DO-A traffic scenarios (event-triggered or periodic) | Subsequent procedures after the first D2R transmission | Observations (e.g., in terms of signaling overhead, resource efficiency, latency, specification impact/effort, other scenarios such as single vs. multiple devices, …) |
| --- | --- | --- | --- |
| Option 1 Msg1-like content | - Event-triggered (Source 1, …) - Periodic (Source 1, …) | - R19 CBRA - R19 CFRA - or describe changes from R19 CBRA/CFRA, if any |  |
| Option 2 DO-A data payload |  |  |  |
| Option 3 DO-A scheduling request |  |  |  |

FL’s assessment: • Updated the table format for further clarification on what/how information would be captured each of the column. • Once table format is confirmed, FL will collect inputs from cm

[Resource allocation/determination for the first D2R transmission for DO-A]

[H] Proposal 4.2.3a

For resource allocation method for the first D2R transmission for DO-A for Device 2b and for Device C, Option 1 (Periodic D2R resource allocation) is feasible and necessary.

FL’s assessment: • Proposal 4.2.3a is based on the majority view from submitted contributions. If it is not agreeable, then FL recommends to capture both options without further discussion on down-selection during SI phase. Following proposal can also be considered: Alternative proposal: For resource allocation method for the first D2R transmission for DO-A for Device 2b and for Device C, both Option 1 (Periodic D2R resource allocation) and Option 2 (Aperiodic D2R resource allocation) are feasible. - Note: Down-selection will not be discussed in SI phase.

[Provisioning of the necessary information for the first D2R transmission for DO-A]

[H] Proposal 4.2.5a

For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, following options are considered feasible.

- Option 1: necessary information provided by broadcast information

- Option 2: necessary information provided by a paging-like R2D message

- Option 3: necessary information derived from a synchronization signal and provided by broadcast information

Note: TR will capture those three options and down-selection will be done in the WI phase.

FL’s assessment: • Each option has enough support and most of companies are okay to capture those 3 options without down-selection in this meeting.

10.3.2.3 Other procedures

Initial frequency/broadcast information acquisition

Previous agreements

Agreement (RAN1#122bis)

For R2D, study at least the following aspects for the study of broadcast information:

- Information included in the broadcast information

- Whether to use PRDCH to carry the broadcast information

- Transmission of broadcast information:

  - Option 1: periodic

  - Option 2: aperiodic

- Time and frequency resources for transmission of the broadcast information, e.g. in relation to the synchronization signal.

Initial frequency acquisition

High level assumptions on initial frequency acquisition including the necessity on cell search like operation are discussed in this sub-clause.

Summary

Companies reports that initial frequency/cell/reader search procedure is necessary for the following reasons. (CMCC, HW, E///, DCM)

- DCM: for flexible operation of A-IoT frequency location in NR band

- CMCC: Different Readers may configure different random access resources, so Readers must be distinguishable from device side, such that devices know which resource to choose for access.

- E///: Device must filter those signals to communicate with a specific reader at a designated frequency (sync-raster point)

It is also reported that for initial frequency search, sync raster is necessary (vivo, HW, E///, IDT)

On the other hand, sources propose not to support it in Rel-20 due to concerns on the increased complexity for ambient IoT devices. (MTK, Xiaomi, OPPO) And one source also reported that that one frequency point can be predefined for synchronization signal transmission. (Xiaomi)

One source also suggested that RAN1 postpone the discussion on initial cell/frequency search until other WGs reach the consensus on: (NEC)

- Whether “multi-reader or cell” should be introduced;

- Whether/how to design sync-raster;

FL would like to check if the following proposal is agreeable. If it is controversial, we will have to capture contradicting views on the necessity of frequency/cell/reader search from companies in TR 38.769.

[H] Proposal 4.1.1a

Initial frequency search procedure is necessary for Device 2b and for Device C for outdoor scenarios, at least for flexible deployment of A-IoT frequency location in NR band.

- FFS how to capture observations on the necessity of cell/reader search procedure.

Cell/reader search procedure is necessary for Device 2b and for Device C for outdoor scenarios, if multiple cell/reader scenario is applicable.

- FFS how to capture observations on the necessity of cell/reader search procedure.

| Company | Y/N | Comments |
| --- | --- | --- |
| LG | Y |  |
| Huawei, HiSilicon | N | We find this proposal inadequate to cover the details of the initial frequency acquisition procedure, the broadcast acquisition procedure and reader ID acquisition. We would encourage companies to consider the following: Proposal 1 Initial frequency search procedure is necessary for Device 2b and for Device C for outdoor scenarios, at least for flexible deployment of A-IoT frequency location in NR band. The device is expected to determine a frequency point to communicate with the reader by the following: - Option 1: searching across a set of pre-defined sync raster points for the transmission of the periodic synchronization signal. - Details on the sync raster design is handled by RAN4. - Option 2: from a pre-defined frequency range. - Option 3: from a single pre-defined frequency point. Proposal 2 Cell/reader search procedure is necessary for Device 2b and for Device C for outdoor scenarios. For the reader ID acquisition, the device is expected to determine the reader ID by one of the following: - Option 1: Reader ID is contained in the broadcast information. Option 2: Reader ID is derived from the sequences associated with the periodic sync signal. |
| TCL |  | If operators can deploy Ambient IoT carriers on any frequency location, then A-IoT devices must have capability for a certain frequency searching to discover the network signals in their area. For devices 2b/C, performing frequency scanning will bring significant implementation complexity and energy consumption burden. Frequency scanning usually requires the device to enable the RF front end and local oscillator, and stay at multiple candidate frequencies for a period of time to detect signals. This means that the device needs to be continuously powered for a period of time. Considering the power consumption level of device 2b, the peak power of several hundred microwatts can only sustain extremely short active operation. If multiple frequencies are to be scanned, it is very likely to consume most of the energy stored in the capacitor. |
| ZTE,Sanechips |  | Sync raster design(e.g. one point or more points etc) should be discussed in RAN4. |
| Panasonic | Y |  |

Information included in the broadcast information

Summary

Based on the contributions, the following is a list of potential information to be included in the broadcast information.

- Cell/reader ID related info (9 sources)

- Power control related information (6 sources)

- L1 control related info (e.g., chip duration, FEC code rate, repetition number) (6 sources)

- Periodic D2R resource configuration for DO-A (6 sources)

- Timing info (e.g., SFN-like information) (5 sources)

- Paging/DRX related information (3 sources)

- Reader/cell selection information (3 sources)

- Information related to frequency resource (2 sources)

- Resource for R2D signals (2 sources)

- SIB1-scheduling information (2 sources)

- RA trigger message (1 source)

- UL frequency info (1 source)

FL suggests that we agree in principle to capture the observations on the broadcast information and continue working on the text proposals based on the above list with some potential down-selection, if possible.

[L] Proposal 4.1.2a

Capture the observations on the broadcast information based on the following list and the corresponding sources in the TR 38.769.

- Cell/reader ID related info

- Power control related information

- L1 control related info (e.g., chip duration, FEC code rate, repetition number)

- Periodic D2R resource configuration for DO-A

- Timing info (e.g., SFN-like information)

- Paging/DRX related information

- Reader/cell selection information

- Information related to frequency resource

- Resource for R2D signals

- SIB1-scheduling information

- RA trigger message

- UL frequency info

| Company | Y/N | Comments |
| --- | --- | --- |
| LG | Y |  |
| vivo |  | For the following bullets, now it is too general to be clearly understood, more calibrations for them are needed. For example, the frequency resource for what? In our contribution, we propose to using broadcast information to carry the frequency information of AIOT carrier, such as the center frequency and bandwidth, if needed. Besides, what is RA trigger message? Moreover, in our contribution, we also propose AIoT-SSB periodicity to be carried in broadcast information added in red. Since we have agreed that periodic sync signal is necessary, the periodicity of sync signal after the initial sync can also be indicated by broadcast information. - Information related to frequency resource - Resource for R2D signals - RA trigger message - UL frequency info - The periodicity of synchronization signal. |
| ZTE,Sanechips |  | As stated in our opinion in Section 4.2, an aperiodic D2R resource can also be considered, with its configuration indicated through broadcast information. Since the specific name of the signal carrying the broadcast information has not yet been determined, we prefer to use the term ‘SIB-like’ instead of ‘SIB’. In our understanding, the information related to frequency resources should be provided when R2D FDM is used. Accordingly, the following modification is proposed: [L] Proposal 4.1.2a Capture the observations on the broadcast information based on the following list and the corresponding sources in the TR 38.769. - Cell/reader ID related info - Power control related information - L1 control related info (e.g., chip duration, FEC code rate, repetition number) - Aperiodic/Periodic D2R resource configuration for DO-A - Timing info (e.g., SFN-like information) - Paging/DRX related information - Reader/cell selection information - Information related to frequency resource for R2D FDM - Resource for R2D signals - SIB1-like scheduling information - RA trigger message - UL frequency info |
| Panasonic | Y |  |
| Nokia | Comments | - DO-A resources can be periodic or aperiodic, hence we propose to either remove the word periodic and leave just “D2R resource configuration for DO-A” or change to “periodic/aperiodic D2R resource configuration for DO-A” - Not clear what exactly “Information related to frequency resource” is referring to - Not clear what exactly “UL frequency info” is referring to |
| Futurewei |  | This may not be a comprehensive list. Given the existing sentence structure, it is unclear how to add “at least” The focus should be on information necessary for RAN1 operation. For example, while it is necessary to support paging, RAN2 is able to provide that |

Whether to use PRDCH to carry the broadcast information

Summary

On how to carry the broadcast information, two options regarding whether to divide the broadcast information into two parts are studied.

- Case B1: Single-part broadcast information (i.e., no division of the broadcast information): Most companies other than vivo, CMCC, HW?

- Case B2: Two-part broadcast information (e.g., MIB-like and SIB1-like): vivo, CMCC, HW,

On whether to use PRDCH to carry the broadcast information,

- Use PRDCH: HW, E///. NEC, ZTE, LGE, DCM, ETRI, SPRD, vivo, CMCC, Xiaomi, MTK,

  - E///: to reduce the design complexity and specification impact

  - DCM: FFS the PRDCH is scheduled by L1 R2D control

  - ETRI: a broadcast destination ID is used to indicate that the transmission is intended for all devices

- Introduce a new PHY channel for the MIB-like information in Case B2 (Two-part broadcast information): vivo, CMCC

- FFS PRDCH or a new PHY channel: FW

Following contributions from sub-agenda 10.3.2.1 discuss whether to use PRDCH to carry the broadcast information

- Separated channel to PRDCH (2 companies): Reported by vivo, and CMCC.

- Transmitted in PRDCH (6 companies): Reported by vivo, CMCC, Huawei, Ericsson, Xiaomi, ZTE, MTK

Furthermore, vivo, CMCC, Huawei suggest, in the contributions submitted in sub-agenda 10.3.2.1, to separate broadcast information into two parts, which are MIB-like part and SIB-like part.

- vivo and CMCC report that the MIB-like part can be transmitted in new physical channel.

- Huawei and CMCC report that the MIB-like part can be transmitted PRDCH. For SIB-like part, vivo, CMCC, Huawei report that it can be transmitted in PRDCH.

FL suggests to capture Case B1 and Case B2 above, and related observations in TR 38.769.

[H] Proposal 4.1.3a

On how to carry the broadcast information, capture in TR 38.769 the following observations with the following two cases regarding whether to divide the broadcast information into two parts.

- Case B1: Single-part broadcast information (i.e., no division of the broadcast information)

- Case B2: Two-part broadcast information (e.g., MIB-like and the SIB1-like broadcast information)

For Case B1, PRDCH is used to carry the broadcast information.

For Case B2, PRDCH is used to carry the SIB1-like broadcast information.

- FFS for MIB-like broadcast information

| Company | Y/N | Comments |
| --- | --- | --- |
| LG | Y |  |
| Huawei, HiSilicon |  | For the case B2, instead of having an FFS, there are only 2 options proposed by companies – one is to use the PRDCH and one is to use a new channel. These can be captured as follows: Proposal: On how to carry the broadcast information, capture in TR 38.769 the following observations with the following two cases regarding whether to divide the broadcast information into two parts. - Case B1: Single-part broadcast information (i.e., no division of the broadcast information) - Case B2: Two-part broadcast information (e.g., MIB-like and the SIB1-like broadcast information) For Case B1, PRDCH is used to carry the broadcast information. For Case B2, - PRDCH is used to carry the SIB1-like broadcast information. - PRDCH or a new channel is used to carry the FFS for MIB-like broadcast information |
| CMCC |  | General fine with the proposal. And for the last FFS, we prefer to list two options for MIB-like broadcast information. [H] Proposal 4.1.3a On how to carry the broadcast information, capture in TR 38.769 the following observations with the following two cases regarding whether to divide the broadcast information into two parts. - Case B1: Single-part broadcast information (i.e., no division of the broadcast information) - PRDCH is used to carry the broadcast information. - Case B2: Two-part broadcast information (e.g., MIB-like and the SIB1-like broadcast information) For Case B1, PRDCH is used to carry the broadcast information. - PRDCH is used to carry the SIB1-like broadcast information. - Two options to carry MIB-like broadcast information - Option 1: A new physical broadcast channel - Option 2: Use PRDCH |
| ZTE,Sanechips |  | To determine the SIB1-like scheduling information, the following options can be considered, Option 1: using predefined/fixed scheduling information, Option 2: indicated by L1 control of the SIB1-like information Option 3: indicated by MIB-like information. Thus, the following modification is proposed: [H] Proposal 4.1.3a On how to carry the broadcast information, capture in TR 38.769 the following observations with the following two cases regarding whether to divide the broadcast information into two parts. - Case B1: Single-part broadcast information (i.e., no division of the broadcast information) - Case B1-1: single-part broadcast information transmitted with predefined scheduling information. - Case B1-2: single-part broadcast information transmitted with scheduling information indicated in corresponding L1 control information. - Case B2: Two-part broadcast information (e.g., MIB-like and the SIB1-like broadcast information) For Case B1, PRDCH is used to carry the broadcast information. For Case B2, PRDCH is used to carry the SIB1-like broadcast information. - FFS for MIB-like broadcast information |
| Panasonic | Y |  |
| Nokia | Y |  |

Transmission of broadcast information (periodic vs. aperiodic)

Summary

Views on the periodicity of the transmission of broadcast information are summarized as follows:

- Option 1: Periodic transmission

  - Supported by [FW], vivo, CMCC, HW, Xiaomi, MTK, Qualcomm, E///, ZTE, Panasonic, LGE, DCM, CEWiT, ETRI, Fraunhofer

  - For Case B2, the time location and periodicity of MIB follow those of periodic R2D sync signal as a block: vivo

  - DCM: following aspects should be considered for the periodicity

    - NW overhead and NW energy saving

    - feasible device sustainable operation time

- Option 2: Aperiodic transmission

  - Supported by

- Option 3: Periodic or aperiodic

  - Supported by NEC, CATT

    - Based on reader implementation (NEC)

In sub-agenda 10.3.2.1, vivo, CMCC, Huawei, Xiaomi, MTK, Qualcomm, Docomo provide discussions on broadcast information transmission, which suggest the transmission of the broadcast information should be periodic.

NO COMPANY mentions aperiodic transmission in its contribution under sub-agenda 10.3.2.1.

As all the companies mentioning periodicity of the broadcast information in their contributions consider periodic transmission, FL would like to try with the following agreement to make a progress.

[H] Proposal 4.1.4a

Support periodic transmission of broadcast information for Device 2b and for Device C for outdoor scenarios, and capture observations on the periodic and aperiodic transmissions of the broadcast information.

| Company | Y/N | Comments |
| --- | --- | --- |
| LG | Y |  |
| Huawei, HiSilicon | Y |  |
| TCL | Y |  |
| vivo | Y | We support periodic transmission of broadcast information. As FL summarized, no companies propose aperiodic only broadcast information transmission. But the last half sentence mentions aperiodic transmission which causes some confusion. Maybe we can just delete the last half sentence since capturing observations is whatever we need to do. |
| CMCC | Y | We support periodic broadcast information for DOA. |
| ZTE,Sanechips | Y |  |
| Panasonic | Y |  |
| Samsung | N | Similar to the synchronization signal, the interval of this broadcasting information does not need to be fixed. As we mentioned in our contribution, the broadcast information transmission interval does not need to be constant. A nominal interval may exist, but it can be shifted as needed. |
| Futurewei |  | The second half of the sentence is unclear with the mention of “aperiodic” given the first half discusses periodic. Can remove the second half. Have similar comment as Samsung |
| Sharp | Y |  |

Time and frequency resources for transmission of the broadcast information

After coordination with the FL of 10.3.2.1, this topic is discussed in sub-agenda 10.3.2.1.

Following information can be passed over to sub-clause 10.3.2.1 for collecting more views:

Summary

For time and frequency resource for broadcast information transmission,

vivo, CMCC, Huawei, MTK, [Qualcomm?] report that broadcast information can be transmitted immediately after R2D periodic sync signal in time domain.

- In which, vivo, CMCC and Huawei report that MIB-like part in broadcast information is transmitted together with R2D periodic sync signal. CMCC also report that the MIB-like part can be transmitted separately with R2D periodic sync signal.

vivo, CMCC, and Huawei further propose the frequency of the MIB-like resource to be the same as periodic synchronization signal.

For time and frequency resource for SIB-like broadcast information,

- vivo report that the time resource for PRDCH with SIB-like information can be predefined or indicated by MIB-like information. And the centre frequency for PRDCH with SIB-like information is aligned with periodic synchronization signal.

- CMCC report that the time resources for PRDCH with SIB-like information can be indicated by MIB-like part or scheduled by the corresponding L1 R2D control information.

- Huawei report that the start time and frequency resource information can either be indicated in the MIB-like broadcast information, or be fixed in relation to the periodic synchronization signal/MIB-like broadcast transmission

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Consider the following proposal: Proposal: For the time and frequency resource for broadcast information transmission - In the case of a single-part broadcast information, it is transmitted immediately after the periodic synchronization signal. - In the case of a two-part broadcast information, - MIB-like broadcast information is transmitted immediately after the periodic synchronization signal. - SIB-like broadcast information is transmitted - Option 1: According to the indication in the MIB-like broadcast information Option 2: In fixed or pre-defined resources [in relation to the periodic synchronization signal/MIB-like broadcast transmission] |
| CMCC | It can be seen from Proposal 4.1.2a that the information payload to be carried by broadcast information is large, and if a single part broadcast information is defined, with predefined MCS by specification, the transmit efficiency will be low. So we prefer to separate it into two part, i.e. MIB-like part and SIB-like part. We propose the following for two-part broadcast information: Proposal: In the case of a two-part broadcast information, - For MIB-like broadcast information, the time/frequency resources with respect to periodic synchronization signal is predefined, and physical channel is either transmitted separately with the synchronization signal or is transmitted immediately after the periodic synchronization signal as a block. - For SIB-like broadcast information, the time/frequency resources resources can be indicated by MIB-like part or scheduled by the corresponding L1 R2D control information |
| ZTE, Sanechips | - Firstly, please add the following ZTE opinions which has been provided in our contribution, - “The broadcast information can be transmitted close in time to the synchronization signal and the period of broadcast information can be larger than or equal to the period of synchronization signal. - The bandwidth of broadcast information is the same as that of the synchronization signal.” - Secondly, we agree to use HW’s proposal as a starting point. However, we prefer to discuss the time resources and frequency resources for broadcast information separately. Considering that the SIP and/or L1 control may be transmitted before the broadcast information, the following proposals are made: Proposal1: For the time resource for broadcast information transmission - In the case of a single-part broadcast information, it is transmitted after the periodic synchronization signal. - In the case of a two-part broadcast information, - If MIB-like broadcast information is supported,MIB-like broadcast information is transmitted after the periodic synchronization signal. - FFS SIB-like broadcast information Proposal2: For the frequency resource for broadcast information transmission, the frequency resource for broadcast information transmission is the same as that of the synchronization signal. |

Procedures for initial frequency and broadcast information acquisition

Summary

Some sources (HW, ZTE, Fraunhofer) provided overall procedures for initial frequency and broadcast information acquisition.

Depending on the progress of other topics in Clause 4.1 and some related discussions in sub-agenda 10.3.2.1, FL plans to try a text proposal based on the contributions, or capture those text proposals/observations provided by the companies directly in the TR.

| Company | Comments |
| --- | --- |

Related proposals/observations (19)

| Source | Proposals and observations |
| --- | --- |
| FUTUREWEI | Proposal 6: Capture the following observations in the TR about the broadcast information: - Broadcast information can follow the synchronization signal o The synchronization signal can provide timing related information to receive the channel containing the broadcast information. o The chip rate, coding (if supported), and the payload size for the broadcast information can be pre-defined. o FFS whether this is a new channel or a PRDCH. - Broadcast information can contain information on how to receive an R2D transmission containing system information. o the chip rate of the L1 R2D control information o relative time location of the R2D transmission containing system information. |
| vivo | Init FA Observation 1: For the design of sync raster, both the initial CFO of device and the flexibility of AIoT deployment should be taken into account. Observation 2: For device 2b with 1000ppm~10000ppm initial CFO range, whether and how to deal with the case that multiple sync raster points within the initial CFO range, leading to undistinguishable for devices, needs further study. Proposal 1: Study the following potential functionalities based on R2D sync signal. - Reader differentiation or identification - Cell (re)selection based on RRM measurement Bcast info Proposal 2: Study broadcast information including both AIoT-MIB and AIoT-SIB: Ÿ The information can be carried by AIoT-MIB: SFN information, AIoT SIB scheduling information. Ÿ The information can be carried by AIoT-SIB: DOA resource configuration, L1 control related information configuration(s), R2D preamble configuration, the frequency information of AIOT carrier, DRX configuration for device power saving, criteria for cell-(re)selection, reader ID, power control related information, AIoT-SSB periodicity. Ÿ Note: other information depends on RAN2 study. Observation 3: For AIoT-SIB, since the transmission structure is the same as other PRDCH, reusing PRDCH to carry AIoT-SIB is enough. Proposal 3: AIoT-SIB is carried by PRDCH. And a new channel to carry AIoT-MIB can be considered. Proposal 4: L1 control information is needed for AIoT-SIB but unnecessary for AIoT-MIB. Observation 5: Broadcast information providing the essential information for active devices to do initial access to a reader should be periodically transmitted by a reader. Proposal 5: Study periodic broadcast information. Observation 6: The time location and periodicity of AIoT-MIB follow that of periodic sync signal as a block i.e., AIoT-SSB. And align the frequency center and bandwidth of AIoT-MIB with that of periodic sync signal can avoid the Rx retuning. Observation 7: The aligned frequency center between AIoT-SSB and AIoT SIB can avoid the unnecessary Rx retune in initial access phase. The device can assume same Rx BW for AIoT-SSB and AIoT-SIB reception, considering typically small M value for AIoT-SSB and AIoT-SIB with signal spectrum concentrating almost within the same single PRB. Observation 8: Regarding the time resources of AIoT-SIB, there can be two options: • Option1: indicated by AIoT-MIB. • Option2: predefined the time resource and periodicity of AIoT-SIB. Proposal 6: The device receives AIoT-SIB assuming aligned frequency center and bandwidth with that of AIoT-SSB reception. Proposal 7: The time domain information of AIoT-SIB including time offset relative to AIoT-SSB and the periodicity can be flexibly indicated by AIoT-MIB. |
| Nokia | Bcast info Proposal 5: Broadcast information, if supported, includes power control parameters such as target received power, allowed increment of transmission power, one or more chip duration values for a PRDCH for L1 control |
| CMCC | Init FA Observation 1：R19 design can support neither cell specific DOA random-access resource configuration nor device mobility. Proposal 1: R20 shall support cell search to acquire the cell ID and cell synchronization, and the exact number of cell IDs is FFS. Proposal 2: Study the three options for conveying the cell ID, • Option 1: Cell IDs are mapped to synchronization sequences. • Option 2: Cell ID is carried in the broadcast message. • Option 3: Combination of Option 1 and Option 2. Bcast info Observation 4: The R19 SIP-based paging detection mechanism is no longer applicable for inventory use case and a DRX-based solution is required. Proposal 6：Study the following information to be included in broadcast information, • Timing information, e.g. SFN-like info. • Periodic random access resource configuration, e.g. periodicity, offset, available resource in each periodicity. • DRX configuration for paging monitoring • Configurations of cell/reader-common L1 R2D control information • Cell ID related information Observation 5: The payload to be carried in the broadcast information is expected to be large, it is more reasonable to split the broadcast information into two parts, i.e., a MIB-like part and a SIB1-like part. Proposal 7：Broadcast information is divided into MIB and SIB part. |
| Huawei, HiSilicon | Proposal 1: For the initial frequency and broadcast information acquisition procedure, we propose the following: • For initial frequency acquisition, the device uses a set of pre-defined sync rasters to blindly detect the periodic synchronization signal. • On detecting the synchronization signal, it obtains coarse frequency synchronization and calibrates its clock accuracy, and on decoding the following MIB-like broadcast information carried on the PRDCH, it obtains the reader ID and SFN information, as well as the resource information on where to expect the SIB-like broadcast information, hence performing reader identification. • The device then detects the following CFO calibration signal to obtain fine frequency synchronization and calibrates the CFO/SFO to the residual level, e.g., 10 ppm, hence acquiring time and frequency synchronization. • The device then decodes the SIB-like transmission to obtain necessary information for DO-A, hence completing the broadcast information acquisition procedure. |
| Ericsson | Initial FA Observation 1 R2D - Reader/Cell search: During RAN1# 122 there were initial discussions around the need for a “reader/cell search procedure” resulting in two initial views 1) A “reader/cell search procedure” is needed for Rel-20 A-IoT outdoor and 2) The Rel-19 approach can be re-used (i.e., there is no need for a “reader/cell search procedure”). Observation 2 R2D - Reader/Cell search: In our view a “reader/cell search procedure” is needed for Rel-20 A-IoT outdoor. This because both the device and the reader (base station) will be outdoors, and even in a licensed spectrum the device will be exposed to a variety of RF signals from multiple sources, therefore the receiver must filter those signals to communicate with a specific reader at a designated frequency (sync-raster point). Observation 3 R2D - Reader/Cell search: During RAN1# 122, initial discussions on FDM suggested that a transmission bandwidth much larger than the ones available in Rel-19 (i.e., 1PRB, 2PRBs, 3PRBs, and 4PRBs) will be needed. If RAN1 concludes that FDM requires a much larger transmission bandwidth, then the 3 MHz CBW (15 PRB transmission bandwidth) or the 5 MHz CBW (25 PRB transmission bandwidth) available in legacy 5G NR could be considered to minimize the specification impact. Observation 4 R2D - Reader/Cell search: In our view, a set of sync-raster points for A-IoT outdoor operation spanning across the NR band should be introduced, however it is important keeping a trade-off between the number of sync-raster points and frequency scanning complexity, thus a limited number of sync-raster points should be considered. Yet, the A-IoT sync-raster points should in principle not overlap with the NR sync-raster points already present in the NR band. Observation 5 R2D - Reader/Cell search: The total number of sync-raster points should be limited (e.g., a dozen or even less than a dozen if that were too complex or too demanding energy consumption-wise), but still the sync-raster design distribution should aim at spanning across the band, which opens the possibility of deploying A-IoT outdoor at different locations within the NR band, which incorporates flexibility and results in a future-proof design. Proposal 1 For R2D in Rel-20 A-IoT, a set of sync-raster points must be supported for deploying A-IoT outdoor operation. This because both the reader and the device are outdoors, and the latter must filter RF signals and communicate with a specific reader at a designated frequency (sync-raster point). Proposal 2 For R2D in Rel-20 A-IoT outdoor, the sync-raster design accounts for the following aspects: 1) The total number of sync-raster points should be limited (e.g., even less than a dozen of sync-raster points if that were too complex or too demanding energy consumption-wise), 2) The design should be sparse (e.g., the step-size between sync-raster points can account for different factors e.g., limited energy storage, non-stationarity, CFO), 3) The sync-raster points should span across the entire NR band, 4) A-IoT Channel Bandwidths potentially deployable anywhere within a band, and 5) Whether the A-IoT sync-raster points should overlap or not with the NR sync-raster points already present in the NR band (e.g., “In-band” deployment should be considered when determining an overlapping or a non-overlapping approach). Bcast info In our view, at least the following information could be included in the broadcast information • Time and frequency resource of the random-access occasions (RO) for the first D2R transmission of DO-A traffic. • Power control parameters Some other information might be included depending on other aspects of the system design: • Time and frequency resource of the paging occasions for Msg0 transmission of DT and DO-DTT traffic. • Reader/cell selection information including certain RSRP level for reader selection/re-selection • D2R transmission configurations/parameters, e.g., size of preambles/midambles Observation 6 Using PRDCH to carry broadcast information could reduce the design complexity and specification impact. Observation 7 Fixed control parameters for PRDCH transmitting broadcast information could increase the reliability and reduce system overhead. Proposal 3 The broadcast information should be transmitted periodically in PRDCH with fixed parameters. |
| NEC | Init FA Proposal 1: RAN1 postpone the discussion on cell/frequency search until other WGs reach the consensus on: • Whether “multi-reader or cell” should be introduced; • Whether/how to design sync-raster; • RAN1 focus on the design of R2D sync signal which is discussed in AI 10.3.2.1. Bcast info Proposal 2: At least Scheduling Info for the first DO-A D2R transmission should be included in the broadcast information. • Child fields of D2R Scheduling Info in Rel-19 can be starting point; • FFS reader ID and the potential other fields. Proposal 3: PRDCH is used to carry the broadcast information, no need to introduce new channel(s) for R2D. Proposal 4: The broadcast information can be periodic or aperiodic, which is based on reader’s implementation. • It is difficult and unnecessary for the device to know the exact transmission time of the broadcast information. Proposal 5: The following two options can be studied for the time resource of broadcast information in relation to the end of the synchronization signal: • R2D sync signal and the corresponding broadcast information are continuous in time; • R2D sync signal and the corresponding broadcast information are non-continuous in time, a pre-defined gap is reserved. • Note: This does not mean all the R2D signals always have the corresponding broadcast information. Proposal 6: The frequency resource of broadcast information is always same to the corresponding R2D sync signal. |
| CATT | Init FA Proposal 1: Both periodic and aperiodic R2D synchronization signals could be supported with the presumption that the A-IoT devices do not require to maintain synchronization with the reader in Rel-20 to support A-IoT device synchronization to the reader. Proposal 2: The active device can complete initial frequency acquisition, i.e., identify the frequency location where the R2D synchronization signal transmitted, by searching only a pre-defined limited set of frequency locations. Proposal 3: R2D synchronization signal should be appended after SIP for timing and frequency synchronization. Bcast info Proposal 4: The reader identification information, which indicates the identifier of the reader, should be carried by R2D synchronization signal. Proposal 5: SFN information should not be broadcast to the device. |
| Xiaomi | Init FA Proposal 1: For frequency acquisition, one frequency point can be predefined for synchronization signal transmission. Bcast info Proposal 2: The broadcast information can be transmitted together with synchronization signal or independently. |
| OPPO | Init FA Proposal 1: Periodic R2D synchronization signal should be studied as baseline in Rel-20 A-IoT. Proposal 2: CFO calibration signal and broadcast information should be as a part of periodic R2D synchronization signal. Proposal 3: The following functionalities should be supported for designing periodic R2D synchronization signal, • CFO calibration • Timing acquisition • Providing necessary A-IoT system information Proposal 4: Study a R2D periodic synchronization signal composed of at least a SIP, a Sine wave, a CAP and a broadcast information. Proposal 5: Sync raster definition should be avoided in Rel-20 A-IoT. Observation 1: It is infeasible for A-IoT devices with up to 1000 ppm residual SFO to operate under NR-level synchronization requirements. Proposal 6: The principle of resource indication/allocation in Rel-19 A-IoT can be reused. • The broadcast information in each periodic R2D synchronization signal indicates X candidate resources in time domain and Y candidate resources in frequency domain for R2D and/or D2R transmission. • Further study the size of time domain guard interval and frequency domain guard band between neighbouring transmission resources. |
| ZTE Corporation, Sanechips | Init FA Proposal 1: For the initial access of an A-IoT device, consider the following steps • the device first receives a synchronization signal to achieve initial timing and frequency synchronization. • It then receives the broadcast information to refine synchronization and obtain paging-related parameters. • Based on the indicated information, the device subsequently monitors paging messages as indicated by the broadcast configuration. Bcast info Proposal 2: The following information can be considered included in the broadcast information: • Paging-related information • Timing information, e.g., SFN • Frequency information for FDM • UL reference frequency position • DO-A resource related information, i.e., time and frequency resource information • Parameters of R2D L1 control, e.g., chip duration, repetition number, code rate • Cell/Reader/Transaction ID. Proposal 3: Use PRDCH to carry the broadcast information. Proposal 4: Periodic broadcast information can be considered. Proposal 5: The broadcast information can be transmitted close in time to the synchronization signal and the period of broadcast information can be larger than or equal to the period of synchronization signal. Proposal 6: The bandwidth of broadcast information is the same as that of the synchronization signal. |
| Panasonic | Init FA Proposal 1: The periodic R2D synchronization signal should be used. How exactly periodic is FFS. Bcast info Proposal 2: The broadcast signal should be carried over each active subcarrier. Proposal 3: The broadcast signal should carry essential access parameters, such as the reader ID and the power control parameters. |
| LG Electronics | Observation #1: It is beneficial to transmit broadcast information periodically (with a time offset from a periodic synchronization signal), and to use the same frequency resource for both the synchronization signal and the broadcast information to enable efficient and synchronized reception at the device. Observation #2: It is preferred to reuse existing PRDCH for broadcast information rather than introducing a new R2D channel dedicated for the broadcast information. |
| InterDigital, Inc. | Proposal 10: Support synchronization raster for initial access procedure. |
| MediaTek Inc. | Init FA Observation 1: Implementing a cell search-like procedure would lead to increased complexity for ambient IoT devices. Proposal 1: Avoid a cell search-like procedure that requires searching for both timing and frequency points Bcast info Proposal 2: The broadcast information is expected to be received right after the initial frequency acquisition |
| NTT DOCOMO, INC. | Init FA Proposal 1: For initial access, study necessity and feasibility on frequency acquisition procedure based on R2D signal. Observation 1: For initial access, - Frequency acquisition from multiple candidate frequency is feasible assuming IF/ZIF receiver with IF/BB filter for device 2b/C. - Frequency acquisition from multiple candidate frequency is necessary to enable flexible operation of A-IoT frequency location in NR band considering co-existence with legacy NR. Bcast info Proposal 2: For initial access, study the procedure to receive broadcast information. - Whether SFO/CFO calibration is necessary before broadcast information reception in addition to the frequency acquisition signal reception. Ø i.e., whether broadcast information can be decoded with initial SFO/CFO or not, or whether its clock can be sufficiently calibrated based on frequency synchronization signal. - If SFO/CFO calibration after frequency acquisition signal reception is necessary before broad cast information reception, how to determine the location of SFO/CFO calibration signal (SCS/CCS). Ø E.g., association with FAS Proposal 3: For the study of broadcast information, following information should be considered. - Information for reader differentiation/identification - Resources of first D2R transmission for DO-A - Resources of R2D signal for SFO/CFO calibration and/or time tracking - FFS: whether broadcast information can trigger random access Proposal 4: For the study of broadcast information, PRDCH should be the baseline to carry the broadcast information. - FFS: The PRDCH which carries broadcast information is scheduled by R2D L1 control Proposal 5: For the study of broadcast information, periodic transmission should be considered. For the study of broadcast information, following aspects should be considered for the periodicity. - NW overhead and NW energy saving - feasible device sustainable operation time Observation 2: For the periodicity of broadcast information, up to a few hundreds ms can be feasible depending on energy storage assumption. Proposal 6: For the study of broadcast information, time and frequency resources for transmission of the broadcast information is determined in relation to R2D synchronization signal for frequency acquisition. - Frequency resource of the broadcast information is the same as the R2D signal. - FFS: The time gap between the R2D sync and the broadcast information |
| CEWiT | Proposal 7: Study periodic broadcast information. Proposal 8：Study the at least following information to be included in broadcast information, • Timing information • Periodic random access resource configuration, e.g. periodicity, offset, available resource in each periodicity. • Cell ID related information • Power related information e.g., reader Tx/ Rx power • Information related to frequency resource |
| ETRI | Bcast info Proposal 7: The broadcast information include at least Reader identification information (e.g., Reader ID) and time synchronization information (e.g., System Frame Number). Proposal 8: The PRDCH is used to carry the broadcast information, where a broadcast destination ID is used to indicate that the transmission is intended for all devices. Proposal 9: The broadcast information is transmitted periodically. Proposal 10: The broadcast information is multiplexed with the synchronization signal in the time domain. |
| Fraunhofer HHI, Fraunhofer IIS | Proposal 3. Adopt periodic R2D synchronization signaling as the baseline approach for A-IoT Device Types 2b/C. Adopt periodic R2D synchronization signaling as the baseline approach for A-IoT Device Types 2b/C. Proposal 4. Introduce a periodic R2D synchronization signal carrying MIB-like information to support coarse timing and frequency acquisition for Device Types 2b/C. |

DO-A

Previous agreements

Agreement (RAN1#122)

Study the following aspects to support DO-A traffic (periodic and event-triggered) for Device 2b and for Device C in Rel-20.

- Timing and frequency synchronization for DO-A transmission

- Resource allocation/determination method for DO-A transmission

- Configuration/signaling of necessary information

Agreement (RAN1#122bis)

For resource allocation method for the first D2R transmission for DO-A for Device 2b and for Device C, study following options considering feasibility:

- Option 1: Periodic D2R resource allocation

- Option 2: Aperiodic D2R resource allocation

Agreement (RAN1#122bis)

For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, study at least following options considering feasibility:

- Option 1: necessary information provided by broadcast information

- Option 2: necessary information provided by a paging-like R2D message

- Option 3: necessary information derived from a synchronization signal and provided by broadcast information

Agreement (RAN1#122bis)

Study whether necessary information for the first D2R transmission for DO-A for Device 2b and for Device C is provided periodically and/or aperiodically by Reader.

Agreement (RAN1#122bis)

For the content of the first D2R transmission for DO-A for Device 2b and for Device C, study the following options:

- Option 1: Msg1-like content

- Option 2: DO-A data payload

  - FFS: with or without device ID

- Option 3: DO-A scheduling request

The content of the first D2R transmission for DO-A

Summary

On the Option 1/2/3 of the following agreement,

Agreement (RAN1#122bis) For the content of the first D2R transmission for DO-A for Device 2b and for Device C, study the following options: - Option 1: Msg1-like content - Option 2: DO-A data payload - FFS: with or without device ID - Option 3: DO-A scheduling request

6 sources (HW, E///(only), OPPO, IDT, Quectel, IIT) support Option 1,

5 sources (SPRD, ZTE, Apple(baseline), IDT, Sharp) support Option 2,

and 9 sources (CATT, ZTE, Panasonic, MTK, DCM, TCL, IIT, CEWiT, Tejas) support Option 3.

Pros and cons of each of the options are captured in the Table below:

|  | Recommended scenarios | Pros | Cons |
| --- | --- | --- | --- |
| Option 1 Msg1-like content | - low probability of DO-A traffic triggering [ZTE] | - Lower latency [LGE] - Reuse existing RA procedure | - Limited access capability [LGE] - High signaling overhead [DCM] |
| Option 2 DO-A data payload | - low probability of DO-A traffic triggering [ZTE] - single-device with period DO-A traffic [ZTE] | - The most efficient approach for short DO-A messages typical in ambient IoT, minimizing latency and signaling overhead [Apple] - Low latency [LGE] - Low signaling overhead [DCM] - Potential risk of large resource waste | - Contention resolution methods should be studied considering contention-based resource allocation. [DCM] |
| Option 3 DO-A scheduling request |  | - Minimum resource overhead by using a common access request sequence/resource for multiple devices [ZTE] - Low power [ZTE] - Low resource waste [ZTE] | - Contention resolution methods should be studied considering contention-based resource allocation. [DCM] - High signaling overhead [DCM] |

[H] Proposal 4.2.1a

Adopt the following table format for capturing in TR 38.769 the pros and cons of the for the content of the first D2R transmission for DO-A for Device 2b and for Device C.

|  | Recommended scenarios | Pros | Cons |
| --- | --- | --- | --- |
| Option 1 Msg1-like content | - low probability of DO-A traffic triggering [ZTE] | - Lower latency [LGE] - Reuse existing RA procedure | - Limited access capability [LGE] - High signaling overhead [DCM] |
| Option 2 DO-A data payload | - low probability of DO-A traffic triggering [ZTE] - single-device with period DO-A traffic [ZTE] | - The most efficient approach for short DO-A messages typical in ambient IoT, minimizing latency and signaling overhead [Apple] - Low latency [LGE] - Low signaling overhead [DCM] - Potential risk of large resource waste | - Contention resolution methods should be studied considering contention-based resource allocation. [DCM] |
| Option 3 DO-A scheduling request |  | - Minimum resource overhead by using a common access request sequence/resource for multiple devices [ZTE] - Low power [ZTE] - Low resource waste [ZTE] | - Contention resolution methods should be studied considering contention-based resource allocation. [DCM] - High signaling overhead [DCM] |

| Company | Y/N | Comments |
| --- | --- | --- |
| LG | Y |  |
| TCL | Y | There are two types for DO-A traffic, i.e., periodic reporting and non-periodic reporting (i.e., occasional emergency reporting). For periodic reporting, option 1/2 is considered. And for non-periodic reporting, option 3 can be considered. |
| CMCC |  | In our contribution, we propose the following procedure for DOA service based on option 1, but the Msg1 is different from R19. Since reusing the R19 Msg1+Msg2 collision resolution procedure for outdoor active device random access leads to high collision probability and excessive D2R resource consumption. For R20 the random access procedure should be updated, - Introduce a preamble based Msg1 that simply informs the reader “some device requests access”, and no ID is carried. This can shorten the Msg1 length and reduce the collision probability. - The device identity is carried in Msg3. - Contention is resolved by Msg4. - Additionally, supporting CDM for Msg1 further lowers the preamble collision probability. Proposal 8: Study for R20 a random access procedure update, including 1)sequence-based Msg1 is used and carries no Random ID, 2) contention is resolved via Msg3 and Msg4. Proposal 9: Study CDM for Msg1 to reduce random access collision. |
| ZTE,Sanechips |  | Recommended scenarios; Subsequent procedures after the first D2R transmission; Observations. Option 1 Msg1-like content; - Event-triggered or periodic - Low probability of DO-A traffic triggering; - CBRA; - Reuse existing RA procedure - High signaling overhead - Collisions between the first D2R transmissions lead to resource waste. - Idle pre-allocated AOs lead to resource waste. Option 2 DO-A data payload; - Single-device with period DO-A traffic; - CFRA; - The most efficient approach for a single device with fixed period - Low latency - Low signaling overhead - Collisions between between the first D2R transmissions lead to large resource waste - Not suitable for multiple devices with Event-triggered DO-A traffic. Option 3 DO-A scheduling request; - Event-triggered or periodic - Single or multiple devices; - Paging + CBRA; - Minimum resource overhead by using a common access request sequence/resource for multiple devices - Low power consumption - Low resource waste - Insensitive to collisions between the first D2R transmissions. - Low signaling overhead. |
| Panasonic |  | Regarding the contention resolutions, we assume that is common for all the options, as shared resources are utilized. So, we suggest removing it from the Cons of Option 2 and Option 3. |
| Samsung |  | The important point to consider is that devices can support combinations of these options. For example, after completing the initial access using Option 1, a device may still perform DO-A using Option 2 for subsequent transmissions. In addition, the down-selection between Option 2 and Option 3 may fall outside the scope of RAN1. What matters is how the resources for these transmissions are provided and the subsequent procedures associated with them. First D2R transmission for DO-A; Recommended scenarios (event-triggered or periodic); Subsequent procedures after the first D2R transmission; Observations. Option 1 Msg1-like content; - Event-triggered; - The device completes the RA procedure (e.g., Msg2/Msg4) and registration with the reader.; - Subsequent DO-A data transmission can occur after registration is completed. - First D2R transmission resources are obtained from broadcast information.. Option 2 DO-A data payload; - Event-triggered - Periodic; - Event-triggered: Additional DO-A transmissions may occur to improve contention success probability. - Periodic: The device transmits DO-A based on configured periodicity.; - Broadcast information, paging-like R2D messages, or device-specific R2D messages can provide DO-A transmission resources.. Option 3 DO-A scheduling request; - Event-triggered; - The device receives data-transmission resources (grant) from the reader and can transmit additional D2R data.; - Broadcast information, paging-like R2D messages, or device-specific R2D messages can provide DO-A transmission resources.. |
| Sharp |  | A correction for FL’s summary, we propose to exclude(not support) option2. |
| NTT Docomo |  | Recommended scenarios; Subsequent procedures after the first D2R transmission; Observations. Option 1 Msg1-like content; - Event-triggered or periodic; - CBRA; - High signaling overhead - Longer latency - Existing random access procedure as starting point. - Applicability needs to be studied if DO-A data can only be transmitted after random access, i.e., after devices are registered to newtork.. Option 2 DO-A data payload; - Periodic; - CBRA; - Waste of the pre-allocated resources if device does not have frequent DO-A traffic to transmit considering event-triggered DO-A traffic. - Contention resolution methods should be studied. Option 3 DO-A scheduling request; - Event-triggered or periodic; - CBRA; - High signaling overhead - Longer latency - Contention resolution methods should be studied. |
| Huawei, HiSilicon |  | First D2R transmission for DO-A; Recommended scenarios (event-triggered or periodic); Subsequent procedures after the first D2R transmission; Observations. Option 1 Msg1-like content; - Periodic; - The subsequent procedure follows the random access procedure defined in Rel-19, where a random ID response message is transmitted by the reader.; - Reuse of the Rel-19 random access procedure does not require further discussions and specification effort. - It also allows the contention resolution mechanism defined in Rel-19 to be used, where the random ID response message echoes the RN16 transmitted in Msg1.. Option 2 DO-A data payload; - Periodic; - CFA-like procedure; - It can only be applied in the CFA-like procedure, which will limit the access capability and increase the collision probability.. Option 3 DO-A scheduling request; - Periodic; - DO-A specific paging message, followed by Msg1-like transmission.; - An additional D2R message, i.e., DO-A scheduling request, and a corresponding additional R2D paging message, i.e., DO-A specific paging message carrying the resource allocation information, are required. Therefore, the resource utilization in the access procedure is lower.. |

Timing and frequency synchronization for DO-A transmission

Summary

Observations & proposals related to the time and frequency synchronization for DO-A transmission.

- Allocated resources should closely follow a synchronization signal / CFO calibration signal. (FW)

- Rely on the new periodic or semi-periodic sync signal for DO-A (SPRD, SS, Lenovo)

- Rely on CFO calibration signal (Quectel)

- rely on periodic R2D sync signal and SIB for initial reader search, and afterwards the SIB can further configure periodic SS associated with the potential D2R transmission in the periodic AO/AR resources (QC)

On triggering the R2D sync signals, two methods were studied with the corresponding resource allocation methods and procedures. (Lenovo)

- Reader-initiated Sync-R2D transmission

- Device-triggered Sync-R2D transmission (upon DO-A traffic intention)

R2D sync signal design is discussed in sub-agenda 10.3.2.1. In this sub-agenda, procedures related to DO-A, if any, can be discussed. From this perspective, FL would like to ask companies which aspects need to be captured in the TR in relation to the timing and frequency synchronization for DO-A transmission. FL thinks that those observations and proposals in the summary including the procedure proposed by Lenovo may be relevant and therefore can be captured in the TR.

[L] Question 4.2.2a

Which aspects need to be captured in the TR in relation to the timing and frequency synchronization for DO-A transmission? FL thinks that those observations and proposals in the summary including the procedure proposed by Lenovo may be relevant and therefore can be captured in the TR.

| Company | Comments |
| --- | --- |
| CMCC | We think based on the new periodic sync signals, device 2b/C are able to support a synchronized system. They get the timing and frequency synchronization from syn signal and broadcast information which provide SFN-like information. |
| ZTE,Sanechips | The D2R resource for the first D2R transmission in DO-A is positioned following a synchronization signal / CFO calibration signal, with the timing reference derived from the synchronization signal and frequency synchronization derived from CFO calibration signal. |
| Huawei, HiSilicon | In our understanding, the timing and frequency synchronization for DO-A transmissions is obtained by the preceding periodic synchronization signal (coarse frequency synchronization and timing synchronization), the broadcast information (for reader ID and SFN) and the CFO calibration signal (fine frequency synchronization and LO frequency calibration). |

Resource allocation/determination for the first D2R transmission for DO-A

Summary

Views on the following two options for the periodicity of resource allocation for the first D2R transmission for DO-A:

- Option 1: Periodic D2R resource allocation

- Option 2: Aperiodic D2R resource allocation

Support Option 1: SPRD, CMCC, HW, E///, OPPO, Panasonic,

Support Option 2:

Support Option 1 and 2: Nokia, SS. ZTE, Apple, IDT, QC,

Sources report that periodic … is necessary

Cons of aperiodic

- Sources report that aperiodic requires more standard effort and consumes more energy before device can send DO-A traffic (CMCC,

- If the resource allocation information is provided aperiodically, it is possible that the number of resources indicated is less than the actual number of devices wanting to transmit D2R transmissions for DO-A, and is not aware of when the next SIB-like transmission containing the resource allocation information can be expected, resulting in a decreased access capability due to the limited resources. (HW,

- For the resource allocation method for the first D2R transmission for DO-A, periodic allocation provides predictable resource availability but may waste resources when device activity is low, whereas polling-based aperiodic allocation improves efficiency by assigning resources only when DO-A traffic is detected (LGE)

Issues when resource is shared b/w DO-A and DO-DTT/DT (NEC)

Resource determination

- With reference to the end of P-SS (Periodic sync signal): SPRD, CT (for Case B1), OPPO, IDT, Quectel, CT

- single-tone sinewave in the P-SS as the reference point in F domain (OPPO)

- also study aperiodic resource allocation multiplexed with other D2R transmissions (IDT)

- With reference to the end of P-SS or the PRDCH for BI: NEC,

- With reference to the end of P-SS and CS (single-tone sine wave)

- Xiaomi: resource pool-based method and Msg3-associated method

FL would like to check if the following proposal is agreeable based on the majority view from submitted contributions.  Otherwise, text proposals for capturing the contradicting views on the periodicity of resource allocation for the first D2R transmission for DO-A will be prepared to be captured in TR 38.769.

[H] Proposal 4.2.3a

For resource allocation method for the first D2R transmission for DO-A for Device 2b and for Device C, Option 1 (Periodic D2R resource allocation) is feasible and necessary.

| Company | Y/N | Comments |
| --- | --- | --- |
| LG | Y |  |
| vivo | Y | We also support periodic D2R resource for the following reasons: aperiodic D2R resource cannot support periodic DoA traffic type. Otherwise, R2D message needed to be periodically transmitted, causing the unnecessary signaling overhead. How does a reader know the timing for scheduling D2R resources for event trigger DoA traffic by using paging-like R2D message is questionable, so periodic D2R resource should be the choice. |
| CMCC | Y | As summarized by FL in 4.1.2, 6 sources propose Periodic D2R resource configuration for DO-A in broadcast information. |
| ZTE,Sanechips |  | The aperiodic resource allocation approach enables dynamic and on-demand resource assignment for DO-A traffic. This approach enhances the flexibility of resource allocation by allowing the network to allocate DO-A resources based on actual traffic requirements and reduce unnecessary resource overhead and reduce the impact of DOA traffic on DT traffic. It is recommended to capture the observations of both periodic and aperiodic DO-A resource allocation in the TR. The selection of the two methods can be considered during WI phase. Proposal Both Option1(periodic D2R resource allocation) and Option2(Aperiodic D2R resource allocation) are considered for DO-A in R20 AIoT SI Note: Further down-selection in WI phase is not precluded. |
| Panasonic | Y |  |
| Lenovo |  | We support the study of both periodic (option 1) and aperiodic (option 2) D2R resource allocation. |
| Samsung | Y | We consider periodic D2R resource allocation an efficient approach because DO-A is autonomously initiated by the device, and periodic resources provide predictable transmission opportunities. This approach supports key scenarios: Event-triggered: RA-like initiation using periodic resources Periodic: Direct DO-A transmission without RA |
| IDCC |  | We support a proposal along the lines of ZTE’s proposal. It is too early to dismiss aperiodic resource allocation. It seems companies may have different understanding of what aperiodic resource allocation means. In one understanding, the reader may allocate periodic resources (with low overhead) where devices may send SR-like signal to indicate data availability. Then, the reader may allocate resources as needed. We think both periodic and aperiodic resource allocation are useful in different scenarios. For example, for event triggered traffic, aperiodic resource allocation can reduce the overhead. |
| Sharp | Y | Periodic resource allocation can be baseline for DOA |
| NTT Docomo | Y |  |
| Huawei, HiSilicon | Y |  |

Necessary information for the first D2R transmission for DO-A

Summary

TBD

FL will take some time to compile the list of proposed necessary information for the first D2R transmission for DO-A as was done in Clause 4.1.2. Then, a text proposal capturing the list of the proposed necessary information and related observations will be prepared to be included in TR 38.769. Companies can still make a comment on this direction, or if views are not correctly captured.

[H] Proposal 4.2.4a

- TBD

| Company | Comments |
| --- | --- |
| CMCC | The time resource configuration includes periodicity, time offset, and available resource within each period. And the frequency resource configuration includes candidate frequency locations in each time occasion. |
| ZTE,Sanechips | The information for the first D2R transmission for DO-A depends on the specific details of the DO-A solution, and can be discussed later. |
| Huawei, HiSilicon | This is list of necessary information is understood to be carried by the broadcast information, and in this case, there is no need to replicate the same list captured under section 4.1.2. It would be adequate to indicate from that list which would be the necessary information for DO-A. |

Provisioning of the necessary information for the first D2R transmission for DO-A

Summary

On the Option 1/2/3 of the following agreement,

Agreement (RAN1#122bis) For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, study at least following options considering feasibility: - Option 1: necessary information provided by broadcast information - Option 2: necessary information provided by a paging-like R2D message - Option 3: necessary information derived from a synchronization signal and provided by broadcast information

9 sources (TH, LGE, DCM, Nokia, CMCC, HW, E///, Apple, Sony,) support Option 1,

8 sources (NEC, CATT, CT, LGE, IDT, Quectel, IIT, Nokia) support Option 2,

and 6 sources (NEC, IDT, SPRD, Apple, Fraunhofer(baseline), HONOR) support Option 3.

FL will directly prepare a text proposal capturing observations on the provisioning of the necessary information for the first D2R transmission for DO-A for Device 2b and for Device C to be included in TR 38.769. Companies can still make a comment on this direction, or if views are not correctly captured.

[H] Proposal 4.2.5a

- TBD

| Company | Comments |
| --- | --- |
| vivo | We support option1. |
| ZTE,Sanechips | We support Option 1, 2 and 3 in our contribution. These three options can be considered and captured in the TR. |
| Lenovo | We support option 1, 2 and 3. |
| Sharp | We support Option 3 for periodic resource allocation, option 2 for aperiodic resource allocation |
| Huawei, HiSilicon | Although we support option 1, given the split among companies, it would make sense to capture observations from companies at this point. |

Overall step/procedures for DO-A transmission

Summary

Sources (vivo, HW, China Telecom) provide their views on the overall steps/procedures to support DO-A.

FL will directly prepare a text proposal capturing observations on the provisioning of the necessary information for the first D2R transmission for DO-A for Device 2b and for Device C to be included in TR 38.769. Companies can still make a comment on this direction, or if views are not correctly captured.

[H] Proposal 4.2.6a

- TBD

| Company | Comments |
| --- | --- |
| ZTE,Sanechips | The overall step/procedures for DO-A can be determined according to the content of the first D2R transmission for DO-A (Proposal 4.2.1a). We have described the overall steps and procedures to support DO-A in our contribution, which could be captured in the TR. |

Related proposals/observations (35)

| Source | Proposals and observations |
| --- | --- |
| FUTUREWEI | 1st D2R content Proposal 3: Capture the following observations in the TR about content of the first D2R transmission for DO-A: - Option 1: Msg1-like content o Reader can associate the content with the appropriate D2R configuration (e.g., chip rate, FEC) for subsequent D2R transmissions. o Procedure would mimic contention based access – which can simplify standardization efforts. - Option 2: DO-A data payload o Payload size can affect procedures, reliability, and resource allocation. o Large payload sizes can lead to unused resources when no device has DO-A content to transmit. o Small payload sizes can lead to unequal error detection if multiple segments are needed o Unclear how to account for different operating conditions (chip rates, FEC) in the resource allocation. - Option 3: DO-A scheduling request o Detection of energy on a common D2R resource triggers next step (e.g., option 1) o Possibly a new signal/message/procedure is needed as well as increased delay as the DO-A scheduling request is first step for access. o When the number of devices is small, there may be resource efficiency. Resource allocation/determination Proposal 4: Capture the following observations in the TR about the resource allocation method of the first D2R transmission for DO-A: - Allocated resources should closely follow a synchronization signal / CFO calibration signal so that a device can update timing / frequency loops for the first D2R transmission. The timing for the allocated resources may be based on the timing of the synchronization / CFO calibration signal(s) - Periodic resource allocation: o A reader has no control to suspend or delay the use of the allocated resources. - Aperiodic resource allocation o A device receives a R2D message indicating that the allocated resources are available. Necessary info Proposal 5: Capture the following observations in the TR about necessary information for the first D2R transmission: - The necessary information includes parameters for the D2R transmission o Chip-rate, FEC information, any power-control-related information, payload size, time domain resources, frequency domain resources, preamble/midamble information. - The necessary information includes the opportunities for the first D2R transmission resource allocation. Examples include o time offset from a synchronization signal o type of resource (periodic / one type event) o availability of resources |
| Spreadtrum, UNISOC | T/F sync Proposal 1: New periodic sync signal should be introduced in Rel-20 for supporting the synchronization of Device 2b/C for DO-A traffic. Resource allocation/determination Proposal 2: The information for resource allocation can be carried by the PRDCH of the periodic sync signal. Proposal 5: For determining the time domain of resource for DO-A, the end of periodic sync signal could be the reference point. Necessary info and its provisioning Proposal 3: Necessary information for the first D2R transmission for DO-A derived from a synchronization signal and provided by broadcast information. Proposal 4: For supporting DO-A, at least following information should be provided by reader: • Time domain resource information for DO-A • Timeline (for determining the exact location in time domain, including the timeline for determining the first time domain resource) • Interval between two time domain resource • Length of a time domain resource • Number of DO-A resource (i.e., number of TDMed DO-A resource) • Frequency domain resource information for DO-A • Frequency location of a typical resource (i.e., a reference frequency resource which can be used for determining other frequency domain resources) • Number of frequency domain resource (i.e., number of FDMed DO-A resource) 1st D2R content Proposal 6: For the content of the first D2R transmission for DO-A for Device 2b and for Device C, DO-A data payload should be supported. |
| vivo | Since these three agreements listed above highly depend on each other, we provide the analyses of the following combination cases. • Combination 1: Option 1: periodic necessary information provided by broadcast information + Option 1: Periodic D2R resource allocation. • Combination 2: Option 2: aperiodic necessary information provided by a paging-like R2D message + Option 2: Aperiodic D2R resource allocation. Note that for Option 3: necessary information derived from a synchronization signal and provided by broadcast information, in our view, it is a further step under option 1: necessary information provided by broadcast information. So, the combination case of option 3 is not separately given. Resource allocation/determination / Necessary info and its provisioning Proposal 9: Capture the pros and cons given in Table 1 of R1-2508426 into TR 38.769. 1st D2R content Proposal 10: Support to study the following three potential procedures by using different contents of the first D2R transmission of DoA traffic: Ÿ Procedure 1: using sequence-based msg1 as the first D2R transmission content and after msg2 transmission, the procedure also includes the contention resolution process. Ÿ Procedure 2: reader transmits broadcast information to indicate CG resource for device to do periodic DOA data transmission (DoA data is the first D2R transmission content). Ÿ Procedure 3: device sends scheduling request (SR) (SR is the first D2R transmission content) to request CG resource for periodic DOA data transmission. |
| Nokia | Resource allocation/determination Proposal 6: Regarding resource allocation/determination methods for DO-A, further study the following methods, including combinations of these methods: • Polling by the reader • Recurring resource configuration provided by the reader Proposal 7: For the resource allocation method for the first D2R transmission for DO-A in aperiodic manner, study a way of including a DO-A indicator within L1 control information, so that a device can skip further processing if it does not have DO-A data traffic. 1st D2R content Regarding the 3 options listed, we observe the following: While Option 1 and Option 3 have content of fixed small size, the payload size in Option 2 could be variable and large. |
| CMCC | Resource allocation/determination Observation 2：Option 2: Aperiodic D2R resource allocation for the first D2R transmission for DO-A requires more standard effort and consumes more energy before device can send DOA traffic. Proposal 3: For resource allocation method for the first D2R transmission for DO-A for Device 2b and for Device C, support Option 1: Periodic D2R resource allocation. Proposal 4: The random access configuration includes periodicity, time offset, and available resource within each period. Necessary info and its provisioning Observation 3: For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, • “Option2: necessary information provided by a paging-like R2D message” means more energy consumption before DOA traffic report than option 1, and has a risk of no enough energy for traffic report; • Option 3 is still based on option 1, and details can be further studied. Proposal 5: For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, study Option 1: necessary information provided by broadcast information |
| Huawei, HiSilicon | Necessary info and its provisioning Proposal 2: Capture the following observations in TR 38.769 on the necessary information for first D2R transmission in DO-A and how it is provided to the device: • Reader ID, the SFN and time and frequency resource allocation information for the SIB-like broadcast information is provided by the MIB-like broadcast information. • Resource allocation information for the first D2R transmission in DO-A is provided by the SIB-like broadcast information. o Information related to the time domain include the start time and the periodicity of ROs. o Information related to the frequency domain include a set of frequency resources, e.g., a list of available sub-channels. Proposal 3: Necessary information for the first D2R transmission for DO-A provided periodically by the reader is supported. Proposal 4: Capture the following observations in TR 38.769 for the periodicity of necessary information for first D2R transmission in DO-A: • The necessary information contained within the broadcast information is transmitted periodically with respect to the reader. • The periodic transmission of this information ensures that the device can receive the necessary information once it is available to receive the broadcast information and the device uses the last received MIB-like and SIB-like broadcast information to determine the necessary information. Proposal 9: Capture the following observations in TR 38.769 on the necessary information for first D2R transmission in DO-A if it is provided aperiodically: • If the necessary information is provided aperiodically, it may cause the device to miss the transmission of the broadcast information, without any knowledge of when the next broadcast information would be transmitted. • If the reader transmits the necessary information aperiodically but more frequently, the device would incur additional resource overhead. • If the reader requires an indication from the device to transmit the necessary information, it is unclear how the device can do so without any resource allocation information for such an indication to be transmitted. Proposal 10: Capture the following observations in TR 38.769 on how the necessary information for first D2R transmission in DO-A is provided to the device: • If the necessary information is provided by a paging-like R2D message (Option 2), if this message is used for polling, there would be an increase in the transmission overhead due to the additional DO-A scheduling request and another paging message to provide resource allocation information for the first D2R transmission. If the message is intended to be triggered by the device, it is unclear how the device can do so without any resource allocation information for such a trigger to be transmitted. • If the necessary information is derived from a synchronization signal and provided by broadcast information (Option 3), it is not clear how and what information can be derived from the synchronization signal and if the devices is going to use the broadcast information anyway, the motivation to derive information from the synchronization signal is unclear. Resource allocation/determination Observation 1: If the resource allocation information is provided aperiodically, it is possible that the number of resources indicated is less than the actual number of devices wanting to transmit D2R transmissions for DO-A, and is not aware of when the next SIB-like transmission containing the resource allocation information can be expected, resulting in a decreased access capability due to the limited resources. Proposal 5: Periodic D2R resource allocation for the first D2R transmission for DO-A is supported. Proposal 6: Capture the following observations in TR 38.769 for the resource allocation method for DO-A: • The resource allocation information provided in the SIB-like broadcast information is periodic for both periodic and event-triggered D2R transmissions for DO-A. • If the resource allocation information is provided aperiodically, it is possible that the number of resources indicated is less than the actual number of devices wanting to transmit D2R transmissions for DO-A, and is not aware of when the next SIB-like transmission containing the resource allocation information can be expected, resulting in a decreased access capability due to the limited resources. 1st D2R content Proposal 7: For the content of the first D2R transmission for DO-A, use of Msg1-like content is supported. Proposal 8: Capture the following observations in TR 38.769 for the content of and steps after the first D2R transmission for DO-A: • The first D2R transmission for DO-A contains Msg1-like content, which is essentially RN16. • The subsequent procedure follows the random access procedure defined in Rel-19, where a random ID response message is transmitted by the reader echoes the RN16 for contention resolution. Proposal 11: Capture the following observations in TR 38.769 for the content of the first D2R message in DO-A: • If the content of the first D2R message is DO-A payload (Option 2), it can only be applied in the CFA-like procedure, which will limit the access capability and increase the collision probability seriously. • If the content of the first D2R message is a DO-A scheduling request (Option 3), an additional D2R message, i.e., DO-A scheduling request, and a corresponding additional R2D paging message, i.e., DO-A specific paging message carrying the resource allocation information, are required. Therefore, the resource utilization in the access procedure is lower. |
| Ericsson | 1st DO-A content Proposal 5 For the content of the first D2R transmission for DO-A for Device 2b and for Device C, consider only Option 1 (Msg1-like content) in Rel-20. Resource allocation/determination Proposal 6 For resource allocation method for Option 1 (Msg1-like content) for DO-A for Device 2b and for Device C, consider only periodic Msg1 resource allocation. Necessary info and its provisioning Proposal 7 For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, the necessary information is provided by broadcast information. |
| NEC | 1st D2R content / Necessary info and its provisioning From our point of view, two candidates generated from above two agreements can be studied further. Candidate 1: option1/option 3 as content of the first D2R transmission for DO-A and option 3 for providing necessary information for the first D2R transmission for DO-A. Candidate 2: option 1/option 2 as content of the first D2R transmission for DO-A and option 2 for providing necessary information for the first D2R transmission for DO-A. Proposal 8: For DO-A traffic transmission procedure with candidate 1, CBRA should be baseline with the following two options about the first D2R transmission: • Option 1-1: Rely on the introduction of sync signal, multiple devices share same resource for transmitting scheduling request, and then monitor A-IOT paging message for Msg1 resource allocation; • Option 1-2: Rely on the introduction of sync signal, multiple devices perform first D2R transmission as Msg1 on separate resources for scheduling request, and then monitor Msg2 for Msg3 resource scheduling. Proposal 10: For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, Option 3 should be adopted as following: • TR->D for time resource determination is pre-defined and the end of the corresponding R2D sync signal or the PRDCH carrying broadcast information is used for the device to determine the time resource for the first D2R transmission. Proposal 11: Other parameters, which are for the indication of following parameters, can be indicated by the broadcast information associated with the corresponding R2D sync signal or pre-defined in the spec. • Number of time domain resource of access occasions, bit duration, frequency resource, coding and repetition scheme and the preamble/midamble related information. Proposal 12: For DO-A traffic transmission with candidate 2, a paging-like R2D message with following two design options can be considered: • Option 2-1: Introduce DO-A traffic specific paging-like R2D message (like, DO-A-paging) with new R2D message type; • Option 2-2: Reuse legacy paging message type but add traffic type indication in legacy paging message. Observation 3: For event triggered DO-A traffic, 2-step RA (Upper layer data transmitted along with Msg1) is of benefit than 3-step RA in terms of latency reduction. Proposal 13: For other necessary information potentially to be indicated by paging-like R2D message, considering following indications: • RA mechanism indication; • Period indication. Proposal 14: Study unified paging design in Rel-20 for reduced signalling of R2D. Proposal 15: The paging-like R2D message for DO-A can be periodic or aperiodic, which is based on reader’s implementation. • It is difficult and unnecessary for the device to know the exact transmission time of the dedicated paging signal for DO-A. Resource allocation/determination Proposal 9: For resource allocation/determination method for DO-A transmission in Option 1-1 and Option 1-2 (in P8) which have the R2D sync signal, at least a subset of periodic signals can have associated PRDCH to carry the scheduling information for the first D2R transmission. • If multiple resources are scheduled, TDM and/or FDM multiplexing schemes can be performed with the same maximum number of X and Y as DO-DTT; • The reference of the scheduling can be the end of the corresponding R2D sync signal or the PRDCH carrying broadcast information; • FFS how to indicate the DO-A types (periodic or event-triggered) and coverage level during the first D2R transmission. Proposal 16: The first D2R transmission for DO-A for Device 2b and for Device C is periodic or aperiodic, which has the same transmission manner as R2D sync signal (may including the broadcast information) or dedicated paging signal for DO-A. |
| CATT | Observation 5: For A-IoT device, the periodic synchronization signal/channel would consume more energy to perform synchronization which has impact on DO-A transmission, especially for Device 2b. Observation 6: The periodic resource allocation for UL DO-A transmission would bring the high energy consumption for A-IoT device and large resource overhead for reader. Proposal 13: It suggests studying the polling mechanism similar to Rel-19 inventory procedure for DO-A transmission. Proposal 14: Based on polling mechanism, the content of the first D2R transmission of DO-A for Device 2b/C is the response information of the D2R responding to polling information, which is used to indicate whether the device needs to transmit DO-A services. Proposal 15: Based on the polling mechanism, how to carry the D2R response information, and on which D2R signal or message to carry, need further study. Proposal 16: For the first D2R transmission for DO-A for Device 2b and Device C, the necessary information at least include the resource configuration information for the first D2R transmission. Proposal 17: It suggests studying the resource allocation and selection for DO-A transmission based on polling mechanism similar to the random access procedure in Rel-19. |
| China Telecom | T/F sync Observation 1: Determine a first OFDM symbol to monitoring synchronization signal is hard to be applied in A-IoT system due to high initial SFO and CFO impact. Proposal 1: Support a synchronization signal with at least time offset or period information to facilitate the network-wide synchronization mechanism. Proposal 2: Using the following formula as a starting point for the absolute temporal alignment if needed: Tmonitoring_occasion = Toffset + m * Tperiod Necessary info Proposal 3: Support a periodicity R2D message containing the common MSG1 resources to carry the necessary information for the first D2R transmission for DO-A traffic. Proposal 4: The broadcast information can be transmitted together with the synchronization signal for a better synchronization performance for the first D2R transmission for DO-A traffic. Proposal 5: If case 1 is supported, it is proposed that the end of synchronization signal reception be established as the reference point for all subsequent transmission and monitoring timelines. Proposal 6: If case 2 is supported, time offset between synchronization signal and SIB-like message should also be studied based on case 1. Proposal 7: In Rel-20, study the feasibility and indication method of the following scheduling information for device 2b/C: - T/F resource configuration for SIB-like message and/or MSG1 - SFN value - repetition index or period index of synchronization signal - time unit index (e.g., half-frame index) - frequency offset between the synchronization signal and the SIB-like message or MSG1 - cell/reader-related information (e.g., cell/reader ID, camp control) - DO/DT-related information Proposal 8: Support paging message to carry system information and a new paging-like message format to carry the system information for the first D2R transmission for DO-A traffic. DO-A access Proposal 9: Study the detailed access procedure under case 1 and case 2 for DO-A traffic, such as the scheduling information content, scheduling method and the access procedures. Proposal 10: The access procedure may contain the following steps: a. The device monitors synchronization signals (or other new R2D signals) to obtain the scheduling information for RO resources and MSG1 transmission. If case 2 is supported, these signals additionally provide the scheduling information for the SIB-like message. b. The device obtains the scheduling information based on synchronization signals (or other new R2D signals) in case 1 and based on SIB-like message in case 2. c. The device determines the potential RO resource and selects one to transmit the MSG1 transmission. d. The device acquires ACK message contained in the MSG2 as well as other necessary information. Proposal 11: Support a SR scheduling mechanism for the devices to report its data actively for DO-A traffic. Proposal 12: Support a DO-A data payload D2R transmission mechanism for the device to report its data actively for DO-A traffic. Proposal 13: Support a SR mechanism without access procedures (SR without MSG1) for the device to report its data actively for DO-A traffic. |
| Xiaomi | Proposal 3: For DO-A resources determination in Rel-20, resource pool-based method and Msg3-associated method can be studied. |
| OPPO | 1st D2R content Proposal 9: Msg 1-like content can be regarded as the baseline for the content of the first D2R transmission in DO-A traffic. Resource allocation/determination Proposal 10: Study periodic resource allocation for the first D2R transmission for DO-A in Rel-20 A-IoT. Proposal 11: The ending of the periodic synchronization signal and the single-tone sine wave in the periodic synchronization signal can be regarded as the reference point in time and frequency domain for candidate resources allocation of DO-A traffic. |
| Tejas Network Limited | Necessary info and its provisioning Proposal 1: The R2D message indicating the resources for DO-A transmission may contain the following necessary information: • R2D Message type • Transaction ID • Device ID • Time and frequency domain resources • Other necessary information Observation 1: The DO-A based traffic can be handled either by sending a DO-A specific periodic DL signal or a DO-A specific event-triggered DL signal. Proposal 2: The following method can be studied to handle DO-A traffic for critical IoT applications • Transmit a D2R energy pulse for a certain duration at a certain frequency with a certain power level as an event-trigger to indicate the reader that the device has some DO-A data to send. Proposal 3: RAN1 should study Handling of DO-A traffic in case of ongoing Inventory and/or Command procedure. |
| Samsung | Observation 1 For the registration and data transmission procedures supporting DO-A, the following directions can be observed: • Direction 1: Network-triggered registration followed by DO-A data transmission • Direction 2: Device-triggered registration followed by DO-A data transmission • Direction 3: Device-triggered registration with combined DO-A data Resource allocation/determination Proposal 1 RAN1 to study resource allocation method for the first and the subsequent D2R transmissions for DO-A transmission. Proposal 2 For providing necessary information for the D2R transmission for DO-A for Device 2b and for Device C, study the following options: • Option 1: necessary information provided by broadcast information • Option 2: necessary information provided by a paging-like R2D message • Option 3: necessary information derived from a synchronization signal and provided by broadcast information • Option 4: necessary information provided by a device-specific R2D message Proposal 3 The necessary information for the D2R transmission(s) for DO-A for Deivce 2b and for Device C is provided by the reader either periodically or aperiodically. Proposal 4 For resource allocation method for the D2R transmission(s) for DO-A for Device 2b and for Device C, study periodic D2R resource allocation. Proposal 5 For event-triggered DO-A transmission, the device randomly selects one resource from the configured periodic resource and initiates DO-A transmission via RA. Proposal 6 For periodic DO-A transmission, the device transmits DO-A data directly using the configured periodic resource, without RA. Proposal 9 Study the following aspects of time domain configuration for DO-A: • Validity of DO-A configuration (e.g., the entire reporting period, maximum number of reports, etc.) Proposal 10 Study on PHY mechanisms for differentiated handling of event-triggered and periodic DO-A should be considered for Rel-20 to ensure higher reliability for critical communications. T/F sync Proposal 7 Study locally synchronous system for DO-A support, where PRDCH-SYNC is semi-periodically transmitted and provides synchronization and system-information-like information for DO-A transmission. Proposal 8 Study PRDCH-SYNC including information provided in the payload. |
| ZTE Corporation, Sanechips | Resource allocation/determination Proposal 7: For resource allocation for the first D2R transmission for DO-A traffic, both periodic and aperiodic methods should be considered in Rel-20 A-IoT. 1st D2R content Proposal 8: For the first D2R transmission for DO-A, access request sequence and DO-A data payload can be considered for the following different cases: • Access request sequence for multiple devices • DO-A data payload for single device with period DO-A traffic |
| Transsion Holdings | Proposal 1 It is recommended that periodic D2R resource allocation can be considered. Proposal 2 It is recommended that necessary information for the first D2R transmission for DO-A provided by broadcast information. Proposal 3 It is recommended that necessary information for the first D2R transmission for DO-A is provided periodically by reader. |
| Panasonic | Proposal 5: Support periodic resources for DO-A transmission within a valid time period, indicated by the reader or preconfigured for the device. Proposal 6: Support sending scheduling requests for the first DO-A transmissions. |
| LG Electronics | 1st D2R content Observation #9: For the content of the first D2R transmission for DO-A, the following observations are made: - Option 1 (i.e., Msg1-like content): Uses Msg1-like content for first D2R transmission, enabling contention-based access but increasing latency due to integrated inventory. - Option 2 (i.e., DO-A data payload): Directly transmits DO-A data payload for low latency, but requires prior inventory and resource configuration. - Option 3 (i.e., Scheduling Request (SR)): Starts with a small SR for resource efficiency, but risks collisions if multiple devices transmit payloads simultaneously. Resource allocation/determination Observation #10: For the resource allocation method for the first D2R transmission for DO-A, periodic allocation provides predictable resource availability but may waste resources when device activity is low, whereas polling-based aperiodic allocation improves efficiency by assigning resources only when DO-A traffic is detected. Necessary info and its provisioning Observation #11: For providing necessary information for the first D2R transmission for DO-A, broadcast information or paging-like messages are preferred for delivering resource information, while synchronization signal-based delivery is less desirable due to increased device complexity. Observation #12: For the period of necessary information for the first D2R transmission for DO-A, the necessary information can be provided periodically or aperiodically, and the choice depends on how efficiently the reader aims to utilize R2D resources. |
| Lenovo | T/F sync Proposal 1: A dedicated Sync-R2D is required to support time/frequency synchronization for DO-A traffic. Proposal 2: The Sync-R2D structure could include at least two key components: • SIP is used to identify the start of the Sync-R2D transmission, • R2D signal is used to enable devices to achieve CFO calibration and/or time/frequency synchronization. Proposal 3: For Sync-R2D transmissions, two methods can be considered: • Reader-initiated Sync-R2D transmission, • Device-triggered Sync-R2D transmission (upon DO-A traffic intention). Proposal 4: In case of reader-initiated Sync-R2D transmission, the following resource allocation can be considered for Sync-R2D: • Dynamic resource allocation and indication by reader • Predefined periodic resource allocation Proposal 5: In case of device-triggered Sync-R2D transmission, a dedicated D2R (Req-D2R) is required for device to send a time/frequency synchronization request. Resource allocation/determination Observation 1: DO-A traffic’s unpredictable arrival and ±90 s timing drift render conventional scheduled resources ineffective, demanding a new collision-robust resource allocation method. Proposal 6: Dedicated band or broadcasting a set of resource is considered for resource allocation in supporting DO-A traffic. |
| ROBERT BOSCH GmbH | Observation 4: To allocate resources for periodic D2R, there is a need for iterative resource allocation by reader. Proposal 6: An ambient IoT device can request periodic resources from a reader providing necessary information like periodicity of D2R traffic using the first message of random-access procedure. Proposal 7: Reader can allocate periodic sets of resources for D2R transmissions of ambient IoT devices to transmit autonomously periodic DO-A traffic. Proposal 8: Paging message can be used to allocate periodic resources. There could be two options: Option 1: Paging message allocates resource for an ambient IoT device for periodic D2R transmissions. Option 2: Paging message may broadcast information about the sets of resources to be used for periodic D2R to all ambient IoT devices. Observation 5: The contention-based random-access procedure is not efficient for periodic D2R of DO-A traffic. Proposal 9: Reader may configure devices to use the contention-free or contention-based random-access based on the type of DO-A traffic. Proposal 10: An ambient IoT device may start contention-based random access to request periodic resources for DO-A. Observation 6: An ambient IoT device with aperiodic DO-A does not need to wait for the next paging occasion for the transmission of its traffic. Proposal 11: Paging message may carry information for the transmission of the first aperiodic D2R message of DO-A traffic. |
| Apple | Resource allocation/determination Observation 1: Periodic resource allocation is efficient for DO-A traffic with predictable patterns, eliminating collisions and providing deterministic latency Observation 2: Aperiodic resource allocation is more applicable for event-triggered DO-A traffic where transmission timing is unpredictable, using contention-based access to shared resource pools Proposal 1: Support both periodic and aperiodic resource allocation for DO-A: - Periodic allocation for devices with regular reporting patterns, with resource configuration provided by reader - Aperiodic allocation using contention-based access for event-triggered traffic, with contention resource pools configured via broadcast information Necessary info and its provisioning Proposal 2: Prioritize broadcast information (Option 1) as the baseline for providing necessary information for DO-A - with the information derived from synchronization signals (Option 3) as a complementary approach to reduce broadcast overhead 1st D2R content Observation 3: Direct data payload transmission (Option 2) provides the most efficient approach for short DO-A messages typical in ambient IoT, minimizing latency and signaling overhead. Proposal 3: Support DO-A data payload (Option 2) as the baseline for first D2R transmission content, with device ID included for applications requiring acknowledgment |
| InterDigital, Inc. | Resource allocation/determination Proposal 1: For resource allocation method for the first D2R transmission for DO-A for Device 2b and for Device C, both periodic D2R resource allocation and aperiodic D2R resource allocation are supported with furth study on: • Periodic resource allocation with reference to an R2D periodic synch signal • Aperiodic resource allocation multiplexed with other D2R transmissions 1st D2R content Observation 1: Rel-19 msg1 needs more resources than a sequence-based msg1 since sequences may be multiplexed on the same time/frequency resources. Observation 2: Scheduling request could be a single sequence that is transmitted by any device to indicate existence of DO-A data. Proposal 2: For the content of the first D2R transmission for DO-A for Device 2b and for Device C, support: • Option 1: Msg1-like content (FFS: msg1 is similar to Rel-19 or sequence-based) • Option 2: DO-A scheduling request Necessary info and its prov Proposal 3: For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, the following options are beneficial: • Option 2: necessary information provided by a paging-like R2D message • Option 3: necessary information derived from a synchronization signal and provided by broadcast information Proposal 4: A D2R resource availability is associated with a measurement (e.g., an RSRP measurement) and/or a device attribute (e.g., device type, CFO, etc.) Proposal 5: The reader can indicate resource availability status. Proposal 6: The reader can select a subset of devices to access a group of resources based on a device measurement and/or device attribute. |
| MediaTek Inc. | Observation 2: Incorporating code domain multiplexing for the DO-A data payload would increase system complexity and negatively impact decoding performance. Proposal 3: The first D2R transmission for DO-A, i.e., a DO-A request signal, should include the random ID or device ID. Observation 3: The DO-A procedure design is highly related with the DO-A traffic characteristics. Proposal 4: Support further study of DO-A traffic characteristics, e.g., DO-A interval, etc. |
| Sharp | Resource allocation/determination Proposal 1: For Device 2b and Device C, both periodic and event-triggered DO-A traffic can be supported by using an autonomously triggered D2R transmission to trigger A-IoT paging. l The D2R transmission does not carry any device-specific information. l The D2R transmission can be performed at any time within a time window allocated by a preceding R2D transmission. Proposal 3: Study whether the resource allocation for the first D2R transmission should be shared for two device types or can be provided separately. 1st D2R content Proposal 2: For the content of the first D2R transmission for DO-A for Device 2b and for Device C, l RAN1 to consider excluding Option 2: DO-A data payload. |
| Quectel | T/F sync Proposal 4: Ÿ A periodic synchronization signal same as CFO calibration signal can be used to provide timing and frequency synchronization can be studied for DO-A traffics for device 2b and device C. 1st D2R content Proposal 5: For the first D2R transmission for DO-A traffic for device 2b/C, Ÿ The resource allocation information for the first DO-A transmission is provided by a paging-like R2D message. Ÿ Support the content of the first D2R transmission for DO-A is Msg1-like content, i.e., Option1. Resource allocation/determination Proposal 6: For DO-A traffics, Ÿ Device derives the location of the first DO-A transmission according to the allocation information in paging-like message, and a reference time point, for example, the end of time resource of periodic sync signal. |
| Qualcomm Incorporated | Observation 1: - For periodic DO-A traffic, intervals may vary according to the traffic pattern, such as every few minutes, and latency requirements may be up to several tens of seconds. - For event-triggered DO-A traffic, stricter latency requirements typically apply, for example, responding to a request within 30 seconds. Resource allocation/determination Observation 2: - The periodic resource allocation is essential for DO-A because: • DO-A traffic cannot rely on aperiodic paging or polling, which occurs unpredictably. • Rel-19’s “A-IoT paging” mechanism is insufficient for DO-A traffic and the paging-based D2R resources is “not applicable at least for the first D2R transmission for DO-A traffic” as in TR38.769. - The periodic resources allow devices to: • Predict transmission opportunities without continuous monitoring. • Schedule wake-up times for R2D monitoring occasions (e.g., synchronization signals, paging). • Harvest energy during sleep intervals to extend device availability and support autonomous DO-A transmissions. - The periodic resources can be allocated for access occasions with random access or access requests without random access, respectively. Observation 3: - The R2D monitoring occasion can be configured for SS before the periodic D2R resources for the first DO-A transmission. - The R2D monitoring occasion can also be configured for potential paging/polling to configure D2R resources, which can be shared for DO-A and DO-DTT/DT to improve resource utilization. - After the first DO-A transmission using the periodic resources, aperiodic resources can be additionally configured by on-demand polling/paging for the following DO-A transmissions. Proposal 1: For support of DO-A traffic, - Periodic resources are needed at least for the first DO-A transmissions. • Periodic Access occasion (AO) resources for device random access to support periodic DO-A traffic. • Periodic Access request (AR) resources for device common request to support for event-triggered DO-A traffic. - Aperiodic resources can be additionally configured by on-demand polling/paging for the following DO-A transmissions. - Consider resource sharing for DO-A and DO-DTT/DT to improve resource utilization. T/F sync Observation 4a: - For initial reader searching, the R2D periodic synchronization signal is transmitted by using predefined frequency location (e.g., sync raster points) and time intervals. - The R2D periodic synchronization signal is required to reach the devices within the reader coverage and further study the sequence length, number of sequences to differentiate at least the adjacent readers. Observation 4b: - After initial reader searching and SIB acquisition, the SIB can configure the R2D periodic synchronization signal associated with periodic D2R resources, where R2D SS is required for time/frequency synchronization after switching from SLEEP to ON mode. Proposal 2: - For initial reader searching, periodic R2D synchronization signal (SS) is needed for time and frequency synchronization and periodic system information (SIB) detection. • Further study the frequency locations (e.g., sync raster points), time intervals and sequence length/number for periodic R2R SS for device 2b and device C. - After initial reader searching and SIB acquisition, the SIB can further configure periodic SS associated with the potential D2R transmission in the periodic AO/AR resources. Necessary info and its provisioning Proposal 3: For support of DO-A traffic, periodic system information (SIB) can provide necessary information including at least: - Configuration of periodic AO resources - Configuration of periodic AR resources - Configuration of frequency locations for anchor/non-anchor carriers - Configuration of periodic R2D monitoring occasions for SS - Configuration of periodic R2D monitoring occasions for potential polling/paging messages - Configuration of power control information, e.g., for different coverage levels - Configuration of L1 control information transmission, e.g., for different coverage levels |
| NTT DOCOMO, INC. | Resource allocation/determination Observation 7: For resource allocation method for the first D2R transmission for DO-A, periodic D2R resource allocation has an advantage of reducing R2D signalling overhead. 1st D2R content Observation 8: l For the content of first D2R Tx for DO-A, option 1 and option 3 have a disadvantage as it requires higher R2D signalling overhead for scheduling D2R data Tx. l For the content of first D2R Tx for DO-A, option 1 and option 3 have a disadvantage as it leads to longer latency for D2R data Tx. l For the content of first D2R Tx for DO-A, option 2 has a disadvantage as it may lead waste of the pre-allocated resources if device does not have frequent DO-A traffic to transmit considering event-triggered DO-A traffic. l For the content of first D2R Tx for DO-A, option 1/2/3 can be applicable in different scenarios. Option 1 and option 3 can be applicable for event-triggered DO-A traffic while option 2 can be applicable for periodic DO-A traffic. Observation 9: For the content of first D2R Tx for DO-A, option 1 has an advantage as it can reuse existing random access procedure as starting point. Proposal 10: For option 3 (DO-A SR) for the content of first D2R Tx for DO-A, further study the following alternatives for the content of DO-A SR: - Alt.1: 1-bit DO-A SR. - Alt.2: device ID in DO-A SR. Proposal 11: For option 2 and option 3 for the content of first D2R Tx for DO-A, contention resolution methods should be studied considering contention-based resource allocation. Observation 10: For content of first D2R Tx for DO-A, Option 1 may not be applicable if DO-A data can only be transmitted after random access. Necessary info and its prov. Observation 11: For providing necessary information for the first D2R transmission for DO-A, existing A-IoT paging message (option 2) is not suitable considering that A-IoT paging triggers D2R for inventory/command use case based on request from NW. Proposal 12: For option 3 for providing necessary information for the first D2R transmission for DO-A, clarify which kind of R2D synchronization signal is used to derive the DO-A resources from the following possible interpretations. - Interpretation 1: DO-A resources are derived from periodic R2D synchronization signals for frequency acquisition, where the periodic R2D synchronization signals are corresponding to periodic R2D broadcast information. - Interpretation 2: DO-A resources are derived from R2D synchronization signals for CFO/SFO calibration for DO-A D2R Tx, where the location of R2D synchronization signals can be provided by broadcast information. Proposal 13: Necessary information for the first D2R transmission for DO-A provided by periodic broadcast information should be considered. |
| TCL | Proposal 3: For DO-A traffic, considering the configuration information from the reader for the D2R resource for the first D2R transmission. The configuration information can include periodicity or quantity of D2R resources. Proposal 4: For periodic DO-A traffic, considering the device reports assistant information to the reader. Proposal 5: For non-periodic DO-A traffic, consider SR-like signaling to requests subsequent resources for the DO-A operation. Proposal 6: Considering periodic synchronization signal/calibration signal to provide R2D synchronization and timing for DO-A traffic. |
| IIT Kanpur | T/F sync Proposal 1: A periodic synchronization signal should be used to provide timing and frequency synchronization for DO-A traffic for device 2b and device C. Observation 1: The resources required for transmitting the first DO-A message can be scheduled after the periodic synchronization signal. These resources do not require explicit scheduling information; instead, they can be implicitly indicated through a defined timing and frequency relationship with the periodic synchronization signal. 1st D2A content Proposal 2: Contents of the first D2R transmission for DO-A for Device 2b and for Device C should be defined before deciding whether the resource allocation for the first D2R transmission for DO-A for Device 2b and for Device C to be provided periodically and/or aperiodically by the reader. Proposal 3: The first D2R transmission for DO-A for Device 2b and for Device C, contains the following: - Option 1: Msg1-like content - Option 2: DO-A scheduling request - Option 3: Combination of Option 1 and Option 2 Necessary info and its prov. Proposal 4: For providing the necessary information provided by a paging-like R2D message for the D2R transmission of DO-A traffic for Device 2b and for Device C. |
| CEWiT | Resource allocation/determination & 1st D2R content Proposal 4: Both periodic and on-demand D2R resource allocation schemes should be studied to support DO-A traffic. Proposal 5: Pre-configured resource allocation for initial D2R resources for periodic DO-A traffic should be supported. Proposal 6: For non-periodic DO-A traffic, devices should use SR-like signaling to request additional D2R resources for DO-A operation. T/F sync Proposal 9: A dedicated periodic Sync-R2D signal should be introduced to provide timing and frequency synchronization for DO-A transmissions. |
| Sony | Proposal 1: Support DO-A data payload in the Msg1 with device ID. Proposal 2: Support grant-free channel access for DO-A traffic for active devices in Rel-20. Proposal 3: When studying possible sequences, consider the power amplifier aspect of constant amplitude. Observation 1: A pre-allocated resource pool only for DO-A transmission may result in unnecessary use of resources as they cannot be used by e.g. DO-DTT traffic. Necessary info and its prov. Proposal 4: Support transmission of DO-A resource allocation information in broadcast information. T/F sync Proposal 5: Let information attached to the synchronization channel indicate changes of sparsely allocated broadcast information. |
| KT Corp. | RAN1 to study the design of R-TAS sequences or synchronization signals that distinguish between Device type, as well as mechanisms for the reader to instruct devices to indicate their device type in first D2R transmission. |
| ETRI | Resource allocation/determination Observation 4: - Option 1 (Periodic D2R Resource Allocation) provides predictable and stable operation with reduced signaling requirements, making it suitable for systems prioritizing scheduling simplicity and low control complexity. - Option 2 (Aperiodic D2R Resource Allocation) offers higher flexibility and better resource utilization by allocating resources only when needed, although it requires additional signaling and may introduce control latency. Proposal 11: Capture Observation 4 in the TR Observation 5: - Periodic provision guarantees predictable operation and readiness for immediate transmission, which is beneficial for systems emphasizing low latency and high reliability. - Aperiodic provision improves efficiency in conditions where configuration changes are infrequent or transmissions are irregular. Proposal 12: Capture Observation 5 in the TR. 1st D2R content Proposal 13: For Option 2 (DO-A data payload), the first D2R transmission includes the device identification information (e.g., Device ID) to allow the reader to identify the transmitting device. |
| Fraunhofer HHI, Fraunhofer IIS | Necessary info and its prov. Proposal 7. Option 3 as the baseline approach for the first D2R transmission for DO-A. Proposal 8. Define the necessary information for the first D2R transmission for DO-A, including offsets, periodicity, and allocation size, to be signaled via broadcast information. |
| HONOR | Proposal 6: For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, support - Option 3: necessary information derived from a synchronization signal and provided by broadcast information. |

D2R power control

Previous agreements

Agreement (RAN1#122)

Study necessity and feasibility of D2R Tx power control for Device 2b and for Device C, including the following aspects

- Open loop and/or closed loop power control

- Required measurements (e.g., signal strength)

- Required information at the device

- Required information at reader side

- Signaling overhead and power consumption

Agreement (RAN1#122bis)

Study the necessity of D2R Tx power control for Device 2b and for Device C for outdoor scenarios

- Target to agree on observations on pros and cons of D2R Tx power control vs. no Tx power control at RAN1#123

Continue studying feasibility of the following open loop power control methods considering device complexity:

- Methods of open loop power control

  - Option 1: Device calculates Tx power based on measured pathloss, a predefined formula, and some configured parameters from reader

    - Required information at the device side at least includes reader Tx power, received target power at the reader, etc.

  - Option 2: Device measures RSRP, and uses a corresponding Tx power when the RSRP falls in an RSRP range.

    - The RSRP range is based on RSRP threshold(s) configured by reader or predefined

    - Required information at the device side at least includes RSRP threshold(s), corresponding D2R Tx power related information per each RSRP range defined by the RSRP threshold(s)

  - FFS details (e.g., required measurements, required information)

Continue studying the necessity and feasibility of closed loop power control for D2R.

Necessity of D2R Tx power control (Pros and cons of Tx power control vs. no Tx power control for D2R)

Summary

Total 22 companies reported that D2R Tx power control is necessary for Device 2b and for Device C for outdoor scenarios for the following reasons:

- it can reduce the interference between different devices (i.e., the Near-Far problem). (Futurewei, LGE, Spreadtrum, vivo, Nokia, Semtech, CMCC, CEWiT, BOSCH, Quectel, InterDigital, TCL, Apple, OPPO, Huawei, Docomo, Ericsson, Xiaomi, Samsung)

- it helps devices remain available longer. (Futurewei, LGE, Spreadtrum, vivo, Semtech, CMCC, NEC, InterDigital, TCL, Apple, OPPO, Xiaomi, Samsung)

- The interference due to a transmission from a device to a reader can be reduced at an unintended reader (Futurewei, Semtech, Apple, Docomo)

On the other hand, one company reported that the D2R Tx power control is not necessary because the inter-channel interference from different devices would be negligible, D2R coverage would be limited, and the complexity and the power consumption required at the device side would be high. (CATT)

FL would like to check if the proposed direction in the following proposal is agreeable first, and then further check if the proposal correctly captures observations from individual companies.

[H] Proposal 4.3.1a

On D2R Tx power control for Device 2b and for Device C, capture the following in TR 38.769.

The necessity of D2R Tx power control for Device 2b and Device C for outdoor scenarios were studied and the following observations were provided:

22 sources (Futurewei, LGE, Spreadtrum, vivo, Nokia, Semtech, CMCC, CEWiT, BOSCH, NEC, InterDigital, TCL, Apple, China Telecom, ZTE, OPPO, Huawei, Docomo, Ericsson, Xiaomi, Samsung, Qualcomm) reported that the D2R Tx power control for Device 2b and Device C is necessary for outdoor scenarios.

- 19 sources (Futurewei, LGE, Spreadtrum, vivo, Nokia, Semtech, CMCC, CEWiT, BOSCH, Quectel, InterDigital, TCL, Apple, OPPO, Huawei, Docomo, Ericsson, Xiaomi, Samsung) state that it can reduce the interference between different devices (i.e., the Near-Far problem).

- 13 sources (Futurewei, LGE, Spreadtrum, vivo, Semtech, CMCC, NEC, InterDigital, TCL, Apple, OPPO, Xiaomi, Samsung) reported that it helps devices remain available longer.

- 4 sources (Futurewei, Semtech, Apple, Docomo) reported that the interference due to a transmission from a device to a reader can be reduced at an unintended reader

1 source (CATT) reported that the D2R Tx power control for Device 2b and Device C is not necessary mentioning in its contribution that the inter-channel interference from different devices is negligible, D2R coverage is limited, and the complexity and the power consumption required at the device side is high.

| Company | Y/N | Comments |
| --- | --- | --- |
| LG | Y |  |
| TCL | Y |  |
| CMCC | Y |  |
| ZTE,Sanechips |  | Our views were overlooked and could be added to the first and second sub-bullets. [H] Proposal 4.3.1a On D2R Tx power control for Device 2b and for Device C, capture the following in TR 38.769. The necessity of D2R Tx power control for Device 2b and Device C for outdoor scenarios were studied and the following observations were provided: 22 sources (Futurewei, LGE, Spreadtrum, vivo, Nokia, Semtech, CMCC, CEWiT, BOSCH, NEC, InterDigital, TCL, Apple, China Telecom, ZTE, OPPO, Huawei, Docomo, Ericsson, Xiaomi, Samsung, Qualcomm) reported that the D2R Tx power control for Device 2b and Device C is necessary for outdoor scenarios. - 19 sources (Futurewei, LGE, Spreadtrum, vivo, Nokia, Semtech, CMCC, CEWiT, BOSCH, Quectel, InterDigital, TCL, Apple, OPPO, Huawei, Docomo, Ericsson, Xiaomi, Samsung, ZTE) state that it can reduce the interference between different devices (i.e., the Near-Far problem). - 13 sources (Futurewei, LGE, Spreadtrum, vivo, Semtech, CMCC, NEC, InterDigital, TCL, Apple, OPPO, Xiaomi, Samsung, ZTE) reported that it helps devices remain available longer. - 4 sources (Futurewei, Semtech, Apple, Docomo) reported that the interference due to a transmission from a device to a reader can be reduced at an unintended reader 1 source (CATT) reported that the D2R Tx power control for Device 2b and Device C is not necessary mentioning in its contribution that the inter-channel interference from different devices is negligible, D2R coverage is limited, and the complexity and the power consumption required at the device side is high. |
| Panasonic | Y |  |
| Nokia | Y |  |
| Samsung | Y |  |
| NTT Docomo | Y |  |

Feasibility study of the two open-loop Tx power control methods for D2R

Summary

Feasibility of the following two methods of open-loop Tx power control for D2R is under study considering device complexity:

- Option 1: Device calculates Tx power based on measured pathloss, a predefined formula, and some configured parameters from reader

  - Required information at the device side at least includes reader Tx power, received target power at the reader, etc.

- Option 2: Device measures RSRP, and uses a corresponding Tx power when the RSRP falls in an RSRP range.

  - The RSRP range is based on RSRP threshold(s) configured by reader or predefined

  - Required information at the device side at least includes RSRP threshold(s), corresponding D2R Tx power related information per each RSRP range defined by the RSRP threshold(s)

Total [15] companies [(LGE, vivo, ASUSTeK, Semtech, CMCC, CEWiT, BOSCH, Quectel, NEC, TCL, ZTE, NTT, Ericsson, Xiaomi, SS)] showed preference for Option 1 over Option 2, while [7] companies [(IIT, BOSCH, Panasonic, Tejas, NEC, Apple, QC) showed preference for Option 2 over Option 1].

Based on the contributions, pros and cons of each of the two methods for open-loop power control for D2R are summarized in the table below. FL asks companies to check if their views are correctly captured in the table below.

| Options | Pros | Cons |
| --- | --- | --- |
| Option 1: Device calculates Tx power based on measured pathloss, a predefined formula, and some configured parameters from reader. | - Adapts to actual link conditions - Works well across varying distances and environments - Higher power control accuracy - Less information required at the device - Can incorporate device-specific calibration | - Complexity due to RSRP measurement - Additional calculation overhead for determining D2R Tx power - Reader must share Tx power and target power |
| Option 2: Device measures RSRP, and uses a corresponding Tx power when the RSRP falls in an RSRP range. | - Simpler implementation - Lower processing complexity - Device can directly select the candidate D2R Tx power based on measured RSRP | - Complexity due to RSRP measurement - Lower power control accuracy - Difficult for the reader to set suitable RSRP thresholds - RSRP thresholds may need frequent updates depending on reader’s Tx power changes - More information required at the device - Determining RSRP threshold involves other WG (e.g.., RAN2) |

FL would like to check if the proposed direction in the following proposal is agreeable first, and then further check if the proposal correctly captures observations from individual companies.

[H] Proposal 4.3.2a

Capture the table below summarizing pros and cons of each of the two methods for open-loop Tx power control for D2R for Device 2b and C in TR 38.769.

| Options | Pros | Cons |
| --- | --- | --- |
| Option 1: Device calculates Tx power based on measured pathloss, a predefined formula, and some configured parameters from reader. | - Adapts to actual link conditions - Works well across varying distances and environments - Higher power control accuracy - Less information required - Can incorporate device-specific calibration | - Complexity due to RSRP measurement - Additional calculation overhead for determining D2R Tx power - Reader must share Tx power and target power |
| Option 2: Device measures RSRP, and uses a corresponding Tx power when the RSRP falls in an RSRP range. | - Simpler implementation - Lower processing complexity - Device can directly select the candidate D2R Tx power based on measured RSRP | - Complexity due to RSRP measurement - Lower power control accuracy - Difficult for the reader to set suitable RSRP thresholds - RSRP thresholds may need frequent updates depending on reader’s Tx power changes - More information required - Determining RSRP threshold involves other WG (e.g.., RAN2) |

| Company | Y/N | Comments |
| --- | --- | --- |
| LG | Y |  |
| TCL | Y |  |
| CMCC | Y |  |
| ZTE,Sanechips | Y |  |
| Panasonic | Y |  |
| Samsung | Y |  |
| NTT Docomo | Y |  |

Necessity and feasibility of closed-loop Tx power control for D2R

Summary

The necessity and feasibility of closed loop power control for D2R is under study. Total 9 companies (ZTE, Spreadtrum, IIT, KT, NTT, CWEiT, Nokia, OPPO, SS) indicated positive views on the necessity and feasibility of the closed loop power control for D2R in their contributions, while 5 (vivo, Semtech, Huawei, Xiaomi, Qualcomm) expressed negative views.

FL would like to check if the proposed direction in the following proposal is agreeable first, and then further check if the proposal correctly captures observations from individual companies.

[H] Proposal 4.3.3a

Regarding closed-loop Tx power control for D2R for Device 2b and for Device C for outdoor scenarios, capture the following observations in TR 38.769:

The necessity and feasibility of closed-loop Tx power control for D2R for Device 2b and Device C for outdoor scenarios were studied and the following observations were provided:

9 sources (ZTE, Spreadtrum, IIT, KT, Docomo, CWEiT, Nokia, OPPO, Samsung) reported that the closed-loop Tx power control for D2R for Device 2b and Device C is necessary and feasible for outdoor scenarios.

- 19 sources (Futurewei, LGE, Spreadtrum, vivo, Nokia, Semtech, CMCC, CEWiT, BOSCH, Quectel, InterDigital, TCL, Apple, OPPO, Huawei, Docomo, Ericsson, Xiaomi, Samsung) stated that it can reduce the interference between different devices (i.e., the Near-Far problem).

  - 4 sources (CWEiT, Nokia, OPPO, Samsung) further stated that open-loop power control can be used for the initial transmission, and closed-loop power control can be applied after the initial transmission.

- 5 sources (vivo, Semtech, Huawei, Xiaomi, Qualcomm) reported that it may not be necessary.

  - 2 sources (Huawei, Qualcomm) further stated that it requires signaling overhead and additional complexity at the reader side.

| Company | Y/N | Comments |
| --- | --- | --- |
| LG | Y with clarification | We can modify the proposal as follows: Regarding closed-loop Tx power control for D2R for Device 2b and for Device C for outdoor scenarios, capture the following observations in TR 38.769: The necessity and feasibility of closed-loop Tx power control for D2R for Device 2b and Device C for outdoor scenarios were studied and the following observations were provided: - 9 sources (ZTE, Spreadtrum, IIT, KT, Docomo, CWEiT, Nokia, OPPO, Samsung) reported that the closed-loop Tx power control for D2R for Device 2b and Device C is necessary and feasible for outdoor scenarios. - 19 sources (Futurewei, LGE, Spreadtrum, vivo, Nokia, Semtech, CMCC, CEWiT, BOSCH, Quectel, InterDigital, TCL, Apple, OPPO, Huawei, Docomo, Ericsson, Xiaomi, Samsung) stated that it can reduce the interference between different devices (i.e., the Near-Far problem). - 4 sources (CWEiT, Nokia, OPPO, Samsung) further stated that open-loop power control can be used for the initial transmission, and closed-loop power control can be applied after the initial transmission. - 5 sources (vivo, Semtech, Huawei, Xiaomi, Qualcomm) reported that it may not be necessary. - 2 sources (Huawei, Qualcomm) further stated that it requires signaling overhead and additional complexity at the reader side. |
| TCL | Y | Open-loop power control helps each device meet a target SNR despite path loss, and closed-loop tuning can further correct any offsets. |
| CMCC |  | We think the following sentence is wrong copied, it seems this is same for open loop power control. - 19 sources (Futurewei, LGE, Spreadtrum, vivo, Nokia, Semtech, CMCC, CEWiT, BOSCH, Quectel, InterDigital, TCL, Apple, OPPO, Huawei, Docomo, Ericsson, Xiaomi, Samsung) stated that it can reduce the interference between different devices (i.e., the Near-Far problem). |
| ZTE,Sanechips |  | The following observation can be captured in the TP: Considering the inaccurate RSRP measurements at the device side under open-loop power control, closed-loop power control enables the reader to adjust the transmit power of the device when necessary. [H] Proposal 4.3.3a Regarding closed-loop Tx power control for D2R for Device 2b and for Device C for outdoor scenarios, capture the following observations in TR 38.769: The necessity and feasibility of closed-loop Tx power control for D2R for Device 2b and Device C for outdoor scenarios were studied and the following observations were provided: 9 sources (ZTE, Spreadtrum, IIT, KT, Docomo, CWEiT, Nokia, OPPO, Samsung) reported that the closed-loop Tx power control for D2R for Device 2b and Device C is necessary and feasible for outdoor scenarios. - 4 sources (CWEiT, Nokia, OPPO, Samsung) further stated that open-loop power control can be used for the initial transmission, and closed-loop power control can be applied after the initial transmission. - 1 source (ZTE) further stated that considering the inaccurate RSRP measurements at the device side under open-loop power control, closed-loop power control enables the reader to adjust the transmit power of the device when necessary. - 5 sources (vivo, Semtech, Huawei, Xiaomi, Qualcomm) reported that it may not be necessary. - 2 sources (Huawei, Qualcomm) further stated that it requires signaling overhead and additional complexity at the reader side. |
| Nokia | Y | We think it is important to add an observation regarding the weaknesses of open loop and how closed loop power control can overcome them; e.g. based on our tdoc: 1. Device’s measurement error when devices measures R2D receive power (e.g. RSRP in Option 2). 2. Device’s power tolerance when setting its TX power. 3. Difference between uplink pathloss and downlink pathloss for FDD |
| Samsung | Y |  |
| NTT Docomo | Y with comment | In our Tdoc, we also report the following pros for closed-loop power control. the interference due to a transmission from a device to a reader can be reduced at an unintended reader. |

Related proposals/observations (31)

| Source | Proposals and observations |
| --- | --- |
| FUTUREWEI | Proposal 1. Capture the following observations in the TR on the pros and cons of D2R Tx power control vs. no Tx power control. - Pros: o With power control, the range in received power of the multiple received signals at the reader is reduced. This allows a reader to process multiple received signals instead of having one high powered signal mask weaker signals. This is especially important for contention based access. o With power control, a device can remain available longer as it can conserve energy by transmitting at a lower power. o With power control, the interference due to a transmission from a device to a reader can be reduced at an unintended reader. - Cons: o Depending on the power control mechanism, transmission of power control values / commands may need to be signaled. o Measurement of power may have a small increase in implementation complexity at the devic Proposal 2. Capture the following observations in the TR for option 1 of open loop power control: - A device capable of measuring the power of a received R2D transmission can estimate the pathloss based on the measured power and information about the transmitted power (provided by a reader) - A device can use a formula to determine the power of the D2R transmission based on the estimated pathloss and information about the target power level of a D2R transmission at the reader (provided by a reader). |
| Spreadtrum, UNISOC | Proposal 9: D2R power control could be supported in Rel-20, and at least closed loop power control for D2R should be supported. |
| vivo | Observation 15: The necessity reasons to study the D2R Tx power control for Device 2b and for Device C: • With D2R Tx power control, it can alleviate the "near and far effect" to be balanced and improve the multi-user multiplexing efficiency or to reduce interference in neighbor cells; • With D2R Tx power control, it can extend device availability life and optimize energy efficiency by adjusting the device transmit power according to the distance from the base station and channel quality. Proposal 11: Capture the pros and cons of open loop power control given in Table 2 of R1-2508426 into TR 38.769. Proposal 12: Deprioritize close loop power control at AIoT device in R20. |
| Nokia | Observation 1: D2R power control is necessary to address near-far problems. The received signal power difference from a Device 2b and Device C may be too large depending on their distance with respect to a reader and their transmission power. Proposal 1: Open-loop power control study should include a case where the device cannot use a required transmission power to achieve the target receive power and/or cannot use an indicated transmission power. Proposal 2: In option 2, corresponding D2R Tx power includes zero power. Observation 2: Assuming a stationary environment, closed loop power control can achieve much more accurate power control than open loop power control, due at least to the following contributions to power control inaccuracy of open loop power control: 1. Device’s measurement error when devices measures R2D receive power (e.g. RSRP in Option 2). 2. Device’s power tolerance when setting its TX power. 3. Difference between uplink pathloss and downlink pathloss for FDD. Proposal 3: For D2R Tx power control, the required information at reader side at least includes available power of A-IoT device. |
| CMCC | Observation 9: Introducing D2R Tx power control for Device 2b and for Device C for outdoor scenarios has the following benefit, • Reduced inter-user interference • Lower device power consumption and longer sustainable operating time Proposal 15: Open loop power control is feasible based on measurement of periodic synchronization signal. Proposal 16: The following option 1 is preferred with finer power control granularity and less signalling overhead. n Option 1: Device calculates Tx power based on measured pathloss, a predefined formula, and some configured parameters from reader u Required information at the device side at least includes reader Tx power, received target power at the reader, etc. |
| Huawei, HiSilicon | Proposal 12: Capture the following observations in TR 38.769 for power control: • Open loop power control is feasible and necessary, and should be supported n It can mitigate interference among devices • Closed loop power control requires signalling overhead and complicated procedure, and should not be supported |
| Ericsson | Observation 8 The benefit of having in place a power control mechanism for A-IoT devices must be pondered with respect to any measurements, power consumption and signalling required for it to work, which are aspects that may go against the necessity of it. Observation 9 An A-IoT outdoor scenario operating without power control will suffer from the “near-far effect,” in which case the signal of the Device closest to the Reader will be orders of magnitude stronger than the one of the farthest Device from the Reader. Observation 10 In relation with the previous observation, if the receiver’s dynamic range were not sufficiently large, the “near-far effect” might cause that the signal from the farthest Device won’t be detectable e.g., because the strongest signal could overwhelm or saturate the receiver, or the strongest signal could mask the weaker one, or the weaker signal can get confused by noise floor, resulting in loss of connectivity. Observation 11 In our view such a power control mechanism would have to be even simpler than the one used in NB-IoT for low cost/low complexity half-duplex devices since some terms are not applicable for A-IoT and will result in unnecessary complexity, signalling overhead and power consumption. Proposal 4 An open-loop power control mechanism should be supported for Rel-20 A-IoT. The NB-IoT power control designed for low cost/low complexity half-duplex devices can be used as a baseline, which would have to be further simplified since some terms are not applicable for A-IoT and will result in unnecessary complexity, signalling overhead and power consumption. |
| NEC | Proposal 17: Both options for open-loop power control can be studied in Rel-20. Proposal 18: Option 2 of open-loop power control has the underlying assumption of constant Tx power at reader side, and the assumption should be clearly stated. |
| CATT | Proposal 18: D2R Tx power control for Device 2b and Device C for outdoor scenarios is not feasible. |
| China Telecom | Proposal 14: Support the open-loop power control of active devices in Rel-20 and some enhancements without any introducing of signalling dedicated to closed-loop power control. Proposal 15: Support the following information pre-configured or indicated by reader side: - Initial target power level - T/F duration related parameters, e.g., repetition number, coding type, frequency bandwidth - Measurement related parameters, e.g., measurement occasion, measurement time duration, measurement thresholds - Power control adjustment related parameters, e.g., power ramping step. |
| Xiaomi | Proposal 7: It is necessary to support the D2R power control. Observation 1: Closed-loop power control requires the device to continuously monitor the power adjustment commands from the reader to determine the accumulated power adjustment value, which increases the power consumption. Proposal 8: Considering limited power and low complexity of AIoT device, closed-loop power control is not feasible for device. Proposal 9: Closed-loop power control for real-time power adjustment is unnecessary. Proposal 10: Open-loop power control shall be supported for AIoT device. OL PC Observation 2: Both option1 and option2 are feasible due to low complexity. Observation 3: Regarding to adjustment accuracy, option1 outperforms than option2 because transmission power is determined based on the actual path loss. Proposal 11: To ensure the reliability of D2R transmission, the option1 shall be supported. Proposal 12: Regard to the measurement, both the measurement quantity and signal need to be further studied. Proposal 13: If the R2D sync signal or reference signal is introduced, RSRP measurement shall be supported, otherwise, RSSI measurement shall be supported. Proposal 14: For the measurement signal, the following options can be considered: Ÿ SIP Ÿ CAP Ÿ Postamble Ÿ PRDCH Ÿ Sync signal (if any) Proposal 15: It is necessary to study the RSSI definition or RSRP definition. Proposal 16: Study whether to apply maximum transmission power based on repetition number. Proposal 17: Study different power control mechanisms for different D2R message types in R20 AIOT. |
| OPPO | Proposal 12: Open-loop and closed-loop power control are considered for Device 2b and Device C. Proposal 13: The Tx power of the reader should be indicated to the device to support open-loop power control. Proposal 14: R2D measurement based on the periodic R2D synchronization signal is supported for open-loop power control. Proposal 15: Multiple power levels can be defined for Device 2b and Device C. Observation 3: Frequent updates for the mapping relationship between RSRP ranges and D2R Tx powers are needed in Option 2, which will increase R2D signalling overhead. Proposal 16: The reader can indicate power offset to the device to support closed-loop D2R Tx power control. |
| Tejas Network Limited | Observation 3: Open loop power control requires less measurement, thereby less complexity and processing time at the device. Therefore, we propose to study the D2R open loop power control. In open loop power control methods, calculating the pathloss from a predefined formula is more computationally complex compared to measuring the RSRP. Proposal 4: We support option 2. o Option 2: Device measures RSRP, and uses a corresponding Tx power when the RSRP falls in an RSRP range. - The RSRP range is based on RSRP threshold(s) configured by reader or predefined - Required information at the device side at least includes RSRP threshold(s), corresponding D2R Tx power related information per each RSRP range defined by the RSRP threshold(s) |
| Samsung | Observation 2 D2R Tx power control is necessary for Rel-20 Ambient IoT to mitigate the near-far problem and to improve device power efficiency. Proposal 11 Study open-loop D2R power control using pathloss estimation (i.e., Option 1) based on comparing the average ON-state power of known R2D signals (e.g., SIP, CAP) with the configured ON-state power. • FFS: R2D signals for pathloss estimation (e.g., Periodic R2D transmission for DO-A support) Proposal 12 Study closed-loop D2R power control for Rel-20 Ambient IoT. Proposal 13 Study different power control mechanisms for D2R transmissions depending on the message purpose or the type of associated R2D message. |
| ZTE Corporation, Sanechips | Proposal 9: Both open-loop and closed-loop D2R power control are considered to support different use cases for Device 2b/C. Proposal 10: For method of open-loop D2R power control, Option 1 (i.e., based measured pathloss and a predefined formula) should be further studied. |
| Panasonic | Proposal 7: Support Option 2 for D2R transmission power control. |
| LG Electronics | Observation #13: D2R Tx power control may help maintain link reliability and improve resource efficiency in outdoor scenarios, particularly by mitigating the near-far problem, but its implementation adds complexity to device operation. Observation #14: For the open loop power control methods, the following observations are made: - Option 1: Pathloss-based power control offers higher accuracy and adaptability to varying radio conditions, making it suitable for scenarios where efficient and reliable communication is critical. - Option 2: RSRP-based power control provides a simpler implementation with lower device complexity, but may be less effective in dynamically changing environments due to its coarser granularity. |
| ROBERT BOSCH GmbH | Observation 1: Large-scale power fluctuation leads to a pronounced near-far problem and this gets further aggravated by affecting duty cycle and overall operations. Proposal 1: Adaptive power control should be required to maintain reliable signal strength, mitigate near–far problem, and ensure energy-efficient connectivity across varying device-to-reader distances. Observation 2: The transmit power determination should be simple and computationally minimal. Proposal 2: For Ambient IoT devices, the transmit power control formulation can be based on following two options: Option 1: Based on the following simple formula pUE = min(pmax,pest) pest = p0 + α.ppl where - pUE: Transmit power from the device, - pmax: maximum permissible power, - p0: Baseline power value set by the reader, - ppl is the pathloss between reader and device, - α: pathloss compensation factor (0 < α < 1) This approach enables a simple, yet adaptive power control mechanism based on path loss estimation, requiring minimal computation. Option 2: The transmit power can be selected from a predefined look-up table indexed by easily measurable link quality metrics such as RSRP or RSSI. Proposal 3: The look-up table can be predefined or dynamically transmitted by the reader using any R2D message. Proposal 4: For ambient IoT devices, the reference signals for computing the key link quality metrics can be based on the following two options: Option 1: The device can use any R2D message such as paging to derive link-quality metrics. Option 2: A dedicated R2D reference signal can specifically be introduced for link-quality metric estimation and power-control operations. Observation 3: Computation of link quality metrics at the device may impose additional energy and processing overheads. Proposal 5: An Ambient IoT device can transmit a D2R reference signal to enable link-quality metric estimation at the reader. There could be two options: Option 1: The device can transmit a dedicated D2R reference signal specifically intended for link-quality metric estimation. Option 2: Existing D2R messages, such as Msg1 from the random-access procedure, can be reused as implicit reference signals for reader-side link-quality metric measurement. |
| Apple | Observation 4: D2R power control is necessary for outdoor scenarios to extend device battery life and manage interference, with benefits outweighing the complexity for both device 2b and device C given their limited power adjustment ranges Observation 5: Path loss based power control (Option 1) provides better energy efficiency through continuous adjustment but requires more processing capability, while RSRP range based control (Option 2) offers simpler implementation with coarser adjustment. Proposal 4: Support RSRP range based power control (Option 2) as baseline for both device 2b and device C, with 3-4 RSRP ranges providing a balance between simplicity and energy efficiency. |
| InterDigital, Inc. | Proposal 7: Support D2R open loop power control using RSRP measurement. Proposal 8: If device type identification is not supported by a receiver, a unified framework for open-loop device power control should be supported to achieve consistent performance regardless of the device type the reader is communicating with. Proposal 9: D2R power control depends on the device energy level. |
| Semtech Neuchatel SA | Observation 1: D2R power control can reduce energy consumption of devices in good coverage. Observation 2: D2R power control helps manage inter-reader interference and reduce collisions. Observation 3: D2R power control can help to reduce power level variations at the reader receiver. Observation 4: D2R power control framework can help with repetition (or coding rate) selection. Proposal 1: Recommend D2R power control be specified in the normative stage, with the following justifications: - Reduced energy consumption of devices in good coverage. - Reduce inter-reader interference and resulting collisions. - Reduce received power level variations at the reader. - Power control framework can help with D2R repetition (or coding rate) selection. Proposal 2: Recommend at least open-loop power control be specified in the normative stage. FSS: support of closed-loop power control. Observation 5: D2R power control open-loop option 1 would provide more accurate power control, with similar computational complexity and less configuration information compared to option 2. Proposal 3: For both open-loop option 1 and option 2, recommend allowing for power control parameters to be dynamically provided by the reader. |
| ASUSTeK | Observation 1: For D2R Tx power control of active devices, there is trade-off in option 2 between required information overhead and power control performance. Proposal 1: For D2R Tx power control of active devices, it supports Option 1: Device calculates Tx power based on measured pathloss, a predefined formula, and some configured parameters from reader. Proposal 2: For the first D2R transmission autonomously triggered by active device, reader implements CBRA triggering which targets devices with pending DO-A data. |
| Quectel | Proposal 1: Ÿ D2R Tx power control for Device 2b and for Device C is necessary for interference mitigation and energy conservation. Proposal 2: Ÿ Study Option 1 for D2R Tx power control for Device 2b and Device C, i.e., device calculates Tx power based on measured pathloss, a predefined formula, and some configured parameters from reader. Proposal 3: Ÿ For D2R Tx power control, reference signal for pathloss estimation is necessary to be studied. - CFO calibration signal of unmodulated sinusoid waveform can be studied for the measurement of pathloss. |
| Qualcomm Incorporated | Observation 5: - It is up to device capability whether to support power control, dependent on its peak power and PA support. Observation 6: (Table on the right) Proposal 4: Consider Option 2 RSRP-threshold-based open-loop power control for devices capable of power control. Measurement Proposal 5: - For R2D measurement, R2D synchronization signal, PRDCH and preamble/midamble with sufficient time duration can be considered for L1-RSRP. • Note: RSRP measurement requirement may be up to RAN4 discussion - For D2R measurement, PDRCH and D2R preamble/midamble with sufficient time duration and stable transmit power can be considered. |
| NTT DOCOMO, INC. | Observation 12: D2R power control is beneficial to reduce power imbalance among devices in D2R which can improve the performance on CDMA and beneficial for interference alleviation in multi-reader scenario. Observation 13: l Device’s measurement of RSRP of R2D signal is feasible. l For D2R open loop power control methods, option 1 has an advantage as it can achieve more accurate D2R Tx power control with finer granularity. l For open loop power control methods, option 2 does not require device to calculate D2R Tx power according to defined formula, however whether the device complexity in the calculation is critical is not clear. Proposal 14: l Study whether D2R power amplifier is feasible for device 2b/C. l For required information at device for D2R open loop power control option 1, further study whether coefficient of pathloss is needed. Observation 14: l Closed-loop power control can be beneficial for interference alleviation in multi-reader scenario. With closed-loop power control, readers can dynamically decrease the transmission power of D2R to avoid interference to other readers or increase the transmission power of D2R if the signal quality is poor. l Closed loop power control requires additional overhead in R2D. The additional overhead on the control information can be minimal, e.g., can be 2-bit same as TPC in NR or even smaller. |
| TCL | Observation 1: D2R power control can greatly enhance link reliability, coverage, and interference management, energy saving for device 2b/C. Observation 2: In outdoor scenarios that require higher reliability coverage and better energy efficiency, Option 1 leverages its superior performance due to precise power control. By using Option 1, Ambient IoT devices can achieve extremely low power consumption while maximizing the success rate of D2R communication and overall network efficiency. Proposal 1: Option 1 is preferred for power control, and measurement quantity such as R2D/D2R RSRP should be introduced for A-IoT. |
| IIT Kanpur | Proposal 5: At least support Option 2 for D2R transmission power control. Proposal 6: For Option 2, it should be clarified that the RSRP measurement is performed under the assumption of constant transmission power at the Reader. Observation 2: A closed-loop power control may simplify device design and mitigate the need to make assumptions regarding constant power, albeit at the cost of higher signaling overhead. Proposal 7: A closed-loop power control mechanism should be further studied. |
| CEWiT | Proposal 1: D2R transmit power control for Devices 2b and C is essential to optimize energy efficiency and minimize interference. Observation 1: Two approaches for D2R transmit power control were analyzed : Option 1 (pathloss-based) and Option 2 (RSRP range-based mapping). • Option 1 (Pathloss-based): o Provides fine-grained power adjustment, better interference mitigation, and consistent link reliability under varying channel conditions. o Higher computational complexity and slightly increased energy consumption at the device. • Option 2 (RSRP range-based mapping): o Low complexity and reduced processing requirements, making it suitable for simpler devices. o Limited accuracy and flexibility increased signaling overhead with finer RSRP granularity, and less effective interference control. Proposal 2: Supported Option 1 for D2R Tx power control for Device 2b and Device C. • Option 1: Device calculates Tx power based on measured pathloss, a predefined formula, and some configured parameters from reader o Required information at the device side at least includes reader Tx power, received target power at the reader, etc. Proposal 3: Study closed-loop D2R power control for Devices 2b and C. |
| KT Corp. | Proposal 3 RAN1 confirms that closed-loop power control scheme as the baseline for D2R Tx power control. |
| Fraunhofer HHI, Fraunhofer IIS | Proposal 6. Introduce configurable power ramping parameters within the open-loop power control framework, allowing the reader to define step size, maximum power, and retry-based adjustments. |
| HONOR | Proposal 4: For D2R open loop power control, support Option 2. Proposal 5: Support to study the closed loop power control. |

Random access

Previous agreements

Agreement (RAN1#122)

At least for CBRA for DT and DO-DTT, study necessity and feasibility of enhancements of D2R TDM(A) for Device 2b and for Device C based on the following:

- X1 > 2 for Msg1 (X1: number of time domain resources for Msg1 transmission indicated by the corresponding R2D message)

- X3 > 1 for Msg3 (X3: number of time domain resources for Msg3 transmission indicated by the corresponding R2D message)

Agreement (RAN1#122bis)

Study necessity and feasibility of D2R CDM(A) for Device 2b and for Device C with sequence-based Msg1 transmission and potential impact to random access procedures.

- FFS: other cases, except for data transmission

Enhancements of D2R TDMA

Summary

Total 14 companies (TCL, ID, CEWiT, Transsion Holdings, LG, NEC, CATT, ZTE, Apple, ASUSTeK, CMCC, Panasonic, NTT, SS) reported that, for CBRA for DT and DO-DTT, introducing a larger number of X1 for Msg1 (i.e., X1 > 2) is feasible and necessary, and some companies further provided the following details:

- For X 1 values for Msg1

  - 2 or 4 or 8: Apple, ASUSTeK

  - 3 or 4: LG

  - 4: SS

  - 4 or 8: NEC

  - 8: NTT

  - <11: CATT

And total 12 companies (TCL, ID, CEWiT, Transsion Holdings, LG, NEC, CATT, Apple, ASUSTeK, CMCC, NTT, SS) reported that, for CBRA for DT and DO-DTT, introducing a larger number of X3 for Msg3 (i.e., X3 > 1) is feasible and necessary, and some companies further provided the following details:

- For X3 values for Msg3

  - 2: ASUSTeK, NTT, CATT (at least 2)

  - 2 or 4: NEC, Apple,

  - 4: SS

On the other hand, 3 sources (Ericsson, Huawei, Spreadtrum) reported that introducing a larger number of X1 for Msg1 (i.e., X1 > 2) is not feasible or not necessary, and 4 sources (Ericsson, Huawei, Spreadtrum, ZTE) stated that introducing a larger number of X3 for Msg3 (i.e., X3 > 1) is not feasible or not necessary.

FL would like to check if the proposed direction in the following proposal is agreeable first, and then further check if the proposal correctly captures observations from individual companies.

[H] Proposal 4.4.1a

On enhancements of D2R TDM(A) for Device 2b and for Device C for CBRA for DT and DO-DTT, capture the following in TR 38.769.

On the enhancements to increase the number of time domain resources indicated by the corresponding R2D message for the Access Random ID message (Msg1) to greater than 2,

- 14 sources (TCL, InterDigital, CEWiT, Transsion Holdings, LGE, NEC, CATT, ZTE, Apple, ASUSTeK, CMCC, Panasonic, Docomo, Samsung) state that introducing a larger number of X1 for Msg1 (i.e., X1 > 2) is feasible and necessary.

- 3 sources (Ericsson, Huawei, Spreadtrum) state that introducing a larger number of X1 for Msg1 (i.e., X1 > 2) is not feasible or not necessary.

On the enhancements to increase the number of time domain resources indicated by the corresponding R2D message for the D2R Upper Layer Data Transfer message (Msg3) to greater than 1,

- 12 sources (TCL, InterDigital, CEWiT, Transsion Holdings, LGE, NEC, CATT, Apple, ASUSTeK, CMCC, Docomo, Samsung) state that introducing a larger number of X3 for Msg3 (i.e., X3 > 1) is feasible and necessary.

- 4 sources (Ericsson, Huawei, Spreadtrum, ZTE) state that introducing a larger number of X3 for Msg3 (i.e., X3 > 1) is not feasible or not necessary.

| Company | Y/N | Comments |
| --- | --- | --- |
| LG | Y |  |
| TCL | Y |  |
| CMCC | Y | This also depends on whether R19 procedure for DT and DO-DTT is reused. If it is reused, then discuss this is useful. However if new procedure is introduced, for example, DRX based paging for inventory is introduced, and no Access trigger is needed, then X1 will be naturally a large number. |
| ZTE,Sanechips | Y |  |
| Panasonic | Y |  |
| Futurewei |  | What is the preferred approach to capture observations / proposals in previous contributions? From RAN1#122bis R1-2506748 Proposal 1: In Rel-20, up to 8 time domain resources should be considered for TDMA of Msg1 Proposal 3: For Rel-20, support more than one time domain resource for Msg3. |
| NTT Docomo | Y |  |

Enhancements of D2R FDMA

Summary

How to determine D2R frequency resource

- Indicated by reader directly: Spreadtrum, NEC, vivo (other than Msg1), LG (sub-band + SFS), NTT (center + offset, or center + SFS), ZTE (center + offset)

- Slotted ALOHA for Msg1: vivo, LG, NEC

Others

- CMCC: study FDM between Msg1 and Msg3

Necessary enhancements on D2R FDMA for Device 2b and Device C are discussed in sub-agenda 10.3.2.2.

How a device determines the D2R frequency resource can be discussed in this sub-agenda or in 10.3.2.2 after some progress on the D2R FDMA is made in sub-agenda 10.3.2.2.

| Company | Comments |
| --- | --- |
| ZTE,Sanechips | Y |

Potential impact of D2R CDM(A) to random access procedures

Necessity and feasibility of D2R CDM(A) for Device 2b and for Device C with sequence-based Msg1 transmission will be discussed in sub-agenda 10.3.2.2.

Summary

Regarding Msg2

- Code would be provided in Msg2 (with T/F index): FW

- Sequence (or sequence index) would be provided in Msg2 (with T/F index): LGE, QC

- Increase the maximum echo random IDs in Msg2, and the frequency resource indication in Msg2 for Msg3 scheduling needs update: CMCC

Contention resolution in Msg4 (CMCC, NEC, QC, DCM)

- contention resolution needed to be added into random access procedure, due to limited number of sequences for Msg1 (CMCC)

- a four-steps random access with contention resolution in Msg4 should be introduced, the details of the new procedure unlike Rel-19 should be discussed and defined in RAN2 (NEC)

- Consider explicit/implicit contention resolution by using Msg3 and Msg4 for CDMA-based Msg1 used in the CBRA procedure (QC)

  - a contention resolution identity with >=9bits can be considered in addition to the 7-bit sequence ID ( 128 sequences) to at least match with the RN16

  - Additionally, if Msg2 includes multiple Msg3 resources associated with the selected sequence ID, devices are afforded the option to select a Msg3 resource, offering an implicit mechanism for contention resolution

- Collision handling for sequence-based Msg1 should be studied considering multiple devices may select same sequence for Msg.1 Tx. Collision handling for sequence-based Msg1 can be solved by minimal enhancement in RAN2 random access procedure.

LGE:

- device shall transmit Msg1 using a predefined D2R modulation, and the D2R modulation for Msg3 PDRCH transmission can be indicated via Msg2 PRDCH.

FL would like to check first if the direction in the following proposal is agreeable.

[H] Proposal 4.4.3a

Capture the potential impact to random access procedures caused by the introduction of D2R CDM(A) for Device 2b and for Device C with sequence-based Msg1 transmission in TR 38.769 considering the following aspects:

- contention resolution in Msg4

- determination of D2R modulation for the sequence-based Msg1

| Company | Y/N | Comments |
| --- | --- | --- |
| LG | Y |  |
| TCL | Y |  |
| CMCC |  | We support CDM of sequence-based Msg1, and support contention resolution in Msg4. For the D2R modulation bullet, we think it is another question and can be removed here. |
| ZTE,Sanechips |  | The feasibility and necessity of CDMA for sequence-based Msg1 needs to be verified first, including the access efficiency gain of a complete random access procedure for multiple devices compared to Rel-19, as well as the impact of SFO and CFO on CDMA. [H] Proposal 4.4.3a Capture the potential impact to random access procedures caused by the introduction of D2R CDM(A) for Device 2b and for Device C with sequence-based Msg1 transmission in TR 38.769 considering the following aspects: - contention resolution in Msg4 - determination of D2R modulation for the sequence-based Msg1 - performance evaluation on access efficiency gain,as well as the impact of SFO and CFO on CDMA. |
| Panasonic |  | We assume the same modulation would be used for Msg1 and other D2R messages. So, the second sub bullet should be removed. |
| IDCC | Y |  |
| Futurewei |  | In AI 10.3.2.2, we provided observations CDMA Proposal 6: Capture the following observations in the TR regarding necessity and feasibility of D2R CDM(A): - For the contention based access procedure, CDMA could increase the number of Msg1 transmissions. However, the bottleneck for contention based access procedure is the transmission of Msg3 as the message 6-8 times longer. - Using CDMA would change the contention based access procedure for Msg2 and scheduling Msg3. - The sensitivity to near – far and CFO needs additional study. We do not support CDMA. We propose to add. Capture the potential impact to random access procedures caused by the introduction of D2R CDM(A) for Device 2b and for Device C with sequence-based Msg1 transmission in TR 38.769 considering the following aspects: - contention resolution in Msg4 - contention based access procedure for Msg2 and scheduling Msg3. - determination of D2R modulation for the sequence-based Msg1 |
| NTT Docomo |  | For first sub-bullet, Msg3 also needs to be included. For second sub-bullet, we can understand the intention, but we feel determination of D2R modulation is not a specific issue for random access procedure, for all D2R Tx type, determination of D2R modulation needs to be studied. |

Potential impact of R2D FDM to random access procedures

Necessity and feasibility will be discussed in sub-agenda 10.3.2.1.

Summary

R2D frequency resource determination when R2D FDM is introduced.

- for sync signal (broadcast info)

  - based on sync raster: Spreadtrum, QC

  - fixed sub-band (or common control channel): LGE (one or all), FW, HW, OPPO, Apple

- for signals/channels other than the sync signal

  - explicit signaling via previous R2D Tx: Spreadtrum, LG, HW (via broadcast), NEC, OPPO, Apple (via broadcast), QC, NTT, vivo (via broadcast for paging message)

  - implicit determination via D2R Tx: NEC, ZTE, NTT, OPPO

  - same as previous R2D Tx: LGE

FL would like to check first if the direction in the following proposal is agreeable.

[H] Proposal 4.4.4a

Capture potential enhancements to random access procedures to support the R2D FDM for Device 2b and for Device C in TR 38.769 focusing on the following aspects:

- R2D frequency resource determination when R2D FDM is supported

| Company | Y/N | Comments |
| --- | --- | --- |
| LG | Y |  |
| TCL | Y |  |
| CMCC |  | If FDM is supported, dynamic adjustment of filter bandwidth and IF center frequency is required for device, which increases the complexity, while the benefit of FDM is not such clear. So we propose not to support FDM within a single carrier. |
| ZTE,Sanechips |  | We also proposed in our contribution that the frequency information for FDM can be indicated in the broadcast information. In addition, please include ZTE as a supporter of the proposal for ‘explicit signaling via previous R2D transmission.’ Summary R2D frequency resource determination when R2D FDM is introduced. - for sync signal (broadcast info) - based on sync raster: Spreadtrum, QC - fixed sub-band (or common control channel): LGE (one or all), FW, HW, OPPO, Apple - for signals/channels other than the sync signal - explicit signaling via previous R2D Tx: Spreadtrum, LG, HW (via broadcast), NEC, OPPO, Apple (via broadcast), QC, NTT, vivo (via broadcast for paging message), ZTE (via broadcast) - implicit determination via D2R Tx: NEC, ZTE, NTT, OPPO - same as previous R2D Tx: LGE It is recommended that the content to be captured is based on the discussion of R2D FDM in section 10.3.2.1. |
| Panasonic | Y |  |
| IDCC | Y |  |

Related proposals/observations (14)

| Source | Proposals and observations |
| --- | --- |
| Vivo | CDMA Observation 9: If msg1 is sequence-based, instead of RN16, then random access procedure would be different compared with that in R19. - The process of contention resolution needed to be added into random access procedure, due to limited number of sequences for Msg1, which is similar with NR logic. TDMA Observation 10: Active AIoT device should be half duplex type device. Rx-Tx switching delay and R2D FEC (if supported) should be the new factors on determining the timing offset value between R2D and corresponding subsequent D2R for device 2b and C. Observation 11: The Tx-Rx or Rx-Tx switching delay for a NB-IoT device is 1ms which could be a reference for that of device 2b or C. Observation 12: Timing offset between R2D and corresponding subsequent D2R provided by a reader e.g., indicated by R2D command is more flexible. Observation 13: The Tx-Rx witching delay due to half-duplex constraints should be an additional factor impacting on TD2R_min. Observation 14: The cases that two consecutive R2D or D2R transmissions for the same device exist. Proposal 8: The timing offset between two consecutive R2D transmissions for the same device i.e., TR2D_R2D_min needs to be defined. |
| CMCC | DO-A Observation 6: Reusing the R19 Msg1+Msg2 collision resolution procedure for outdoor active device random access leads to high collision probability and excessive D2R resource consumption. Proposal 8: Study for R20 a random access procedure update, including 1)sequence-based Msg1 is used and carries no Random ID, 2) contention is resolved via Msg3 and Msg4. Proposal 9: Study CDM for Msg1 to reduce random access collision. Inventory and command Proposal 10: For active device to support inventory, the following two alternatives need to be studied, • Alt 1: Reuse Release 19 inventory procedure modification • Alt 2: Design new inventory procedure for active devices Proposal 11: If R19 random access procedure is reused for R20 active devices, the following modification can be studied, • Support X1 > 2 for Msg1 (X1: number of time domain resources for Msg1 transmission indicated by the corresponding R2D message), and the related Toffset values for occasions with X1>2. • Increase the maximum echo random IDs in Msg2, and the frequency resource indication in Msg2 for Msg3 scheduling needs update. • Support X3 > 1 for Msg3 (X3: number of time domain resources for Msg3 transmission indicated by the corresponding R2D message), and the related Toffset values for occasions with X3>1 • Support Msg3 retransmission. Observation 7: If random access procedure of active devices is based on R19 random access procedure, the handling of time collision for Msg1/Msg3 between inventory service and autonomous DOA traffic is complex Observation 8: If new inventory procedure for active devices is designed, the R2D and D2R timing can base on R2D control indication, overlapping of Msg1/Msg3 between inventory service and autonomous DOA traffic can be avoided based on Reader indication. General (for both DO-A and inv. and cmd. Proposal 12: For Device 2b/C design, Reader can transmit and receive simultaneously in FDD band, for example, detecting Msg1 and transmitting Msg2 at the same time. Proposal 13: Monitoring window is needed for Msg2 monitoring to save device energy consumption. Proposal 14: Study whether to support Msg3 restransmission for both DOA and DT/DO-DTT traffic of device 2b/C, to improve efficiency. |
| Huawei, HiSilicon | Proposal 13: Capture the following observations in TR 38.769 for D2R TDMA: • The performance gain provided with X1 > 2 for Msg1 is limited • The performance gain provided with X3 > 1 for Msg3 is limited • Supporting X3 > 1 requires designing a new common Msg2, which is a lot of specification workload |
| NEC | Observation 1: D2R CDM(A) by sequence-based Msg1 is very difficult to be adopted for device 2b due to the large residual SFO/CFO, which may result in “the same air interface for Device 2b and Device C” cannot be achieved. Observation 2: D2R CDM(A) by sequence-based Msg1 may have impacts on the A-IoT random access procedure, where a four-steps random access with contention resolution in Msg4 should be introduced, the details of the new procedure unlike Rel-19 should be discussed and defined in RAN2. Proposal 7: For the study of binary sequence-based Msg1 for the study of necessity and feasibility of D2R CDM(A), RAN1 should consider the large residual SFO/CFO of device 2b and the impacts on random access procedure. |
| CATT | Proposal 6: The maximum number of time domain resources for Msg1 transmission should not be larger than 11 for Device 2b/C. Observation 1: For sequence-based Msg1, the length of sequence is dependent on the following factors including SFO impact, cell coverage and the device number. Proposal 8: For sequence-based Msg1, sequence length for sequence-based Msg1 should not be shorter than 31. Proposal 9: For the number of sequence-based Msg1, NR preamble number could be as starting point for Device 2b/C in outdoor scenario. |
| OPPO | Observation 2: It is beneficial to introduce FDM-based R2D transmissions in random access procedure. Proposal 7: Paging message is transmitted on a R2D channel indicated by broadcast information. Proposal 8: Study FDM-based transmissions for Msg 2 in random access procedure. |
| Samsung | TDMA Observation 3 With the more accurate timing capability of device 2b/C, the number of access occasions for Msg 1 transmission can be further increased, though this number needs to be limited due to the limitations of A-IoT devices. Observation 4 The capacity for Msg 3 transmission should match that of Msg 1, while the Rel-19 design permits the Msg 3 transmission capacity to be only half of the Msg 1 capacity. Proposal 14 Study the feasibility of increasing the number of access occasions for both Msg 1 and Msg 3 to 4. Observation 5 In the Rel-19 design, each Msg 2 is followed by its corresponding Msg 3, which has a drawback of much longer monitoring time for devices addressed in later Msg 2 transmissions and also negatively impacts device energy consumption. Proposal 15 Study the feasibility of burst Msg 2 transmissions. Proposal 16 Study differentiating Msg2 monitoring occasions based on: • Option 1: Msg1 resources • Option 2: Random ID in Msg 1 and Msg 2 |
| Transsion Holdings | Proposal 4 It is recommended that the number of time domain resources allocated for Msg1 transmission can be increased. Proposal 5 It is recommended that the number of time domain resources allocated for Msg3 transmission can also be increased. |
| Panasonic | Proposal 4: Support larger transmission occasions (X>2) for Msg1 transmissions. |
| LG Electronics | TDMA Observation #3: Regarding the Msg1 TDM enhancement, the following observations are made: l Device 2b/C may exhibit smaller residual SFO compared to Device 1, which can be beneficial in enabling the use of more than two TDM resources (e.g., X1 = 3 or 4). l When more than two TDM resources are supported, n a larger TR→D value may be needed to ensure reliable reception and synchronization. n considering additional synchronization signaling may be beneficial to ensure reliable Msg1 transmission. Observation #4: Regarding the Msg3 TDM enhancement, the following observations are made: l Device 2b/C may exhibit smaller residual SFO compared to Device 1, which can be beneficial in enabling the use of multiple TDM resources (e.g., X3 > 1). l Enhancements may be needed for Msg2 PRDCH to indicate the corresponding Msg3 transmission resources, for example by indicating Msg3 TDM/FDM resources per echoed random ID. l When multiple TDM resources are supported, a TR→D value similar to that for Msg1 may be needed to ensure reliable operation. CDMA Observation #5: Msg1 TDMA/FDMA has already been defined in Rel-19 A-IoT, and Msg1 TDMA enhancements are under consideration; thus, the necessity of Msg1 CDMA remains unclear. Observation #6: If Msg1 CDMA is required in Rel-20, the following observations are made: l Regarding sequence type: n Gold sequence: Offers superior cross-correlation properties for interference mitigation, but is more complex to implement. n m-sequence: Simpler to implement with good auto-correlation for timing detection, but has relatively poor cross-correlation performance. l Regarding sequence length, it is observed that Rel-19 A-IoT used a 16-bit random number plus 6-bit CRC, but for binary sequence-based Msg1, excluding CRC and adopting a sequence length of around 15 bits appears reasonable. l Regarding number of sequences, it is observed that multiplexing capacity depends on the number of sequences (e.g., for length 15, Gold sequence supports up to 15 distinct sequences, while m-sequence supports up to 2 distinct sequences). Proposal #1: If Msg1 CDMA is required in Rel-20, the device expects the reader to include the binary sequence (or sequence index) in Msg2 PRDCH. Proposal #2: If Msg1 CDMA is required in Rel-20, the device shall transmit Msg1 using a predefined D2R modulation. l The D2R modulation for Msg3 PDRCH transmission can be indicated via Msg2 PRDCH. |
| Qualcomm Incorporated | TDMA Proposal 6: Consider the TDMed D2R transmission for Msg1/Msg3 with small time gap to improve efficiency of CBRA procedure. CDMA Proposal 7: Consider explicit/implicit contention resolution by using Msg3 and Msg4 for CDMA-based Msg1 used in the CBRA procedure. |
| NTT DOCOMO, INC. | TDMA Observation 3: l X1>2 for Msg1 and X3>1 for Msg3 can be beneficial for; Ø reducing the time of the whole random access procedure from system perspective. Ø Increasing multiplexing capacity, e.g., to reduce collision probability on contention-based access for Msg1. l When the number of X1 for Msg1 and X3 for Msg3 are increased, guard period required between TDMed D2Rs is increased. l From the perspective of guard period between D2R transmissions, at least X1=8 and X3=2 can be feasible. l When the number of X1 for Msg1 and X3 for Msg3 are increased, depending on the CFO / SFO drift rate and time duration between the latest CFO / SFO calibration signal and Msg.1/Msg.3 transmission occasion, the SFO and CFO drift may affect to the performance on FDMA or CDMA. l CFO calibration signal / SFO calibration signal before each one or multiple Msg.1/Msg.3 transmission occasions can be beneficial for minimizing the CFO/SFO drift. Proposal 7: Study feasible number on X1 for Msg1 and X3 for Msg3 considering required guard period between D2Rs and CFO/SFO drift. CDMA Proposal 8: Collision handling for sequence-based Msg1 should be studied considering multiple devices may select same sequence for Msg.1 Tx. Observation 4: Collision handling for sequence-based Msg1 can be solved by minimal enhancement in RAN2 random access procedure. |
| CEWiT | TDMA Proposal 10: Study to support for increasing the number of time-domain access occasions for Msg1 for Device Types 2b/C. Proposal 11: Study to support TDMA for Msg 3 for device 2b/C. |
| KT Corp. | Proposal 2 RAN1 to study introduction of a MSG2 monitoring window configuration that allows Device 2b and Device C to determine the start timing and duration for MSG2 monitoring based on reference timing parameters signaled by the reader. |

Tx timing

Previous agreements

Agreement (RAN1#122)

At least for DT and DO-DTT, study necessity and feasibility of changes to existing timing offsets for Device 2b and for Device C, e.g.,

- timing offset value between R2D and corresponding subsequent D2R (e.g., Toffset1/2/3/4)

- timing offset value between D2R and corresponding subsequent R2D (e.g., T_D2R_min)

  - Note: RAN1 only discusses potential reasons that may impact the value of T_D2R_min, and if any provides those reasons to RAN4

- timing offset between R2D and corresponding subsequent D2R provided by a reader

- timing offset between D2R and corresponding subsequent R2D provided by a reader

At least for DT and DO-DTT, study necessity and feasibility of introduction of new timing offsets for Device 2b and for Device C, e.g.,

- timing offset between two consecutive R2D or D2R transmissions for the same Device, if such scenario exists

Timing offset value b/w R2D and corresponding subsequent D2R (e.g., Toffset1/2/3/4)

Summary

Timing offset value between R2D and corresponding subsequent D2R (e.g., Toffset1/2/3/4)

- Toffset2 should be smaller due to smaller SFO after calibration (SPRD, Xiaomi)

  - Values: SPRD,

- Can be considered in WI phase (ZTE)

- If R2D FEC is introduced, from the device perspective, decoding time is required upon PRDCH reception, which may necessitate an increase in the minimum TR->D value. (LGE)

- For timing offset value between R2D and corresponding subsequent D2R, it should be revisited from that for Rel-19 considering following aspects; (DCM)

  - improved clock speed for device 2b/C

  - impact on processing time by new/enhanced features, e.g., introduction on FEC/interleaving for R2D, repetition for R2D, etc. if supported

  - new chip duration, e.g., M=1, for R2D, if supported

  - improved SFO for device 2b/C especially for Toffset2

  - RTT for target maximum coverage in outdoor scenario especially for Toffset2

- If channel coding or different multiplexing is supported for device 2b/C, device may have different processing time for R2D transmission from device 1. The value of TR→D would be changed for device 2b/C comparing with device 1, in case the D2R is MSG1, MSG3, or data from upper layers, respectively. (TCL)

FL will directly prepare text proposal capturing observations on the necessity and feasibility of changes to the timing offset value b/w R2D and corresponding subsequent D2R (e.g., Toffset1/2/3/4) to be included in TR 38.769. Companies can still make a comment on this direction, or if views are not correctly captured.

[H] Proposal 4.5.1a

- TBD

| Company | Comments |
| --- | --- |
| Vivo | Our following observations on the factors on determining the timing offset value between R2D and corresponding subsequent D2R seems not be captured in the above summary (don’t know if the summary part above is the text proposal or not?) Observation 10: Active AIoT device should be half duplex type device. Rx-Tx switching delay and R2D FEC (if supported) should be the new factors on determining the timing offset value between R2D and corresponding subsequent D2R for device 2b and C. Observation 11: The Tx-Rx or Rx-Tx switching delay for a NB-IoT device is 1ms which could be a reference for that of device 2b or C. |
| ZTE,Sanechips | - The new values of Toffset1, Toffset2, Toffset3 and Toffset4 for Device 2b/C are considered in WI phase. Other potential timing offsets between R2D and corresponding subsequent D2R are based on specific use cases. |
| Futurewei | What is the preferred approach to capture observations / proposals in previous contributions? From RAN1#122 R1-2505155 Observation 3: The latency for TDMA can be reduced with a lower SFO value. Observation 4: For the active devices, the effects of the propagation delay may need to be considered for TDMA when the SFO for the active device is on the order of 103 or lower. Proposal 1: For the active devices, the coefficient of the expression TR→D=1+2te1-teToffset+Tmsg1 should be updated according to the value of SFO supported. |

Timing offset value b/w D2R and corresponding subsequent R2D (e.g., T_D2R_min)

Summary

Timing offset value between D2R and corresponding subsequent R2D (e.g., T_D2R_min)

Note: RAN1 only discusses potential reasons that may impact the value of T_D2R_min, and if any provides those reasons to RAN4

- There is no need for the reader to provide the timing offset between D2R and corresponding subsequent R2D. Because after transmitting the D2R message, device is expected to receive the corresponding R2D message continuously subject to processing delay requirement (HW)

- TD2R_min is up to RAN4 (ZTE)

- If R2D FEC is introduced, from the reader perspective, additional encoding time for PRDCH may be needed after receiving PDRCH, which may require an increase in the minimum TD->R value. (LGE)

- For timing offset value between D2R and corresponding subsequent R2D, it is not necessary to change the value of T_D2R_min for device 2b/C from what is supported for device 1 in Rel-19. (DCM)

- The value of TD→Rmin would be different between device 2b/C and device 1, which would be depended on RAN4 (TCL)

FL will directly prepare text proposal capturing observations on the necessity and feasibility of changes to the timing offset value b/w R2D and corresponding subsequent D2R (e.g., Toffset1/2/3/4) to be included in TR 38.769. Companies can still make a comment on this direction, or if views are not correctly captured.

[H] Proposal 4.5.2a

- TBD

| Company | Comments |
| --- | --- |
| vivo | Our following observation on the factor impacting on TD2R_min seems not be captured in the above summary (don’t know if the summary part above is the text proposal or not?) Observation 13: The Tx-Rx witching delay due to half-duplex constraints should be an additional factor impacting on TD2R_min. |
| Futurewei | What is the preferred approach to capture observations / proposals in previous contributions? From RAN1#122bis R1-2506748 Proposal 4: For Rel-20, support a timing offset value between D2R and corresponding subsequent R2D to allow a minimum time for a device to switch from transmit mode to receive mode. |

Timing offset provided by a reader

Summary

Timing offset between R2D and corresponding subsequent D2R provided by a reader

Timing offset between D2R and corresponding subsequent R2D provided by a reader

- Not necessary (SPRD)

- Reader indicating the time offset between R2D and corresponding D2R is feasible and beneficial, and should be supported (HW)

- It is not necessary to consider the dynamic signalling to indicate the timing offset for R2D and D2R transmission in Rel-20 (CATT)

- For DT and DO-DTT, it is feasible to support the timing offset between R2D and corresponding subsequent D2R or between D2R and corresponding subsequent R2D to be indicated by a reader. (Xiaomi)

- Study indicating TR2D, TD2R timing relationship in the preceding PRDCH including timing of each access occasions in a TDMA-based access. (SS)

  - Observation 6 In the Rel-19 design, all timing relationships are predefined in a specification, which are arguably overly pessimistic, lack technical justification and provide no flexibility to the reader.

- Observation #8: Providing the timing offset between R2D and the subsequent D2R (or vice versa) enables the UE to anticipate the timing of the next communication event, facilitating timely CFO calibration and synchronization. However, if the interval is long, frequency drift may degrade the accuracy of the subsequent transmission or reception. (LGE)

- Consider the configurable time offset between R2D and corresponding subsequence D2R, and between D2R and corresponding subsequence R2D. (QC)

  - Further study the minimum processing time required for active devices with consideration of the R2D FEC and interleaving impact.

- For timing offset values between R2D and corresponding subsequent D2R, it is beneficial from scheduling flexibility point of view to use timing offset value provided by reader instead of using fixed predefined value. (DCM)

- For timing offset values between D2R and corresponding subsequent R2D, device can save its energy if R2D reception timing is indicated by previous R2D. (DCM)

Source [Nokia] mentions R2D FEC, SFO, longer RTT as potential factors affecting the existing timing offset for Device 2b and for Device C for outdoor scenarios.

Source [CATT] mentions that the timing offset configuration should be updated in Rel-20 and consider the following impact:

- The change of maximum number X of time domain resources for Msg1 transmission and Msg3 transmission;

- The more accurate SFO timing accuracy of Device 2b and Device C;

- The FEC scheme for R2D transmission.

FL will directly prepare text proposal capturing observations on the necessity and feasibility of changes to the timing offset value b/w R2D and corresponding subsequent D2R (e.g., Toffset1/2/3/4) to be included in TR 38.769. Companies can still make a comment on this direction, or if views are not correctly captured.

[H] Proposal 4.5.3a

- TBD

| Company | Comments |
| --- | --- |
| Vivo | Our following observation on the benefit of Timing offset provided by a reader seems not be captured in the above summary (don’t know if the summary part above is the text proposal or not?) Observation 12: Timing offset between R2D and corresponding subsequent D2R provided by a reader e.g., indicated by R2D command is more flexible. |
| ZTE,Sanechips | Predefined values and rules can be applied to simplify signaling overhead. Both indicated and predefined timing offset can be captured in the TR. |
| Futurewei | What is the preferred approach to capture observations / proposals in previous contributions? From RAN1#122bis R1-2506748 Proposal 5: For Rel-20, support a timing offset between R2D and corresponding subsequent D2R provided by a reader. |

Timing offset between two consecutive R2D or D2R transmissions for the same Device

Summary

Scenarios where introduction of timing offset b/w two consecutive R2D transmissions for the same Device

- b/w A-IoT SSB and a subsequent R2D transmission, e.g., R2D paging (vivo)

  - a device is not expected to do the latter R2D reception if the time gap between two consecutive R2D transmissions is smaller the defined minimum time offset (vivo)

- when R2D sync signal is followed immediately by an R2D transmission, such as R2D paging or SSB like R2D transmission (CEWiT)

- b/w consecutive Msg2 PRDCH transmissions (if supported) (LGE)

  - LGE thinks supporting this is beneficial for device energy saving and less buffer requirement

- b/w L1 R2D control with a separate CRC and the following PRDCH for data payload (DCM, CEWiT)

  - Timing offset between two consecutive R2Ds should be studied at least for the case where R2D control information is encoded with separate CRC from R2D data. (DCM)

  - the need for such an offset becomes relevant, particularly for DT or DO-DTT cases where R2D control information is coded with a separate CRC from the data. In such scenarios, two PRDCHs may be transmitted consecutively, and the question arises whether a time gap between them is necessary. The necessity largely depends on whether the R2D control must be decoded prior to R2D data reception. If this is the case, a timing offset is required to account for the processing time needed to decode the control information. (CEWiT)

Scenarios where introduction of timing offset b/w two consecutive D2R transmissions for the same Device

- when different traffic types are supported by the same device, the scenario can happen b/w different traffic types for the same device (e.g., D2R transmission for non-DOA traffic is followed by D2R transmission for DoA traffic) (vivo, CEWiT)

  - While for this case maybe TD2R_D2R_min has no need to be defined since a device can control the later D2R transmission e.g., defer it by implementation if needed. (vivo)

Others

- Study the Msg2 monitoring window in Rel-20 A-IoT. (LGE)

FL will directly prepare text proposal capturing observations on the necessity and feasibility of changes to the timing offset value b/w R2D and corresponding subsequent D2R (e.g., Toffset1/2/3/4) to be included in TR 38.769. Companies can still make a comment on this direction, or if views are not correctly captured.

[H] Proposal 4.5.4a

- TBD

| Company | Comments |
| --- | --- |
| ZTE,Sanechips | Consecutive R2D transmissions, such as between an SIB and another R2D transmission (e.g., a trigger, Msg2, or paging), may occur during network scheduling. However, whether a device is required to receive both R2D transmissions depends on the specific device behavior. Therefore, timing offset between two consecutive R2D should be considered after clarifying the specific use case and target device behavior. |
| Futurewei | What is the preferred approach to capture observations / proposals in previous contributions? From RAN1#122bis R1-2506748 Observation 2: If the scenario for consecutive R2D transmissions for the same device is supported, there may be a need for a timing offset to allow a device to complete processing the first R2D transmission. |

Related proposals/observations (15)

| Source | Proposals and observations |
| --- | --- |
| Spreadtrum, UNISOC | Observation 1: since active device has better performance for SFO calibration, Toffset2 for active device can be 0.248 (Toffset1 + Tmsg1) less than that for Device 1. Proposal 7: the definition for the existing timing offsets for Device 1, i.e., Toffset1, Toffset2, Toffset3, Toffset4, and TD2R,min, could be reused for Device 2b and for Device C. Proposal 8: Not support the indication of existing timing offset (between R2D and corresponding D2R, and between D2R and corresponding R2D) for Device 2b/C by reader. |
| Nokia | Proposal 10: RAN1 identifies factors which potentially affect the existing timing offset such as R2D FEC, SFO, longer RTT, by supporting Device 2b/C especially for outdoor scenarios. |
| vivo | Observation 10: Active AIoT device should be half duplex type device. Rx-Tx switching delay and R2D FEC (if supported) should be the new factors on determining the timing offset value between R2D and corresponding subsequent D2R for device 2b and C. Observation 11: The Tx-Rx or Rx-Tx switching delay for a NB-IoT device is 1ms which could be a reference for that of device 2b or C. Observation 12: Timing offset between R2D and corresponding subsequent D2R provided by a reader e.g., indicated by R2D command is more flexible. Observation 13: The Tx-Rx witching delay due to half-duplex constraints should be an additional factor impacting on TD2R_min. Observation 14: The cases that two consecutive R2D or D2R transmissions for the same device exist. Proposal 8: The timing offset between two consecutive R2D transmissions for the same device i.e., TR2D_R2D_min needs to be defined. |
| Huawei, HiSilicon | Proposal 14: Capture the following observations in TR 38.769 for timing relationship: • If the number of time domain resources for Msg1 or Msg3 is larger than that in Rel-19, more timing offset need to be defined. Therefore, using pre-defined timing offset is very complicated and should not be supported • Reader indicating the time offset between R2D and corresponding D2R is feasible and beneficial, and should be supported • There is no need for the reader to provide the timing offset between D2R and corresponding subsequent R2D. Because after transmitting the D2R message, device is expected to receive the corresponding R2D message continuously subject to processing delay requirement |
| Ericsson | Observation 12 For Device 1 in Rel-19, time interval from the end of a R2D transmission to the starting time of the corresponding D2R time domain resource, and vice-versa, is fixed. Proposal 8 For Device 2b/C in Rel-20, support a more flexible and dynamic time interval between R2D transmission and D2R reception, and vice-versa, where relavant as shown in the table above. |
| CATT | Proposal 11: The timing offset configuration should be updated in Rel-20 and consider the following impact: l The change of maximum number X of time domain resources for Msg1 transmission and Msg3 transmission; l The more accurate SFO timing accuracy of Device 2b and Device C; l The FEC scheme for R2D transmission. Proposal 12: It is not necessary to consider the dynamic signalling to indicate the timing offset for R2D and D2R transmission in Rel-20. |
| Xiaomi | Proposal 4: For CBRA in Rel-20, the existing timing relationships defined in Rel-19 can be studied as a starting point. Proposal 5: For DT and DO-DTT, it is feasible to support the timing offset between R2D and corresponding subsequent D2R or between D2R and corresponding subsequent R2D to be indicated by a reader. Proposal 6: For Toffset2, a smaller value than Device 1 of 1.25(Toffset+MD2R,chip*TD2R,chip) can be considered. |
| Samsung | Observation 6 In the Rel-19 design, all timing relationships are predefined in a specification, which are arguably overly pessimistic, lack technical justification and provide no flexibility to the reader. Proposal 1 Study indicating TR2D, TD2R timing relationship in the preceding PRDCH including timing of each access occasions in a TDMA-based access. |
| ZTE Corporation, Sanechips | Proposal 1: For timing of D2R response to R2D, new values of Toffset1, Toffset2, Toffset3 and Toffset4 for Device 2b/C are considered in WI. Proposal 12: For timing of R2D response to D2R, the value of TD2R_min is up to RAN4. |
| LG Electronics | Observation #7: If R2D FEC is introduced: - from the device perspective, decoding time is required upon PRDCH reception, which may necessitate an increase in the minimum TR->D value. - from the reader perspective, additional encoding time for PRDCH may be needed after receiving PDRCH, which may require an increase in the minimum TD->R value. Observation #8: Providing the timing offset between R2D and the subsequent D2R (or vice versa) enables the UE to anticipate the timing of the next communication event, facilitating timely CFO calibration and synchronization. However, if the interval is long, frequency drift may degrade the accuracy of the subsequent transmission or reception. Proposal #3: Study the Msg2 monitoring window in Rel-20 A-IoT. Proposal #4: Study the timing offset between two consecutive R2D transmissions (TR2D_R2D) for Msg2 PRDCH transmission. |
| Qualcomm Incorporated | Proposal 8: Consider the configurable time offset between R2D and corresponding subsequence D2R, and between D2R and corresponding subsequence R2D. • Further study the minimum processing time required for active devices with consideration of the R2D FEC and interleaving impact. |
| NTT DOCOMO, INC. | Observation 5: l For timing offset value between R2D and corresponding subsequent D2R, it should be revisited from that for Rel-19 considering following aspects; Ø improved clock speed for device 2b/C Ø impact on processing time by new/enhanced features, e.g., introduction on FEC/interleaving for R2D, repetition for R2D, etc. if supported Ø new chip duration, e.g., M=1, for R2D, if supported Ø improved SFO for device 2b/C especially for Toffset2 Ø RTT for target maximum coverage in outdoor scenario especially for Toffset2 l For timing offset values between R2D and corresponding subsequent D2R, it is beneficial from scheduling flexibility point of view to use timing offset value provided by reader instead of using fixed predefined value. Observation 6: l For timing offset value between D2R and corresponding subsequent R2D, it is not necessary to change the value of T_D2R_min for device 2b/C from what is supported for device 1 in Rel-19. l For timing offset values between D2R and corresponding subsequent R2D, device can save its energy if R2D reception timing is indicated by previous R2D. Proposal 9: Timing offset between two consecutive R2Ds should be studied at least for the case where R2D control information is encoded with separate CRC from R2D data. |
| TCL | Proposal 2: The following time offset would be changed for device 2b/C comparing with device 1: TR→D in case the D2R is MSG1, MSG3, or data from upper layers, respectively; TD→Rmin. |
| CEWiT | Proposal 13: Support to study the timing offset between consecutive R2D transmissions. Proposal 14: Support to study the timing offset between consecutive D2R transmissions. |
| HONOR | Observation 1: Cross-message block-level repetitions are candidate scenarios where there are consecutive R2D/D2R transmissions for the same device. |

Others

Other proposals/observations related to A-IoT procedures

| Source | Proposals and observations |
| --- | --- |
| Nokia | Impact of CFO Proposal 4: RAN1 study CFO mitigation mechanisms (e.g., preamble-based scheme and/or calibration signal design) |
| Nokia | Device (un)availability Observation 3: For device 2b and especially for device C, peak power consumption is much higher than typical energy harvester output power. This could result in extended periods of full or partial device unavailability. Proposal 8: Study device energy management, in particular solutions to address the issue of device (un)availability due to energy harvesting. Proposal 9: Study solutions to assist the reader in predicting device (un)availability, e.g. based on device reporting the following information to the reader: device’s energy storage level, achievable energy harvester output power, power/energy required for RX and TX. |
| CATT | R2D FDM Observation 2: The complexity of Device 2b and Device C to support R2D FDM transmission is too high. The necessity and feasibility needs further study. Msg3 TDMA Observation 3: The transmission latency can be reduced by supporting TDMA for Msg3. Observation 4: The larger the TDMA number, the slower the reduction in latency. Proposal 10: At least X3=2 can be supported for the TDMA of Msg3 (X3: number of time domain resources for Msg3 transmission indicated by the corresponding R2D message). |
| China Telecom | Device (un)availability Proposal 16: Support to introduce a new state ‘SLEEP’ and further study the time duration, transaction latency, and the triggering events for the ‘SLEEP’ state. |
| Xiaomi | Device (un)availability Observation 4: Standardized device unavailability confers advantages in prolonging the device's active duration and minimizing the ambiguity in the communication between the reader and the device. Observation 5: The state transitions related to the OFF state can depend on the implementation of the device. Proposal 18: RAN1 should investigate the switching method between the ON and SLEEP states of the device. |
| Samsung | Device (un)availability Proposal 18 For Device 2b/c, study whether and how the device availability during communication procedures depends on energy aspects. The two directions in Rel-19 Ambient IoT study can be considered as a starting point: • Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable. • Direction 2: Reader can provide information to a device based on which the device may become available/unavailable. Proposal 19 Study the necessity and gain of new timing parameters for Direction 1, including at least the following: • A minimum and a maximum time between two consecutive PRDCHs providing triggering messages. • A maximum time for a device, during which a device successfully finished inventory is not required to monitor a potential R2D transmission. • A minimum time required for a device to continuously monitor a potential R2D transmission, when it starts monitoring to guarantee a reachability from a reader to the device. Proposal 20 Study energy status report (ESR) procedure at A-IoT PHY layer |
| Quectel | Device (un)availability Proposal 7: the following Options can be considered for direction 2 of device (un)availability, Ÿ Option 1: The reader provides periodic information for device. The device periodically switches to ON state or sleep state. Ÿ Option 2: The reader sends wake-up or sleep indication. The device switches to ON state or sleep state according to the indication. |
| NTT DOCOMO, INC. | Clock assumption Observation 14: l Closed-loop power control can be beneficial for interference alleviation in multi-reader scenario. With closed-loop power control, readers can dynamically decrease the transmission power of D2R to avoid interference to other readers or increase the transmission power of D2R if the signal quality is poor. l Closed loop power control requires additional overhead in R2D. The additional overhead on the control information can be minimal, e.g., can be 2-bit same as TPC in NR or even smaller. |
| CEWiT | Device (un)availability Proposal 12: Support direction 2 on device unavailability in Rel-20. R2D sync Proposal 15: Support CFO/SFO calibration in Rel-20 for Device 2b/C, either by reusing existing R2D signals or introducing new dedicated sync signals. |
| HONOR | Device (un)availability Proposal 7: Study solutions under direction 2 to address unavailability. Positioning/proximity Proposal 8: Study positioning/proximity determination under solution 2. |

References

- RP-252964, “Revised SID: Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices”, RAN#109, Beijing, China, Sep, 2025

- TR 38.769	Study on solutions for ambient IoT (Internet of Things)

- R1-2508332	Discussion on Other Air Interface Procedures	FUTUREWEI

- R1-2508382	Discussion on other procedure for Ambient IoT	Spreadtrum, UNISOC

- R1-2508426	Discussion on Other Procedures for R20 AIoT	vivo

- R1-2508441	Other procedures for AIoT	Nokia

- R1-2508450	Discussion on other procedures	CMCC

- R1-2508502	Study on other procedures	Huawei, HiSilicon

- R1-2508514	Other procedures of A-IoT	Ericsson

- R1-2508545	Study on other procedures for Device 2b and C	NEC

- R1-2508591	Study of general procedures of A-IoT enhancement for device 2b/C	CATT

- R1-2508606	Discussion on other procedures for Ambient IoT	China Telecom

- R1-2508678	Discussion on other procedures for Device 2b/C	Xiaomi

- R1-2508721	Discussion on other procedures for Rel-20 A-IoT	OPPO

- R1-2508759	Study AIoT air interface for DO-A traffic	Tejas Network Limited

- R1-2508796	Study on other procedures of air interface for Device 2b/C	Samsung

- R1-2508820	Discussion on other procedures for active Ambient IoT device	ZTE Corporation, Sanechips

- R1-2508836	Discussion on other aspects of Device 2b_C	Transsion Holdings

- R1-2508860	Discussion on A-IoT other procedures	Panasonic

- R1-2508902	Other procedures for Device 2b/C	LG Electronics

- R1-2508959	Discussion on other procedures for R20 Ambient IoT	Lenovo

- R1-2509041	Procedures for Air Interface of Devices 2b/C	ROBERT BOSCH GmbH

- R1-2509104	On other procedures for device 2b/C	Apple

- R1-2509122	Other Procedures for Active Devices in Ambient IoT	InterDigital, Inc.

- R1-2509123	Considerations for A-IoT other procedures	Semtech Neuchatel SA

- R1-2509155	Other procedures for Ambient IoT	MediaTek Inc.

- R1-2509176	Discussion on other procedures	Sharp

- R1-2509190	Discussion on procedures for A-IoT active device	ASUSTeK

- R1-2509195	Discussion on other procedures for Device 2b/C 	Quectel

- R1-2509225	Study of other procedures for Device 2b/C	Qualcomm Incorporated

- R1-2509274	Study on other procedures for Ambient IoT outdoor for active device	NTT DOCOMO, INC.

- R1-2509324	Discussion on other procedures for Ambient IoT	TCL

- R1-2509332	Discussion on other aspects of Rel-20 AIoT	IIT Kanpur

- R1-2509345	Discussion on other procedures for Ambient IoT	CEWiT

- R1-2509424	System information and DO-A aspects for Ambient IoT outdoor devices	Sony

- R1-2509432	Discussion on other procedures for Device 2b/C	KT Corp.

- R1-2508967	Discussion on air interface for device 2b and device C	ETRI

- R1-2508980	Discussion on Air Interface Enhancements for R20 A-IoT	Fraunhofer HHI, Fraunhofer IIS

- R1-2508991	Views on air interface for Device 2b&C for Ambient IoT	HONOR

Annex A : RAN1 agreements

RAN1#122 (Bengaluru)

Agreement

For R2D, study necessity of addition/removal of M values from Rel-19 M value set for Device 2b and for Device C for at least the following M values:

- Addition of M=1

- Addition of M=32

- Removal of M=24

Agreement

Study necessity and feasibility of FEC for R2D for Device 2b and for Device C including the following aspects:

- Coding schemes: convolutional code (e.g TBCC), RM code

  - Note: focus on coding schemes already specified in LTE or NR

- FEC code rates: at least 1/2, 1/3

- Interleaver or LTE bit collection scheme for TBCC

Agreement

Study necessity and feasibility of R2D repetitions for R2D for Device 2b and for Device C considering the following repetition types:

- Block-level repetitions

- Bit-level Type-2 repetitions

- Chip-level repetitions (in case of M=1 with FEC if applied)

Agreement

For R2D signal(s), study at least the following functionalities (if needed), and study whether to reuse/enhance existing R2D signal(s) or to introduce new R2D signal(s) for the following functionalities (if needed):

- R2D chip duration determination

- indication of the start of R2D

- indication of the end of PRDCH

- frequency synchronization (including frequency acquisition)

- CFO estimation / LO calibration

- SFO calibration

- timing synchronization/tracking

- measurement (e.g., signal strength)

- reader identification/differentiation

Agreement

For D2R signal(s), study at least the following functionalities (if needed), and study whether to reuse/enhance existing D2R signal(s) or to introduce new D2R signal(s) for the following functionalities (if needed):

- CFO estimation

- SFO estimation

- timing acquisition

- channel estimation

- measurement (e.g., signal strength, interference estimation)

- Device differentiation e.g. for collision resolution, for interference randomization

Agreement

Study necessity and feasibility of reuse, modifications or enhancements of existing Rel-19 D2R modulations, and the potential introduction of new modulations schemes, for Device 2b and for Device C, including at least the following aspects:

- introduction of new modulation schemes (e.g., MSK or DBPSK)

- down-selection of existing modulation schemes

- design considerations related to spectrum shaping and D2R signals

Agreement

Study the following aspects to support DO-A traffic (periodic and event-triggered) for Device 2b and for Device C in Rel-20.

- Timing and frequency synchronization for DO-A transmission

- Resource allocation/determination method for DO-A transmission

- Configuration/signaling of necessary information

Agreement

Study necessity and feasibility of enhancements on existing Rel-19 D2R FEC for Device 2b and for Device C including the following aspects:

- additional FEC code rate(s) (e.g., 1/2, 1/4)

- interleaver, or LTE bit collection scheme for TBCC

- Note: no change to Rel-19 TBCC polynomial

Agreement

For PDRCH, study necessity and feasibility of larger (>2) number of repetitions for block-level repetitions for Device 2b and for Device C.

Agreement

For PDRCH, study necessity of scrambling for Device 2b and for Device C.

Agreement

At least for CBRA for DT and DO-DTT, study necessity and feasibility of enhancements of D2R TDM(A) for Device 2b and for Device C based on the following:

- X1 > 2 for Msg1 (X1: number of time domain resources for Msg1 transmission indicated by the corresponding R2D message)

- X3 > 1 for Msg3 (X3: number of time domain resources for Msg3 transmission indicated by the corresponding R2D message)

Agreement

Study necessity and feasibility of enhancements of D2R for Device 2b and for Device C, considering the following aspects or potential enhancements:

- Considering that Device 2b and Device C can adjust the LO towards a desired frequency, whether to also support baseband small FS for Device 2b and Device C

- D2R frequency resources for FDM(A) (e.g., considering D2R transmission bandwidth, guard-band size between FDMed D2R transmissions, CFO, SFO, etc.)

- How a device determines the frequency resource for D2R transmission

- FDM(A) for D2R transmissions other than Msg1 and Msg3

Agreement

Study necessity and feasibility of D2R Tx power control for Device 2b and for Device C, including the following aspects

- Open loop and/or closed loop power control

- Required measurements (e.g., signal strength)

- Required information at the device

- Required information at reader side

- Signaling overhead and power consumption

Agreement

At least for DT and DO-DTT, study necessity and feasibility of changes to existing timing offsets for Device 2b and for Device C, e.g.,

- timing offset value between R2D and corresponding subsequent D2R (e.g., Toffset1/2/3/4)

- timing offset value between D2R and corresponding subsequent R2D (e.g., T_D2R_min)

  - Note: RAN1 only discusses potential reasons that may impact the value of T_D2R_min, and if any provides those reasons to RAN4

- timing offset between R2D and corresponding subsequent D2R provided by a reader

- timing offset between D2R and corresponding subsequent R2D provided by a reader

At least for DT and DO-DTT, study necessity and feasibility of introduction of new timing offsets for Device 2b and for Device C, e.g.,

- timing offset between two consecutive R2D or D2R transmissions for the same Device, if such scenario exists

RAN1#122bis (Prague)

Agreement

Study L1 R2D control information with separate CRC for Device 2b and for Device C considering the following aspects

- Necessary information to be included in the L1 R2D control

- Details on how the L1 R2D control information is transmitted

Agreement

Study necessity and feasibility of R2D FDM from the same reader for Device 2b and for Device C considering at least the following aspects:

- complexity of device to support FDM

- how a device determines the frequency resource for receiving one of the FDMed R2D transmissions

Agreement

Study necessity and feasibility of D2R CDM(A) for Device 2b and for Device C with sequence-based Msg1 transmission and potential impact to random access procedures.

- FFS: other cases, except for data transmission

Agreement

For the functionality of indicating the start of the R2D transmission that contains PRDCH, study at least the following options if the functionality relies on SIP:

- Option 1:  binary sequence-based SIP (e.g M-sequence or Golay sequence).

- Option 2: Reuse the Rel-19 SIP.

Agreement

For the functionality of indicating the end of the PRDCH, study the following option:

- Option 1: Use the L1 R2D control information to indicate the TBS.

[H] Proposal 4.2a-r1

For PRDCH, study necessity and feasibility of scrambling for Device 2b and for Device C to achieve the following:

- Interference randomization

- To reduce continuous 1s or 0s if needed

Agreement

For the functionality of chip duration determination of the data payload in the PRDCH, study the following options:

- Option 1: same as the chip duration of the corresponding L1 R2D control information

- Option 2: indicated by the corresponding L1 R2D control information

Agreement

For the functionality of chip duration determination of the L1 R2D control information, study at least the following options:

- Option 1: Fixed chip duration. No use of CAP for chip duration determination.

- Option 2: Blind detection from a predefined set of the chip durations of L1 R2D control information. No use of CAP for chip duration determination.

- Option 3: Use CAP

  - Alt.3-1 Rel-19 CAP pattern

  - Alt.3-2 Enhanced CAP

    - Alt.3-2a: Rel-19 CAP pattern with repetition

    - Alt.3-2b: CAP pattern and the association between chip durations for CAP and chip durations of L1 R2D control information

- Option 4: Use a set of different binary sequences with fixed or variable length

  - Alt.4-1: using binary sequences for SIP. No use of CAP for chip duration determination.

  - Alt.4-2: using binary sequences for CAP

- Option 5: Use broadcast information (if any) to indicate the chip duration

Agreement

For the functionality of CFO estimation/LO calibration at the device side, study CFO calibration signal with the following aspects:

- Whether the signal is

  - Option a: unmodulated sinusoid single tone

  - Option b: unmodulated multiple sinusoid single tones

  - Option c: modulated multiple tones

    - Whether the CFO calibration signal is the same or different from the synchronization signal (if supported)

- Whether the CFO calibration signal is transmitted periodically and/or as needed (aperiodically)

- The time location(s) of CFO calibration signal.

  - Option 1: in relation to the L1 R2D control information

  - Option 2: in relation to the data payload of the PRDCH

  - Option 3: in relation to the PDRCH

  - Option 4: in relation to the synchronization signal (if supported)

  - Option 5: in relation to broadcast information (if supported)

  - Combination of Options is not precluded

- Other details (e.g., time duration, frequency location(s) of the CFO calibration signal)

Proposal 4.4.g

For R2D signals, for the functionality of timing synchronization/tracking, study the following options:

- Option 1: New periodic synchronization signals

- Option 2: R2D midamble(s)

- Option 3: Manchester encoding

Agreement

Study R2D synchronization signal at least for the functionality of frequency synchronization (including frequency acquisition) and for the functionality of timing synchronization/tracking.

- Alt1: periodic synchronization signal

- Alt2: aperiodic synchronization signal

Agreement

For resource allocation method for the first D2R transmission for DO-A for Device 2b and for Device C, study following options considering feasibility:

- Option 1: Periodic D2R resource allocation

- Option 2: Aperiodic D2R resource allocation

Agreement

For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, study at least following options considering feasibility:

- Option 1: necessary information provided by broadcast information

- Option 2: necessary information provided by a paging-like R2D message

- Option 3: necessary information derived from a synchronization signal and provided by broadcast information

Agreement

Study whether necessary information for the first D2R transmission for DO-A for Device 2b and for Device C is provided periodically and/or aperiodically by Reader.

Agreement

For the content of the first D2R transmission for DO-A for Device 2b and for Device C, study the following options:

- Option 1: Msg1-like content

- Option 2: DO-A data payload

  - FFS: with or without device ID

- Option 3: DO-A scheduling request

Agreement

The study assumes single-carrier waveform for D2R.

- FFS whether to use 2SB or 1SB for D2R

Agreement

Regarding D2R preamble for PDRCH, study the following aspects of the D2R preamble.

- Preamble length (e.g., longer than Rel-19 D2R preamble)

- Sequence type

- Number of sequences using the same length (e.g., for reader/device differentiation)

- Considering other factors affecting preamble design (e.g., D2R modulation and detection schemes)

Agreement

Regarding D2R midamble for PDRCH, study the following aspects of the D2R midamble.

- Midamble pattern(s) including the length(s) and interval(s) within (and possibly at the end of) a PDRCH

- Sequence type, and whether the same or different sequence type is used for D2R preamble and D2R midamble

- Considering other factors affecting midamble design (e.g., D2R modulation and detection schemes)

Proposal 4.1a

For R2D, it is beneficial to add M=1 to the set of M values from Rel-19 in order to support larger coverage distances.

R1-2508129	FL summary #7 for 10.3.2 “Study of air interface for Device 2b/C”	Moderator (LG Electronics)

Agreement

Study the necessity of D2R Tx power control for Device 2b and for Device C for outdoor scenarios

- Target to agree on observations on pros and cons of D2R Tx power control vs. no Tx power control at RAN1#123

Continue studying feasibility of the following open loop power control methods considering device complexity:

- Methods of open loop power control

  - Option 1: Device calculates Tx power based on measured pathloss, a predefined formula, and some configured parameters from reader

    - Required information at the device side at least includes reader Tx power, received target power at the reader, etc.

  - Option 2: Device measures RSRP, and uses a corresponding Tx power when the RSRP falls in an RSRP range.

    - The RSRP range is based on RSRP threshold(s) configured by reader or predefined

    - Required information at the device side at least includes RSRP threshold(s), corresponding D2R Tx power related information per each RSRP range defined by the RSRP threshold(s)

  - FFS details (e.g., required measurements, required information)

Continue studying the necessity and feasibility of closed loop power control for D2R.

Agreement

Study interleaving for PDRCH with FEC, including the necessity and feasibility of the following:

- Alt 1: reuse both LTE sub-block interleaver and LTE bit collection scheme

- Alt 2: reuse LTE bit collection scheme without LTE sub-block interleaver

- Alt 3: based on LTE bit collection scheme with some update (e.g., for memory reduction) without LTE sub-block interleaver

- Combination of alternatives can be studied

Agreement

Study interleaving for PRDCH with FEC, including the necessity and feasibility of the following:

- Alt 1: reuse both LTE sub-block interleaver and LTE bit collection scheme

- Alt 2: reuse LTE bit collection scheme without LTE sub-block interleaver

- Alt 3: based on LTE bit collection scheme with some update (e.g., for memory reduction) without LTE sub-block interleaver

- Combination of alternatives can be studied

Agreement

For PDRCH, further study the necessity of scrambling with following details:

- Scrambling applied after block-level repetition (if applied) and channel coding (if applied, including interleaving if supported)

- Scrambling sequence type and generation method

Agreement

Regarding D2R modulation for device 2b and device C, further study the following:

- Option 1: BPSK

  - FFS the corresponding preamble and midamble design for coherent detection at reader side

- Option 2: OOK

  - FFS the corresponding preamble and midamble design for coherent or non-coherent detection at reader side

- Option 3: DBPSK

  - FFS the corresponding preamble and midamble design for non-coherent detection at reader side

- Option 4: MSK

  - FFS the corresponding preamble and midamble design for coherent or non-coherent detection at reader side

Agreement

The minimum R2D transmission bandwidth of 1 PRB is the baseline for Rel-20.

Agreement

For R2D signals, study the necessity for the functionality of SFO calibration, and study the feasibility of the following options:

- Option 1: Reuse CAP and/or Manchester encoding

- Option 2: sequence-based SIP with or without R2D midamble(s)

- Option 3: using the CFO calibration signal

R1-2508130	FL summary #8 for 10.3.2 “Study of air interface for Device 2b/C”	Moderator (LG Electronics)

Agreement

For the study of necessity and feasibility of FEC for R2D for Device 2b and for Device C, further study the following aspects:

- Coding schemes

  - LTE TBCC

  - Tailed convolutional code with the same polynomial as LTE TBCC

  - RM code

- FEC code rates: 1/2, 1/3, 1/4

Proposal

For the study of necessity and feasibility of enhancements on existing Rel-19 D2R FEC for Device 2b and for Device C, further study the following codes:

- Code rate = 1/2 obtained by Rel-19 TBCC polynomial followed by puncturing

- Code rate = 1/4 obtained by Rel-19 TBCC polynomial followed by repetition and then puncturing

- Code rate = 1/4 obtained by adding a new polynomial for the fourth branch

Agreement

For R2D and D2R, study the order of channel coding (including interleaver if supported) and block-level repetition (if supported) for complexity reduction, considering necessity and feasibility.

Proposal

For the other factors affecting preamble and midamble design in the two agreements below, consider only the following:

- D2R modulation (BPSK, OOK, DBPSK, MSK)

- Detection schemes at the reader side (coherent or non-coherent detection)

Proposal

For R2D, for the study of addition/removal of M values from Rel-19 M value set for Device 2b and for Device C, assume a constant power within each OFDM symbol containing at least one OOK ON chip.

Agreement

For the study of R2D binary sequence-based SIP, study the following aspects:

- Sequence type

  - Option 1: m-sequence

  - Option 2: Golay sequence

- Length of sequence

- Number of sequence(s)

- FFS: whether to apply Manchester coding to the sequence

- Note: the same aspects can be studied for R2D midamble if applied with a sequence-based SIP

Agreement

For the study of binary sequence-based Msg1 for the study of necessity and feasibility of D2R CDM(A) for Device 2b and for Device C, study at least the following aspects:

- Sequence type (e.g. Gold sequence, m-sequence)

- Length of the sequence

- Number of sequences

Agreement

For R2D, study at least the following aspects for the study of broadcast information:

- Information included in the broadcast information

- Whether to use PRDCH to carry the broadcast information

- Transmission of broadcast information:

  - Option 1: periodic

  - Option 2: aperiodic

- Time and frequency resources for transmission of the broadcast information, e.g. in relation to the synchronization signal.

Agreement

For the L1 R2D control information preceding and scheduling the data payload of PRDCH, study the following aspects:

- Time location of L1 R2D control information:

  - Option 1: L1 R2D control information and data payload of PRDCH are contiguous in time.

  - Option 2: L1 R2D control information and data payload of PRDCH are not contiguous in time.

- Study frequency resource of L1 R2D control information

- Whether L1 R2D control information is included in the PRDCH that carries that data payload, or in another PRDCH, or in another channel