3GPP TSG RAN WG1 #123                                                                                 R1-2508957

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	10.3.2.1

Source:	Lenovo

Title:	Discussion on R2D for R20 Ambient IoT

Document for:	Discussion

# Introduction

In 3GPP RAN#108 new SID [1] on study on enhancements for solutions for A-IoT in NR outdoor for active devices was approved with following objectives on RAN1 scope:

- Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - Deployment scenario 4 with topology 1 with no external carrier wave. - Traffic types DO-DTT, DT, DO-A. - Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. - Assumption of the same air interface for Device 2b and Device C. - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 - Positioning for outdoor scenarios is under objective 2. This objective begins after RAN#108 (June 2025) RAN#111 (March 2026) will make a decision on whether to include outdoor scenarios in Rel-20 normative work. This objective includes: RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. - While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range - Maximum transmit power of -20 dBm and -10 dBm for device 2b Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources. - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations. - Define link budget calculation for coverage. RAN4-led - Study coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios. - Use band n8 as an example band. NOTE: Strive to minimize evaluation cases in RAN1 and RAN4.

In this contribution, we present our perspective on the design of R2D Timing and Sync Signals and associated L1 Control information.

# Discussion

## R2D Timing and Sync Signals and L1 Control

Regarding R2D signal(s), the following agreement was reached at the RAN1#122 meeting.

Agreement For R2D signal(s), study at least the following functionalities (if needed), and study whether to reuse/enhance existing R2D signal(s) or to introduce new R2D signal(s) for the following functionalities (if needed): - R2D chip duration determination - indication of the start of R2D - indication of the end of PRDCH - frequency synchronization (including frequency acquisition) - CFO estimation / LO calibration - SFO calibration - timing synchronization/tracking - measurement (e.g., signal strength) - reader identification/differentiation

### CFO estimation / LO calibration

About CFO estimation at the device side, the following agreement was reached at the RAN1#122bis meeting.

Agreement For the functionality of CFO estimation/LO calibration at the device side, study CFO calibration signal with the following aspects: - Whether the signal is - Option a: unmodulated sinusoid single tone - Option b: unmodulated multiple sinusoid single tones - Option c: modulated multiple tones - Whether the CFO calibration signal is the same or different from the synchronization signal (if supported) - Whether the CFO calibration signal is transmitted periodically and/or as needed (aperiodically) - The time location(s) of CFO calibration signal. - Option 1: in relation to the L1 R2D control information - Option 2: in relation to the data payload of the PRDCH - Option 3: in relation to the PDRCH - Option 4: in relation to the synchronization signal (if supported) - Option 5: in relation to broadcast information (if supported) - Combination of Options is not precluded - Other details (e.g., time duration, frequency location(s) of the CFO calibration signal)

In this section, more details will be discussed about CFO estimation at the device side.

CFO calibration signal (CCS) types

For A-IoT devices, potential CFO calibration signals include: (1) a single-tone RF signal and (2) an OOK-based signal, corresponding to Option A and Option C in the agreement above, respectively. These two signal types will be analyzed separately.

Rel-20 A-IoT technology is expected to provide complexity and power consumption orders of magnitude lower than the existing 3GPP LPWA technologies (e.g. NB-IoT and eMTC), and the features of 6G relevant to IoT. A single-tone RF signal, consisting of a single sinusoidal waveform at a specific frequency (as illustrated by Figure 1), is particularly suitable for CFO calibration in Ambient IoT for the reasons including requiring minimal processing complexity, being robust against noise and interference, and being compatible with low-cost, low-power device.

When a single-tone RF signal is used for CCS, the device can estimate the CFO using either of the following methods. Time-domain method (phase rotation detection) typically requires down-conversion, offers robustness against interference and fading but increases implementation complexity. Frequency-domain method (peak detection) avoids down-conversion but is less resilient to interference and fading. The choice between these methods can be made based on its computational capabilities and performance requirements.

Figure 1 An example of CFO calibration signal by using single-tone RF signal

OOK serves as the physical layer modulation scheme for R2D in R19 A-IoT. OOK signal can also be considered as CFO calibration signal for active device in R20 A-IoT. Multiple CFO estimation methods using OOK signal are available, among which pulse width distortion analysis is promising to be used for A-IoT device due to simplicity. This method is performed by using OOK signal with known pattern (e.g., 101010…), as illustrated by Figure 2. Since the CFO is calculated based on the carrier frequency for the signal transmission, the transmitted pulse width (PW) of the ON-OFF and the estimated rising/falling edge intervals, the pulse width, i.e., duration of ‘1’needs to be known by device. In addition, timer resolution at device is required to be much less than the difference between estimated pulse width and the transmitted pulse width.

Figure 2 An example CFO Calibration signal by using OOK-based signal

Proposal 1: Regarding different CFO calibration signal types, the following aspects need to be further studied:

- What specific signal information need be indicated to the device?

- What is the implementation requirements of the estimation method on the device?

- What level of estimation accuracy is the method capable of?

The time location(s) of CFO calibration signal

To facilitate frequency calibration at the active device, enabling the device to acquire the CCS within the R2D, two information must be known by the device: 1) the location of the CCS within the R2D, and 2) the duration of the CCS transmission. In previous meeting, we agreed on multiple options regarding the time location of CFO calibration signal. However, the difference of CCS locations within the R2D structure remains under discussion.

Regarding the positioning of the CCS within the R2D, Figure 3 illustrates the following configuration alternatives. The dashed box indicates that the corresponding component is optional.

- P1: the CFO calibration signal locates immediately after R2D SIP

- P2: the CFO calibration signal locates after other signal(s), e.g., CAP, or synchronization signal, following SIP

- P3: the CFO calibration signal locates after L1 control information (L1-CI)

- P4: the CFO calibration signal locates after PRDCH

Figure 3 Example placement of CFO calibration signal in R2D

For P1 and P2, the device determines the CCS start position based on the end timing of SIP or the end timing of Other Signal. For P3, the device determines the CCS starting position based on the start position as well as duration of L1-CI, which can be either prior knowledge or indicated in the L1 control information. For P4, the device determines the CCS starting position based on start position of PRDCH and TBS of PRDCH indicated in L1-CI, which was agreed upon in RAN1#122b.

In addition, a key advantage of P4 is that it positions the CCS more closely to the subsequent D2R transmission than other options. This proximity is particularly beneficial when the PRDCH has a large TBS, given the time-sensitive nature of CFO calibration.

Proposal 2: It is beneficial to place the CFO calibration signal after the PRDCH, particularly when the PRDCH has a large TBS.

The time duration of CFO calibration signal

The definition of CCS duration parameters depend on the specific signal type used for CCS transmission.

In the case of CCS using single-tone RF signal, given that the device can continuously count in terms of chips, the chip-based measurement can be utilized to determine the duration of CCS. However, the CCS must be sufficiently long to ensure adequate calibration accuracy. As a result, using the number of chips as the time unit for a single-tone RF signal is inefficient, particularly when a large value of M is employed for R2D. Considering the R2D transmission is aligned with NR OFDM symbol (OS), OFDM symbol-based measurement can be utilized to determine the duration of single-tone RF signal.

In the case of OOK-based CCS, either chip duration or OS duration can be used to as time units for duration measurement. Furthermore, since the CCS is constructed using repeated ON-OFF cycles, its complete duration can be specified by the individual ON-OFF pattern duration and the number of pattern repetitions.

Proposal 3: For single-tone RF signal based CCS, the duration estimation can be performed using either chip duration, or OFDM symbol duration as the reference time unit.

Proposal 4: For OOK-based CCS, the duration estimation can be performed by using the individual ON-OFF pattern duration and the number of pattern repetitions.

### Synchronization signals

About synchronization signals carried by R2D, the following agreement was reached at the RAN1#122bis meeting.

Agreement Study R2D synchronization signal at least for the functionality of frequency synchronization (including frequency acquisition) and for the functionality of timing synchronization/tracking. - Alt1: periodic synchronization signal - Alt2: aperiodic synchronization signal

According to pervious agreement, periodic synchronization signals could be supported in R20 AIoT. In addition, in the Rel-19 A-IoT study, the SLEEP state of A-IoT devices has been analyzed: the device keeps clock #3 (the low-power real-time counter) running, while clock #5 (the high-frequency crystal oscillator) is disabled; upon returning to the ON state, clock #5 is re-enabled.

| Clock purpose # | Description | Clock speed | Applicable devices | Power consumption | Initial clock Accuracy [ppm] | Accuracy after clock sync / calibration at device side [ppm] |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | Time counting (if supported) | e.g. tens of kHz | 1 (if applicable) 2a, 2b | <0.1 μW | 104 - 105 | Calibration may not always be feasible for this clock purpose |
| 5 | LO for carrier frequency (for up/down conversion) | According to carrier frequency e.g., 900 MHz | 2b | 10s - 100s of µW | 103 - 104 | Within 10s – 200 (Note 2), there is no need to determine a single value for the study |

Observation 1: Given that distinct clocks are employed during the ON and SLEEP states, their respective CFO drift rates impose distinct requirements on the design of synchronization signals.

If the device detects and receives the synchronization signal only while in the ON state, it must rely on its internal timer (clock#3) to wake up after a period in SLEEP. However, because of carrier-frequency-offset (CFO) drift, the timing alignment between reader and device is no longer accurate. Consequently, the reader may transmit while the device is still asleep, or the device may wake up early and miss the transmission due to energy shortage or forcing it back into SLEEP without having received any data. Moreover, the differing CFO drift rates in ON and SLEEP imply that the synchronization signals for these two states should be transmitted at different granularities.

Proposal 5: The synchronization signals for the ON and SLEEP states require distinct transmission granularities.

### Indication of the start of R2D

On indication of the start of R2D, the following agreements were reached at the RAN1#122bis meeting.

Agreement For the functionality of indicating the start of the R2D transmission that contains PRDCH, study at least the following options if the functionality relies on SIP: - Option 1: binary sequence-based SIP (e.g M-sequence or Golay sequence). - Option 2: Reuse the Rel-19 SIP. Agreement For the study of R2D binary sequence-based SIP, study the following aspects: - Sequence type - Option 1: m-sequence - Option 2: Golay sequence - Length of sequence - Number of sequence(s) - FFS: whether to apply Manchester coding to the sequence - Note: the same aspects can be studied for R2D midamble if applied with a sequence-based SIP

The R2D binary sequence-based SIP is beneficial because it provides information to the device prior to the decoding of the L1 control information. Regarding the information provided by the SIP, study at least the following functionalities:

- Use R2D SIP for reader identification/differentiation

If a multi-reader deployment scenario is supported, to quickly identify the target reader for device is highly beneficial for reducing device power consumption. This can be achieved by assigning a different sequence to each reader.

- Use R2D SIP for R2D structure identification/differentiation

Based on the study, the R20 R2D structure demonstrates significant versatility in its composition—specifically regarding the inclusion or exclusion of L1 control information, CFO calibration signal, synchronization signal, or PRDCH. Quickly identifying the target R2D structure is highly beneficial for the device, as it facilitates the decoding of the remaining R2D parts.

Proposal 6: For R2D binary sequence-based SIP, study at least the following functionalities:

- Use R2D SIP for reader identification/differentiation

- Use R2D SIP for R2D structure identification/differentiation

If Rel-19 SIP is reused for indicating the start of the Rel-20 R2D transmission, the functionalities of reader identification/differentiation and R2D structure identification/differentiation may be supported by using L1 control information.

Proposal 7: If Rel-19 SIP is reused for indicating the start of the Rel-20 R2D transmission, study at least the following functionalities by using L1 control information:

- reader identification/differentiation

- R2D structure identification/differentiation

# Conclusion

In this contribution, we focus on  key aspects: R2D Timing and Sync Signals and associated L1 Control information as follows:

Proposal 1: Regarding different CFO calibration signal types, the following aspects need to be further studied:

- What specific signal information need be indicated to the device?

- What is the implementation requirements of the estimation method on the device?

- What level of estimation accuracy is the method capable of?

Proposal 2: It is beneficial to place the CFO calibration signal after the PRDCH, particularly when the PRDCH has a large TBS.

Proposal 3: For single-tone RF signal based CCS, the duration estimation can be performed using either chip duration, or OFDM symbol duration as the reference time unit.

Proposal 4: For OOK-based CCS, the duration estimation can be performed by using the individual ON-OFF pattern duration and the number of pattern repetitions.

Observation 1: Given that distinct clocks are employed during the ON and SLEEP states, their respective CFO drift rates impose distinct requirements on the design of synchronization signals.

Proposal 5: The synchronization signals for the ON and SLEEP states require distinct transmission granularities.

Proposal 6: For R2D binary sequence-based SIP, study at least the following functionalities:

- Use R2D SIP for reader identification/differentiation

- Use R2D SIP for R2D structure identification/differentiation

Proposal 7: If Rel-19 SIP is reused for indicating the start of the Rel-20 R2D transmission, study at least the following functionalities by using L1 control information:

- reader identification/differentiation

- R2D structure identification/differentiation

# References

RP-251884, “SID Study on enhancements for solutions for A-IoT in NR outdoor for active devices”, RAN#108 June 2025.