3GPP TSG RAN WG1 #123	R1- 2509110

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	11.3.1

Source:	Apple

Title:	Waveforms for 6GR air interface

Document for:	Discussion

# 1	Introduction

Waveform design is fundamental in meeting the diverse requirements of 6G, including wideband coverage and stringent power efficiency. While CP-OFDM and DFT-s-OFDM serve as proven baselines in 5G NR, they confront challenges such as high peak-to-average power ratio (PAPR), spectral leakage, mobility impairments, and sensitivity to RF non-idealities. This contribution revisits waveform design and proposes enhanced schemes to address these limitations for the 6G systems

# 2	Low PAPR Waveforms

## 2.1	Motivation

In RAN1 #122, the following agreements were reached [3]

Agreement CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded. Agreement CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink - Enhancements/modifications on CP-OFDM will be studied as potential additions - DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink Note: proponents to identify at least the target use cases, signals/channels to use the waveform, and how the proposal is intended (if applicable) to support multiplexing with CP-OFDM, including MRSS, and how multi-user multiplexing is supported, etc. Note: Proponents are encouraged to provide more detailed information on their proposals for the next meeting, e.g.: Proponents to characterize the main motivation for modification/additional waveform proposals: - Targeted link direction, i.e. DL, UL or both - Targeted use case (e.g. NTN, specific frequency range, etc.), if any - Potential motivations metrics used, and quantified gains for a proposal, e.g. - Coverage - Network energy efficiency - UE energy efficiency - Spectral efficiency - High speed tolerance - Scheduling flexibility - Integration with ISAC Proponents provide information on the following aspects, if applicable - MRSS compatibility - Target channels/signals, e.g. all channels, PxSCH only, etc. - MIMO (SU and MU-MIMO) compatibility - Target modulations, and impact to other modulations, if applicable - Multi-user multiplexing/scheduling flexibility - Multiplexing/coexistence with baseline waveforms - Impact on synchronization and initial access - Expected specification impact Transmitter/receiver complexity and impact to power consumption

OFDM has been adopted since 4G and is known to have high Peak to Average Power Ratio (PAPR). DFT-s-OFDM was introduced in the uplink of 4G systems and carried over to the 5G uplink design to address primarily the high PAPR issues of OFDM. A waveform with high PAPR requires linear amplification with considerable power back-off to avoid saturation in power amplifiers. This reduces the overall power efficiency and increases the risk of nonlinear distortion in the transmitters.

To mitigate those issues, some RF and Digital Front End (DFE) features (e.g., Envelope Tracking (ET), Digital Pre-Distortion (DPD), and Crest Factor Reduction (CFR)) with the high PAPR waveforms are used. From an RF implementation point of view, if we can achieve perfect ET and DPD, the nonlinear distortion before saturation as well as the PA efficiency which are caused by high PAPR of the waveform will be completely resolved. However, there are implementation imperfectness and difficulties. Overall, a low PAPR waveform is helpful to relax the burdens on the RF implementation aspects.

On the other hand, even with perfect ET and DPD, the saturation power of PA (Psat) is still limited, and clipping is unavoidable when we need to increase the average output power higher in coverage limited scenarios. The Psat clipping causes distortion and higher spectrum leakage. In particular, the sharp clipping creates undesired spectral components outside the intended signal band and degrades the ACLR performance. The CFR methods by a combination of clipping and filtering, can balance the ACLR and EVM while reducing the PAPR, and hence allow higher average output power while having a tolerable increase of ACLR and EVM.

However, even though the CFR clipping and filtering method strikes a trade-off between ACLR, EVM and PAPR, it is a heuristic compromise in nature and does not consider the intrinsic design of the waveform. On the other hand, directly designing a low PAPR waveform brings in another dimension for the optimization, and will provide the best trade-off between ACLR, PAPR, and spectral efficiency.

In summary we can say that there are three main advantages of using Low PAPR Waveform:

  - Mitigation of PA nonlinearity — This can also be fully achieved through ideal digital predistortion (DPD).

  - Improved PA efficiency — This can also be fully achieved with ideal envelope tracking (ET) + DPD

  - Coverage enhancement — This CANNOT be fully achieved with crest factor reduction (CFR) + ET/DPD

Table 1: Low PAPR Waveform Advantages

|  | EVM | Energy | Coverage |
| --- | --- | --- | --- |
| High PAPR | PA nonlinearity → signal distortion | Signal Amp < Vdd → wasted energy & heat | Backoff from PA sat. → limits output power |
| DFE/RF Solution | DPD (Digital Pre-Distortion) | ET (Envelope Tracking) | CFR (Crest Factor Reduction) or costly PA |
| Low PAPR Waveform | Reduce EVM without DPD | Reduce Energy consumption and Heat Dissipation without ET | Systematic optimization beyond CFR |

Based on the agreements from RAN1 #122 [3], the following table details the motivations of Low PAPR waveforms for 6G

Table 2: Low PAPR Waveform Motivation

| Motivation/Information | Comments |
| --- | --- |
| Targeted link direction | UL |
| Motivation | Uplink Coverage Enhancement, Energy Efficiency, Mitigation of PA non-linearity |
| MRSS compatibility | Yes |
| Target channels/signals | PUSCH |
| MIMO (SU and MU-MIMO) compatibility | N/A |
| Target modulations | pi/2-BPSK, QPSK |
| Multi-user multiplexing/scheduling flexibility | same as baseline DFT-S-OFDM |
| Multiplexing/coexistence with baseline waveforms | Yes |
| Impact on synchronization and initial access | N/A |
| Expected specification impact | RAN1 Specification RAN4 Requirements |
| Receiver Complexity | Transparent Non-transparent |
| Impact to power consumption | Improved PA efficiency |

Observation 1:  Low PAPR Waveform provides benefit in coverage (allows a higher output power without waveform saturation) and energy efficiency (depending on the PA type and operating mode).

Proposal 1: The following table details the motivations of Low PAPR waveforms for 6G

| Motivation/Information | Comments |
| --- | --- |
| Targeted link direction | UL |
| Motivation | Uplink Coverage Enhancement, Energy Efficiency, Mitigation of PA non-linearity |
| MRSS compatibility | Yes |
| Target channels/signals | PUSCH |
| MIMO (SU and MU-MIMO) compatibility | N/A |
| Target modulations | pi/2-BPSK, QPSK |
| Multi-user multiplexing/scheduling flexibility | same as baseline DFT-S-OFDM |
| Multiplexing/coexistence with baseline waveforms | Yes |
| Impact on synchronization and initial access | N/A |
| Expected specification impact | RAN1 Specification RAN4 Requirements |
| Receiver Complexity | Transparent Non-transparent |
| Impact to power consumption | Improved PA efficiency |

## 2.2	Summary and Evaluation Metrics for Low PAPR Waveforms

In RAN1 #122-bis, the following agreement was made [4]

Agreement - For uplink low-PAPR proposals, the link level performance evaluation criterion is Net Gain assuming same spectrum efficiency as the reference - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER - A realistic PA model should be used - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account. - For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged. - FFS: Other evaluation metrics - Note: Companies to report how to calculate the Tx power gain, modulation and coding

Low PAPR waveform design in 6G must be evaluated not only on instantaneous envelope statistics but by its overall impact on system-level performance. While waveform shaping and bandwidth extension methods such as FDSS and FDSS-SE reduce PAPR, their true value must be assessed under practical constraints such as EVM, ACLR compliance, and coverage enhancement.

Net gain was agreed as the link level performance criterion in RAN1 #122-bis [4], defined with a realistic PA model as

Net Gain = Δ𝑆𝑁𝑅 + ΔPower

Δ𝑆𝑁𝑅 captures the gain or loss in SNR compared with basic DFT-S-OFDM due to the use of a low PAPR waveform at a desired BLER (typically a loss). The ΔPower captures the gain or loss in UE output power captured by the PAPR difference with the basic DFT-S-OFDM or the Maximum Power Reduction (MPR) represented by difference in the maximum Pout under ACLR constraints.

MPR implicitly accounts for the RF requirement and EVM performance of a waveform under a specific Power Amplifier (PA) model, offering a more realistic measure of achievable output power.

A key objective for 6G is to enable significant power boosting for UEs beyond their nominal power class (e.g., PC2 or PC3) to achieve substantial coverage gains. This draws inspiration from the high-power capabilities of GSM handheld devices, which supported a nominal maximum output power of 2W (33 dBm) for GSM class 4 handheld UE [TS 45.005] by leveraging the constant envelope GMSK waveform. To realize a similar capability in 6G, near-constant envelope waveforms based on DFT-s-OFDM, such as those using π/2-BPSK with GMSK approximation filters, are critical.

However, achieving this level of power boost requires a re-evaluation of the PA models and RF requirements. The PA model in R4-164542, while useful for legacy evaluations, may not accurately reflect the behaviour of a high-power PA in a power-boosted mode. Furthermore, operating at significantly higher power levels, such as 33 dBm, necessitates a careful reconsideration of RF requirements, including potentially tighter RF requirement, to ensure coexistence and regulatory compliance.

Therefore, a dedicated study is needed to define a high-power PA model that captures these non-linearities and to establish the associated RF requirements for enabling a power boost feature in 6G.

Observation 2: The existing PA model from, e.g. R4-164542 may be insufficient for evaluating true high-power performance. A new realistic PA model to accurately assess the benefits of near constant envelop waveforms may be required to study near constant envelop waveform especially in a power-boost mode.

Observation 3: Enabling a high-power boost feature necessitates a re-evaluation of RF requirements to ensure spectral compliance and manage potential interference

Proposal 2: To evaluate MPR or power gain, PA model and RF requirement should be revisited to support power boost feature in 6G.

### 2.3 Low PAPR Waveforms

In RAN1 #122, the following agreements were reached [3]

Agreement CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded.

In this section, we discuss specific low PAPR enhancements to DFT-S-OFDM. We study UL π/2-BPSK DFT-S-OFDM with support for FDSS, FDSS-SE and GMSK approximation filters to reach near constant envelope waveform with both transparent and non-transparent schemes.

Proposal 3: Study UL π/2-BPSK DFT-S-OFDM with support for FDSS, FDSS-SE and GMSK approximation filters to reach near constant envelope waveform using transparent and non-transparent schemes.

#### 2.3.1 DFT-s-OFDM with FDSS and FDSS-SE

Low PAPR waveform design has been proposed to 5G NR. That is mainly based on DFT-s-OFDM, π/2-BPSK with Frequency- Domain Spectral Shaping (FDSS). The study in 5G was based on DFT-s-OFDM with Frequency- Domain Spectral Shaping (FDSS) and FDSS with spectral Extension (FDSS-SE). Within 3GPP, RAN4 has specified nominal FDSS filter options, typically 2-tap and 3-tap FIR structures. There are numerous proposals, contributions and suggestion submitted to 3GPP on the FDSS, ranging from 2 to 3 tap filters. While this approach reduces PAPR significantly, the resulting waveform still exhibits a PAPR of ~1.5–2 dB, leaving a gap from the ideal 0 dB PAPR of true constant-envelope signals. In 5G NR, FDSS is transparent and is up to UE implementation in the current spec. At the receiver side, DMRS is used to learn the FDSS for demodulation. FDSS is also applied to QPSK, i.e., there is DFTs-OFDM QPSK + FDSS.

#### 2.3.2 DFT-s-OFDM with GMSK Approximation

Constant envelope waveforms are especially appealing for achieving high transmit efficiency, as they enable the power amplifier (PA) to operate in saturation without the need for signal clipping. However, these waveforms are generally more suitable for lower data rate applications. Among them, Minimum Shift Keying (MSK)—and particularly its Gaussian-filtered variant, Gaussian Minimum Shift Keying (GMSK)—is widely used. GMSK is a type of Continuous Phase Modulation (CPM) characterized by its constant envelope and low peak-to-average power ratio (PAPR), which has led to its adoption in systems such as 2G GSM, IEEE WPAN, and Bluetooth. Notably, 5G NR’s π/2-BPSK combined with frequency-domain spectral shaping (FDSS) is also motivated by an approximation of GMSK using Laurent’s linear decomposition.

As was mentioned, using GMSK is beneficial for transmission with low PAPR and efficient use of spectrum. But it has been used for narrowband transmission like in GSM with 200 KHz channel bandwidth or in Bluetooth transmission. Traditionally GMSK has not been used for wideband transmissions mainly due to large number of ISI taps caused by the frequency selective channel. If we use GMSK for wideband transmission, we need to employ a linear frequency equalizer either by appending cyclic prefix/unique word to it or use it with DFT-s-OFDM.  Since DFT-s-OFDM is the candidate waveform for the 6G, it motivates to use it jointly together with GMSK.

Pierre Laurent in his original paper proposed a decomposition method that simplifies the GMSK signal into a sum of Pulse Amplitude Modulation (PAM) components, making it easier to implement it with reduced complexity compared to other CPM schemes [2]. The main advantage of this decomposition is that it transforms a nonlinear modulation problem into a linear combination of simpler PAM, making it easier to analyse and implement in practical systems. The good news of using this approximation is that in practice, only the first few Laurent pulses are needed to approximate the signal accurately, depending on the value of the bandwidth-time product —a key design parameter in the GMSK waveform. We can define the Laurent basis in such a way that matches with FDSS pulses defined in 5G NR. However, this configuration does not achieve the ideal 0 dB PAPR. In practice, even with a Gaussian filter of BT = 0.5, a bandwidth extension is needed to maintain fidelity to the GMSK waveform. A typical 20% bandwidth extension has been studied during Release 18. Yet, even under this extension, the minimum achievable PAPR remains around 1-2dB. Reducing the BT parameter to, e.g., 0.25 can help constrain the bandwidth, but in that case, the one-pulse approximation becomes insufficient to capture GMSK characteristics. To address this, we propose including the second Laurent pulse in the approximation. This allows better fidelity to the GMSK structure while still limiting bandwidth extension. A block diagram of 2-pulse GMSK approximation filter in DFTs-OFDM framework is illustrated in Figure 1 below.

Figure 1 Block diagram of the GMSK DFT-s-OFDM with two Laurent pulses, where J0n≜S(n),  n=1, 2, ..., M is the pi/2-BPSK modulation, and J1n≜S(n-2).[SnSn-1],  n=1, 2, ..., M with S0=SM and S-1=SM-1, is the second modulation stream derived and modified from Laurent’s PAM coefficients of second pulse [2].

2.3.3 Transparent and Non-transparent receivers

#### In 5G NR, FDSS is transparent and the presence or absence of FDSS is not signalled to the receiver with limits to the FDSS filter governed through RAN4 spectral flatness requirements. Typically, the effect of the FDSS filter is captured during channel estimation and handled by UE implementation. It is possible that details of the FDSS filter can be used by the receiver e.g. in channel estimation, in a non-transparent receiver.

## 2.4 Net Gain Evaluation for several Low PAPR Waveforms

In this section, we discuss evaluation parameters and evaluation results for several low-PAPR waveforms.

### 2.4.1 Evaluation Parameters

Table 3 details the evaluation parameters used in the study.

Table 3: Evaluation Parameters

| Parameter | Value | Comments |
| --- | --- | --- |
| Waveform | DFT-S-OFDM | No FDSS |
|  | Frequency- Domain Spectral Shaping (FDSS & FDSS-SE) | 3-tap: [ 0.175 1 0.175] 3-tap: [0.28 1 0.28] SE bandwidth: 20% |
|  | GMSK Approximation Filter | BT = 0.5, 1-pulse BT = 0.35, 1-pulse BT = 0.25, 1 & 2-pulse BT =0.18, 1 & 2-pulse SE bandwidth: 20% |
| Metrics | SNR @ 10%BLER, PAPR, MPR, Net Gain |  |
| Frequency | 3.7 GHz |  |
| SCS | 30 kHz |  |
| Channel | TDL-C | Delay Spread 300 ns |
| Frequency | 3.7 GHz |  |
| SCS | 30 kHz |  |
| UE speed | 3 km/h |  |
| Antenna Configuration | 1 x 4, 1 layer |  |
| Receiver Description |  | DMRS-Based Channel Estimation with MMSE Equalization |
| RB Allocation | 100 MHz with 24 RB Allocation |  |
| DMRS | 2 DMRS symbols | π/2-BPSK DMRS |
| Modulation | π/2-BPSK |  |
| Code Rate /Rate Matching | 0.3066 | NOTE: The SE for all schemes are kept constant based on the code rate of the evaluation. |
| ACLR requirement | 31 dBc, 37 dBc |  |
| PA Model | Nominal PA for highly efficient PA with RAN4 calibration |  |
| BW Extension | Symmetric Bandwidth extension | Subcarrier combining at the receiver |

#### 2.4.1.1 Note on Bandwidth Extension

To implement bandwidth extension, we use the symmetrical bandwidth extension shown in Figure 2, (a). At the receiver side we do subcarrier combining by summing up for those subcarrier amounts which have been extended in Tx side. In order to keep the spectral efficiency for all schemes constant, in case of bandwidth extension we adjust the code rate. In our evaluation we use 24RBs and the code rate of 0.3066. For 20% of bandwidth extension, we adjust the code rate by 1,2*0,3066=0,36792 for bandwidth extension scenarios. This ensures that the agreement in RAN1 #122-bis in which the Net Gain should assume the same spectrum efficiency as the reference is kept.

Figure 2 Symmetric bandwidth extension (a) Tx side (b) Rx side

### 2.4.2 Evaluation Results

In this section we discuss simulation results for different low power PAPR waveform scenarios. We

- evaluate DFT-S-OFDM with π/2-BPSK, and FDSS or FDSS-SE

- assume both legacy FIR filters and a GMSK approximation filter

- Set the ACLR requirement to 31 dBc and 37 dBc.

When we apply FDSS, we evaluate the performance when the FDSS is unknown at the receiver (Transparent) or known at the receiver (non-Transparent). In the non-Transparent case with the GMSK approximation filters and 2 pulse transmission we assume that only the first or main pulse is known at the receiver.

For these results we have considered Net Gains as follows:

Metric 1: Net Gain = ΔSNR + ΔPAPR

Metric 2: Net Gain = ΔSNR + ΔPout

Table 4 summarizes some example cases where we have considered two 3-tap FDSS filters along with a few GMSK cases (1- or 2-pulse with different BT parameter values).We compare the results in terms of their Δ𝑆𝑁𝑅s (dB) at 10% BLER, 99.99% ΔPAPR (dB),  ΔPout, and the Net Gains as defined above. For the Net Gains with ΔPout we have considered two ACLR values as the limiting factor of Tx power under a realistic PA nonlinearity that are suitably configured for near constant envelop waveform (i.e. high efficiency PA).  We emphasize that when we mention in the table GMSK, we mean that we still transmit π/2-BPSK symbols with DFT-S-OFDM, but as the FDSS filters we are using GMSK approximation filters as we have demonstrated in Figure 1.

Table 4 Net Gain Results

| Transparent vs non-Transparent; with BW 20% extension or without BW extension | Modulation (with DFT-s-OFDM and Coding Rate=0,3066) | Δ𝑆𝑁𝑅 @10% BLER (dB) | 99.99% ΔPAPR (dB) | ΔPout (dBm) @ 31dBc ACLR | ΔPout (dBm) @ 37 dBc ACLR | Net Gain (dB), ΔPAPR-based | Net Gain (dB), ΔPout-based @ 31 dBc ACLR | Net Gain (dB), ΔPout-based @ 37 dBc ACLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transparent, No BW Extension | π/2-BPSK | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | π/2-BPSK + FDSS [0.28 1 0.28] | -0,51 | 3,56 | 4,54 | 4,88 | 3,05 | 4,03 | 4,37 |
|  | π/2-BPSK + FDSS [0.175 1 0.175] | -0,21 | 2,07 | 2,8 | 2,88 | 1,86 | 2,59 | 2,67 |
|  | GMSK 1-pulse (BT=0.5) | -0,44 | 3,76 | 4,12 | 4,38 | 3,32 | 3,68 | 3,94 |
|  | GMSK 1-pulse (BT=0.35) | -0,54 | 4,26 | 4,75 | 5,12 | 3,72 | 4,21 | 4,58 |
|  | GMSK 1-pulse (BT=0.25) | -0,77 | 4,51 | 4,93 | 6,1 | 3,74 | 4,16 | 5,33 |
|  | GMSK 2-pulse (BT=0.25) | -0,83 | 4,89 | 5,1 | 7,81 | 4,06 | 4,27 | 6,98 |
|  | GMSK 1-pulse (BT=0.18) | -1,03 | 4,27 | 4,95 | 7,15 | 3,24 | 3,92 | 6,12 |
|  | GMSK 2-pulse (BT=0.18) | -1,28 | 5,11 | 5,11 | 8,13 | 3,83 | 3,83 | 6,85 |
| non-Transparent, No BW Extension | π/2-BPSK | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | π/2-BPSK + FDSS [0.28 1 0.28] | -0,58 | 3,56 | 4,54 | 4,88 | 2,98 | 3,96 | 4,3 |
|  | π/2-BPSK + FDSS [0.175 1 0.175] | -0,23 | 2,07 | 2,8 | 2,88 | 1,84 | 2,57 | 2,65 |
|  | GMSK 1-pulse (BT=0.5) | -0,32 | 3,76 | 4,12 | 4,38 | 3,44 | 3,8 | 4,06 |
|  | GMSK 1-pulse (BT=0.35) | -0,42 | 4,26 | 4,75 | 5,12 | 3,84 | 4,33 | 4,7 |
|  | GMSK 1-pulse (BT=0.25) | -0,51 | 4,51 | 4,93 | 6,1 | 4 | 4,42 | 5,59 |
|  | GMSK 2-pulse (BT=0.25) | -0,61 | 4,89 | 5,1 | 7,81 | 4,28 | 4,49 | 7,2 |
|  | GMSK 1-pulse (BT=0.18) | -0,58 | 4,27 | 4,95 | 7,15 | 3,69 | 4,37 | 6,57 |
|  | GMSK 2-pulse (BT=0.18) | -0,81 | 5,11 | 5,11 | 8,13 | 4,3 | 4,3 | 7,32 |
|  | Modulation (with DFT-s-OFDM and Coding Rate=1,2*0,3066=0,36792) |  |  |  |  |  |  |  |
| Transparent, with 20% BW Extension | π/2-BPSK | -0,31 | 0,82 | -1,47 | -1,45 | 0,51 | -1,78 | -1,76 |
|  | π/2-BPSK + FDSS [0.28 1 0.28] | -0,63 | 3,67 | 4,07 | 4,28 | 3,04 | 3,44 | 3,65 |
|  | π/2-BPSK + FDSS [0.175 1 0.175] | -0,35 | 2,57 | 1,46 | 1,51 | 2,22 | 1,11 | 1,16 |
|  | GMSK 1-pulse (BT=0.5) | -0,85 | 4,81 | 5,1 | 6,74 | 3,96 | 4,25 | 5,89 |
|  | GMSK 1-pulse (BT=0.35) | -1,04 | 4,92 | 5,1 | 7,96 | 3,88 | 4,06 | 6,92 |
|  | GMSK 1-pulse (BT=0.25) | -1,32 | 4,84 | 4,98 | 7,97 | 3,52 | 3,66 | 6,65 |
|  | GMSK 2-pulse (BT=0.25) | -1,43 | 5,46 | 5,11 | 8,14 | 4,03 | 3,68 | 6,71 |
|  | GMSK 1-pulse (BT=0.18) | -1,68 | 4,38 | 4,99 | 7,98 | 2,7 | 3,31 | 6,3 |
|  | GMSK 2-pulse (BT=0.18) | -1,93 | 5,39 | 5,11 | 8,15 | 3,46 | 3,18 | 6,22 |
| non-Transparent, with 20% BW Extension | π/2-BPSK | -0,31 | 0,82 | -1,47 | -1,45 | 0,51 | -1,78 | -1,76 |
|  | π/2-BPSK + FDSS [0.28 1 0.28] | -0,72 | 3,67 | 4,07 | 4,28 | 2,95 | 3,35 | 3,56 |
|  | π/2-BPSK + FDSS [0.175 1 0.175] | -0,35 | 2,57 | 1,46 | 1,51 | 2,22 | 1,11 | 1,16 |
|  | GMSK 1-pulse (BT=0.5) | -0,66 | 4,81 | 5,1 | 6,74 | 4,15 | 4,44 | 6,08 |
|  | GMSK 1-pulse (BT=0.35) | -0,73 | 4,92 | 5,1 | 7,96 | 4,19 | 4,37 | 7,23 |
|  | GMSK 1-pulse (BT=0.25) | -0,8 | 4,84 | 4,98 | 7,97 | 4,04 | 4,18 | 7,17 |
|  | GMSK 2-pulse (BT=0.25) | -0,86 | 5,46 | 5,11 | 8,14 | 4,6 | 4,25 | 7,28 |
|  | GMSK 1-pulse (BT=0.18) | -0,86 | 4,38 | 4,99 | 7,98 | 3,52 | 4,13 | 7,12 |
|  | GMSK 2-pulse (BT=0.18) | -1,07 | 5,39 | 5,11 | 8,15 | 4,32 | 4,04 | 7,08 |

From the results in the Table, we observe the following:

Observation 4: FDSS Filtering

- For ACLR 31 and ACLR 37, waveforms with π/2-BPSK, DFT-S-OFDM and the GMSK approximation filter show consistently better Net Gains compared to those of legacy filtering and no filtering

Observation 5: Transparency vs Non-Transparency

- For ACLR 31 and ACLR 37, the non-Transparent scheme shows improvement in Net Gain for the π/2-BPSK with DFT-S-OFDM using the GMSK approximation filters

- For ACLR 31 and ACLR 37, the non-Transparent scheme does not show any Net Gain for the legacy 3-tap filters

Observation 6: Symmetric Bandwidth Extension

- For ACLR 31, symmetric BW extension shows Net Gains for π/2-BPSK with DFT-S-OFDM using the GMSK approximation filter only with BT = 0.5. For other BT values we do not observe improvement.

- For ACLR 37, symmetric BW extension shows Net Gains for all the π/2-BPSK with DFT-S-OFDM using the GMSK approximation filter (i.e., 1 pulse or 2 pulse, with different BT values)

- For ACLR 31 and ACLR 37, symmetric BW extension does not show any Net Gains for the legacy 3-tap filters

From the analysis, results and observations, we propose following proposals:

Proposal 4: Consider near constant envelope waveform and potential impact to RAN4 for example in terms of amount of power boost and associated RF requirement.

Proposal 5: Study Transparent/non-Transparent scheme with the GMSK approximation filter to approximate constant envelope waveform with π/2-BPSK and DFT-S-OFDM for uplink coverage enhancement

Proposal 6: Send LS to RAN4, to inform RAN4 that near constant envelope waveform shall be considered for 6G, and request RAN4 to evaluate the PA model, RF requirements, maximum power boost, and Tx power gain of a near constant envelope (sub-1dB) waveforms.

# 3	DFT-S-OFDM in the DL

In RAN1 #122-bis, there were multiple discussions on proposing DFT-S-OFDM in the DL for 6G.

The advantages for DFT-S-OFDM in the DL have been discussed to include reducing the transmit PAPR, increasing energy savings in TN networks and improving coverage.

However, the use of DFT-S-OFDM in the DL has the following disadvantages:

- Large Specification Impact: There may need to be a re-design of multiple channels to support DFT-S-OFDM.

- Separate handling of physical channel and signals: There is a need for the UE to implement separate handling for CP-OFDM and DFT-S-OFDM for one or multiple physical channel and signals.

- Multiple Channel and Signal Multiplexing: Multiplexing multiple signals and channels with DFT-S-OFDM may be challenging.

- Frequency domain user allocation and user multiplexing: Multiplexing multiple users in the frequency domain with DFT-S-OFDM may be challenging.

- Use case Clarification: Multiple use cases in terrestrial networks and NTN have been discussed but there is a need to identify a specific use case to drive the choice of signals/channels to be designed.

- Evidence based study: There needs to be a well-defined evaluation methodology to evaluate the need for DFT-S-OFDM in the DL e.g. for energy savings a well-defined energy model is needed.

From the points above, DFT-S-OFDM in the DL should not be supported until the discussed disadvantages are resolved by proponents.

Proposal 7: Do not support DFT-S-OFDM in the DL

# 4	Multi-Layer DFT-S-OFDM

In RAN1 #122-bis, the following agreement was made [4]:

Agreement - Study the evaluation method for evaluating DFT-s-OFDM for UL with number of layers > 1.

There is currently support for only single layer DFT-S-OFDM in 5G NR with multi-layer uplink transmission supported for CP-OFDM only. However, in 4G LTE, there is support for 2-layer DFT-S-OFDM with non-coherent pre-coding. The evaluation should use the 4G LTE scheme as baseline.

In terms of the evaluation metric, the use of multi-layer DFT-S-OFDM is for increased SE at the cell edge i.e. higher spectral efficiency in a coverage enhancement scenario. As such, similar to the baseline DFT-S-OFDM coverage enhancement study, the Net Gain should be used as the evaluation metric. Any further evaluation on spectral efficiency improvements, comparison with CP-OFDM should be left to the MIMO study.

Proposal 8: Use the Net Gain as Evaluation Metric for Multi-Layer DFT-S-OFDM

- Baseline scheme is 2-layer DFT-S-OFDM with non-coherent precoding

# Conclusion

In this contribution, we provided our views on waveforms for the 6G air interface. We have the following observations and proposals:

Observation 1:  Low PAPR Waveform provides benefit in coverage (allows a higher output power without waveform saturation) and energy efficiency (depending on the PA type and operating mode).

Proposal 1: The following table details the motivations of Low PAPR waveforms for 6G

| Motivation/Information | Comments |
| --- | --- |
| Targeted link direction | UL |
| Motivation | Uplink Coverage Enhancement, Energy Efficiency, Mitigation of PA non-linearity |
| MRSS compatibility | Yes |
| Target channels/signals | PUSCH |
| MIMO (SU and MU-MIMO) compatibility | N/A |
| Target modulations | pi/2-BPSK, QPSK |
| Multi-user multiplexing/scheduling flexibility | same as baseline DFT-S-OFDM |
| Multiplexing/coexistence with baseline waveforms | Yes |
| Impact on synchronization and initial access | N/A |
| Expected specification impact | RAN1 Specification RAN4 Requirements |
| Receiver Complexity | Transparent Non-transparent |
| Impact to power consumption | Improved PA efficiency |

Observation 2: The existing PA model from, e.g. R4-164542 may be insufficient for evaluating true high-power performance. A new realistic PA model to accurately assess the benefits of near constant envelop waveforms may be required to study near constant envelop waveform especially in a power-boost mode.

Observation 3: Enabling a high-power boost feature necessitates a re-evaluation of RF requirements to ensure spectral compliance and manage potential interference

Proposal 2: To evaluate MPR or power gain, PA model and RF requirement should be revisited to support power boost feature in 6G.

Proposal 3: Study UL π/2-BPSK DFT-S-OFDM with support for FDSS, FDSS-SE and GMSK approximation filters to reach near constant envelope waveform using transparent and non-transparent schemes.

Observation 4: FDSS Filtering

- For ACLR 31 and ACLR 37, waveforms with π/2-BPSK, DFT-S-OFDM and the GMSK approximation filter show consistently better Net Gains compared to those of legacy filtering and no filtering

Observation 5: Transparency vs Non-Transparency

- For ACLR 31 and ACLR 37, the non-Transparent scheme shows improvement in Net Gain for the π/2-BPSK with DFT-S-OFDM using the GMSK approximation filters

- For ACLR 31 and ACLR 37, the non-Transparent scheme does not show any Net Gain for the legacy 3-tap filters

Observation 6: Symmetric Bandwidth Extension

- For ACLR 31, symmetric BW extension shows Net Gains for π/2-BPSK with DFT-S-OFDM using the GMSK approximation filter only with BT = 0.5. For other BT values we do not observe improvement.

- For ACLR 37, symmetric BW extension shows Net Gains for all the π/2-BPSK with DFT-S-OFDM using the GMSK approximation filter (i.e., 1 pulse or 2 pulse, with different BT values)

- For ACLR 31 and ACLR 37, symmetric BW extension does not show any Net Gains for the legacy 3-tap filters

Proposal 4: Consider near constant envelope waveform and potential impact to RAN4 for example in terms of amount of power boost and associated RF requirement.

Proposal 5: Study Transparent/non-Transparent scheme with the GMSK approximation filter to approximate constant envelope waveform with π/2-BPSK and DFT-S-OFDM for uplink coverage enhancement

Proposal 6: Send LS to RAN4, to inform RAN4 that near constant envelope waveform shall be considered for 6G, and request RAN4 to evaluate the PA model, RF requirements, maximum power boost, and Tx power gain of a near constant envelope (sub-1dB) waveforms.

Proposal 7: Do not support DFT-S-OFDM in the DL

Proposal 8: Use the Net Gain as Evaluation Metric for Multi-Layer DFT-S-OFDM

- Baseline scheme is 2-layer DFT-S-OFDM with non-coherent precoding

# References

- TR 38.830, Study on NR coverage enhancements

- Laurent Pierre A., “Exact and Approximate Construction of Digital Phase Modulations by Superposition of Amplitude Modulated Pulses (AMP)”, IEEE Trans. on Communications, COL. COM-34, No. 2, Feb 1986.

- Final Minutes Report of 3GPP TSG RAN WG1 #122, August 2025

- Draft Report of 3GPP TSG RAN WG1 #122-bis, October 2025