3GPP TSG RAN WG1 Meeting #76bis		R1-141701

Shenzhen, China, March 31st – April 4th 2014

Source:	QUALCOMM Incorporated

Title:	Design of UL DPCCH ACK channel for DL FET

Agenda item:		6.2.4

Document for:	Discussion/Decision

# 1	Introduction

Design of UL ACK to support DL FET was discussed in RAN1 #76, and a way forward was agreed in [1], where the slot format for UL DPCCH is agreed to have 6 pilot symbols, 2 TPC, and the remaining two symbols may be used for TFCI or ACK signalling.  In this contribution, a design for ACK signalling based on two-slot BPSK is considered and evaluated.

# 2	UL DPCCH structure with ACK

UL DPCCH with slot format (nPilot = 6, nTFCI/ACK/NACK =2, nTPC = 2) is considered with TFCI sent over the first 10 slots, using (20,5) Reed Muller code specified in Table 15A and mapping described in Section 4.7.2.2 of [2], allowing for 32 TFCI values, i.e, the TFCI bit sequence of (0,0,0,0,0) is also allowed unlike CQI bit sequence (0,0,0,0,0).

To send the ACK, two consecutive slots are used to repeat the ACK message. Thus, there is an ACK opportunity every alternate slot starting from slot 10, after TFCI information is sent, with the last ACK opportunity occurring at slot pairs #24 and #25, where enumeration of slots starts from 0 over two radio frames. In other words, the first slot of a slot pair carrying ACK is always an even-numbered slot. Slot pairs (26,27), (28,29) and (29,30) are considered too late to send an ACK message and always carry NACK. The ACK in each slot is sent using BPSK modulation at the same level as DPCCH pilot. When ACK is not being sent, NACK is sent in every slot.

Table 1 summarize simulation assumptions for evaluations.

Table 1: Two slot BPSK, single link

| Parameter | Configuration |
| --- | --- |
| UL transmission time | 10ms |
| TFCI encoding | First 10 slots carry TFCI using (20,5) Reed Muller Code, allowing 32 |
| T2P (FULL,SID to DPCCH) | 2.6940, -5.46 dB |
| Target false alarm | 3e-4 (per-ACK false alarm |
| Target BLER | 1% |
| DL TPC error rate | 4% |
| ACK opportunities | Every alternate slot beginning at slot #10 and ending with slot#25, where slots are enumerated starting from 0 over two radio frames |

# 3	Performance Results

## 3.1	Performance of ACK

Table 2 and Table 3 show the performance of ACK signalling in single link and soft-handover with two-link with zero imbalance between the two links. As can be seen in this table, missed detection rate for target false alarm rate of 3e-4 per ACK is less than 2% in single link for all cases, and is about 12% on the average, with maximum of about 17%, in the two-link soft handover scenario. The low missed detection rate and stringent false alarm rate guarantees efficient DL FET operation in single link and multi-link scenarios.  The relatively higher missed detection rate in soft handover is still low enough to provide significant gains for DL FET (80% likelihood to receive the ACK and terminate the transmission).

Proposal 1: ACK shall be sent in pairs over consecutive pairs of slots starting from slot number 10 with a total of 8 ACK opportunities every 20ms, i.e., over slot pairs (10,11), (12,13),…,(24,25) (where slots in a TTI are numbered 0 through 29).

Table 2: Two-slot BPSK, single link

| Channel | Packet Type | BLER at 10ms | TFCI error rate | DPCCH Ec/No setpoint (dB) | Per ACK False Alarm Rate | Overall False Alarm Rate | Missed detection rate |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PA3 | FULL | 0.0102 | 0.0002 | -15.2964 | 0 | 0 | 0.0133 |
| PA3 | SID | 0.0104 | 0.0004 | -17.8513 | 0.0001 | 0.0002 | 0.0048 |
| PB3 | FULL | 0.0103 | 0.0003 | -15.1361 | 0 | 0.0001 | 0.0092 |
| PB3 | SID | 0.0103 | 0.0003 | -17.8074 | 0 | 0.0002 | 0.0035 |
| VA30 | FULL | 0.0103 | 0.0006 | -14.5308 | 0.0001 | 0.0004 | 0.0125 |
| VA30 | SID | 0.0103 | 0.0003 | -17.2659 | 0.0001 | 0.0004 | 0.0049 |
| VA120 | FULL | 0.0103 | 0.0003 | -14.2912 | 0.0003 | 0.001 | 0.0198 |
| VA120 | SID | 0.0103 | 0.0003 | -16.9817 | 0.0002 | 0.0005 | 0.0117 |

Table 3: Two slot BPSK, soft handover

| Channel | Packet Type | BLER at 10ms | TFCI error rate | DPCCH Ec/No setpoint (dB) | Per ACK False Alarm Rate | Overall False Alarm Rate | Missed detection rate |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4 | 0.0103 | 0.0002 | -17.2895 | 0 | 0 | 0.0913 |
| 1 | 5 | 0.0103 | 0.0002 | -19.9101 | 0 | 0.0001 | 0.0497 |
| PA3 | FULL | 0.0104 | 0.0004 | -17.0818 | 0.0003 | 0.001 | 0.1748 |
| PA3 | SID | 0.0104 | 0.0003 | -19.6968 | 0.0001 | 0.0004 | 0.1258 |
| PB3 | FULL | 0.0106 | 0.0005 | -16.1815 | 0.0001 | 0.0003 | 0.1269 |
| PB3 | SID | 0.0106 | 0.0005 | -18.9961 | 0.0001 | 0.0002 | 0.0875 |
| VA30 | FULL | 0.0106 | 0.0006 | -15.6895 | 0 | 0.0001 | 0.1254 |
| VA30 | SID | 0.0112 | 0.0011 | -18.4821 | 0.0001 | 0.0002 | 0.0887 |
| VA120 | FULL | 0.0106 | 0.0006 | -15.2243 | 0.0001 | 0.0002 | 0.1411 |
| VA120 | SID | 0.0107 | 0.0006 | -18.0815 | 0.0001 | 0.0002 | 0.1053 |

## 3.2	Performance of TFCI

The decoding error rate of TFCI is also presented in Table 2 and Table 3. Since the effective code rate for TFCI with the (20,5) code is 1/4, compared to the 1/3 coding rate for the data channel, TFCI appears to be sufficiently protected and the maximum error rate for TFCI is about 0.1% which is negligible compared to BLER target of 1%.  Since having 32 TFCI values is more than enough for most vocoders and specifically for AMR family of codecs, it is proposed to adopt the (20,5) coding for TFCI protection in UL DPCCH.

Proposal 2: TFCI shall have 32 possible values and encoded using (20,5) Reed Muller code specified in Table 15A of Section 4.7.2.2 of TS25.212.

# 4	Conclusions

Performance of ACK and TFCI in UL DPCCH channel using two-slot BPSK signalling and (20,5) Reed Muller coding, respectively, was evaluated in this contribution and shown to be acceptable.  Based on these evaluations and previous discussion in RAN1, it is proposed that:

Proposal 1: ACK shall be sent in pairs over consecutive pairs of slots starting from slot number 10 with a total of 8 ACK opportunities every 20ms, i.e., over slot pairs (10,11), (12,13),…,(24,25) (where slots in a TTI are numbered 0 through 29).

Proposal 2: TFCI shall have 32 possible values and encoded using (20,5) Reed Muller code specified in Table 15A of Section 4.7.2.2 of TS25.212.

# 5	References

- R1-141040, “Way forward on aspects of DCH enhancements,” Qualcomm et al.

- TS 25.212, 3GPP Technical Specification