3GPP TSG RAN WG1 Meeting #77,	R1-142479

Seoul, Korea, 19th – 23rd May 2014

Source:	Kyocera

Title:	        Physical resource design for D2D discovery

Agenda Item:	6.2.5.1.1

Document for:	Discussion/Decision

Introduction

During RAN1#76bis, the message size of D2D discovery was discussed [1]. SA2 proposed that the message size of D2D discovery signal is around 200 bits [2]. In RAN1 the message size is 104bits was agreed as a working assumption. As discussed in [3] [4], the message size has a big impact on the D2D discovery physical design. In this contribution, we reconsider the Physical design of D2D discovery especially the number of RBs necessary for the D2D discovery signal.

Number of RBs needed for D2D discovery

In this paper, we investigate the number of RBs necessary for D2D discovery signal using link level evaluation. The simulation assumption is listed in Appendix A. For simplicity in this simulation we assume no gaps and we ignore the need for D2D and UL WAN transition gap. We assume the performance baseline is 2PRB-pair of 104bits. The simulation results are below and we conclude the number of PRBs needed for D2D discovery should be 3 PRBs.

Table 1: Performance degradation at BLER = 0.01

| Number of RBS | Performance degradation from 104bits (2RBs) |
| --- | --- |
| 2 | 2.0 dB |
| 3 | 0.0 dB |

Figure 1: BLER performance

Proposal:

If the message size is 200bits, the number of PRBs needed for D2D discovery should be 3 PRBs.

Conclusions

Proposal:

If the message size is 200bits, the number of PRBs needed for D2D discovery should be 3 PRBs.

References

[1] Chairman's Notes RAN1#76bis

[2] R1-140185(S2-140568), LS reply on discovery message size, SA2

[3] R1-141115, Discussion on discovery message size, Huawei, HiSilicon

[4] R1-134075, D2D discovery message size, Huawei, HiSilicon

- Appendix A: Simulation Assumptions

The physical format is shown in Figure 1. Simulation assumptions are listed in the Table 2.

Data Symbol

DMRS

Figure 1: D2D discovery physical format

Table 2: Simulation assumptions

| Carrier frequency | 2000 [MHz] |
| --- | --- |
| System Bandwidth | 10 [MHz] |
| Channel model | ITU-R Umi NLOS w/ dual-mobility |
| Moving direction of Tx UE and Rx UE | Uniform Random(-180-180[deg.]), independently |
| Number of Tx Antennas | 1 |
| Number of Rx Antennas | 2 |
| Number of RB | 2 |
| Modulation | QPSK |
| Bit length | 200 bits |
| Channel coding | Turbo coding |
| CRC | 24 bits |
| UE velocities pairs (Tx, Rx) | (3,3) [km/h] |
| Channel Estimation | ZF with linear interpolation in time |
| Receiver Type | MRC |