3GPP TSG RAN WG1 Meeting #80			R1-150596

Athens, Greece, 9th – 13th February 2015

Agenda Item:	7.2.4.2.1

Source: 	CATR

Title: 	Performance Evaluations of 3D MIMO Phase I

Document for:	Discussion and Decision

Introduction

In RAN1#78bis meeting, new SID of elevation beamforming/Full-Dimension (FD) MIMO for LTE [1] is approved. The purpose of phase 1 is to identify antenna configurations and evaluation scenarios, and evaluate the performance of Rel-12 downlink MIMO using 3D channel model with realistic non full buffer traffic model.

The simulation assumptions for homogeneous network is agreed during RAN1#78bis meeting and the following email discussion, the details are shown as below:

Case1: 3D-UMa with ISD=500m, 2 GHz:  100 deg

Case2：3D-UMa with ISD=200m, (2 GHz mandatory, 3.5 GHz optional): 104 deg

Case3：3D-UMi with ISD=200m, (2 GHz, 3.5 GHz – both mandatory): 100 deg

The vertical antenna element spacing and number of antenna elements with the same polarization in each column are (dV , M): (0.8λ, 8) respectively for homogeneous networks.

In this contribution, we give initial evaluation results of Rel-12 DL MIMO using 3D channel model with full buffer traffic model.

Simulation assumptions and Evaluation results

In this section, the performance of Rel-12 DL MIMO using 3D-UMA and 3D-UMI channel model, including both full buffer and non full buffer traffic model is evaluated, and only SU-MIMO transmission is evaluated.

Table 1 Simulation results of phase 1 with full buffer

| Scenario | Mean UPT (Mbps) | 5% UPT (Mbps) |
| --- | --- | --- |
| 3D-UMi ISD 200m (FDD) | 2.21 | 0.054 |
| 3D-UMA ISD 500m (FDD) | 2.38 | 0.058 |
| 3D-UMA ISD 200m (FDD) | 2.28 | 0.052 |

Conclusion

In this contribution, we give initial evaluation results of Rel-12 DL MIMO using 3D channel model, including both full buffer traffic model.

References

- RP-141644 New SID Proposal: Study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE, Samsung, Nokia Networks.

- R1-144444,“WF on simulation assumptions”, Nokia Networks, Nokia Corporation, CMCC, Samsung.

Appendix

Table A.1 Simulation parameters for homogeneous network scenarios

| Parameters | Assumptions |
| --- | --- |
| Cellular Layout | Hexagonal grid, 7 sites, 3 Macro cells per site, geographical based wrap-around |
| Channel Model | 3D UMa ISD 500m,3D UMa ISD 200m, 3D UMi |
| Operating bandwidth (BW) | 10 MHz |
| Tx Power | 3D UMa ISD 500m: 46dBm 3D UMa ISD 200m: 41dBm 3D UMI :41 dBm |
| UE Speed | 3km/h |
| Antenna configuration | Transmitter: cross-polarized horizontal antenna array with 8TXRUs and (M,N,K)=(8,4,8) Receiver: 2Rx cross-polarized antenna at UE |
| Downtilt angle | θetilt = 100 degree for 3D-UMa ISD 500m, 3D-UMi ISD 200m θetilt = 104 degree for 3D-UMa ISD 200m |
| Antenna element spacing | (dV,dH)=( 0.8λ, 0.5λ,) |
| CQI/PMI reporting interval and frequency granularity | 5ms for CQI/PMI, 6RB |
| Feedback scheme | Rel-12 PUSCH feedback mode 3-2 with Rel-10 8Tx codebook |
| Delay for scheduling and AMC | 6ms |
| Scheduler | Proportional Fair |
| Receiver | MMSE-IRC With non-ideal interference covariance matrix estimation by using complex Wishart distribution with 12 degrees of freedom (Model in TR36.829 with DMRS based sample covariance matrix) |
| HARQ Scheme | Chase Combining |
| Maximum number of retransmissions | 4 |
| Traffic model | - FTP Model 1 with packet size 0.5 Mbytes (low ~20% RU, medium ~50% RU, high ~70%RU) - Full buffer model |
| Channel estimation | Non-ideal modeling of channel estimation error modeling is used, based on CSI-RS for channel measurements, based on DMRS for data demodulation, based on IMR for interference measurement |
| Handover margin | 3dB |
| Wrapping method | Geographical distance based |