# 3GPP TSG RAN WG1 Meeting #80bis	 R1-151476

# Belgrade, Serbia, 20th – 24th April 2015

# Title: 		Evaluation results for LAA and Wi-Fi Coexistence with DL-Only LAA

# Source: 	Alcatel-Lucent Shanghai Bell, Alcatel-Lucent

# Agenda Item:	7.2.4.2

# Document for: 	Discussion/Decision

# Introduction

In RAN1 #80, LBT schemes were classified into four categories for LAA channel access evaluations [1].

- Classify the evaluated LBT schemes according to the following categories:

  - Category 1: No LBT

  - Category 2: LBT without random back-off

  - Category 3: LBT with random back-off with fixed size of contention window

  - Category 4: LBT with random back-off with variable size of contention window

Note: Contention window is the maximum possible random back-off value

Note: Category classification does not restrict a LBT design investigation

Note: Company is encouraged to evaluate many categories as much as possible

- Illustrative examples

  - FBE procedure as defined in EN BRAN V1.8.0 belongs to category 2

  - LBE procedure with a fixed q for the contention window as defined in EN BRAN V1.8.0 belongs to category 3

- LBE procedure Op A with a variable q for the contention window as defined in EN BRAN V1.8.0 belongs to category 4

Also, the following simulation assumptions were agreed:

- 256 QAM is mandatory for all cases

- LDPC codes should be used in the simulations for all cases for the Wi-Fi network

In this contribution, the simulation results taking the above agreed simulation assumptions into account are given and updated for Wi-Fi and LAA coexistence on top of our previous contribution [2].

# Simulation scenarios and assumptions

The following Wi-Fi and LAA coexistence scenarios are evaluated as agreed in [3] :

- Indoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and FTP traffic  (LAA without licensed carrier)

- Outdoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and FTP traffic (LAA without licensed carrier)

In this contribution, the performance of LBT category 2, category 3, and category 4 are evaluated for Wi-Fi and LAA coexistence scenario. The FTP-3 traffic model is considered for all simulation cases in this contribution. The detailed LBT parameters and additional simulation assumptions can be found in the appendix. In addition, the evaluation results of LAA-LAA coexistence can be found in our companion contribution [5].

# Simulation results

The average user perceived throughput (UPT), latency/delay and buffer occupancy as performance metrics are given. Herein 𝜆 denotes the traffic arrival rate, and 𝜌 denotes the ratio of the mean served cell throughput to the mean offered cell throughput for a given traffic arrival rate [3].

## Indoor Scenarios

In Table 1 and Table 2, the coexistence simulation results for indoor scenario are provided without 256 QAM for LAA/Wi-Fi and with  BCC codec for Wi-Fi.

Table 1: Indoor without licensed CC, LBE (Category 3)

| Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in step 1 | Wi-Fi In step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 |
| UPT CDF [Mbps] | 5% | 10.29 | 26.35 | 40.37 | 5.10 | 3.10 | 15.15 | 3.53 | 2.22 | 16.14 |
|  | 50% | 67.38 | 74.19 | 73.19 | 47.34 | 52.49 | 58.66 | 35.02 | 34.58 | 44.68 |
|  | 95% | 101.44 | 103.10 | 95.68 | 88.82 | 84.77 | 86.05 | 76.72 | 70.14 | 75.22 |
|  | Mean | 63.23 | 70.98 | 71.76 | 47.82 | 49.19 | 56.96 | 36.19 | 35.50 | 44.54 |
| Delay CDF [s] | 5% | 0.032 | 0.032 | 0.036 | 0.032 | 0.032 | 0.036) | 0.032 | 0.032 | 0.036 |
|  | 50% | 0.064 | 0.057 | 0.055 | 0.095 | 0.092 | 71.00 | 0.163 | 0.159 | 0.100 |
|  | 95% | 0.852 | 0.426 | 0.171 | 1.629 | 1.930 | 375.0 | 2.675 | 3.857 | 0.683 |
|  | Mean | 0.202 | 0.119 | 0.077 | 0.362 | 0.389 | 0.133 | 0.585 | 0.715 | 0.201 |
| ρ |  | 0.95 | 0.96 | 0.88 | 0.87 | 0.88 | 0.84 | 0.79 | 0.75 | 0.79 |
| BO |  | 0.24 | 0.17 | 0.13 | 0.38 | 0.37 | 0.22 | 0.55 | 0.56 | 0.33 |
| λ |  | 0.70 |  |  | 0.85 |  |  | 1.00 |  |  |
| Additional comments |  | LBT category 3: q=32, Maximum Occupancy Time = 10 ms No 256 QAM for both LAA and Wi-Fi, and BCC Codec for Wi-Fi |  |  |  |  |  |  |  |  |

Table 2: Indoor without licensed CC, FBE  (Category 2)

| Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi In step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi In step 2 | LAA in step 2 |
| UPT CDF [Mbps] | 5% | 10.29 | 26.57 | 24.62 | 5.10 | 8.22 | 8.10 | 3.53 | 5.71 | 5.73 |
|  | 50% | 67.38 | 87.49 | 70.39 | 47.34 | 66.75 | 51.14 | 35.02 | 51.76 | 36.38 |
|  | 95% | 101.44 | 112.93 | 93.52 | 88.82 | 102.28 | 83.74 | 76.72 | 92.47 | 70.62 |
|  | Mean | 63.23 | 82.39 | 67.47 | 47.82 | 61.56 | 49.09 | 36.19 | 49.98 | 36.83 |
| Delay CDF [s] | 5% | 0.032 | 0.032 | 0.037 | 0.032 | 0.032 | 0.037 | 0.032 | 0.032 | 0.037 |
|  | 50% | 0.064 | 0.038 | 0.056 | 0.095 | 0.066 | 0.086 | 0. 163 | 0.094 | 0.134 |
|  | 95% | 0.852 | 0.339 | 0.304 | 1.629 | 1.086 | 0.847 | 2.675 | 1.838 | 2.048 |
|  | Mean | 0.203 | 0.119 | 0.146 | 0.362 | 0.246 | 0.221 | 0.585 | 0.418 | 0.423 |
| ρ |  | 0.95 | 0.95 | 0.85 | 0.87 | 0.91 | 0.78 | 0.79 | 0.74 | 0.59 |
| BO |  | 0.24 | 0.15 | 0.17 | 0.38 | 0.28 | 0.31 | 0.55 | 0.43 | 0.48 |
| λ |  | 0.7 |  |  | 0.85 |  |  | 1 |  |  |
| Additional comments |  | LBT category 2: frame period: 10ms(Maximum Occupancy Time = 9.5ms, idle time = 0.5ms) No 256 QAM for both LAA and Wi-Fi, and BCC Codec for Wi-Fi |  |  |  |  |  |  |  |  |

For the case without licensed carrier, we observe the following:

- For LBE-based LAA,

  - For the low and medium traffic load, LAA provides fair coexistence with Wi-Fi, while maintaining good performance itself.

  - However, for high traffic load, the Wi-Fi performance becomes slightly worse than the performance in Wi-Fi/Wi-Fi coexistence scenario due to the fact that the advantage of LBE-based LAA in channel access compared to Wi-Fi becomes more obvious.

- For FBE-based LAA,

  - Wi-Fi in Wi-Fi/LAA coexistence performs better that that in Wi-Fi/Wi-Fi coexistence across the range of the traffic load.

  - However, LAA does not perform very well, mainly due to its disadvantage in channel access compared to Wi-Fi, which on the other hand boosts the Wi-Fi performance. The relative performance of LAA compared to Wi-Fi becomes worse as the traffic load increases. Note that in these simulations, a periodicity of 10 ms is used for FBE, which means that LAA only has the chance to access the channel once every 10 ms.

- LBE-based LAA has better UPT than FBE-based LAA under various traffic loads. That is mainly because LBE could obtain more transmission opportunities than FBE when contending the unlicensed channel with Wi-Fi.

Observation 1: For the indoor scenario, in terms of UPT and latency, Wi-Fi has good performance in Wi-Fi/LAA coexistence for both FBE-based and LBE-based LBT mechanisms in the low and medium traffic load. When there is a performance difference between Wi-Fi and LAA due to unfair channel access, the difference become larger as the traffic load increases.

In addition, the coexistence simulation results for indoor scenario are provided with 256 QAM for LAA/Wi-Fi and LDPC codec for Wi-Fi in Table 3 and Table 4.

Table 3: Indoor without licensed CC, LBE (Category 3)

| Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in step 1 | Wi-Fi In step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 |
| UPT CDF [Mbps] | 5% |  |  |  | 3.28 | 2.28 | 20.04 |  |  |  |
|  | 50% |  |  |  | 48.12 | 53.38 | 66.95 |  |  |  |
|  | 95% |  |  |  | 113.97 | 115.53 | 119.23 |  |  |  |
|  | Mean |  |  |  | 52.73 | 54.54 | 69.52 |  |  |  |
| Delay CDF [s] | 5% |  |  |  | 24.00 | 24.00 | 24.00 |  |  |  |
|  | 50% |  |  |  | 100.00 | 99.00 | 65.00 |  |  |  |
|  | 95% |  |  |  | 1817.00 | 2275.00 | 536.00 |  |  |  |
|  | Mean |  |  |  | 388.52 | 474.10 | 152.37 |  |  |  |
| ρ |  |  |  |  | 0.92 | 0.79 | 0.74 |  |  |  |
| BO |  |  |  |  | 0.49 | 0.53 | 0.32 |  |  |  |
| λ |  | 0.70 |  |  | 1.10 |  |  | ? |  |  |
| Additional comments |  | LBT category 3: q=32, Maximum Occupancy Time = 10 ms 256 QAM for both LAA and Wi-Fi, and LDPC Codec for Wi-Fi |  |  |  |  |  |  |  |  |

Table 4: Indoor without licensed CC, LBE (Category 4)

| Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in step 1 | Wi-Fi In step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 |
| UPT CDF [Mbps] | 5% |  |  |  | 3.28 | 9.98 | 11.73 |  |  |  |
|  | 50% |  |  |  | 48.12 | 66.93 | 61.09 |  |  |  |
|  | 95% |  |  |  | 113.97 | 125.63 | 102.52 |  |  |  |
|  | Mean |  |  |  | 52.73 | 67.40 | 59.26 |  |  |  |
| Delay CDF [s] | 5% |  |  |  | 24.00 | 25.00 | 25.00 |  |  |  |
|  | 50% |  |  |  | 100.00 | 62.00 | 79.00 |  |  |  |
|  | 95% |  |  |  | 1817.00 | 2132.00 | 1008.00 |  |  |  |
|  | Mean |  |  |  | 388.52 | 409.50 | 250.36 |  |  |  |
| ρ |  |  |  |  | 0.92 | 0.89 | 0.69 |  |  |  |
| BO |  |  |  |  | 0.49 | 0.41 | 0.40 |  |  |  |
| λ |  | 0.70 |  |  | 1.10 |  |  | ? |  |  |
| Additional comments |  | LBT category 4: follows the ETSI standard 256 QAM for both LAA and Wi-Fi, and LDPC Codec for Wi-Fi |  |  |  |  |  |  |  |  |

When coexisting with Wi-Fi, the LAA of category 3 outperforms the LAA of category 4. That is because the contention window is fixed for the former one, while the latter doubles its contention window till the maximum value when the eCCA could not find N unoccupied eCCA slots within the current contention window size.

On the other hand, the Wi-Fi coexisting with the LAA of category 4 achieves higher UPT that of the Wi-Fi co-existing with LAA base on LBT of category 3.

Observation 2: When co-existing with Wi-Fi, LAA Category 3 achieves higher UPT than Category 4 due to the fixed contention window compared with varied contention window

## Outdoor Scenarios

In Table 5 and Table 6, the coexistence simulation results for outdoor scenario are provided without 256 QAM for LAA/Wi-Fi and with  BCC codec for Wi-Fi.

Table 5: Outdoor without licensed CC, LBE (Category 3)

| Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 |
| UPT CDF [Mbps] | 5% | 23.39 | 16.23 | 32.40 | 2.30 | 4.59 | 14.80 | 1.36 | 1.16 | 7.08 |
|  | 50% | 65.92 | 55.35 | 72.11 | 28.07 | 21.06 | 46.48 | 18.34 | 11.51 | 39.11 |
|  | 95% | 104.37 | 95.87 | 96.68 | 79.00 | 62.16 | 80.14 | 63.51 | 51.66 | 72.57 |
|  | Mean | 65.21 | 55.46 | 69.33 | 32.86 | 25.24 | 47.38 | 24.74 | 17.18 | 39.75 |
| Delay CDF [s] | 5% | 0.032 | 0.032 | 0.036 | 0.032 | 0.037 | 0.036 | 0.036 | 0.044 | 0.038 |
|  | 50% | 0.064 | 0.084 | 0.054 | 0.184 | 0.265 | 0.087 | 0.258 | 0.434 | 0.107 |
|  | 95% | 0.535 | 0.886 | 0.229 | 2.390 | 2.506 | 0.514 | 2.967 | 3.710 | 0.688 |
|  | Mean | 0.139 | 0.242 | 0.092 | 0.535 | 0.635 | 0.160 | 0.731 | 0.987 | 0.215 |
| ρ |  | 0.98 | 0.94 | 0.87 | 0.81 | 0.84 | 0.77 | 0.71 | 0.70 | 0.75 |
| BO |  | 0.17 | 0.21 | 0.12 | 0.49 | 0.57 | 0.25 | 0.64 | 0.74 | 0.33 |
| λ |  | 0.60 |  |  | 0.85 |  |  | 1.00 |  |  |
| Additional comments |  | LBT category 3: q=32, Maximum Occupancy Time = 10 ms No 256 QAM for both LAA and Wi-Fi, and BCC Codec for Wi-Fi |  |  |  |  |  |  |  |  |

Table 6: Outdoor without licensed CC, FBE (Category 2)

| Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 |
| UPT CDF [Mbps] | 5% | 23.39 | 23.20 | 38.74 | 2.30 | 4.40 | 6.93 | 1.36 | 1.55 | 4.55 |
|  | 50% | 65.92 | 68.23 | 69.00 | 28.07 | 29.96 | 41.61 | 18.34 | 17.88 | 30.24 |
|  | 95% | 104.37 | 107.73 | 96.80 | 79.00 | 77.27 | 72.83 | 63.51 | 67.99 | 59.05 |
|  | Mean | 65.21 | 67.72 | 68.17 | 32.86 | 34.51 | 41.27 | 24.74 | 23.68 | 30.97 |
| Delay CDF [s] | 5% | 0.032 | 0.033 | 0.037 | 0.032 | 0.033 | 0.037 | 0.036 | 0.035 | 0.037 |
|  | 50% | 0.064 | 0.060 | 0.056 | 0.184 | 0.180 | 0.107 | 0.258 | 0.339 | 0.166 |
|  | 95% | 0.535. | 0.481 | 0.266 | 2.390 | 1.647 | 0.835 | 2.967 | 3.344 | 1.465 |
|  | Mean | 0.139 | 0.130 | 0.092 | 0.535 | 0.449 | 0.231 | 0.731 | 0.803 | 0.379 |
| ρ |  | 0.98 | 0.94 | 0.85 | 0.81 | 0.89 | 0.74 | 0.71 | 0.75 | 0.68 |
| BO |  | 0.17 | 0.15 | 0.12 | 0.49 | 0.46 | 0.31 | 0.64 | 0.66 | 0.46 |
| λ |  | 0.60 |  |  | 0.85 |  |  | 1 |  |  |
| Additional comments |  | LBT category 2: frame period: 10ms(Maximum Occupancy Time = 9.5ms, idle time = 0.5ms) No 256 QAM for both LAA and Wi-Fi, and no LDPC for Wi-Fi |  |  |  |  |  |  |  |  |

The results for the outdoor case roughly follow the same trend as those in the indoor case. However, in terms of the absolute performance, it can be seen that Wi-Fi performs worse in LAA/Wi-Fi than in Wi-Fi/Wi-Fi for LBE-based LBT even with low and medium load. This is likely due to more interference observed by each node (including the interference from neighboring clusters) in the outdoor scenario, which magnifies the effect of the channel access advantage of LBE compared to the indoor scenario.

In addition, the coexistence simulation results for outdoor scenario are provided with 256 QAM for LAA/Wi-Fi and LDPC codec for Wi-Fi in Table 7 and Table 8.

Table 7: Outdoor without licensed CC, LBE (Category 3)

| Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in step 1 | Wi-Fi In step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 |
| UPT CDF [Mbps] | 5% |  |  |  |  |  |  | 2.82 | 1.95 | 8.17 |
|  | 50% |  |  |  |  |  |  | 27.36 | 25.51 | 60.76 |
|  | 95% |  |  |  |  |  |  | 97.09 | 79.72 | 104.32 |
|  | Mean |  |  |  |  |  |  | 36.34 | 30.37 | 57.95 |
| Delay CDF [s] | 5% |  |  |  |  |  |  | 27.00 | 27.00 | 26.00 |
|  | 50% |  |  |  |  |  |  | 185.00 | 240.00 | 71.00 |
|  | 95% |  |  |  |  |  |  | 1914.00 | 2954.00 | 589.00 |
|  | Mean |  |  |  |  |  |  | 458.15 | 670.03 | 174.29 |
| ρ |  |  |  |  |  |  |  | 0.82 | 0.77 | 0.71 |
| BO |  |  |  |  |  |  |  | 0.55 | 0.63 | 0.28 |
| λ |  |  |  |  |  |  |  | 1.00 |  |  |
| Additional comments |  | LBT category 3: q=32, Maximum Occupancy Time = 10 ms 256 QAM for both LAA and Wi-Fi, and LDPC Codec for Wi-Fi |  |  |  |  |  |  |  |  |

Table 8: Outdoor without licensed CC, LBE (Category 4)

| Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in step 1 | Wi-Fi In step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 |
| UPT CDF [Mbps] | 5% |  |  |  |  |  |  | 2.82 | 2.65 | 9.66 |
|  | 50% |  |  |  |  |  |  | 27.36 | 30.84 | 52.74 |
|  | 95% |  |  |  |  |  |  | 97.09 | 88.27 | 98.33 |
|  | Mean |  |  |  |  |  |  | 36.34 | 35.94 | 53.43 |
| Delay CDF [s] | 5% |  |  |  |  |  |  | 27.00 | 25.00 | 25.00 |
|  | 50% |  |  |  |  |  |  | 185.00 | 205.00 | 88.00 |
|  | 95% |  |  |  |  |  |  | 1914.00 | 2712.00 | 843.00 |
|  | Mean |  |  |  |  |  |  | 458.15 | 450.81 | 179.13 |
| ρ |  |  |  |  |  |  |  | 0.82 | 0.82 | 0.68 |
| BO |  |  |  |  |  |  |  | 0.55 | 0.58 | 0.33 |
| λ |  |  |  |  |  |  |  | 1.0 |  |  |
| Additional comments |  | LBT category 4: follows the ETSI standard 256 QAM for both LAA and Wi-Fi, and LDPC Codec for Wi-Fi |  |  |  |  |  |  |  |  |

The results for the outdoor case roughly follow the same trend as those in the indoor case.  So we have the further observations:

Observation 3: For both the indoor and outdoor scenarios, when co-existing with Wi-Fi, the LBE-based LAA mechanism performs better than FBE-based LAA because LBE can obtain more transmission opportunities.

Observation 4: For both the indoor and outdoor scenarios, Wi-Fi coexisting with the LAA of category 4 achieves higher UPT that of the Wi-Fi co-existing with LAA base on LBT of category 3.

Note that all these observations are made based on the specific assumptions that were chosen for evaluation, so they may have dependency on some of the parameters chosen for the LBT mechanism.

The results certainly suggest that further investigation is needed on the LBT mechanism, including the parameter tuning, in order to ensure fair coexistence between LAA and Wi-Fi. This is especially important for the high load case.

# Conclusion

In this contribution, we provided initial LAA and Wi-Fi coexistence simulation results for DL only LAA. Based on the simulation results, we made the following observations:

Observation 1: For the indoor scenario, in terms of UPT and latency, Wi-Fi has good performance in Wi-Fi/LAA coexistence for both FBE-based and LBE-based LBT mechanism in the low and medium traffic load. When there is a performance difference between Wi-Fi and LAA due to unfair channel access, the difference become larger as the traffic load increases.

Observation 2: When co-existing with Wi-Fi, LAA Category 3 achieves higher UPT than Category 4 due to the fixed contention window compared with varied contention window

Observation 3: For both the indoor and outdoor scenarios, when co-existing with Wi-Fi, the LBE-based LAA mechanism performs better than FBE-based LAA because LBE can obtain more transmission opportunities.

Observation 4: For both the indoor and outdoor scenarios, Wi-Fi coexisting with the LAA of category 4 achieves higher UPT that of the Wi-Fi co-existing with LAA base on LBT of category 3.

The simulation results suggest that further study is needed on the LBT mechanism, including the parameter tuning, in order to ensure fair coexistence between LAA and Wi-Fi, which is especially important for the high load case.

# References

3GPP TR 36.889, “Study on Licensed-Assisted Access to Unlicensed Spectrum”, Nov. 2014.

R1-151477, “Evaluation results for LAA and LAA Coexistence with DL-only LAA”, Alcatel-Lucent Shanghai Bell, Alcatel-Lucent, Mar. 2015.

R1-150890, “Table templates for DL only LAA coexistence evaluation results”, Ericsson, Feb. 2015.

R1-150189, “Coexistence Evaluation Results for DL-only LAA”, Alcatel-Lucent Shanghai Bell, Alcatel-Lucent, Feb. 2015.

R1-151477, “Evaluation results for LAA and LAA Coexistence with DL-only LAA”, Alcatel-Lucent Shanghai Bell, Alcatel-Lucent, Mar. 2015.

# Appendix: Simulation assumptions

According to the agreement on the simulation assumption, the following parameters are provided for details. The others are aligned with [1], which are not mentioned here.

Table 5 Coexistence evaluation assumptions

| Parameters | LAA | Wi-Fi |
| --- | --- | --- |
| Outdoor scenario layout | Based on SCE# 2a + unlicensed band X=4, Y=1; 10 UEs per operator per carrier; 10 m for min. distance between small cells of different operators |  |
| Indoor scenario layout | Based on SCE#3 + unlicensed band X=4, Y=1; 10 UEs per operator per carrier; 3 m minimal distance between small cells of different operators |  |
| Traffic model | FTP Model 3 file size: 0.5 Mbytes |  |
| Network synchronization | For the same operator, the network is ideally synchronized Small cells of different operators are not synchronized |  |
| Total BS TX power | 18 dBm | 18 dBm |
| Antenna configuration | 2Tx2Rx in DL, Cross-polarized; adaptive stream | 2Tx2Rx in DL, Cross-polarized; Open-loop with 2 fixed streams |
| CCA-ED | -73 dBm/MHz+23-PH / (1 MHz) | -62 dBm |
| Channel selection | None | None |
| Link adaptation | Realistic | Rate control - Minstrel algorithm |

Table 6 Wi-Fi additional simulation parameter

| Parameter |  | value |
| --- | --- | --- |
| MCS |  | 802.11ac MCS table (256QAM) |
| LDPC |  | Yes |
| MPDU size |  | 1 ms |
| Max PPDU duration |  | 3 ms |
| MAC | Coordination | DCF |
|  | SIFS, DIFS | SIFS, DIFS |
|  | Detection | Energy detection only |
|  | RTS/CTS | None |
|  | Contention window | Per DCF |
| ACK Modeled |  | Ideal reception, but resource utilization and interference caused by ACK is modelled. |

Table 7 LTE additional simulation parameter

| Parameter | value |
| --- | --- |
| Transmission schemes | TM10, QPSK/16QAM/64QAM/256QAM |
| Scheduler | Proportional fair |
| UE receiver | MMSE-IRC |
| Scheduling delay for LAA | 4 ms (Channel ON Status) |
| CQI/RI/PMI Feedback for LAA | Sub-band feedback with 1 ms feedback period and 1ms feedback delay |
| MAC HARQ | Number of maximum ReTx: 3; minimal interval of ReTx: 8 ms (Channel ON Status) |
| Outer loop link adaptation | Enabled |