3GPP TSG-RAN WG1 Meeting #80bis	R1-152379

Belgrade, Serbia, 20th - 24th April 2015

Agenda item:	7.2.2.2.2

Source:		Nokia Networks

Title:	Summary of offline discussion on link level simulation assumptions for uplink

Document for:	Discussion and Decision

# 1	Introduction

This document summarizes the outcome of the offline discussion on link level simulation assumptions for uplink for Work Item “LTE Carrier Aggregation Enhancement Beyond 5 Carriers”. The proposed simulation assumptions are presented in Table 1.

Table . Link level simulation assumptions for uplink

| Parameter | Setting |
| --- | --- |
| Carrier Frequency | 2 GHz |
| System Bandwidth | 10 MHz |
| Channel Model | EPA, ETU, 3 km/h, optionally 120 kmph, AWGN interference |
| Antenna Setup | 1Tx, 2 Rx, for TDD optionally 8 Rx, assumption on antenna correlation to be described by the proponent |
| Channel coding | Assumption to be described by the proponent |
| DMRS Structure | Assumption to be described by the proponent |
| Channel Estimation | Practical, non-ideal |
| Number of PRBs for PUCCH | Assumption to be described by the proponent |
| Transmit power | Assumption to be described by the proponent |
| PUCCH frequency hopping | Assumption to be described by the proponent |
| CRC length (if any) | Assumption to be described by the proponent [8, 16] |
| Performance Metric | ACK missed detection probability (1 %), NACK-to-ACK error probability (0.1%); DTX-to-ACK probability 1% With CRC, in case CRC check fail eNodeB considers all bits as “NACK” |