3GPP TSG RAN WG1 Meeting #81		                                               R1- 152671

Fukuoka, Japan, 25th – 29th May, 2015

# Title: 	Discovery transmission in multiple carriers

# Source: 	Alcatel-Lucent Shanghai Bell, Alcatel-Lucent

# Agenda Item:	6.2.3.2.2

# Document for: 	Discussion/Decision

# Introduction

The work item for enhanced LTE device to device proximity services [1] needs to support sidelink direct discovery announcements in a non-serving carrier and/or secondary cell. In the RAN2#89bis meeting, RAN2 discussed discovery transmission on a non-primary carrier and achieved the following agreements (Intra-PLMN scenario) [2]:

The option to configure a UE via RRC signalling to transmit discovery in another carrier can be allowed. The RRC signalling can be used to configure either Type 1 or Type 2 discovery configuration for non-primary carrier.

This contribution discusses the necessary enhancement to support discovery transmission in non-primary carrier regardless of whether it is intra-PLMN or inter-PLMN. It is assumed that the UE is authorized for inter-PLMN scenarios.

# Power setting for discovery on a non-primary carrier

When a UE wants to announce discovery on a non-primary carrier, it needs to identify a resource pool on that frequency first. According to RAN2 agreements, both Type 1 and Type 2 discovery need to be supported on a non-primary carrier via RRC signalling.

Once a UE is permitted to announce discovery on a non-primary carrier, power control for the announcement is necessary to avoid intra-frequency interference. In release 12, the transmission power for discovery message transmission is determined from the path loss of the PCell, which is not applicable for discovery announcement on a non-primary carrier illustrated in Fig.1. The path loss between the UE and the non-serving cell, where UE is in coverage of this non-serving cell, should be used for transmission power setting. If Type 2 discovery is configured for UE on the non-primary carrier, the PCell should indicate the non-serving cell for which cell the path loss needs to be estimated. If Type 1 discovery is configured for the UE on the non-primary carrier, the PCell may indicate resource pools associated with different non-serving cells such as Cell2 or Cell3 on the non-primary carrier. The UE needs to estimate which cell’s coverage is best on the non-primary carrier, and then the UE can select a transmission resource from the  resource pool of the corresponding non-serving cell. Because path loss is used in power control for discovery transmission, it is natural for the UE to use the RSRP on the non-primary carrier to determine which cell to use. The UE can use the resource pool corresponding to the non-serving cell with the strongest RSRP.

Fig.1 UE announcing in non-primary carrier

Proposal 1: If a UE is permitted to announce discovery on a non-primary carrier, the path loss between the UE and the non-serving cell should be used for transmission power setting.

Proposal 2: If Type 1 discovery is configured for the UE on the non-primary carrier, the UE should select a resource pool of the non-serving cell with the strongest RSRP.

# Conclusions

In this contribution, we discuss discovery transmission on a non-primary carrier and  make the following proposals:

Proposal 1: If a UE is permitted to announce discovery on a non-primary carrier, the path loss between the UE and the non-serving cell should be used for transmission power setting.

Proposal 2: If Type 1 discovery is configured for the UE on the non-primary carrier, the UE should select a resource pool of the non-serving cell with the strongest RSRP.

# References

[1] RP-142311, “Work item proposal for enhanced LTE device to device proximity services”, Qualcomm.

[2] R2-151722, Report of the LTE break-out session (ProSe).