3GPP TSG RAN WG1 Meeting #81	  R1-153035

Fukuoka, Japan, 25th - 29th May 2015

Source:	CMCC

Title:	Evaluation of MU-MIMO performance and legacy UE pairing

Agenda Item:	6.2.5.3

Document for:	Discussion and Decision

- Introduction

In 3GPP RAN #65 meeting, a new study item (SI) on EBF and Full Dimension MIMO (3D-MIMO) was approved for Release 13 [1].  One of the objectives of the SI is,

- Identify/evaluate potential enhancements required for implementing the SU/MU-MIMO transmission schemes that would provide the identified performance benefits including

  - Evaluate the need for defining additional measurement antenna ports up to 64 at maximum.

  - Evaluate the need for reference signal design enhancements (including SRS, CSI-RS, and DMRS).

  - Evaluate the need for enhancement of codebook and feedback mechanism for SU/MU-MIMO (including CQI definition, layer mapping, and precoder/rank indication).

  - Evaluate the need for enhancement of channel reciprocity based operation.

  - …

The simulation assumptions on DMRS enhancement evaluation have reached agreement in RAN1 #80 meeting, where full buffer traffic load was encouraged to evaluate the performance of up to 8-layer MU-MIMO transmission. This contribution discusses issues on legacy UE pairing and gives the system level simulation results of up to 8-layer MU-MIMO transmission.

- Discussion on legacy UE pairing

As discussed in contribution [2], 6 alternatives were introduced for up to 8-layer MU-MIMO transmission and following proposals were made.

Proposal 1. Alt.1 is slightly preferred to support up to 8-layer multi-user transmission with up to 8 orthogonal DMRS ports.

- Alt. 1. Reuse DMRS design of 8-layer single user transmission. This alternative provide 8 orthogonal DMRS ports using FDM (24 REs) and OCC length = 4.

Table 1. DMRS design for 8-layer MU-MIMIO of Alt.1

| FDM (orthogonal) | OCC (orthogonal) | Port 7 | [+1 +1 +1 +1] | Layer1 |
| --- | --- | --- | --- | --- |
|  |  | Port 8 | [+1 -1 +1 -1] | Layer2 |
|  |  | Port 11 | [+1 +1 -1 -1] | Layer3 |
|  |  | Port 13 | [+1 -1 -1 +1] | Layer4 |
|  | OCC (orthogonal) | Port 9 | [+1 +1 +1 +1] | Layer5 |
|  |  | Port 10 | [+1 -1 +1 -1] | Layer6 |
|  |  | Port 12 | [-1 -1 +1 +1] | Layer7 |
|  |  | Port 14 | [-1 +1 +1 -1] | Layer8 |

Fig. 2. RE mapping of DMRS design Alt.1

Proposal 2. Alt.6 is slightly preferred to support up to 8-layer multi-user transmission with up to 4 orthogonal DMRS ports.

- Alt. 6.  Provide 4 orthogonal DMRS ports using OCC length = 4 with 12REs, besides 2 scrambling sequences could be used to support other 4 quasi-orthogonal layers if needed.

Table 2. DMRS design for 8-layer MU-MIMIO of Alt.6

|  |  |  | Scrambling (quasi-orthogonal) |  |
| --- | --- | --- | --- | --- |
|  |  |  | nSCID = 0 | nSCID = 1 |
| OCC (orthogonal) | Port 7 | [+1 +1 +1 +1] | Layer 1 | Layer3 |
|  | Port 8 | [+1 -1 +1 -1] | Layer2 | Layer4 |
|  | Port 9 | [+1 +1 -1 -1] | Layer5 | Layer7 |
|  | Port 10 | [+1 -1 -1 +1] | Layer6 | Layer8 |

Fig. 2.  RE mapping of DMRS design Alt.6

2.1.  Discussion on max number of orthogonal DMRS

Since legacy UEs use OCC length of 2 to estimate DMRS, but OCC length of 4 is designed for new UEs. In order to guarantee the DMRS channel estimation accuracy of legacy UEs, orthogonality of DMRS design for new UEs need to be discussed while pairing with legacy UE.

For alt.1, if a legacy UE using port 7 ([1 1]) and a new UE using port 11 ([1 1 -1 -1]) are paired together, DMRS of new UE would interfere legacy UE DMRS estimation. However, it is possible to pair legacy UE on port 7 with a new UE on port 8 and/or port 13 on the same REs. Besides, port 9, 10, 12 and 14 using the different REs can provide other 4-layer transmission. Therefore the maximum number of orthogonal paired UE is 7 (but not 8) if a legacy UE is included in MU-pairing.

For alt. 6, if a legacy UE using port 7 ([1 1]) and new UE using port 9 ([1 1 -1 -1]) are paired together, DMRS of new UE would interfere legacy UE DMRS estimation. However, it is possible to pair legacy UE on port 7 with two new UEs on port 8 or port 10. Therefore the maximum number of orthogonal paired UE is 3 (but not 4) if a legacy UE is included in MU-pairing.

2.2.  Discussion on PDSCH puncture of legacy UEs

Legacy UEs only occupy 12 REs for MU-MIMO DMRS, however, total 24 REs may be occupied in alt.1. Therefore, the REs used for transmission of DMRS ports 9, 10, 12 and 14 may conflict with legacy UEs’ PDSCH transmission. In order to optimize channel estimation of DMRS ports 9, 10, 12 and 14, the PDSCH of legacy UE may be punctured on the corresponding REs.  Since there is no signal to inform that puncture, the data demodulation will be affected for legacy UEs.

The original alt.1 implies that 24 REs will be used for DMRS transmission while total number of transmit layer is more than 2, since port 7/8 and port 9 are distributed on different REs.  Thus, PDSCH of legacy UE will be punctured if total number of transmit layer is more than 2 while a legacy UE is included in MU-pairing. In order to reduce overhead of the new DMRS and reduce the probability of PDSCH puncture of legacy UE, only 12 REs may be used if total number of transmit layer is less than or equal to 4, and the other 12REs will be occupied if the total number of transmit layer is more than 4. Thus, another way of DMRS design of alt.1 is listed in Table 3.

Table 3.  Another DMRS design for 8-layer MU-MIMIO of Alt.1

| FDM (orthogonal) | OCC (orthogonal) | Port 7 | [+1 +1 +1 +1] | Layer1 |
| --- | --- | --- | --- | --- |
|  |  | Port 8 | [+1 -1 +1 -1] | Layer2 |
|  |  | Port 9 | [+1 +1 -1 -1] | Layer3 |
|  |  | Port 10 | [+1 -1 -1 +1] | Layer4 |
|  | OCC (orthogonal) | Port 11 | [+1 +1 +1 +1] | Layer5 |
|  |  | Port 12 | [+1 -1 +1 -1] | Layer6 |
|  |  | Port 13 | [-1 -1 +1 +1] | Layer7 |
|  |  | Port 14 | [-1 +1 +1 -1] | Layer8 |

Alt.6 has the same RE occupying as the conventional MU-MIMO, thus, it does not need to puncture PDSCH of legacy UE while pairing with legacy UE.

- System level simulation of higher order MU-MIMO

Performance evaluation results of the conventional MU-MIMO (up to 2-layer) and higher order MU-MIMO (up to 8-layer) are shown in this section. Assume UE only has one transmit antenna for SRS transmitting, and single layer is transmitted per UE. 3D-UMi with 200m ISD and antenna configuration (8,4,2,64) with one-to-one mapping are applied in the simulation. Table 4 and 5 give the simulation results of the conventional MU-MIMO and higher order MU-MIMO using FTP and Full Buffer traffic model, respectively.

Table 4. Simulation results of higher order MU-MIMO using FTP model ()

| 3D-UMi, 200m ISD (8,4,2,64) One-to-one | Up to 2-layer MU-MIMO | Up to 8-layer MU-MIMO with total 12 REs (e.g. Alt.6) | Up to 8-layer MU-MIMO with total 24REs (e.g. Alt.1) |
| --- | --- | --- | --- |
| 5%UPT(bps/Hz) | 0.531 | 1.008 | 0.896 |
| 50%UPT(bps/Hz) | 2.007 | 2.586 | 2.299 |
| Mean UPT(bps/Hz) | 1.914 | 2.301 | 2.045 |
| RU | 68% | 51% | 51% |

From Table 4, we can see that up to 8-layer MU-MIMO with total 12 REs improves 5% UPT by 90%, 50% UPT by 29% and mean UPT by 20% compared to 2-layer MU-MIMO. Considering total 24REs overhead, the 5%, 50% and mean UPT are still improved by 69%, 15% and 7%. Besides, the RU is decreased from 68% to 51%.

Table 5. Simulation results of higher order MU-MIMO using Full Buffer model

| 3D-UMi, 200m ISD (8,4,2,64) One-to-one | Up to 2-layer MU-MIMO | Up to 8-layer MU-MIMO with total 12 REs (e.g. Alt.6) | Up to 8-layer MU-MIMO with total 24REs (e.g. Alt.1) |
| --- | --- | --- | --- |
| Cell average(bps/Hz) | 4.11525 | 6.489 | 5.768 |
| Cell edge(bps/Hz) | 0.09324 | 0.0954 | 0.0848 |

From Table 5, we can see that up to 8-layer MU-MIMO has almost the same performance for cell-edge users compared to 2-layer MU-MIMO. However, the cell average throughput is increased by 40%~58% corresponding to the DMRS overhead.

- Conclusion

This contribution discusses issues on legacy UE pairing and gives the system level simulation results of up to 8-layer MU-MIMO transmission. The system level simulation shows 40%~58% cell average throughput improvement while using up to 8-layer MU-MIMO transmission.

References

- RP-141644, “Study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE,” Samsung, Nokia Networks, 3GPP RAN #65, Edinburgh, Scotland, 9 - 12 September, 2014.

- R1-150449, “DMRS design for Higher Order MU-MIMO”, CMCC, 3GPP TSG RAN WG1 Meeting #80, Athens, Greece, 9 – 13 February, 2015.

Annex

Table 1: 3D-MIMO Evaluation Assumptions

| Parameters | Values |
| --- | --- |
| Homogeneous scenarios | 3D-Umi ISD 200m |
| Polarized antenna modeling | Model-2 from 36.873 |
| Traffic model | FTP and Full Buffer（15 users per cell） |
| Wrapping method | Geographical distance based |
| Handover margin | 3dB |
| System bandwidth | 10MHz (50 PRBs) |
| UE attachment | Based on RSRP (formula) from CRS port 0 |
| Carrier Frequency | 3D-Umi ISD 200m: 2GHz |
| Downtilt | 3D-Umi ISD 200m: 100 degree |
| Network synchronization | Synchronized |
| UE Speed | 3km/h |
| UE distribution | according to 36.873 |
| UE array orientation | ΩUT,a uniformly distributed on [0,360] degree, ΩUT,b = 90 degree, ΩUT,g = 0 degree |
| UE antenna pattern | Isotropic antenna gain pattern A’(θ’,ф’) = 1 |
| Receiver | Non-ideal channel estimation and interference modeling, LMMSE-IRC receiver, detailed guidelines according to Rel-12 [71-12] assumptions, |
| UE Rx configuration | 2 Rx x-polar (+90/0) |
| Duplex Mode | TDD |
| Feedback | PUSCH 3-0 for reciprocity based operation |
| SRS | CQI reporting triggered per 5ms |
| Transmission scheme | Feedback delay is 5 ms |
| SRS | 1Tx, 5 ms periodicity, wideband |
| Transmission scheme | TM10, single CSI process, dynamic SU/MU-MIMO with rank adaptation (no CoMP) |
| Overhead | 3 symbols for DL CCHs, 2 CRS ports and DM-RS with 12 or 24 REs per PRB |
| Scheduler | Frequency selective scheduling (multiple UEs per TTI allowed) |

Table 2: BS Antenna Configuration

| Parameter | Values |
| --- | --- |
| Number of columns (N) | 4 |
| Antenna Polarization (P) | P = 2: cross-pol (eNB: +/- 45 deg, UE: 0/90 deg) |
| Horizontal antenna element spacing dH | 0.5λ |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8 dBi |
| Vertical antenna element spacing and Number of antenna elements with the same polarization in each column (dV , M) | (0.8 λ, 8) |