3GPP TSG RAN WG1 Meeting #81	R1-153124

Fukuoka, Japan, 25th - 29th May 2015

Source:	Ericsson

Title:	Coexistence Evaluation Results for DL+UL LAA with Fast DL Control

Channel LBT

Agenda Item:	6.2.4.2

Document for:	Discussion and Decision

# Introduction

Substantial progress was made for the Study Item on Licensed-Assisted Access (LAA) in RAN1 WG #80bis [1] with additional simulation assumptions for DL+UL LAA.

In [2], we evaluated coexistence of Wi-Fi and DL+UL LAA with a regular Cat 3 UL LBT algorithm. We found that DL+UL LAA with Cat 3 UL LBT can coexist in a fair manner with Wi-Fi by enabling the non-replaced Wi-Fi network to achieve better performance than when operating on the channel with another Wi-Fi network. However, we also identified several issues for further study:

The LTE/LAA UL transmission when being self-scheduled from the eNB, requires two successful LBT procedures to commence transmission: the eNB needs to succeed at sending the UL grant and the scheduled UE needs to succeed in initiating UL transmissions. In the Wi-Fi network, UL transmissions are initiated by the mobile units autonomously. This results in the following behavior.

- The Wi-Fi network with both DL and UL traffic manages to achieve higher served traffic than the LAA network with both DL and UL traffic when the offered loads to both networks are identical.

- As the Wi-Fi network attempts to serve more and more UL traffic, the amount of the offered UL traffic that the LAA network is able to serve drops very fast.

It is concluded that further study is needed to enhance LAA UL transmission success rates while considering the performance of both LAA and coexisting Wi-Fi networks. Examples include faster LBT for UL transmissions and/or control signaling for UL grants.

The IEEE 802.11ax TG has recently approved the following motion on UL OFDMA operations [3]:

An UL MU PPDU (MU-MIMO or OFDMA) is sent as an immediate response (IFS TBD) to a Trigger frame (format TBD) sent by the AP.

That is, this new scheduled Wi-Fi UL mode will allow multiple transmissions to follow the grant with a short delay and without performing any LBT by any of the scheduled Wi-Fi stations. This is in addition to the reverse direction grant in the IEEE 802.11n protocol, which also allows the reverse direction transmission to follow the first transmission without LBT (within the TXOP of the first transmitter).

In light of these findings, we investigate in this contribution the coexistence performance of Wi-Fi and DL+UL LAA when fast DL control channel is applied when no DL data exists at the eNB buffer. The UL grant transmission is based on self-scheduling with a grant delay of 4 ms.

# Description of DL and UL LBT algorithms

## LAA DL Data Channel LBT algorithm

Category 4 LBT algorithm based on LBE LBT with NACK based contention window increase [6]

In addition to the main Category 3 LBT loop, the transmitter also maintains a variable contention window size CW, which is initialized to CWmin = 15. The details of the algorithm are given below.

- Whenever a random backoff counter is needed in the LBT loop,

  - If the latest received HARQ feedback is NACK, CW is doubled.

    - The maximum size of the contention window is limited to CWmax = 1023.

  - If the latest received HARQ is ACK, CW is reset to CWmin.

  - The random number N is drawn from [0, CW].

- The CCA slot duration T1 is reduced to 9 μs to align with Wi-Fi slot duration.

- The transmitter can occupy the channel for 4 ms following a successful LBT attempt.

## LAA Control Channel LBT algorithm for sending UL grant

When eNB has no data in buffer for DL transmission, EPDCCH follows the fast DL LBE LBT algorithm as followed:

Category 3 LBT algorithm based on LBE LBT [5][6]

- Same as in the DL Category 3 LBT algorithm based on LBE LBT except Q = 1

Note that this fast UL LBT algorithm can also be considered as a special case of the Category 4 LBT with CWmin = CWmax = 1. In this case, the CCA slot time reduces to 9 μs.

This fast LBT scheme can also be considered as the initial CCA step defined in [7].

In the other case when eNB has data in buffer, UL grant is transmitted together with data transmissions, which follow DL data channel LBT algorithms as described in Section 2.1 above.

## LAA UL Data Channel LBT algorithm

Category 4 LBT algorithm based on LBE LBT with NACK based contention window increase [6]

- Same as in the DL except CWmin = 7 and CWmax =15

This set of minimum and maximum CW is the same as that for the voice access class in Wi-Fi EDCA.

# Wi-Fi – LAA coexistence evaluation results and discussion

The results presented in Figure 1 and also captured in Table 1 provide an overview on the coexistence of LAA with Wi-Fi when both networks carry both DL and UL traffic. The system performance results show that not only does LAA with DL and UL traffic coexists in a friendly manner with Wi-Fi but also boosts Wi-Fi performance as compared to the case where two Wi-Fi networks coexist with each other.

Some of the major elements which make LAA a good neighbor to Wi-Fi in the unlicensed band where both technologies have to share the medium are discussed in the following. The LAA uplink traffic is scheduled by the LAA eNBs, which reduces the number of contending nodes at any given time. In the Wi-Fi network, all UEs with non-empty UL buffer contend for access to the medium. In contrast, in the LAA network, only the few LAA UEs that are scheduled at a given time attempt to access the medium. LTE, as compared to Wi-Fi, has more robust interference mitigation, error correction and retransmission schemes which result in greater efficiency in serving the traffic and reducing resource utilization thus providing more opportunities for other systems to access the medium. LAA uses the same sensing threshold irrespective of transmissions from LAA or Wi-Fi nodes as opposed to Wi-Fi which uses a higher sensing threshold for LAA as compared to Wi-Fi.

Observation:

- A DL+UL LAA network operating Category 4 LBT for data and fast LBT for DL control channel to send UL grants can coexist with a Wi-Fi network.

Figure 1 shows the benefits of introducing fast LBT algorithm for the eNB to send UL grants when the DL data buffer is empty. It can be observed that:

- Fast LBT for DL control channel to send UL grants has no impact to the DL performance of both the LAA and the Wi-Fi network.

- Fast LBT for DL control channel to send UL grants allows large improvement to the LAA UL performance and no impact to Wi-Fi UL performance. This enables the LAA and Wi-Fi UL performance to become more comparable at high loads.

Observation:

- Fast LBT for DL control channel to send UL grants enables large improvement to the LAA UL performance with no impact to Wi-Fi UL and DL and LAA DL performance. At high loads, the DL and UL performance of LAA and Wi-Fi networks become more comparable with the use of fast LBT for the LAA DL control channel.

- DL user throughputs				(b) UL user throughputs

Figure 1:  Mean user throughputs of the indoor test scenario with FTP traffic in DL on the left and UL on the right. Each network has 4 eNBs/APs and 20 UEs. Operator B has 2 VoIP UEs. Both operator A and B networks have 50% DL and 50% UL traffic. For LAA, licensed band PCell is not used for DL traffic in this test.

Figure 2:  Ratio of served traffic in uplink over the offered traffic in uplink.

Table 1: Coexistence evaluation results when LAA uses fast DL control channel LBT. Indoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and Mixed traffic. Operator A network has only DL traffic and Operator B network has DL and UL traffic with 50/50 split. Each network has 4 eNBs/APs and 20 UEs. The non-replaced Wi-Fi network is operator B. Operator B has 2 VoIP UEs.

| Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi Opt.B in step 1 | Wi-Fi Opt.A in step 1 | Wi-Fi Opt. B in step 2 | LAA Opt.A in step 2 | Wi-Fi Opt.B in step 1 | Wi-Fi Opt.A in step 1 | Wi-Fi Opt. B in step 2 | LAA Opt.A in step 2 | Wi-Fi Opt.B in step 1 | Wi-Fi Opt.A in step 1 | Wi-Fi Opt. B in step 2 | LAA Opt.A in step 2 |
| DL: UPT CDF [Mbps] | 5% | 28.780 | 26.730 | 49.490 | 70.380 | 7.2600 | 8.6100 | 40.610 | 58.910 | 1.6200 | 1.9800 | 29.660 | 45.990 |
|  | 50% | 66.990 | 67.230 | 82.540 | 99.590 | 47.320 | 48.100 | 74.430 | 91.380 | 24.400 | 25.910 | 66.990 | 85.060 |
|  | 95% | 93.090 | 93.790 | 102.38 | 114.11 | 80.040 | 82.680 | 96.500 | 109.99 | 64.690 | 66.710 | 91.680 | 105.79 |
|  | Mean | 65.970 | 66.550 | 82.050 | 98.770 | 46.200 | 47.570 | 74.510 | 90.870 | 29.710 | 30.880 | 66.740 | 83.390 |
| DL: Delay CDF [s] | 5% | 0.0380 | 0.0420 | 0.0340 | 0.0300 | 0.0400 | 0.0590 | 0.0370 | 0.0320 | 0.0470 | 0.0480 | 0.0390 | 0.0330 |
|  | 50% | 0.2340 | 0.2000 | 0.0520 | 0.0410 | 0.3700 | 0.6550 | 0.0670 | 0.0500 | 1.2440 | 1.4730 | 0.0860 | 0.0620 |
|  | 95% | 3.2800 | 2.9240 | 0.1200 | 0.0740 | 11.493 | 13.264 | 0.2120 | 0.1400 | 17.307 | 15.763 | 1.1350 | 0.4950 |
|  | Mean | 0.8840 | 0.8720 | 0.0670 | 0.0470 | 2.5910 | 2.9840 | 0.0990 | 0.0700 | 4.4350 | 4.4920 | 0.3180 | 0.1670 |
| UL: UPT CDF [Mbps] | 5% | 24.010 | 25.450 | 41.460 | 61.220 | 6.4200 | 6.6500 | 34.200 | 43.000 | 1.5800 | 1.5700 | 25.230 | 26.400 |
|  | 50% | 64.330 | 62.840 | 78.190 | 87.200 | 43.330 | 42.840 | 69.250 | 70.190 | 23.230 | 23.700 | 61.650 | 53.920 |
|  | 95% | 90.720 | 88.990 | 100.71 | 101.14 | 77.150 | 77.920 | 93.630 | 86.570 | 60.650 | 62.870 | 89.590 | 69.800 |
|  | Mean | 63.470 | 62.400 | 77.670 | 86.570 | 44.160 | 44.070 | 69.290 | 69.910 | 28.110 | 28.750 | 62.260 | 53.270 |
| UL: Delay CDF [s] | 5% | 0.0380 | 0.0390 | 0.0340 | 0.0350 | 0.0440 | 0.0410 | 0.0370 | 0.0420 | 0.0480 | 0.0550 | 0.0390 | 0.0520 |
|  | 50% | 0.2580 | 0.2690 | 0.0580 | 0.0490 | 0.7100 | 0.6560 | 0.0850 | 0.0710 | 1.8020 | 1.3710 | 0.1080 | 0.1010 |
|  | 95% | 2.7330 | 2.9920 | 0.1680 | 0.1150 | 10.961 | 9.2730 | 0.3430 | 0.2660 | 14.655 | 14.784 | 1.0410 | 1.3730 |
|  | Mean | 0.7570 | 0.8300 | 0.0820 | 0.0620 | 2.7490 | 2.2940 | 0.1380 | 0.1090 | 4.2740 | 3.9020 | 0.3150 | 0.3490 |
| VoIP outage |  | 0.4100 | N/A | 0.2500 | N/A | 0.6300 | N/A | 0.3300 | N/A | 0.8300 | N/A | 0.3800 | N/A |
| VoIP outage(DL) |  | 0.3400 | N/A | 0.2300 | N/A | 0.5700 | N/A | 0.3300 | N/A | 0.7800 | N/A | 0.3600 | N/A |
| VoIP outage(UL) |  | 0.1600 | N/A | 0.0200 | N/A | 0.2800 | N/A | 0.0100 | N/A | 0.5200 | N/A | 0.0600 | N/A |
| 𝜌DL |  | 0.9500 | 0.9500 | 1.0000 | 1.0000 | 0.8100 | 0.8300 | 1.0000 | 1.0000 | 0.6700 | 0.6900 | 0.9900 | 0.9900 |
| 𝜌UL |  | 0.9600 | 0.9500 | 1.0000 | 1.0000 | 0.8500 | 0.8700 | 1.0000 | 1.0000 | 0.7500 | 0.7600 | 0.9900 | 0.9800 |
| BO |  | 0.2000 | 0.1900 | 0.0700 | 0.0500 | 0.4000 | 0.3900 | 0.1100 | 0.0800 | 0.6000 | 0.5900 | 0.1600 | 0.1300 |
| 𝜆 |  | 0.203897 |  |  |  | 0.246513 |  |  |  | 0.276395 |  |  |  |
| Company/tdoc: Ericsson / R1-153124 LBT category: Category 4 on DL and UL with self-scheduling Additional information: Sensing threshold used: -82 dBm Whether defer periods are used or not: yes CCA and ECCA slot length: 20 μs Whether or not intra and/or inter-RAT detection is assumed: only CCA-ED Any significant deviations from evaluations methodology and assumptions: DL CWmin=15, DL CWmax=1023, UL CWmin=7, UL CWmax=15, fast LBT (Cat 3 LBE with Q=1) for DL control channel to send UL grants when the eNB DL data buffer is empty |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Conclusions

In this contribution, we report coexistence evaluation results for LAA with both DL and UL traffic in indoor deployments when the Wi-Fi network also supports both UL and DL traffic. We provide an analysis of the UL LAA performance with self-scheduling. We observe the following.

Observations:

- A DL+UL LAA network operating Category 4 LBT for data and fast LBT for DL control channel to send UL grants can coexist with a Wi-Fi network.

- Fast LBT for DL control channel to send UL grants enables large improvement to the LAA UL performance with no impact to Wi-Fi UL and DL and LAA DL performance. At high loads, the DL and UL performance of LAA and Wi-Fi networks become more comparable with the use of fast LBT for the LAA DL control channel.

Based on the investigation, we propose the following.

Proposal:

- Fast LBT for DL control channel to send UL grants when the eNB DL data buffer is empty should be included as a component for LAA to achieve fair coexistence with a Wi-Fi network and improve the performance of both LAA and Wi-Fi networks.

- Fast LBT scheme should be adopted for LAA discovery signal transmissions.

# References

Chairman’s notes, 3GPP RAN1 WG #80bis

R1-152109 Coexistence Evaluation Results for DL+UL LAA and Wi-Fi, Ericsson

IEEE TGax March 2015 Minutes 11-15/0309r0.

Section 9.25 “Reverse Direction Protocol” IEEE Std 802.11-2012, Part II: Wireless LAN MAC and PHY Specifications

R1-150584 Further details on LBT design in DL for LAA, Ericsson

R1-151996 Discussion on LBT Protocols, Ericsson

R1-152413, WF on Cat4 Channel Access Scheme, Intel, LG, Ericsson, Broadcom, NTT DoCoMo, Mediatek

3GPP TR 36.899 Study on Licensed-Assisted Access to Unlicensed Spectrum, v.0.4.0

# Appendix

Annex A: Coexistence Evaluation Assumptions

The simulation assumptions are based on the agreed coexistence assumptions in [5]. However our simulation settings on the assumptions that remained optional or need clarifications when results are presented are provided below. In all the indoor coexistence evaluations, the transmit power of the base station in the unlicensed band is assumed to be 18 dBm. Moreover, FTP model 3 is used for generating FTP traffic.

Table 2: Additional Wi-Fi system evaluation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| MCS |  | 802.11ac MCS table with 256 QAM |
| Antenna configuration |  | 2Tx2Rx, Cross-polarized Baseline: open loop 2x2 MIMO |
| Channel coding |  | LDPC |
| Frame aggregation |  | A-MPDU |
| MPDU size |  | 1500B MSDU + 14 B header |
| Max PPDU duration |  | Baseline:< 4 ms (Asynchronous to LTE timing) |
| MAC | Coordination | EDCA |
|  | SIFS, DIFS | SIFS, DIFS |
|  | Detection | Energy detection & preamble detection |
|  | RTS/CTS | No |
|  | Contention window | Per EDCA |
| CCA-CS |  | -82dBm and preamble decoding (Note preamble occupies the 20MHz system bandwidth with rate 1/2 coding and BPSK modulation) |
| CCA-ED |  | -62dBm |
| ACK Modeled (successful reception, resources utilized) |  | Yes |
| DL/UL Duplexing |  | For the DL-only LAA coexistence evaluations: - DL traffic only for the replaced Wi-Fi network - DL and UL for the non-replaced Wi-Fi network |
| Rate control |  | Minstrel algorithm |
| Channel selection |  | Based on the minimum interference level while ensuring that each unlicensed carrier is shared by two operators in each cluster |
| OFDM symbol length |  | 4 micro second |

Table 3: Additional LAA system evaluations assumptions

| Parameters | Value |
| --- | --- |
| PCI planning for each NW | Planned |
| Antenna configuration | 2Tx2Rx, Cross-polarized. |
| Transmission schemes | Open loop 2x2 MIMO based on TM10, QPSK/16QAM/64QAM/256QAM |
| Turbo code block interleaving depth | Per LTE specs (1-14 LTE OFDM symbols dependent on MCS and PRB allocation) |
| Scheduling | Proportional fair |
| Link adaptation | Realistic |
| CCA-ED | -82 dBm |
| Cyclic Prefix | Normal |