3GPP TSG RAN WG1 Meeting #82                                                                            	                      R1-153963

Beijing, China, 24th – 28th August 2015

Source:	Panasonic

Title: 	Performance evaluation on M-PDCCH

Agenda Item:		7.2.1.2

Document for:	Discussion and Decision

Introduction

In RAN1#81 meeting [1], the following agreements were made on physical downlink control channel for MTC (M-PDCCH):

Agreements:

- Within subframes for M-PDCCH transmission for UEs operating coverage enhancements,

- Confirm working assumption with the following revised proposal

  - The UE may assume the same precoder per antenna port applies at least on the same PRB for at least X subframes.

  - FFS: The UE may assume the same precoder per antenna port applies on a PRG for at least X subframes.

      - FFS: Details on PRG size

      - Companies are encouraged to evaluate potential performance benefits due to PRG, especially comparing with precoding diversity. Impact due to frequency offsets can also be considered.

In this contribution, we evaluate following points related to M-PDCCH.

- PRB bundling and PRG size

- System band frequency hopping and M-PDCCH coding over the subframes

- M-PDCCH assignment within a narrowband

M-PDCCH evaluations

PRB bundling and PRG size

Cross-PRB channel estimation (= PRB bundling) is evaluated in the condition of EPA, fD = 1 Hz, and RFO = 100 Hz. The further detailed parameters are listed in the Appendix A and detailed simulation results with BLER performance are shown in Appendix B. When the number of the repetition is 8 and more, cross-subframe channel estimation with 4 subframes and frequency hopping with hopping interval of 4 subframes are used. When the number of the repetition is 4, cross-subframe channel estimation with 4 subframes and no frequency hopping is used. When the number of the repetition is 1, neither cross-subframe channel estimation nor frequency hopping is used. The frequency difference of frequency hopping is 5 MHz.

Table 1 summarizes the required SINR for achieving BLER = 102.

Table 1	Required SINR for achieving BLER = 102

| Number of repetitions |  | 1 | 4 | 8 | 16 | 32 | 64 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No PRB bundling | No precoder cycling among PRBs | 2.9 dB | -1.3 dB | -7.9 dB | -9.1 dB | -11.6 dB | -13.8 dB |
|  | Precoder cycling among PRBs | 2.1 dB | -2.1 dB | -8.6 dB | -9.8 dB | -12.4 dB | -14.6 dB |
| 2 PRBs are bundled | No precoder cycling among bundled PRBs | 2.1 dB | -2.2 dB | -8.9 dB | -10.2 dB | -12.8 dB | -15.1 dB |
|  | Precoder cycling among bundled PRBs | 1.5 dB | -2.9 dB | -9.5 dB | -10.9 dB | -13.5 dB | -15.5 dB |
| 6 PRBs are bundled | - | 1.3 dB | -3.3 dB | -10.3 dB | -11.7 dB | -14.5 dB | -16.9 dB |

When PRB bundling is not used and 2 PRBs are bundled, precoder cycling among (bundled) PRBs can provide slightly better performance than no precoder cycling among PRBs. The coverage enhancement gain of precoding diversity is 0.4~0.8 dB.

The coverage enhancement gain of cross-PRB channel estimation is larger than that of precoder cycling. The coverage enhancement gain of cross-PRB channel estimation is 0.8~1.3 dB when 2 PRBs are bundled and 1.6~3.1 dB when 6 PRBs are bundled.

Based on the above evaluation, our observation is following.

Observation 1: For PRB bundling, PRG size should be 6 PRBs in order to achieve the best coverage enhancement gain.

System band frequency hopping and M-PDCCH coding over the subframes

The performance difference with/without frequency hopping and the coding gain in no/small repetition case is evaluated in the condition of ETU, fD = 1 Hz, and RFO = 100 Hz. The further detailed parameters are also listed in the Appendix A.

At first, we evaluate the performance difference when the total ECCE usage is 2. The BLER performance of M-PDCCH performance among following M-PDDCH mapping options described in Fig. 1 is evaluated.

- Option 1: Aggregation level (L) = 2, 2 ECCEs are mapped in a subframe

- Option 2: L = 1, same ECCE encoding is repeated over 2 subframes, frequency hopping is applied between subframes

- Option 3: L = 2, 2 ECCEs are mapped over 2 subframes, frequency hopping is applied between subframes

Table 2 summarizes the required SINR for achieving BLER = 102. Detailed simulation results with BLER performance are shown in Fig. 6 of Appendix C.

It can be seen that Option 3 has significant gain compared to Option 1 and Option 2 because of much coding gain and frequency diversity gain.

Fig.1 M-PDCCH mapping options for 2 ECCE usage.

Table 2		Required SINR for achieving BLER = 102 for 2 ECCE usage

| MTC control channel resource (PRB) in a subframe (Intra-narrowband diversity) | Option1 | Option2 | Option3 |
| --- | --- | --- | --- |
| 2 PRBs | 12.7 dB | 13.0 dB | 9.1 dB |
| 4 PRBs | 9.7 dB | 11.0 dB | 7.9 dB |
| 6 PRBs | 9.4 dB | - | 7.5 dB |

Next, we evaluate the performance difference when the total ECCE usage is 24. The BLER performance of M-PDCCH performance among following M-PDDCH mapping options described in Fig. 2 is evaluated.

- Option 1: L = 24, 24 ECCEs are mapped in a subframe

- Option 2: L = 12, same ECCE encoding is repeated over 2 subframes, frequency hopping is applied between subframes

- Option 3: L = 24, 24 ECCEs are mapped over 2 subframes, frequency hopping is applied between subframes

Table 3 summarizes the required SINR for achieving BLER = 102.  Detailed simulation results with BLER performance are shown in Fig. 7 of Appendix C.

Fig.2 M-PDCCH mapping options for 24 ECCE usage.

Table 3		Required SINR for achieving BLER = 102 for 24 ECCE usage

| Option1 | Option2 | Option3 |
| --- | --- | --- |
| -1.4 dB | -3.6 dB | -3.6 dB |

It can be seen that Option 2 and 3 provides significantly better BLER performance than Option 1 because of the frequency diversity gain. For higher aggregation level, the effect of frequency diversity gain would be larger than that of coding gain and then comparing Option 2 and 3, almost the same performance is achieved.

Based on above evaluation results, we provide the following observations.

Observation 2: To realize total aggregation level higher than 2 (L × R ≥ 2) by multiple subframes with frequency hopping can improve the BLER performance.

Observation 3: For lower aggregation level, encoding across multiple subframes is beneficial to obtain coding gain and frequency diversity simultaneously.

Observation 4: For higher aggregation level, encoding across multiple subframes is not required as coding rate is sufficiently low in a subframe.

To transmit M-PDCCH by multiple subframes with frequency hopping increases the latency. If cell throughput is important, it would be better to use multiple subframes with frequency hopping as much as possible. While, if latency or user throughput is important, it would be better to minimize the repetition as much as possible. In this case, up to L = 24, no frequency hopping operation would be possible.

M-PDCCH assignment within a narrowband

Contiguous and non-contiguous PRB locations in narrowband for L = 8 with 2 PRBs usage case is evaluated in the condition of ETU, fD = 1 Hz, and RFO = 100 Hz. Table 4 summarizes the required SINR for achieving BLER = 102. Detailed simulation results with BLER performance are shown in Fig. 8 of Appendix D.

Non-contiguous PRB location can achieve better BLER performance than contiguous PRB location. On the other hand, there seems additional diversity gain more than 3 PRBs is not so much.

Observation 5: Non-contiguous PRB location can achieve better BLER performance than contiguous PRB location while the additional diversity gain more than 3 PRBs is not so much.

Table 4 	Required SINR for achieving BLER = 102

| Contiguous PRB location | Non-contiguous PRB location (2PRB difference) | Non-contiguous PRB location (3PRB difference) |
| --- | --- | --- |
| 5.0 dB | 2.9 dB | 2.6 dB |

Conclusion

In this contribution, we provided link performance of M-PDCCH. Based on the simulation results, we provide the following observations:

Observation 1: For PRB bundling, PRG size should be 6 PRBs in order to achieve the best coverage enhancement gain.

Observation 2: To realize total aggregation level higher than 2 (L × R ≥ 2) by multiple subframes with frequency hopping can improve the BLER performance.

Observation 3: For lower aggregation level, encoding across multiple subframes is beneficial to obtain coding gain and frequency diversity simultaneously.

Observation 4: For higher aggregation level, encoding across multiple subframes is not required as coding rate is sufficiently low in a subframe.

Observation 5: Non-contiguous PRB location can achieve better BLER performance than contiguous PRB location while the additional diversity gain more than 3 PRBs is not so much.

- Reference

- 3GPP RAN1#81, Chairman’s note

- Appendix A: Simulation parameters

| Parameters | For evaluations in 2.1 | For evaluations in 2.2 and 2.3 |
| --- | --- | --- |
| Number of antennas | 2 × 1 with low correlation | 2 × 1 with low correlation |
| Number of repetitions | NRep = 1, 4, 8, 16, 32, 64 | NRep = 1, 2 |
| Channel model | EPA(Doppler frequency, fD = 1 Hz) | ETU (Doppler frequency, fD = 1 Hz) |
| DCI format | 0 (payload size = 21 bits) | 0 (payload size = 21 bits) |
| Aggregation level | 24 | 1, 2, 8, 12, 24 |
| M-PDCCH type | Distributed | Distributed |
| Carrier frequency | 2 GHz | 2 GHz |
| System bandwidth | 10 MHz (The frequency difference of frequency hopping is 5 MHz.) | 10 MHz (The frequency difference of frequency hopping is 5 MHz.) |
| Timing error | Ideal | Ideal |
| Residual frequency offset (RFO) | 100 Hz | 100 Hz |
| Channel estimation | Realistic cross-subframe channel estimation and/or realistic cross-PRB channel estimation | Realistic single-subframe channel estimation |
| Legacy control channel region | 3 OFDM symbols | 3 OFDM symbols |
| Transmit diversity | RBF | RBF |

- Appendix B: Simulation results on 2.1

Figure 3 plots the BLER performance of M-PDCCH with number NRep of repetitions as a parameter. Cross-PRB channel estimation is not used. No precoder cycling among PRBs is assumed in Fig.3(a) and precoder cycling among PRBs is applied in Fig.3 (b).

Figure 4 plots the BLER performance of M-PDCCH when cross-PRB channel estimation with 2PRBs is used. No precoder cycling among bundled PRBs is assumed in Fig.4(a) and precoder cycling among bundled PRBs is applied in Fig.4 (b).

Figure 5 plots the BLER performance of M-PDCCH when cross-PRB channel estimation with 6 PRBs is used.

(a) No precoder cycling among PRBs                                 (b) Precoder cycling among PRBs

Fig.3	BLER performance of MPDCCH (no PRB bundling).

(a) No precoder cycling among bundled PRBs                     (b) Precoder cycling among bundled PRBs

Fig.4	BLER performance of MPDCCH (PRB bundling with 2 PRBs).

Fig.5	BLER performance of MPDCCH (PRB bundling with 6 PRBs).

- Appendix C: Simulation results on 2.2

(a) No. of PRB = 2                                                           (b) No. of PRB = 4

(c) No. of PRB = 6

Fig.6	Performance comparison on M-PDCCH mapping (Lower aggregation level).

Fig.7	Performance comparison on M-PDCCH mapping (Higher aggregation level).

- Appendix D: Simulation results on 2.3

Fig.8	Performance comparison between contiguous and non-contiguous PRB locations.