3GPP TSG RAN WG1 Meeting #82bis                                                                  	                      R1-155337

Malmö, Sweden, 5th – 9th October 2015

Source:	Panasonic

Title: 	Evaluation on M-PDCCH

Agenda Item:		7.2.1.2

Document for:	Discussion and Decision

Introduction

In this contribution, we evaluate following points related to M-PDCCH.

- Cross-PRB channel estimation, including the effects of precoder cycling for both distributed and localized

- Performance of localized mapping vs distributed mapping.

M-PDCCH evaluations

Cross-PRB channel estimation, including the effects of precoder cycling

Distributed mapping

Cross-PRB channel estimation (= PRB bundling) for distributed mapping is evaluated in the condition of EPA, fD = 1 Hz, and RFO = 100 Hz. The further detailed parameters are listed in the Appendix A and detailed simulation results with BLER performance are shown in Appendix B. When the number of the repetition is 8 and more, cross-subframe channel estimation with 4 subframes and frequency hopping with hopping interval of 4 subframes are used. When the number of the repetition is 4, cross-subframe channel estimation with 4 subframes and no frequency hopping is used. When the number of the repetition is 1, neither cross-subframe channel estimation nor frequency hopping is used. The frequency difference of frequency hopping is 5 MHz. When precoder cycling is used, precoding vector is randomly selected at each PRB or each bundled PRBs.

Table 1 summarizes the required SINR for achieving BLER = 102.

Table 1	Required SINR for achieving BLER = 102

| Number of repetitions |  | 1 | 4 | 8 | 16 | 32 | 64 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No PRB bundling | No precoder cycling among PRBs | 2.9 dB | -1.3 dB | -6.9 dB | -9.1 dB | -11.6 dB | -13.8 dB |
|  | Precoder cycling among PRBs | 2.1 dB | -2.1 dB | -7.6 dB | -9.8 dB | -12.4 dB | -14.6 dB |
| 2 PRBs are bundled | No precoder cycling among bundled PRBs | 2.1 dB | -2.2 dB | -7.9 dB | -10.2 dB | -12.8 dB | -15.1 dB |
|  | Precoder cycling among bundled PRBs | 1.5 dB | -2.9 dB | -8.5 dB | -10.9 dB | -13.5 dB | -15.5 dB |
| 3 PRBs are bundled | No precoder cycling among bundled PRBs | 1.7 dB | -2.7 dB | -8.5 dB | -10.8 dB | -13.6 dB | -15.8 dB |
|  | Precoder cycling among bundled PRBs | 1.2 dB | -3.3 dB | -9.0 dB | -11.3 dB | -14.0 dB | -16.3 dB |
| 6 PRBs are bundled | - | 1.3 dB | -3.3 dB | -9.5 dB | -11.7 dB | -14.5 dB | -16.9 dB |

When PRB bundling is not used and 2 PRBs are bundled, precoder cycling among (bundled) PRBs can provide slightly better performance than no precoder cycling among PRBs. The coverage enhancement gain of precoding diversity is 0.4~0.8 dB.

Cross-PRB channel estimation can provide significantly better performance regardless of with or without precoder cycling. The coverage enhancement gain of cross-PRB channel estimation is 0.6~1.3 dB, 0.9~2.0 dB, and 0.8~1.3 dB when 2 PRBs, 3PRBs, and 6PRBs are bundled, respectively.

Based on the above evaluation, our observation is following.

Observation 1: For distributed mapping, PRG size of 6PRBs achieves the best coverage gain.

Localized mapping

Cross-PRB channel estimation (= PRB bundling) for localized mapping is evaluated in the condition of EPA, fD = 1 Hz, and RFO = 100 Hz. The further detailed parameters are listed in the Appendix A and detailed simulation results with BLER performance are shown in Appendix B. The same condition as the distributed mapping case above is assumed with regard to frequency hopping and cross-subframe channel estimation. Precoder cycling is always used and precoding vector is randomly selected at each PRB or each bundled PRBs.

Table 2 summarizes the required SINR for achieving BLER = 102.

Table 2	Required SINR for achieving BLER = 102

| Number of repetitions | 1 | 4 | 8 | 16 | 32 | 64 |
| --- | --- | --- | --- | --- | --- | --- |
| No PRB bundling | 1.6 dB | -2.8 dB | -8.4 dB | -10.7 dB | -13.4 dB | -15.6 dB |
| 2 PRBs are bundled | 1.9 dB | -2.7 dB | -8.8 dB | -11.3 dB | -13.8 dB | -16.2 dB |
| 3 PRBs are bundled | 3.4 dB | -1.3 dB | -8.4 dB | -10.9 dB | -13.5 dB | -15.8 dB |
| 6 PRBs are bundled | 8.3 dB | 3.0 dB | -7.0 dB | -9.5 dB | -12.2 dB | -14.7 dB |

When the number of repetitions is small (=1 and 4), the cross-PRB channel estimation degrades the BLER performance. When the number of repetitions is larger than 4 (=8, 16, 32, and 64), 2 PRB bundling provides the best performance. The reason that different trend is seen from distributed mapping would be the difference of precoding gain. In this evaluation, precoding vector is randomly selected at each PRB or each bundled PRBs, and then, there are possibility to use the same precoding vector among PRBs or bundled PRBs as PRB bundling size increases (When 6PRBs are bundled, the same precoding vector is always used among PRBs). Therefore, the probability that precoding gain could not be obtained would be higher than that of distributed mapping. For localized mapping, 2 PRB bundling would be optimal bundling size (when precoding vector is randomly selected). On the other hand, for distributed mapping, at least two antenna diversity gain could be always obtained, the performance is improved as the bundling size increases.

In order to improve the cross-PRB channel estimation gain for localized mapping, selection of precoding vector should be considered. For example, different precoding vector is always selected among bundled PRBs. Table 3 summarizes the required SINR for achieving BLER=102 when 3 PRBs are bundled. The precoding vector is randomly selected or different precoding vector is always used among bundled PRBs.

It can be seen from Table 3 that when different precoding vector is always used among bundled PRBs, the BLER performance when 3 PRBs are bundled can be improved and PRG size of 3PRBs achieves the best coverage gain.

Table 3 Required SINR for achieving BLER = 102

| Number of repetitions |  | 1 | 4 | 8 | 16 | 32 | 64 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3 PRBs are bundled | Precoding vector is randomly selected | 3.4 dB | -1.3 dB | -8.4 dB | -10.9 dB | -13.5 dB | -15.8 dB |
|  | Different precoding vector is used among bundled PRBs. | 1.2 dB | -3.3 dB | -9.4 dB | -11.9 dB | -14.6 dB | -16.9 dB |

Based on the above evaluation, our observation is following.

Observation 2: For localized mapping, PRG size of 2 or 3 PRBs achieves the best coverage gain.

Performance of localized mapping vs distributed mapping

The performance of localized mapping vs distributed mapping is summarized in Table 4. For no PRB bundling case, localized mapping provides about 1dB gain than distributed mapping. This would be because the transmit power of DMRS for localized mapping can be greater than that of other REs. When the use of cross-PRB channel estimation and precoder cycling are assumed, localized mapping with 3PRB bundling and distributed mapping with 6PRB bundling provides almost the same performance in the case of AL=24 (especially large repetition case).

Table 4	Required SINR for achieving BLER = 102

| Number of repetitions |  | 1 | 4 | 8 | 16 | 32 | 64 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No PRB bundling | Distributed | 2.1 dB | -2.1 dB | -7.6 dB | -9.8 dB | -12.4 dB | -14.6 dB |
|  | Localized | 1.6 dB | -2.8 dB | -8.4 dB | -10.7 dB | -13.4 dB | -15.6 dB |
| 3 PRBs are bundled | Distributed | 1.2 dB | -3.3 dB | -9.0 dB | -11.3 dB | -14.0 dB | -16.3 dB |
|  | Localized | 1.2 dB | -3.3 dB | -9.4 dB | -11.9 dB | -14.6 dB | -16.9 dB |
| 6 PRBs are bundled | Distributed | 1.3 dB | -3.3 dB | -9.5 dB | -11.7 dB | -14.5 dB | -16.9 dB |
|  | Localized | 8.3 dB | 3.0 dB | -7.0 dB | -9.5 dB | -12.2 dB | -14.7 dB |

Based on the above evaluation, our observation is following.

Observation 3: Assuming the use of cross-PRB channel estimation with appropriate PRG size and precoder cycling, localized mapping and distributed mapping provide almost the same coverage gain.

In order to avoid multiple channel estimation trials for different aggregation levels for low complexity [1], PRG size is limited to the PRBs corresponding to the smallest aggregation levels. Our search space proposal for no/small repetition case is the minimum aggregation level is one [2]. Therefore, it is difficult to have PRG bundling for no/small repetition case. On the other hand, our search space proposal for large repetition case is only to have 24 eCCE aggregations. Therefore, PRG bundling can be applied based on the performance without additional complexity.

From eCCE usage perspective, for 24 eCCEs aggregation case, as all eCCEs in 6 PRBs are used, localized or distributed does not make the difference. In this case either one would be sufficient. Based on the observation 3, either localized mapping with 2(or 3) PRBs bundling or distributed mapping with 6PRBs bundling only may be sufficient.

Proposal 1: For large repetition case, if only AL=24 is supported, either localized mapping with 2(or 3) PRBs bundling or distributed mapping with 6PRBs bundling is supported.

Proposal 2: For no/small repetition case, if the minimum aggregation level is one, PRB bundling is not supported.

Conclusion

In this contribution, we provided link performance of M-PDCCH. Based on the simulation results, we provide the following observations:

Observation 1: For distributed mapping, PRG size of 6PRBs achieves the best coverage gain.

Observation 2: For localized mapping, PRG size of 2 or 3 PRBs achieves the best coverage gain.

Observation 3: Assuming the use of cross-PRB channel estimation with appropriate PRG size and precoder cycling, localized mapping and distributed mapping provide almost the same coverage gain.

Proposal 1: For large repetition case, if only AL=24 is supported, either localized mapping with 2(or 3) PRBs bundling or distributed mapping with 6PRBs bundling is supported.

Proposal 2: For no/small repetition case, if the minimum aggregation level is one, PRB bundling is not supported.

- Reference

[1]	R1-153846, “Physical Downlink Control Channels,” Qualcomm Incorporated, RAN1#82

[2]	R1-155334, “UE-specific search space for MTC,” Panasonic, RAN1#82bis

- Appendix A: Simulation parameters

| Parameters | Values |
| --- | --- |
| Number of antennas | 2 × 1 with low correlation |
| Number of repetitions | NRep = 1, 4, 8, 16, 32, 64 |
| Channel model | EPA(Doppler frequency, fD = 1 Hz) |
| DCI format | 0 (payload size = 21 bits) |
| Aggregation level | 24 |
| M-PDCCH type | Distributed, Localized |
| Carrier frequency | 2 GHz |
| System bandwidth | 10 MHz (The frequency difference of frequency hopping is 5 MHz.) |
| Timing error | Ideal |
| Residual frequency offset (RFO) | 100 Hz |
| Channel estimation | Realistic cross-subframe channel estimation and/or realistic cross-PRB channel estimation |
| Legacy control channel region | 3 OFDM symbols |
| Transmit diversity | RBF |

- Appendix B: BLER performance

Figure 1 plots the BLER performance of distributed M-PDCCH with number NRep of repetitions as a parameter.

Figure 2 plots the BLER performance of localized M-PDCCH with number NRep of repetitions as a parameter.

(a) No PRB bundling                                                          (b) 2 PRBs are bundled

(c) 3 PRBs are bundled                                                          (d) 6 PRBs are bundled

Fig.1	BLER performance of distributed MPDCCH.

(a) No PRB bundling                                                          (b) 2 PRBs are bundled

(c) 3 PRBs are bundled                                                          (d) 6 PRBs are bundled

Fig.2	BLER performance of localized MPDCCH.