3GPP TSG RAN WG1 Meeting #82bis	R1-155358

Malmö, Sweden, 5th - 9th October 2015

Source:                 Panasonic

Title: 	Discussion on resource allocation mechanism in V2X

Agenda Item:		7.2.8.2.1

Document for:	Discussion and Decision

Introduction

This document would discuss resource allocation mechanism which is one of objectives described in [1].

Discussion

Compared with D2D, there are more UEs in V2X scenario so it is important to reduce signalling overhead whatever from lower layer to higher layer. Crowded transmission of SA and data would increase the collision probability and at the same time transmitting UE may miss many messages due to half duplex issue.  It may cause critical outcome for vehicles.

Semi-static scheduling (SPS) is one of mechanisms adopted in early LTE releases and basically a good approach to reduce SA overhead especially considering one of typical traffics in V2X is periodic transmission.  Some aspects have been discussed in [2].

But based on our understanding purely reusing legacy SPS like mechanism may not work well for V2X especially considering frequent change of vehicle group topology. So some enhancements are necessary.

Proposal 1: SPS like mechanism should be further evaluated for periodic traffic of V2X

Another issue is about message transmission cross multiple SA periods. This case may often happen due to crowed resources within one SA period and relatively large size of V2X messages.  Based on legacy D2D framework SA needs to be transmitted for each SA period and it will increase collision probability especially in SA subframes. In that case, basically we think one SA indicating data of multiple SA periods would be beneficial to reduce SA overhead. In order to reflect frequent change of vehicle group topology, multiple SA periods should be sufficiently short. More detailed solution could be studied further. So we propose

Proposal 2: SA scheduling multiple SA periods should be further evaluated.

Conclusion

In this document we discussed resource allocation mechanisms for V2X. Basically we propose

Proposal 1: SPS like mechanism should be further evaluated for periodic traffic of V2X

Proposal 2: SA scheduling multiple SA periods should be further evaluated.

- Reference

[1] RP-151109, “New SI proposal: Feasibility Study on LTE-based V2X Services LG Electronics”, CATT, Vodafone, Huawei

[2] R1-154361, “Impact of SA transmissions”, Huawei, HiSilicon