# 3GPP TSG-RAN1 #82bits	R1-156017

# 5th – 9th Oct 2015

# Malmö, Sweden

Source:                    	Ericsson

Title:  	NB-LTE – Impacts of Downlink TX filter

Document for:        	Discussion and decision

Agenda Item:         	7.2.6.3.1

# Introduction

At GERAN#62, a new feasibility study named Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (WI code: FS_IoT_LC)  was approved, see  [1]. At GERAN#67, the FS_IoT_LC study item was completed and the TR 45.820 v2.1.0 [4] approved. Among the candidates included in the TR is the NB-LTE candidate solution.

At RAN#69, a narrow band IOT (NB-IOT) work item is approved, where the downlink design is included as one of the candidates [5].  In this contribution, we discuss the downlink TX filter design of the NB LTE system in the stand alone case.

# General design principles

In order to fulfil the GSM PSD mask in the stand alone deployment, a downlink TX filter is needed. In [4], a 21-tap FIR filter is used, which makes the transmit signal fulfil the GSM PSD mask and introduces very limited impact of the EVM of the transmit signal.

In this contribution, we continue the discussions and evaluations. As an alternative design to the downlink TX filter in [4], we show that by using a 19-tap FIR filter, we can make the transmit signal fulfill the GSM PSD mask, and we also simulate the BLER performance at different MCL’s. The simulation results show that by applying the downlink TX filter, little BLER impacts can be observed.

The 19-tap FIR filter is designed at a sampling rate of 1.92 MHz. Therefore, the impulse response of the filter is around 10 µs. This is longer than the normal CP of NB LTE, however more than 97% of the energy in this filter is contained in the central 9 taps, which corresponds to the length of the normal CP used in NB LTE at the sampling rate of 1.92 MHz. The filtered downlink NB LTE signal and the GSM PSD mask are plotted in Figure 1. As we can see from the plot, the filtered signal is well under the GSM PSD mask.

# Downlink Physical Layer

In order to understand the impact of the TX filter on the received BLER at different MCL values, we resort to link level simulations. The M-PDSCH link simulation parameters are given in Table 1. The transmitted signal is passed through the TX filter in order to fulfil the GSM PSD mask.

Table 1 Assumptions for M-PDSCH link simulations

| Parameter | Value |
| --- | --- |
| Frequency band | 900 MHz |
| Sampling rate | 1.92 MHz |
| Modulate Scheme | QPSK |
| Propagation channel model | TU |
| Doppler spread | 1 Hz |
| Antenna configuration | BS:1T, MS:1R |
| Frequency error | Uniformly drawn from the set {-50 Hz, 50 Hz} |
| Timing uncertainty | Uniformly drawn from the cyclic prefix range (+ and -). |
| Number of channel realizations | 10,000 |

Figure 1 Filtered NB LTE signal

Figure 2 compares the BLER performance of a 100-byte packet when using the MCS designed for different coverage classes (see [2]). In [2], an RRC TX filter with bandwidth of 240 KHz is assumed. In addition, the BLER performance of MCS with higher code rate, i.e., code rate of 0.7, is also provided. As we can see from the plot, after applying the 19-tap FIR TX filter, the BLER performance has no significant differences at the 10% BLER target. Therefore, even though the downlink TX filter is longer the length of normal CP used by NB LTE, little impact on the BLER can be expected. The impact of the TX filter on cell search is presented in [5], and compared to the results given in [6], no significant degradation is observed. Note, in order to fulfil the performance target defined in [1], NB LTE does not require to use higher order modulations than QPSK.

Figure 2 BLER comparison of filtered and unfiltered NB LTE signal

# Conclusions

In this contribution, we show an alternative downlink TX filter design example to the one used in [4]. With a 19-tap downlink TX filter, the NB LTE signal fulfils the GSM PSD spectrum mask. No significant degradation on the coverage is observed. The downlink TX filter used in here is just an example, and further optimization of the downlink TX filter can be expected.

# References

GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

GP-150782, “Narrowband LTE – PDSCH and PUSCH design and performance”, source Ericsson LM, Nokia Networks, GERAN#67, August 10th to 13th, 2015; Yin Chuan, China.

3GPP TR 45.820, “Technical Specification Group GSM/EDGE Radio Access Network; Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things,” Aug. 2015, v.2.1.0.

RP-151560, “NB-LTE - Co-existence with GSM/UMTS/LTE”, source Ericsson, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Nokia, Intel, ZTE, Samsung, LGE, RAN#69, Sep 14th to 16th, 2015, Phoenix, USA.

R1-156009, “Narrowband LTE – Synchronization Channel Design and Performance”, source Ericsson, RAN1#82bits, Oct 5th to 10th, Malmo, Sweden.

RP-151392, “NB-LTE - Improved indoor coverage”, source Ericsson, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Intel, LG Electronics, Nokia Networks, Samsung Electronics, ZTE, RAN#69, Sep 14th to 16th, 2015, Phoenix, USA.

RP-151621, “New Work Item: NarrowBand IOT (NB-IOT)”, source Qualcomm Incorporated, RAN#69, Sep 14th to 16th, 2015, Phoenix, USA.