3GPP TSG RAN1 Meeting #82bis							R1-156018

Malmö, Sweden, 5th – 9th October, 2015

Source: 	Ericsson

Title:	Coverage analysis in inband operation

Agenda Item:	7.2.6.3.3

Document for:	Discussion

# Introduction

At RAN#69 a new work item on narrowband IoT was agreed. In this contribution we do a coverage analysis for the proposed channels for NB-LTE as described in [1] following the methodology given in in TR45.820 [2]. This analysis is for the inband case.

# Link Budget Simulation Assumptions

The simulation assumptions used in the evaluations are listed in Table 1.

Table 1: Assumptions for Link Level Simulations

| Parameter | Value |
| --- | --- |
| Legacy system bandwidth | 10 MHz |
| Frequency band | 900 MHz |
| Propagation channel model | TU |
| Doppler spread | 1 Hz |
| Interference/noise | Sensitivity |
| Antenna configuration (Transmission mode) | Case 1) BS: 2T, MS: 1R (TM2, SFBC) Case 2) BS: 1T, MS: 1R (TM1) |
| Frequency error | F_offset(t) = F_est_error + (F_drift_inactive *T_inactive) + (F_drift_active * t). |
| NB LTE specific frequency error (F_est_error) | Randomly chosen from [-50, 50] Hz |
| Frequency drift rate (F_drift_active) | 22.5 Hz/second |
| Thermal noise density (dBm/Hz) | -174 |
| BS Receiver noise figure (dB) | 3 |
| MS Receiver noise figure (dB) | 5 |
| Interference margin (dB) | 0 |
| Receiver processing gain (dB) | 0 |
| # of PRB | 1 |
| Power boosting (Note1) | 3dB power boosting for data/control + 6dB CRS power boosting |

Note 1: In case that no power boosting or different power boosting level is considered, this will be indicated in the evaluation results.

# Link Budget Performance

In this section, the evaluation results of coverage are presented for the inband deployment option of NB-LTE. The results show that 164dB MCL target can be achieved. For reference, the link budget and data rate for coupling loss of 155.7dB (eMTC MCL target) is shown below.

## M-PBCH

In this section, simulation results are presented to evaluate NB-LTE PBCH coverage performance.

Table 2: NB-LTE M-PBCH Coverage Evaluation

| Deployment | In-band with 10 MHz LTE |
| --- | --- |
| Acquisition time (ms) | 1920 |
| Transmitter |  |
| Max Tx power (dBm) | 46 |
| (1) Actual Tx power (dBm) | 35 |
| Receiver |  |
| (2) Thermal noise density (dBm/Hz) | -174 |
| (3) Receiver noise figure (dB) | 5 |
| (4) Interference margin (dB) | 0 |
| (5) Occupied channel bandwidth (Hz) | 180,000 |
| (6) Effective noise power | -116.4 |
| (7) Required SINR (dB) | -12.6 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -129.0 |
| (9) Rx processing gain | 0 |
| (10) MCL = (1) -(8) + (9) (dB) | 164.0 |

Here, the payload consists of 34 bits and 16-bit CRC. Antenna configuration case 1 with SFBC is used. Convolution coding is used. The M-PBCH is transmitted every 10ms in subframe #0 (using 5 OFDM symbols). The M-PBCH content changes every 640ms and 3 decoding attempts are used to achieve 10% BLER. More detailed description can be found in [1].

## M-PDSCH

The sourcing companies have evaluated the M-PDSCH coverage performance, which are summarized in Table 3.

Table 3: NB-LTE M-PDSCH Coverage Evaluation

| Deployment | In-band with 10 MHz LTE |
| --- | --- |
| Date Rate (kbps) | 1.03 |
| Repetitions (#) | 65 |
| Transmitter |  |
| Max Tx power (dBm) | 46 |
| (1) Actual Tx power (dBm) | 35 |
| Receiver |  |
| (2) Thermal noise density (dBm/Hz) | -174 |
| (3) Receiver noise figure (dB) | 5 |
| (4) Interference margin (dB) | 0 |
| (5) Occupied channel bandwidth (Hz) | 180,000 |
| (6) Effective noise power | -116.4 |
| (7) Required SINR (dB) | -12.6 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -129 |
| (9) Rx processing gain | 0 |
| (10) MCL = (1) -(8) + (9) (dB) | 164.0 |

Here, the payload consists of 776 bits and a 24 bit CRC is appended to it, resulting in a total of 800 bits. Antenna configuration case 1 with SFBC is used. 6 dB power boosting is assumed, and 16 dedicated CRS within a subframe (8 for each transmit antenna) are used. In addition, the first 3 OFDM symbols are unused to avoid collision with legacy PDCCH and other resource elements corresponding to legacy CRS are punctured, leaving a total of 100 resource elements within a subframe. Convolutional coding with QPSK modulation gives a total of 2400 bits and 1200 symbols, which are then mapped to 12 subframes (12 ms).

## M-EPDCCH

Coverage evaluation based on link simulation results from multiple sources is summarized in Table 4.

Table 4: NB-LTE M-EPDCCH Coverage Evaluation

| Deployment | In-band with 10 MHz LTE |
| --- | --- |
| Date Rate (kbps) | 0.533 |
| Repetition (#) | 20 |
| Transmitter |  |
| Max Tx power (dBm) | 46 |
| (1) Actual Tx power (dBm) | 35 |
| Receiver |  |
| (2) Thermal noise density (dBm/Hz) | -174 |
| (3) Receiver noise figure (dB) | 5 |
| (4) Interference margin (dB) | 0 |
| (5) Occupied channel bandwidth (Hz) | 180,000 |
| (6) Effective noise power | -116.4 |
| (7) Required SINR (dB) | -13.5 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -129.9 |
| (9) Rx processing gain | 0 |
| (10) MCL = (1) -(8) + (9) (dB) | 164.9 |

Here, the payload consists of 48 bits and a 16 bit CRC is appended to it, resulting in a total of 64 bits. Antenna configuration case 1 with SFBC is used. 6 dB power boosting is assumed, and 16 dedicated CRS within a subframe (8 for each transmit antenna) are used. In addition, the first 3 OFDM symbols are unused to avoid collision with legacy PDCCH and other resource elements corresponding to legacy CRS are punctured, leaving a total of 100 resource elements within a subframe. Convolutional coding with QPSK modulation gives a total of 1200 bits and 600 symbols, which are then mapped to 6 subframes (6 ms).

## Cell search

For inband cell search performance, see [3].

# Conclusions

This contribution presents coverage evaluation for the inband deployment of NB-LTE. The evaluation methodology follows all the simulation assumptions detailed in [2]. Results from multiple independent evaluations confirm that all the physical channels and physical procedures of NB-LTE achieve 164 dB MCL also in the inband case.

# References

- R1-156010, “NB-LTE – General L1 concept description”, Ericsson

- 3GPP TR 45 820, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things” v13.0.0.

- R1-156009, “NB-LTE – Synchronization channel design and performance”, Ericsson