3GPP TSG RAN WG1 Meeting #83                                 R1-156833

Anaheim, USA, 15th – 22nd November 2015

# Source:         ZTE

Title:            Rank-2 Codebook Design for Class A

Agenda Item:    6.2.4.3.1

Document for:   Discussion and Decision

- Introduction

Kronecker Product (KP) type codebook for non-precoded CSI-RS has been approved in [1]. In WF [2], the structure of  and class A rank-1 codebook are determined upon four configurations. In RAN1#82bis[3], the valid  combinations for each antenna array layout  was finalized. For rank-2 and higher ranks codebook design, companies provide various codebook schemes via e-mail discussion.

In this contribution, first we discuss rank 2 codebook design based on the rank 1 codebook. Then we give the beam selection rules for rank 2. We present our simulation results and analysis. Our proposal is given as a conclusion.

- Rank 2 codebook Design

In WF [3], four configs are proposed for Rank 1 codebook structure, which correspond to different beam selection patterns as shown in Figure 1. The four configs are configured by RRC signaling according to different scenarios.

Figure 1: Beam selection of rank-1 for different configs

The design of Rank 2 codebook should be based on the four configs. In the email discussion, we have proposed our design of Rank 2 codebook, whose beam selection scheme is shown in Figure 2. The codewords of Rank 2 is given in Appendix.

Figure 2: Beam selection of rank-2 for different configs

Compared with the scheme in [4], our proposal for config 2 covers all the cases in which different beams happen for the larger dimension. Some cases allow both different horizontal and different vertical beams for the two layers.  In general, the larger dimension has more contribution to support rank 2 transmission. So it is expected that these cases happen more often in the practical transmission.

- Performance

In the email discussion, several companies post their proposals for Rank 2 codebook. We simulate 3 schemes: scheme 1 is the proposal in [4], scheme 2 is the proposal posted by Qualcomm, and scheme 3 is our proposal in Appendix. Scheme 1 is set as the baseline for comparison. The codebooks for config 1 and 4 remain the same in the three schemes. Compared with the baseline, scheme 2 redesigns config 2 and 3, whereas scheme 3 only redesigns config 3. We conduct simulations for 3D-Umi with different oversampling. We set  and . The simulation results are shown in Table 1 and Table 2.  From the evaluation results, it seems that different schemes have similar performance.

Table 1: Performance of different codebooks with heavy load, N1,N2,O1,O2=(4,2,4,4)

|  |  | scheme 1 | scheme 2 | scheme 3 |
| --- | --- | --- | --- | --- |
| Config 1 | RU | 0.7204 | The same as scheme 1 | same as scheme 1 |
|  | 5% | 4.5255 |  |  |
|  | 50% | 18.6584 |  |  |
|  | Mean | 21.1494 |  |  |
| Config 2 | RU | 0.7281 | 0.7297 | 0.7273 |
|  | 5% | 4.8431 | 4.7777 | 4.7968 |
|  | 50% | 19.1331 | 19.0671 | 19.0763 |
|  | Mean | 21.4154 | 21.3781 | 21.4524 |
| Config 3 | RU | 0.7213 | 0.7209 | same as scheme 1 |
|  | 5% | 4.2101 | 4.3414 |  |
|  | 50% | 18.2648 | 17.8237 |  |
|  | Mean | 21.1110 | 20.965 |  |

Table 2: Performance of different codebooks with heavy load, N1,N2,O1,O2=(4,2,8,4)

|  |  | scheme 1 | scheme 2 | scheme 3 |
| --- | --- | --- | --- | --- |
| Config 1 | RU | 0.7004 | same as scheme 1 | same as scheme 1 |
|  | 5% | 4.5505 |  |  |
|  | 50% | 17.4013 |  |  |
|  | Mean | 21.0325 |  |  |
| Config 2 | RU | 0.7241 | 0.7255 | 0.7272 |
|  | 5% | 4.7960 | 4.8587 | 4.7753 |
|  | 50% | 18.9956 | 18.8961 | 18.7670 |
|  | Mean | 21.4048 | 21.4106 | 21.4564 |
| Config 3 | RU | 0.7088 | 0.7096 | same as scheme 1 |
|  | 5% | 4.7019 | 4.6517 |  |
|  | 50% | 17.5663 | 17.5165 |  |
|  | Mean | 20.9140 | 20.8362 |  |

- Conclusion

We analyze the rank 2 codebook design and provide evaluation results of 3 different designs.  The three schemes have similar performance. Consider that it is easier for the larger dimension to have different beams corresponding two streams and two streams may also have different vertical and horizontal directions in the practical Rank 2 transmission, it seems to be more reasonable to cover all the cases with different beams in larger dimension as used in our design proposal for config 2.

- References

- 3GPP TR36.897 v13.0.0, “Study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE”, June. 2015.

- R1-156217, WF on class A and class B CSI reporting for Rel.13 EB/FD-MIMO. AT&T, BeijingXinwei Telecom Tech., CATR, CATT, CHTTL, CMCC, Deutsche Telekom, Ericsson, ETRI, Huawei, HiSilicon, ITRI, Kathrein-Werke KG, KDDI, KT Corporation, Nokia Networks, NTT DOCOMO, Samsung, Sony Corporation.

- 3GPP TSG RAN1 #82bis, chairman’s Notes, Oct 2015.

- R1-156381, “Joint Proposal on Rank 2 codebook for Class A CSI reporting”, Samsung, Ericsson, NTT DOCOMO, CATT

- Appendix

Rank-2 codebook is defined as follows.

i1,1=0,1,…,O1N1/s1-1

i1,2=0,1,…,O2N2/s2-1

p1=1 and p2=1

The second PMI selected by UE is based on Config.

| Config | Selected i2 indices | (s1, s2) |
| --- | --- | --- |
| Config 1 | 0-1 | (1,1) |
| Config 2 | 0-3, 8-9, 16-19, 22-23, 28-31 | (2,2) |
| Config 3 | 0-1, 4-5, 8-9, 12-13, 18-21, 24-27 | (2,2) |
| Config 4 | 0-15 | (2,2) |

|  | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| i1,1,i1,2 |  |  |  |  |
|  | 4 | 5 | 6 | 7 |
| i1,1,i1,2 |  |  |  |  |
|  | 8 | 9 | 10 | 11 |
| i1,1,i1,2 |  |  |  |  |
|  | 12 | 13 | 14 | 15 |
| i1,1,i1,2 |  |  |  |  |
|  | 16 | 17 | 18 | 19 |
| i1,1,i1,2 |  |  |  |  |
|  | 20 | 21 | 22 | 23 |
| i1,1,i1,2 |  |  |  |  |
|  | 24 | 25 | 26 | 27 |
| i1,1,i1,2 |  |  |  |  |
|  | 28 | 29 | 30 | 31 |
| i1,1,i1,2 |  |  |  |  |