3GPP TSG RAN WG1 Meeting #85                                 	           R1-164781

# Nanjing, China 23rd - 27th May 2016

# Source:        Samsung

Title:           LC based explicit CSI feedback and simulation results

Agenda Item:   6.2.3.2.3

Document for:  Discussion and Decision

- Introduction

In RAN #71, a new WID related to enhancements to FD-MIMO (eFD-MIMO) has been approved [1]. The objective of the eFD-MIMO WID as captured in [1] includes the following:

- Extend specification support for CSI reporting in the following areas [RAN1]

  - As second priority, evaluate and, if needed, specify enhancement on CSI reporting based on non-precoded and beamformed CSI-RS to improve eNB precoding (such as new feedback methodologies in addition to codebook-based CSI feedback) and interference measurement to support efficient multi-user transmissions (e.g. further enabling interference estimation from NZP or ZP CSI-RS)

    - Analog feedback is not precluded

Based on this objective, the following conclusion was made in RAN1#84bis.

Conclusions:

- Evaluate proposed advanced CSI schemes

- Baseline for comparison:

  - Implicit CSI feedback based on Rel.13 codebook (as well as its extension to >16 ports) or other legacy codebooks if applicable

- Performance and feedback overhead should be provided

- Simulation scenarios:

  - FTP model 1 with 70% and 50% traffic loading

  - Evaluations should include dynamic switching between MU- and SU-MIMO

  - Other simulation details can refer to 36.897 EB/FD MIMO SI

Candidates for PMI Enhancement:

- Codebook based (implicit feedback)

  - Linear combination codebook (enhanced W2) for Non-precoded CSI-RS and beamformed CSI-RS

  - MU CSI (e.g., additional i1, i1,1, or i1,2)

  - Other new or modified codebooks

- Explicit feedback

  - Channel quantization

  - Eigenvector quantization

  - Covariance matrix quantization

- Analog feedback

- Note: other schemes are not precluded

Candidates for CQI Enhancement:

- CQI Derivation

  - Interference estimation based on NZP CSI-RS

  - MU-CQI conditioned on MU hypotheses and CSI-IM

- Reduced CQI feedback delay

- Note: other schemes are not precluded

This contribution proposes an enhancement on CSI reporting based on explicit feedback. In particular, linear combination (LC) based explicit feedback framework is proposed to report a form of DL channel explicitly. Simulation results are provided to show performance of the proposed explicit feedback scheme.

- Linear Combination Based Explicit Feedback

Figure 1: LC based explicit feedback framework

In Figure 1, an LC based explicit CSI feedback framework is proposed in which a form of the DL channel is reported using a double codebook for explicit feedback: W = W1W2, where

- W1 codebook is for WB and long-term basis vectors {bl:l=0,1,…,L-1} feedback,

- W2 codebook is for SB and short-term LC coefficients {cl:l=0,1,…,L-1} feedback, and

- L is the size of the basis vector set.

A few examples of the forms of DL channel and their LC representations are as follows:

- Channel: For a subcarrier k in SB f, the DL channel is represented as Hk≈l=0L-1clbl;

- Covariance matrix: For SB f, the covariance matrix is represented as K=1|f|k∈fHkHkH≈l=0L-1clblblH; and

- Eigenvector: For SB f, the dominant eigenvector of the covariance matrix K is represented as e≈l=0L-1clbl.

For W1 codebook, the Rel. 13 Class A codebook framework can be re-used with the following two alternatives for the basis vectors (as shown in Table 1):

- (Alt 0) L = 4: Config = 2, 3, and 4 of Rel. 13 Class A codebook; and

- (Alt 1) L = 8: (4, 2) and (8, 1), where 1st dimension is the longer dimension.

Table 1: Config to beam group mapping

| L | Number of beams (L1,L2) | Config | Beam group |
| --- | --- | --- | --- |
| 4 | (2,2) | 2 |  |
|  | (4,2) | 3 |  |
|  | (4,1) | 4 |  |
| 8 | (4,2) | - |  |
|  | (8,1) | - |  |

The W2 codebook has two components:

- Co-phase φll=0L-1: the co-phase belong to {1,j,-1,-j}. Two alternatives for the co-phase are as follows.

  - Common: same co-phase for L beams.

  - Different: different co-phase for L beams.

- Coefficients cll=0L-1: The unquantized LC coefficients can be obtained as the least-square solution to minimize the squared error. For example, for eigenvector feedback, the unquantized coefficients are cll=0L-1=argmincll=0L-1e-l=0L-1clblφlbl2. The resultant solution is obtained by pre-multiplying the dominant eigenvector e with the pseudo-inverse of the basis vector set, i.e., cll=0L-1=B*B-1B*e, where B=[b0, b1,…,bL-1] is basis matrix in which columns are basis vectors.

Proposal 1: For advanced CSI reporting, consider explicit reporting of a form of DL channel using a LC framework, where

- the LC framework corresponds to a double codebook for explicit feedback: W1W2

  - W1 codebook is for the WB selection of L basis vectors;

  - W2 codebook is for SB co-phase and coefficient selection for the selected L basis vectors; and

- the form of DL channel can be DL channel itself or covariance matrix or eigenvector.

Proposal 2: For W1 codebook, the Rel. 13 Class A codebook framework can be re-used with the following two alternatives for the basis vectors (Table 1):

- (Alt 0) L = 4: Config = 2, 3, and 4 of Rel. 13 Class A codebook; and

- (Alt 1) L = 8: (4, 2) and (8, 1), where 1st dimension is the longer dimension.

Proposal 3: For W2 codebook, there are two alternatives for the co-phase

- Common co-phase for L beams, and

- Different co-phase for L beams.

The unquantized coefficients can be obtained by pre-multiplying the form of DL channel with the pseudo-inverse of the basis matrix obtained by column concatenation of L basis vectors.

The proposed LC based explicit feedback scheme can also be applied to enhance Class B, K ≥ 1 CSI reporting. For instance, for K = 1 and P beam-formed ports, the UE can report explicit CSI, which represents P-port beam-formed channel using the proposed scheme. Similarly, the proposal is applicable to enhance hybrid CSI reporting scheme in which two types of CSI-RS resources, for example non-precoded and beamformed CSI-RS, are associated with two CSI reports. One of the two reported CSIs can be the proposed LC based explicit CSI.

Observation 1: Proposal 1-3 can be extended to Class B and hybrid CSI enhancement in which at least one of the reported CSIs indicates LC based explicit CSI.

In the proposed explicit CSI feedback scheme, the reported CSI comprises of the following components:

- 1st PMI (i1,1, i1,2): indicating L beams or basis vectors, similar to Rel. 13 Class A codebook;

- 2nd PMI i2: indicating

  - co-phase for the L selected beams from the QPSK codebook {i,j,-1,-j}, and

  - LC coefficients to combine the beams;

- (Optional) CQI: indicating dominant eigenvalues; and

- (Optional) RI: indicating the preferred rank.

Proposal 4: The explicit CSI report includes the following components:

- 1st PMI (i1,1, i1,2) indicating L beams or basis vectors, similar to Rel. 13 Class A codebook;

- 2nd PMI i2 indicating QPSK co-phase and coefficients for the L selected beams; and

- (optional) CQI and RI.

- Simulation Results

To demonstrate the performance of the proposed LC based explicit feedback scheme, simulation-level simulation results are provided for the following proposed LC based explicit CSI feedback schemes to represent the dominant eigenvector of the DL channel:

- Config 2, common co-phase for 4 beams

- Config 3, common co-phase for 4 beams

- Config 4, common co-phase for 4 beams

- (4,2) beam group, common co-phase for 8 beams

- (8,1) beam group, common co-phase for 8 beams

- Config 2, different co-phase for 4 beams

- Config 3, different co-phase for 4 beams

- Config 4, different co-phase for 4 beams

- (4,2) beam group, different co-phase for 8 beams

- (8,1) beam group, different co-phase for 8 beams.

For the co-phase QPSK codebook is assumed and results are provided for unquantized coefficients. The first PMI (i1,1, i1,2) reporting overhead is the same for all schemes. The second PMI i2 reporting overhead is according to Table 2 where K is the number of bits to represent each coefficient.

Table 2: i2 reporting overhead

| L | Beam group | Co-phase type | Co-phase (#bits) | Coefficients (#bits) | Total (#bits) |
| --- | --- | --- | --- | --- | --- |
| 4 | Config 2 | Common | 2 | 4K | 2 + 4K |
|  | Config 3 |  |  |  |  |
|  | Config 4 |  |  |  |  |
| 8 | (4,2) |  |  | 8K | 2 + 8K |
|  | (8,1) |  |  |  |  |
| 4 | Config 2 | Different | 8 | 4K | 8 + 4K |
|  | Config 3 |  |  |  |  |
|  | Config 4 |  |  |  |  |
| 8 | (4,2) |  | 16 | 8K | 16 + 8K |
|  | (8,1) |  |  |  |  |

The non-full-buffer system-level evaluation is carried out for UMa-200m and UMi-2GHz channel models in heavy (70% target RU) traffic loading scenarios. The detailed results can be found in Table 4 and Table 5 in the Appendix. The results are provided for 16 and 32 antenna ports with (N1,N2) = (4,2)  and (N1,N2) = (4,4), respectively. Here, we assume that the first dimension is horizontal and the second dimension is vertical. The downtilt angles in the elevation domain are chosen according to [2]. In these simulations, full-port non-precoded CSI-RS is used for CSI estimation, and the corresponding CSI-RS overhead is taken into account in the final throughput calculation. Cell association antenna pattern is approximated by one-TXRU pattern, and proportional fair scheduling (max 4 layers per time-frequency resource) have been used. For MU-MIMO, SLNR precoding is considered. The relevant simulation parameters are enlisted in Table 3. The rest of the simulation assumption is according to [2]. For comparison, performance results for Class A, Config 2, 3, and 4 based implicit CSI feedback, and ideal CSI feedback in which dominant eigenvector is assumed to be known at eNB are also provided. The performance gains with “Class A Config 2 based implicit scheme” as reference are summarized in Figure 2 - Figure 5.

Figure 2: Performance for 16 ports, (N1,N2) = (2,4), UMa-200m

Figure 3: Performance for 16 ports, (N1,N2) = (2,4), UMi-2GHz

Figure 4: Performance for 32ports, (N1,N2) = (4,4), UMa-200m

Figure 5: Performance for 32ports, (N1,N2) = (4,4), UMi-2GHz

From the SLS results, we can make the following observation.

Observation 2: From the SLS results, we can make the following observation:

- Proposed explicit feedback scheme shows significant gain over implicit scheme

- Comparison between Config 2, 3, and 4 (L = 4) based proposed explicit feedback scheme show that the performance of Config 2 and 3 are similar, but that of Config 4 is the worst.

- L = 8 shows significant performance gain over L = 4.

- Performance gain is maintained on increasing the number of ports from 16 to 32.

- Conclusion

This document proposes an LC based explicit CSI feedback. The proposals and observations made can be summarized as follows.

Proposal 1: For advanced CSI reporting, consider explicit reporting of a form of DL channel using a LC framework, where

- the LC framework corresponds to a double codebook for explicit feedback: W1W2

  - W1 codebook is for the WB selection of L basis vectors;

  - W2 codebook is for SB co-phase and coefficient selection for the selected L basis vectors; and

- the form of DL channel can be DL channel itself or covariance matrix or eigenvector.

Proposal 2: For W1 codebook, the Rel. 13 Class A codebook framework can be re-used with the following two alternatives for the basis vectors (Table 1):

- (Alt 0) L = 4: Config = 2, 3, and 4 of Rel. 13 Class A codebook; and

- (Alt 1) L = 8: (4, 2) and (8, 1), where 1st dimension is the longer dimension.

Proposal 3: For W2 codebook, there are two alternatives for the co-phase

- Common co-phase for L beams, and

- Different co-phase for L beams.

The unquantized coefficients can be obtained by pre-multiplying the form of DL channel with the pseudo-inverse of the basis matrix obtained by column concatenation of L basis vectors.

Observation 1: Proposal 1-3 can be extended to Class B and hybrid CSI enhancement in which at least one of the reported CSIs indicates LC based explicit CSI.

Proposal 4: The explicit CSI report includes the following components:

- 1st PMI (i1,1, i1,2) indicating L beams or basis vectors, similar to Rel. 13 Class A codebook;

- 2nd PMI i2 indicating QPSK co-phase and coefficients for the L selected beams; and

- (optional) CQI and RI.

Observation 2: From the SLS results, we can make the following observation:

- Proposed explicit feedback scheme shows significant gain over implicit scheme

- Comparison between Config 2, 3, and 4 (L = 4) based proposed explicit feedback scheme show that the performance of Config 2 and 3 are similar, but that of Config 4 is the worst.

- L = 8 shows significant performance gain over L = 4.

- Performance gain is maintained on increasing the number of ports from 16 to 32.

References

[1] RP-160623, “New WID Proposal: Enhancements on Full-Dimension (FD) MIMO for LTE,” Samsung.

[2] 3GPP, TR36.897v1.0.1, “Study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE”

[3] R1-164779, “Advanced CSI reporting paradigm” Samsung.

[4] R1-164781, “LC based explicit CSI feedback and simulation results” Samsung.

Appendix: Simulation Assumptions and Results

Table 3: Simulation Parameters

| Parameters | Values |
| --- | --- |
| Simulation Type | Non-full-buffer (Heavy load 70% Target RU) |
| Channel model | UMi-2GHz, UMa-200m |
| Number of BS (H,V) antenna elements | (8,8), x-polarized, subarray partition |
| (N1,N2, P) | 16, 32 ports: (4,2,2), (4,4,2) |
| BS (H,V) antenna spacing | (0.5, 0.8)λ |
| BS and MS antenna polarizations | BS: (+45°,-45°); MS: (0°, 90°) |
| Number of UE antennas | 2 |
| SU/MU pre-coding | SLNR |
| Scheduling | MU, Proportional fair, up to 4 layers |
| Channel estimation | Ideal |
| Transmission rank | 1 |
| Receiver | MMSE-IRC |
| Explicit CSI schemes | Reference: ideal CSI LC: Rel. 13 Class A W1 codebook as framework 4 beams: Config 2,3,4 8 beams: (4,2) and (8,1) beam group Co-phase: QPSK Coefficient: Unquantized |

Table 4: Non-full buffer simulation results for 16 ports

| Channel | Lambda | Scheme | Co-phase | RU | Avg UPT | 50% UPT | 5% UPT | Avg UPT gain | 50% UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UMa-200m | 4 | Class A, Config 2 | - | 59.0% | 17.07 | 17.13 | 5.29 | 100.0% | 100.0% | 100.0% |
|  |  | Class A, Config 3 | - | 59.2% | 16.90 | 16.88 | 5.14 | 99.0% | 98.5% | 97.1% |
|  |  | Class A, Config 4 | - | 59.4% | 16.93 | 16.92 | 5.19 | 99.2% | 98.8% | 98.0% |
|  |  | Config 2 | Common | 56.3% | 18.18 | 18.61 | 6.41 | 106.5% | 108.6% | 121.0% |
|  |  | Config 3 |  | 55.9% | 18.30 | 18.65 | 6.48 | 107.2% | 108.9% | 122.5% |
|  |  | Config 4 |  | 58.5% | 17.14 | 17.17 | 5.43 | 100.4% | 100.2% | 102.7% |
|  |  | (4,2) beams |  | 55.0% | 18.76 | 19.14 | 7.46 | 109.9% | 111.7% | 140.9% |
|  |  | (8,1) beams |  | 56.5% | 17.90 | 18.10 | 6.51 | 104.9% | 105.7% | 123.0% |
|  |  | Config 2 | Different | 52.0% | 20.52 | 21.62 | 9.37 | 120.2% | 126.2% | 177.0% |
|  |  | Config 3 |  | 51.9% | 20.51 | 21.62 | 9.44 | 120.2% | 126.2% | 178.3% |
|  |  | Config 4 |  | 58.2% | 17.30 | 17.51 | 5.32 | 101.3% | 102.2% | 100.5% |
|  |  | (4,2) beams |  | 50.6% | 21.44 | 22.86 | 10.92 | 125.6% | 133.4% | 206.2% |
|  |  | (8,1) beams |  | 51.5% | 20.66 | 21.98 | 9.79 | 121.0% | 128.3% | 185.0% |
|  |  | Ideal | - | 49.8% | 21.97 | 23.53 | 11.74 | 128.7% | 137.3% | 221.8% |
| UMi-2GHz | 4 | Class A, Config 2 | - | 60.4% | 17.04 | 16.95 | 5.59 | 100.0% | 100.0% | 100.0% |
|  |  | Class A, Config 3 | - | 60.1% | 17.08 | 17.08 | 5.69 | 100.2% | 100.8% | 101.8% |
|  |  | Class A, Config 4 | - | 60.2% | 17.03 | 16.88 | 5.58 | 99.9% | 99.6% | 99.9% |
|  |  | Config 2 | Common | 57.1% | 18.48 | 18.90 | 7.14 | 108.4% | 111.5% | 127.8% |
|  |  | Config 3 |  | 56.8% | 18.55 | 19.14 | 7.24 | 108.9% | 112.9% | 129.6% |
|  |  | Config 4 |  | 59.7% | 17.23 | 17.11 | 6.01 | 101.1% | 101.0% | 107.6% |
|  |  | (4,2) beams |  | 55.5% | 19.23 | 19.80 | 8.07 | 112.8% | 116.9% | 144.3% |
|  |  | (8,1) beams |  | 57.4% | 18.18 | 18.35 | 6.88 | 106.7% | 108.3% | 123.0% |
|  |  | Config 2 | Different | 53.0% | 20.81 | 22.22 | 9.33 | 122.1% | 131.1% | 167.0% |
|  |  | Config 3 |  | 53.2% | 20.75 | 22.16 | 9.21 | 121.8% | 130.8% | 164.9% |
|  |  | Config 4 |  | 59.5% | 17.34 | 17.47 | 6.05 | 101.8% | 103.1% | 108.3% |
|  |  | (4,2) beams |  | 51.3% | 21.72 | 23.26 | 10.58 | 127.5% | 137.2% | 189.3% |
|  |  | (8,1) beams |  | 52.2% | 20.93 | 22.22 | 9.64 | 122.8% | 131.1% | 172.4% |
|  |  | Ideal | - | 50.7% | 22.22 | 23.95 | 11.33 | 130.4% | 141.4% | 202.7% |

Table 5: Non-full buffer simulation results for 32 ports

| Channel | Lambda | Scheme | Co-phase | RU | Avg UPT | 50% UPT | 5% UPT | Avg UPT gain | 50% UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UMa-200m | 4 | Class A, Config 2 | - | 56.2% | 18.50 | 18.96 | 7.12 | 100.0% | 100.0% | 100.0% |
|  |  | Class A, Config 3 | - | 56.1% | 18.46 | 18.95 | 7.14 | 99.8% | 100.0% | 100.3% |
|  |  | Class A, Config 4 | - | 56.6% | 18.35 | 18.69 | 7.17 | 99.2% | 98.6% | 100.8% |
|  |  | Config 2 | Common | 54.0% | 19.42 | 20.00 | 8.49 | 105.0% | 105.5% | 119.3% |
|  |  | Config 3 |  | 54.0% | 19.44 | 20.05 | 8.38 | 105.1% | 105.8% | 117.7% |
|  |  | Config 4 |  | 54.7% | 18.78 | 19.42 | 7.41 | 101.5% | 102.4% | 104.1% |
|  |  | (4,2) beams |  | 53.2% | 19.85 | 20.51 | 9.09 | 107.3% | 108.2% | 127.8% |
|  |  | (8,1) beams |  | 54.4% | 19.21 | 19.70 | 8.20 | 103.8% | 103.9% | 115.2% |
|  |  | Config 2 | Different | 50.8% | 21.29 | 22.66 | 10.47 | 115.1% | 119.5% | 147.1% |
|  |  | Config 3 |  | 50.9% | 21.23 | 22.60 | 10.34 | 114.8% | 119.2% | 145.3% |
|  |  | Config 4 |  | 55.5% | 18.73 | 19.32 | 7.49 | 101.2% | 101.9% | 105.3% |
|  |  | (4,2) beams |  | 49.4% | 22.03 | 23.81 | 11.24 | 119.1% | 125.6% | 157.9% |
|  |  | (8,1) beams |  | 50.2% | 21.49 | 23.26 | 10.35 | 116.2% | 122.7% | 145.4% |
|  |  | Ideal | - | 48.6% | 22.63 | 24.39 | 12.31 | 122.3% | 128.7% | 173.0% |
| UMi-2GHz | 4 | Class A, Config 2 | - | 55.9% | 18.99 | 19.80 | 7.78 | 100.0% | 100.0% | 100.0% |
|  |  | Class A, Config 3 | - | 55.9% | 18.89 | 19.79 | 7.82 | 99.5% | 99.9% | 100.5% |
|  |  | Class A, Config 4 | - | 55.9% | 18.83 | 19.60 | 7.85 | 99.1% | 99.0% | 100.9% |
|  |  | Config 2 | Common | 54.1% | 20.00 | 21.04 | 8.85 | 105.3% | 106.2% | 113.8% |
|  |  | Config 3 |  | 53.9% | 20.06 | 21.05 | 8.80 | 105.6% | 106.3% | 113.2% |
|  |  | Config 4 |  | 55.5% | 19.07 | 19.80 | 7.51 | 100.4% | 100.0% | 96.5% |
|  |  | (4,2) beams |  | 53.2% | 20.49 | 21.51 | 9.44 | 107.9% | 108.6% | 121.4% |
|  |  | (8,1) beams |  | 54.5% | 19.69 | 20.51 | 8.59 | 103.7% | 103.6% | 110.4% |
|  |  | Config 2 | Different | 51.1% | 21.79 | 23.30 | 11.07 | 114.7% | 117.7% | 142.3% |
|  |  | Config 3 |  | 51.4% | 21.71 | 23.19 | 10.86 | 114.3% | 117.1% | 139.7% |
|  |  | Config 4 |  | 55.6% | 19.15 | 20.02 | 7.93 | 100.8% | 101.1% | 101.9% |
|  |  | (4,2) beams |  | 49.4% | 22.45 | 24.24 | 11.49 | 118.2% | 122.4% | 147.7% |
|  |  | (8,1) beams |  | 49.7% | 21.94 | 23.67 | 10.55 | 115.5% | 119.5% | 135.6% |
|  |  | Ideal | - | 49.0% | 23.16 | 25.16 | 13.07 | 121.9% | 127.0% | 168.0% |