3GPP TSG-RAN WG1 Meeting #85	R1-164922

Nanjing, China, 23 - 27 May 2016

#### Agenda item:		6.2.10.1 Channel design for shortened TTI

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:					sPUCCH Design for shortened TTI

Document for:		Discussion and Decision

# 1. Introduction

Based on the outcome of RAN#67 captured in the SI description in RP-150465, the following items have been identified specifically for RAN1 studies on TTI shortening and reduced processing times:

- Assess specification impact and study feasibility and performance of TTI lengths between 0.5ms and one OFDM symbol, taking into account impact on reference signals and physical layer control signaling

- Backwards compatibility shall be preserved (thus allowing normal operation of pre-Rel 13 UEs on the same carrier);

In this document, we discuss the principles of PUCCH design for short TTI. Short TTI related aspects of UL DMRS, UL HARQ, and TDD specific aspects are considered in [1], [2], and [3], respectively.

# 2. Design Targets for sPUCCH

PUCCH design is one of the key aspects in facilitating latency reduction. In order to be able to reduce HARQ round-trip time, the UL control channel conveying HARQ-ACK will need to be shortened similarly as the related PDSCH carrying the data transport block(s). Therefore it seems apparent that a shortened PUCCH will need to be specified to carry at least HARQ-ACK. In principle, it should be straight forward to apply the same design for conveying Scheduling Request as well, which in turn may provide a minor decrease in the observed latency.

On the other hand, the need for using shortened PUCCH for CSI is far less clear. In LTE, periodic PUCCH reports are transmitted on PUCCH when there is no simultaneous PUSCH transmission. However, periodic CSI reporting involves inherently relatively large latency depending on the reporting periodicity, and the length of PUCCH – 1 ms or less – does not have a significant impact on the overall latency. Furthermore, it should be possible for the network to trigger Aperiodic CSI report to be transmitted using shortened PUSCH, which further reduces the need for transmitting periodic CSI over shortened PUCCH.

Observation #1: Primary contents for sPUCCH are HARQ-ACK (+SR). The need for supporting periodic CSI transmission over sPUCCH is unclear.

As for the PUCCH supported payload, the baseline design should provide support for single-CC case (i.e. 1 or 2 bit HARQ-ACK + possibly SR) with a robust performance. Support for moderate carrier aggregation configurations should be considered as well, so that ~10 bits can supported.

When it comes to sPUCCH design, a few requirements need to be considered. In LTE, the coverage of the whole system is often limited by the PUCCH. In order to mitigate this, various means have been specified including e.g. PUCCH repetition to enhance HARQ-ACK coverage. Obviously, for shorter TTI operation to make sense, the impact on coverage should be minimized. On the other hand, shorter TTI means that the energy per bit for e.g. HARQ-ACK transmission is inevitable reduced, and hence there will be some penalty in terms of cell coverage. This issue is more pronounced with TTI lengths less than 0.5 ms and may easily end up limiting the coverage of sTTI feature as a whole.

Observation #2: PUCCH coverage may easily become a bottleneck for sTTI, especially if sTTI length of less than 0.5 is considered.

In order to make shorter TTI operation an attractive feature, special attention needs to be put on sPUCCH coverage. We see that the as a starting point, one needs to keep the basic design properties of 1-ms PUCCH, namely frequency hopping to achieve frequency diversity. Furthermore, sPUCCH waveform should have low cubic metric to guarantee uncompromised operation at maximum TX power.

Proposal #1): sPUCCH should aim at as large coverage as possible, and

  - Retain support for frequency diversity (i.e. hopping across band edges)

  - Have low cubic metric similar or close to that of SC-FDMA

Another key design target for LTE PUCCH has been low overhead, which has been achieved with high multiplexing capability. Depending on the PUCCH format (and correspondingly payload), more than 10 UEs can share the same PUCCH PRB. We see that the same design target is equally important with shorter TTIs as well, and efficient multiplexing of different sPUCCH UEs, as well as both sPUCCH and legacy PUCCH UEs should be supported within a PRB.

Observation #3: Increase in UL control overhead due to sTTI should be minimized.

Proposal #2: Multiplexing of sPUCCHs should be supported within the same PRB.

Proposal #3: Multiplexing of sPUCCH and legacy PUCCH should be supported within the same PRB.

# 3. sPUCCH Design Candidates

Based on the design targets discussed above, we present here two sPUCCH designs meeting the target criteria. The designs are targeting 0.5 ms TTI, and support payload sizes similar to legacy PUCCH Formats 1b and 2, respectively. However, the presented designs are in principle scalable and applicable also with shorter than 0.5 ms TTI lengths.

Figure 1 shows the design for a PUCCH Format 1b-like design, denoted here sPUCCH Format 1, capable of supporting 1 or 2 bit HARQ-ACK transmission, as well as SR transmission through channel selection as in LTE Rel-8. The design is based on QPSK-modulated length-12 UL DM-RS sequences. Similarly as with legacy PUCCH format 2, user multiplexing with cyclic shifts can be performed, allowing for six simultaneous UEs on a single PUCCH PRB. Furthermore, it is also possible to assign some of the cyclic shifts for legacy PUCCH format 2 users as well, allowing for minimized UCI overhead and UL resource fragmentation.

Figure 1. sPUCCH Format 1 design for 1-bit or 2- bit HARQ-ACK

Figure 2. illustrates another candidate design for moderate UCI payloads of up to 11 bits (“sPUCCH Format 2”). Also here, Rel-8 length-12 UL DM-RS sequences are applied, with a distinct cyclic shift assigned for each sequence. In the DM-RS symbols (i.e. symbol #1, #4), one unmodulated DM-RS sequence is transmitted (e.g. CS_0), whereas in data symbols two QPSK-modulated sequences are transmitted in parallel (e.g. M-CS_0, M-CS_1), separated with cyclic shifts. Frequency hopping is applied between “short slots” hence maintaining frequency diversity similarly as the legacy PUCCH formats. The described arrangement results in having 20 uncoded bits in a 0.5 ms TTI, i.e. the same number as with the current LTE PUCCH format 2. This allows for directly reusing the PUCCH Format 2 Reed-Muller channel coding scheme. Similarly as with the 1/ 2 bit design, also in this case legacy PUCCH format 2 users - as well as sPUCCH Format 1 users – can be multiplexed on the same PRB(s). The design principle scales also to shorter TTI lengths, although at the cost of supported payloads. Therefore we conclude that designs based on modulated LTE length-12 DMRS sequences are promising candidates for sPUCCH and should be considered further.

Figure 2. sPUCCH Format 2 design supporting up to 11 bit payload with 0.5 ms TTI.

Proposal #4: Designs based on modulated LTE length-12 DMRS sequences can co-exist with PUCCH format 1/1a/1b and 2/2a/2b and should be considered further as candidates for sPUCCH.

# 4. Summary

In this contribution we consider aspects impacting PUCCH design for sTTI on a high level. Based on the discussion, we make the following observations and proposals related to PUCCH design:

Observation #1: Primary contents for sTTI PUCCH are HARQ-ACK (+SR). The need for supporting periodic CSI transmission over shortened PUCCH is unclear.

Observation #2: PUCCH coverage may easily become a bottleneck for sTTI, especially if sTTI length of less than 0.5 is considered.

Proposal #1): sTTI PUCCH should aim at as large coverage as possible, and

  - Retain support for frequency diversity (i.e. hopping across band edges)

  - Have low cubic metric similar or close to that of SC-FDMA

Observation #3: Increase in UL control overhead due to sTTI should be minimized.

Proposal #2: Multiplexing of sPUCCHs should be supported within the same PRB.

Proposal #3: Multiplexing of sPUCCH and legacy PUCCH should be supported within the same PRB.

Proposal #4: Designs based on modulated LTE length-12 DMRS sequences can co-exist with PUCCH format 1/1a/1b and 2/2a/2b and should be considered further as candidates for sPUCCH.

# 5. References

- R1-160779 “DMRS for PUSCH with short TTI”, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, RAN1#84

- R1-160799, “Considerations on UL HARQ for Supporting Latency Reduction”, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, RAN1#84

- R1-160780, “Shorter TTI enhancements for TDD”, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, RAN1#84