3GPP TSG-RAN WG1 Meeting #86			     R1- 166596

Goteborg, Sweden, August 22-26, 2016

Agenda item:	7.2.10

Source: 	Apple

Title: 	Higher performance targets for Further Enhanced MTC.

Document for:		Discussion and Decision

# Introduction

Working Item for Release-14 “Further Enhanced MTC” [1] has been approved in RAN Meeting #72. This Working Item has, among others, the following Objectives (RAN1-led):

“Higher data rates [RAN1, RAN2, RAN4]

- Specify HARQ-ACK bundling in CE mode A in HD-FDD

- Larger maximum TBS

- Larger max. PDSCH/PUSCH channel bandwidth in connected mode at least in CE mode A in order to enhance support e.g. voice and audio streaming or other applications and scenarios

- Up to 10 DL HARQ processes in CE mode A in FD-FDD”

This contribution discusses the higher data rate, HD-FDD design targets and provides some high-level proposals.

# Discussion and proposals

3GPP work in Release-13 eMTC specified new features allowing cellular modem form-factor cost and complexity reduction, battery life extension and coverage enhancements.

These features were designed for a set of devices/use cases, such as smart meters, sensors and smart readers that requires neither real time services nor high data rates.

Today, wearable devices usage is expanding at fast pace and these devices provide their own set of services and applications that could be summarized as follows:

- HD Voice and Multimedia Calls

- HD Music streaming

- Video streaming

- Human-interface originated operations.

It is undeniable that wearable devices share some common requirements with other MTC devices such as form factor cost & complexity, battery life and coverage enhancements. On top to these requirements, wearables enable new services that are delay-sensitive and require a higher peak data rate in both normal and enhanced coverage modes.

Proposal 1: RAN1 to work on enhancing R-13 eMTC features based on wearables services/applications requirements when operating in normal coverage and enhanced coverage in TDD, FDD and HD-FDD.

## Higher Data Rates

Release 13 Cat-M1 devices support a maximum of 6 PRBs and 1000 bits TBS [1]. This configuration allows a modest maximum peak data rate of:

- 1Mbps for Full duplex FDD.

- ~330kbps for Half duplex FDD, as the device cannot support simultaneous transmission and reception.

- ~200kbps UL/ ~700kbps DL for a typical TDD configuration 2.

These numbers indicate that a theoretical 1Mbps target of CAT-M1 device is hardly achievable in most network configurations in FDD and impossible to achieve in TDD or HD-FDD modes.

Applications such HD music could require a high peak unicast traffic-throughput in normal coverage (at least in downlink).

Applications such audio/video calls require a uniform user experience between normal coverage and cell edge, hence an increase in device data rate is desirable in enhanced coverage situations.

In addition to that, a user’s access to short videos or files download through wearable interfaces requires a high peak data rate for a short time. This aspect of response delay is really important for an acceptable user experience.

Maximum TBS size:  The first limitation of R-13 eMTC devices is the max TBS size of 1000 bits, therefore supporting a larger TBS for FDD, TDD and HD-FDD in both normal coverage and enhanced coverage should be investigated.

Maximum PDSCH/PUSCH bandwidth: The second limitation on data rates in Release 13 eMTC devices is the 6PRB restriction, especially in downlink transmission. In order to support higher peak data rates, eMTC devices should be able to support larger number of PRBs in both normal and enhanced coverage for PUSCH and PDSCH.

Proposal 2: RAN1 to investigate larger TBS for eMTC in both normal coverage and enhanced coverage.

Proposal 3: RAN1 to investigate larger PDSCH/PUSCH bandwidth for eMTC in both normal coverage and enhanced coverage.

## Higher Data Rates Optimisations

HD-FDD form-factors provide certain advantages such as cost and power consumption when compared to Full duplex FDD form factors.  On the other hand, this mode of operation does not allow PDSCH reception while PUSCH transmission, hence a lower peak throughput.

Some optimisations could be considered to improve HD-FDD operation such as:

- Bundling of HARQ-ACK (at least for Downlink), could unleash higher pick throughput [3].

Proposal 4: RAN1 to investigate optimisations of HD-FDD mode through HARQ-ACK bundling.

# Conclusion

In this contribution, we listed some important design targets for eMTC based in wearable use-cases and we propose the following directions to enable necessary higher performances:

Proposal 1: RAN1 to work on enhancing R-13 eMTC features based on wearables services/applications requirements when operating in normal coverage and enhanced coverage in TDD, FDD and HD-FDD.

Proposal 2: RAN1 to investigate larger TBS for eMTC in both normal coverage and enhanced coverage.

Proposal 3: RAN1 to investigate larger PDSCH/PUSCH bandwidth for eMTC in both normal coverage and enhanced coverage.

Proposal 4: RAN1 to investigate optimisations of HD-FDD mode through HARQ-ACK bundling.

# 4. Reference

[1] RP-161321 “New WI proposal on Further Enhanced MTC”, Ericsson

[2] 3GPP TS 36.306 v13.2.0  “User Equipment (UE) radio access capabilities”

[3] R1-153111 “M-PDCCH and HARQ Options for HD-FDD in Normal Coverage”, Sierra Wireless