3GPP TSG RAN WG1 Meeting #86                          R1-166675

Gothenburg, Sweden 22nd - 26th August 2016

Agenda Item:	   8.1.3.2

Source: 	   Fujitsu

Title:            Guard period setting in TDD based NR frame structure

Document for:   Discussion/Decision

# Introduction

In RAN1#85, the following agreements have been achieved on TDD based NR frame structure:

- At least the following should be supported for NR in a frequency portion - A time interval X which can contain one or more of the following - DL transmission part - Guard - UL transmission part - FFS length(s) of time interval X - At least the following time domain structures should be studied/evaluated for NR - DL transmission region (containing data assignments and data), guard region, UL transmission region (containing UCI) - DL transmission region (containing data assignments), guard region, UL transmission region (containing data, UCI)

Based on the above agreements, it is worth noting that the duration of the guard period (GP) in the time-domain structure could impact the efficiency, especially when the time interval X is set to a small values, e.g., 0.5ms, 0.25ms, 0.125ms. Thus, in this contribution, we discuss the GP setting in NR.

# 2. Guard period in LTE/LTE-A

In LTE/LTE-A, the special sub-frame is defined to allow the switch from DL to UL, which consists of DwPTS (DL part), GP and UpPTS (UL part). To address different cell coverages, 10 special sub-frame configurations are given in [1]. In OFDM system, the minimum time-domain resource granularity is an OFDM symbol. Thus, for each special sub-frame configuration, DwPTS and UpPTS always contain the integer number of OFDM symbols with either normal or extended CP length. Then, the remaining part in the special sub-frame is left to the GP. Considering the aim of supporting flexible future-proof configuration of NR, perhaps, it does not make sense to assume directly adopting the guard period configuration method in LTE into NR. Therefore, studying the guard period configuration for NR becomes necessary.

# 3. Guard period setting in NR

In this section, the factors of determining the NR guard period are defined. In order to intuitively understand how the factors related to the guard period setting, in addition, a typical example in NR is illustrated.

## Factors of determining the guard period setting

There are two factors related to the guard period setting:

- Factor 1: Cell size and hardware-capability

In general, as shown in Fig. 1, the simplest GP configuration considers one period of switching from DL to UL, TDU, and one period of switching from UL to DL, TUD [2], i.e., TDU+TUD, where TDU=max⁡(TRTT+TUE, Rx-Tx, TeNB, Tx-Rx), and TUD=max⁡(TUE, Tx-Rx, TeNB, Rx-Tx). Here, TRTT is the maximum round trip time (RTT) determined by the cell size. TUE, Rx-Tx, TeNB, Tx-Rx, TUE, Tx-Rx, and TeNB, Rx-Tx are parameters related to the hardware capability at both eNB and UE sides, with the reference values as listed in Table 1. In other words, once the cell size and the hardware of eNB and UE have been determined, the minimum GP length, i.e., TGPmin=TDU+TUD, is determined.

Fig. 1 Component of TGPmin

Table 1 Reference values for hardware-related parameters [3]

| TeNB,Tx-Rx [sec] | TUE,Rx-Tx [sec] | TUE,Tx-Rx [sec] | TeNB,Rx-Tx [sec] |
| --- | --- | --- | --- |
| 10-15 | 10-40 | 10-20 | 10-20 |

- Factor 2: Length of DL/UL transmission

As agreed in RAN1#85, a time interval X with DL transmission, GP and UL transmission should be supported. Thus, the GP length in time interval X is denoted as TGP=TX-TDL-TUL, where TX, TDL, and TUL are the lengths of the time interval X, DL and UL transmissions, respectively. Similarly, in NR, the minimum time-domain resource granularity is an OFDM symbol. Thus, the time interval X should contain an integer number of OFDM symbols in both DL and UL transmission parts.

Combining the above two factors, we can propose the following working assumption when setting GP in NR:

Possible working assumption: the guard period in the time interval X should be larger than or equal to the minimum guard period value, i.e., TGP≥TGPmin, under the constraint that the DL/UL transmission contains an integer number of OFDM symbols.

## Example of guard period setting in NR

In RAN1#85 meeting, the working assumptions associated with the numerology are:

- RAN1 concludes on alternative 1 (15 kHz) as the baseline design assumption for the NR numerology - RAN1 concludes on scale factors N =2n for subcarrier spacing as the baseline design assumption for the NR numerology

Then, the possible lengths of an OFDM symbol are given in Table 2.

Table 2 Symbol length with normal CP for different numerologies

| Subcarrier spacing | 15KHz | 30KHz | 60KHz |
| --- | --- | --- | --- |
| OFDM symbol length (us) | 71.88/71.35 | 35.94/35.68 | 17.97/17.84 |
| Note: the 1st and 2nd values are the lengths of OFDM symbols with the CP lengths of 5.21us and 4.69us, respectively. |  |  |  |

With subcarrier spacing of 15/30/60KHz, the length of the time interval X may be set to 0.5/0.25/0.125ms, respectively. Assuming the length of the time interval X corresponding to 7 OFDM symbols with the same numerology, we calculate the GP length and the resultant overhead for different scenarios and different lengths of time interval X, as listed in Table 3.

Table 3 GP duration and overhead in NR for different lengths of time interval X

| Scenario |  | Indoor hotspot | Dense urban | Rural |  | Urban macro |
| --- | --- | --- | --- | --- | --- | --- |
| ISD(m) |  | 20 | 200 | 1732 | 5000 | 500 |
| Radius(m) |  | 10 | 133.3 | 1154.7 | 3333.3 | 333.3 |
| RTT value (us) |  | 0.0667 | 0.889 | 7.7 | 22.23 | 2.22 |
| TGPmin (us) (Note1) |  | 20.0667 | 20.889 | 27.7 | 42.23 | 22.22 |
| Length of the configured GP, i.e., TGP, (us) (Note2) | X=0.5ms | 71.35(14.27%) 4.01% | 71.35(14.27%) 4.18% | 71.35(14.27%) 5.54% | 71.35(14.27%) 8.45% | 71.35(14.27%) 4.44% |
|  | X=0.25ms | 35.68(14.27%) 8.03% | 35.68(14.27%) 8.35% | 35.68(14.27%) 11.1% | 71.36(28.54%) 16.9% | 35.68(14.27%) 8.89% |
|  | X=0.125ms | 35.68(28.54%) 16.1% | 35.68(28.54%) 16.7% | 35.68(28.54%) 22.16% | 53.53(42.82%) 33.8% | 35.68(28.54%) 17.8% |
| Note 1: TGPmin is calculated by setting TUE, Rx-Tx, TeNB, Tx-Rx, TUE, Tx-Rx, and TeNB, Rx-Tx to be the minimum values in Table 1. Note 2: the blue value denotes the GP overhead for the configured GP length, i.e., TGP/X., and the red value denotes the target GP overhead, i.e., TGPmin/X. |  |  |  |  |  |  |

From Table 3, it can be observed that the configured GP length (TGP) is much larger than the minimum GP length (TGPmin).

Observation 1: By keeping the symbols in the DL/UL transmission part being the same numerology, the configured GP results in much larger overhead than the minimum GP length.

# Methods of reducing guard period in NR

In this section, two proposed methods are introduced, which are capable of reducing the guard period in NR, with the proven evaluation results.

According to the above Possible working assumption, the configured GP in the time interval X can be denoted as,

TGP=Textra+TGPmin=Textra+maxTRTT+TUE, Rx-Tx, TeNB, Tx-Rx+max⁡(TUE, Tx-Rx, TeNB, Rx-Tx)

where Textra is the extra part with the value of TGP-TGPmin illustrated in Fig.2, which is unuseable for any purpose due to the requirement of the integer number of OFDM symbols in both DL and UL transmission parts. Among the components in the configured GP, the hardware-related parameters, i.e. TUE, Rx-Tx, TeNB, Tx-Rx, TUE, Tx-Rx, and TeNB, Rx-Tx, cannot be changed further once the hardware is determined. In such a circumstance, thus, we propose two methods, aiming at reducing the GP length by means of reducing Textra and TRTT, in the following sections.

Fig. 2 Components of guard period in time interval X

## 4.1 Method 1: Multiplexing OFDM symbols with different numerologies using TDM

This method focuses on reducing the length of Textra. As an example in Table 3, in indoor hotspot scenario, Textra in the configured GP for X=0.5ms is equal to 51.2833us. Although this length cannot accommodate an OFDM symbol with subcarrier spacing of 15KHz, it is sufficient to contain either an OFDM symbol with subcarrier spacing of 30KHz or two OFDM symbols with subcarrier spacing of 60KHz. In other words, the configured GP can be reduced by means of including single or multiple short OFDM symbols in DL/UL transmission part, as shown in Fig. 3.

Fig. 3 GP length reduction via adding one short OFDM symbol

Note that this method is aligned with the agreement in RAN1#85, i.e.,

- Forward compatibility of NR shall ensure smooth introduction of future services and features with no impact on the access of earlier services and UEs - Multiplexing different numerologies within a same NR carrier bandwidth (from the network perspective) is supported - FDM and/or TDM multiplexing can be considered

## 4.2 Method 2: UE-specific GP configuration

In this method, we give two options in consideration of the GP length reduction through reducing TRTT.

In configured GP (TGP), the parameter TRTT is used to determine TGPmin. TRTT should be calculated  according to the cell radius in order to guarantee that the GP in time interval X can be applied to all UEs served by an eNB. In other words, TRTT is associated with the RTT value of the farthest UE in the cell. In this sense, each UE in the cell can guarantee that its UL transmission reaches the eNB at the same time after finishing the DL reception. Fig. 4 (a) illustrates that the UL transmissions of both near UE and the farthest UE can reach the eNB at T1. This results in, however, that the near UE has a much larger unused part than the farthest UE (i.e., TUE, Rx-Tx) does. Thus, to reduce the unused part, we can consider some options for UE-specific GP, i.e.,

- Option 1 (UE-specific GP based on distance between UE and eNB): The GP setting depends on the distance between UE and eNB. Specifically, different UEs served by the same cell may have different TGPmin, i.e.,

Near UE: TGP1min=maxTRTT1+TUE, Rx-Tx, TeNB, Tx-Rx+max⁡(TUE, Tx-Rx, TeNB, Rx-Tx)

Far UE: TGP2min=maxTRTT2+TUE, Rx-Tx, TeNB, Tx-Rx+max⁡(TUE, Tx-Rx, TeNB, Rx-Tx)

where TGP1min≤TGP2min. Thus, the configured GPs for near UE and far UE (e.g., TGP1 and TGP2) can be different as long as TGP1≥TGP1min and TGP2≥TGP2min. In this sense, TGP1 could be smaller than TGP2. For example, as shown in Fig. 4 (b), the configured GP for the near UE is smaller than that for the farthest UE.

Fig. 4 GP setting without/with Method 2 (Option 1)

In Option1, we assume that the UE has both DL and UL transmissions in the same time interval. Thus, the consideration of RTT in TGPmin can guarantee that: 1) the UE has enough time to finish DL reception and 2) the UE can start UL transmission earlier than the beginning of UL reception at eNB side. However, in practice, the UE may have either only DL or only UL transmission in time interval X. For example, as shown in Fig. 5 (a), UE1 and UE2 have DL transmission and UL transmission, respectively, in time interval X. If the eNB performs the TX-to-RX switching at the illustrated timing, the eNB may have some unused part before switching from DL to UL. Thus, another option for UE-specific GP becomes necessary in order to reduce the unused part, i.e.,

- Option2 (UE-specific GP based on DL/UL transmission allocation of the UE): the GP setting depends on whether there are DL and UL transmissions for the same UE in the same time interval. Specifically, if the UEs in DL part of time interval X are totally different from those in UL part, the eNB can set TGPmin without consideration of RTT, i.e.,

TGPmin*=maxTUE, Rx-Tx, TeNB, Tx-Rx+max⁡(TUE, Tx-Rx, TeNB, Rx-Tx)

In this sense, the configured GP satisfying TGP≥TGPmin* may be smaller than that with consideration of RTT in TGPmin. As shown in Fig. 5(b), the UE1 can have more time duration for DL reception. In this case, UE2 can start it early to guarantee that the UL transmission of UE2 reaches at the eNB from the beginning of UL part. However, this method may introduce inter-UE interference (e.g., UE2 may cause interference to UE1, as shown in Fig.5(b)). It can be mitigated via the scheduling, e.g., the UE1 and UE2 are selected to have a large distance between them.

Fig. 5 GP setting without/with Method 2 (Option 2)

Therefore, with the abovementioned two options, the GP length can be reduced by multiplexing OFDM symbols with different numerologies so that TGP-TGPmin does not necessarily accommodate a full OFDM symbol.

## 4.3 Evaluation

We evaluate the two proposed methods. For method 2, the option 1 aims at reducing RTT value in TGPmin for different UEs while the option 2 targets at ignoring RTT in TGPmin. Thus, method 2 is evaluated based on option 2. Table 4 gives the evaluation results for various specified values of time interval X, e.g., X=0.5, 0.25, 0.125ms, where

- S1/ S2/ S3: the OFDM symbol length with short CP for sub-carrier spacing of 15/30/60KHz, i.e., 71.35/35.68/17.84us.

- TGP: the configured GP length without enhancement, i.e., TGP≥TGPmin and both DL and UL parts in time interval X contain an integer number of OFDM symbols with the same numerology

- TGP1: the configured GP length with method 1, i.e., TGP1≥TGPmin, and the DL/UL parts in time interval X may contain an integer number of OFDM symbols with different numerologies. For example, in indoor hotspot scenario, when X=0.5ms, TGP1is set to 1*S1-1*S2 or 1*S1-2*S3. Compared to TGP, method 1 reduces GP length by one S2 or two S3.

- TGP2: the configured GP length with method 2 (option 2), i.e., TGPmin is reduced to TGPmin*=maxTUE, Rx-Tx, TeNB, Tx-Rx+max⁡(TUE, Tx-Rx, TeNB, Rx-Tx), and TGP2≥TGPmin with an integer number of OFDM symbols with different numerologies in DL/UL part. For example, in rural scenario (ISD=5000m), when X=0.5ms, TGPmin is reduced to 20us, TGP2 is set to 1*S1-1*S2 or 1*S1-2*S3. Compared to TGP1, method 2 further reduces GP length by one S3.

Table 4 Evaluation results of reduced GP

| Scenario |  | Indoor hotspot | Dense urban | Rural |  | Urban macro |
| --- | --- | --- | --- | --- | --- | --- |
| ISD(m) |  | 20 | 200 | 1732 | 5000 | 500 |
| TGPmin (us) |  | 20.0667 | 20.889 | 27.7 | 42.23 | 22.22 |
| X=0.5ms | TGP | 1*S1(14.27%) | 1*S1(14.27%) | 1*S1(14.27%) | 1*S1(14.27%) | 1*S1(14.27%) |
|  | TGP1 (method 1) | 1*S1-1*S2 or 1*S1-2*S3 (7.13%) | 1*S1-1*S2 or 1*S1-2*S3 (7.13%) | 1*S1-1*S2 or 1*S1-2*S3 (7.13%) | 1*S1-1*S3 (10.7%) | 1*S1-1*S2 or 1*S1-2*S3 (7.13%) |
|  | TGP2 (method 2) | -- | -- | -- | 1*S1-1*S2 1*S1-2*S3 (7.13%) | -- |
|  | TGPmin/X | 4.01% | 4.18% | 5.54% | 8.45% | 4.44% |
| X=0.25ms | TGP | 1*S2 (14.27%) | 1*S2(14.27%) | 1*S2(14.27%) | 2*S2 (28.54%) | 1*S2(14.27%) |
|  | TGP1 (method 1) | -- | -- | -- | 2*S2-1*S3 (21.4%) | -- |
|  | TGP2 (method 2) | -- | -- | -- | 1*S2 (14.27%) | -- |
|  | TGPmin/X | 8.03% | 8.35% | 11.1% | 16.9% | 8.89% |
| X=0.125ms | TGP | 2*S3(28.54%) | 2*S3(28.54%) | 2*S3(28.54%) | 3*S3 (42.82%) | 2*S3(28.54%) |
|  | TGP2 (method 2) | -- | -- | -- | 2*S3 (28.54%) | -- |
|  | TGPmin/X | 16.1% | 16.7% | 22.16% | 33.8% | 17.8% |

Based on the results listed in Table 4, we have the observation as follows:

Observation 2: The GP length can be reduced by multiplexing OFDM symbols with different numerologies in the time-domain and supporting UE-specific GP setting.

Thus, we propose

Proposal: The GP length in time interval X can be reduced by considering the following two methods:

  - Multiplexing OFDM symbols with different numerologies in a TDM manner

  - UE-specific GP configuration

# 5. Conclusions

In this contribution, we analyse the GP setting in NR, and derive the possible working assumption for GP configuration as

Possible working assumption: the guard period in the time interval X should be larger than or equal to the minimum guard period value, i.e., TGP≥TGPmin, under the constraint that the DL/UL transmission contains an integer number of OFDM symbols.

As demonstrated by the evaluation results of the two proposed methods, the GP length can be further reduced. Thus, we propose:

Proposal: The GP length in time interval X can be reduced by considering the following two methods:

  - Multiplexing OFDM symbols with different numerologies in a TDM manner

  - UE-specific GP configuration

# References

[1] 3GPP TS36.211 v13.2.0.

[2] R1-081563, “On UL/DL frame timing for TDD”, RAN1#52bis, Mar. 2008.

[3] R1-081181, “Response to LS on switch time requirements for LTE TDD”, RAN1#52bis, Mar. 2008.