3GPP TSG RAN WG1 Meeting #86	 	R1-166961

Gothenburg, Sweden, 22-26 August 2016

Source: 	Ericsson

Title:	On the Need for V2V Gaps

Agenda Item:	7.2.2.6

Document for:	Discussion and Decision

# Proposal for V2V Gaps Standardization

During RAN1#84bis there was some attempt of discussion around the topic of V2V gaps [1].

Agreement:

- The followings are supported for the purpose of coexistence between PC5-based V2V and WAN:

  - Sidelink open loop power control is re-used for SL TX for V2V

    - FFS RSRP based resource selection

  - SL TX for V2V can be prioritized over WAN TX

    - FFS the details (e.g., applicability to Mode 1 and/or Mode 2, etc), especially whether existing D2D mechanism can be reused,

    - The prioritization is managable by eNB. Details FFS.

  - The same receiver capability of D2D communication UEs is assumed for V2V UEs. That is, a Rx chain is available at all time to receive V2V signals without affecting WAN reception (from RAN1 perspective) when the UE is configured to receive V2V.

Essentially, the idea is that at least in some case SL TX for V2V is prioritized over WAN TX, under eNB control.

We first observe that, from a UE RF implementation perspective, the complexity and cost saving enabled by optimizing tx chain reuse is very limited. Indeed the PA and most of the analog front end can unlikely be shared between a 5.9GHz carrier and a lower frequency carrier typically used for WAN. It is also debatable whether the potential cost saving by sharing some baseband resources would justify the additional implementation cost for optimized sharing, especially for a first release of products.

We also observe that the WID completion timeline is approaching and the remaining number of critical unresolved issues is large. This is even more critical in RAN2 where RRC completion is urgent.

We thus propose that V2V gaps, if configured by eNB, are only used for mode-1. Additional optimizations for mode-2 may be considered for a future release in case of interest.

Observations:

- The complexity and cost saving enabled by optimizing tx chain reuse between V2V at 5.9GHz and WAN is very limited.

- The WID completion timeline is reached and the remaining number of critical unresolved issues is large.

Proposals:

- V2V gaps, if configured by eNB, are only used for mode-1.

  - Additional optimizations for mode-2 may be considered for a future release in case of interest.

# References

- RAN1#84bis, Chairman’s notes