# 3GPP TSG RAN WG1 Meeting #86	 R1- 167808

# Gothenburg, Sweden

# 22nd - 26th August 2016

Source:                    	CEWiT, IIT-H, IIT-M, Reliance Jio, Tejas Networks

Title:  	flexi-OFDMA, a new waveform for NR

Document for:        	Discussion

Agenda Item:         	8.1.2.1

# Introduction

The Study on New Radio Access Technology [1] aims to develop a NR access technology to meet a broad range of use cases including enhanced mobile broadband, massive MTC, critical MTC, and additional requirements defined during the RAN requirements study. It is suggested to study a waveform based on OFDM, with potential support of non-orthogonal waveform and multiple access. Further SID urges to study and identify technical features necessary for enabling new radio access including efficient multiplexing of traffic for different services and use cases on the same contiguous block of spectrum.

RAN WG1 meeting #84bis agreed on

- Waveform is based on OFDM

  - Multiple numerologies are supported

  - Additional functionality on top of OFDM such as DFT-S-OFDM, and/or variants of DFT-S-OFDM, and/or filtering/windowing, and/or OTFS is further considered

    - Complementary non-OFDM based waveform is not precluded for some specific use cases (e.g., mMTC use case)

Further, in 3GPP RAN WG1 #85 meeting, agreed [2] to OFDM-based waveforms OFDM with CP and DFT-s-OFDM with CP should be used as RAN1 NR waveform performance reference:

This contribution proposes a new waveform namely flexi-OFDM for the New Radio, which is a derivative of OFDM. It also discusses the advantages of flexi-OFDM over filtered / windowed OFDM.

# Overview

For the design of the waveform for New Radio, different requirements and design principles have been put forth in the previous meetings by various companies [3][4]. The basic requirements of the NRAT includes

- Reduced Out Of Band Emission (OOBE) to adjacent numerologies in the same band

- Reduced PAPR for better Power Amplifier performance

- Flexible CP to reduce CP overhead

- Dynamic scheduling of different numerologies

- Grant-free asynchronous transmission

# Flexi OFDM

Fig. 1 Flexi-OFDMA Transceiver Chain

The flexi-OFDMA is a waveform where the data over each stream with different rates is transmitted over different bands with different subcarrier spacing, still maintaining orthogonality. The transceiver chain of the flexi-OFDMA is shown in the Fig. 1. The flexi-OFDMA waveform works by dividing the carrier bandwidth into N subbands and transmission in each subband is performed as OFDM/OFDMA using a small IFFT in the transmitter and FFT in the receiver, respectively. Defining Xm as the input signal at the mth subband sml as the lth OFDM symbol in the mth band, the output of stage 1 IFFT can be written as shown below:

sml=FMm†Xml-1Mm:lMm-1=FMm†Xml

where FMm† represents an Mm-pt IDFT matrix. To proceed to stage 2, assume that Mmax is the maximum IFFT size required of all the subbands and any Mm is a divisor of Mmax. Now s is defined as a block of size N×Mmax over which stage 2 IFFT operates Mmax times. With symbol boundaries aligned across all subbands, s and x over the observation window T can be represented as shown below:

s=s11⋯s1MmaxM1s21⋯s2MmaxM2⋮sN1⋯sNMmaxMN , and

x=FN†s

where FN† represents an N-pt IDFT matrix. If Mm<Mmax in the mth subband, the mth row of the block s will have Mmax/Mm OFDM symbols. After parallel to serial conversion, the transmission vector x is passed through a multipath channel h. With appropriate equalization at the receiver to counter the multipath, the received signal at stage 1 of the receiver can be written as shown below:

y=Hs+n

where

s=s11⋯s1MmaxM1 ,s21⋯sNMmaxMN and

H=H(1)IMmax00H(2)IMmax⋯00⋮⋮00⋯⋱00H(N)IMmax

with H=FNh and H(i) represents the ith element of H. After the N-pt FFT, the estimates of the transmit signal Xml with frequency domain equalization (FDE) either in stage 2 / stage 1 can be written as shown below:

Xml=WmH(m)Xml+nm

where Wm is the single tap equalizer for the mth subband. The equalizer Wm can be based on MMSE/MRC as in the existing LTE systems.

# flexi-OFDMA for multiple numerologies

Fig. 2  flexi-OFDMA with 2 numerologies

The main feature of flexi-OFDMA lies in its flexibility to multiplex different numerologies without the loss of orthogonality.  Fig. 2 depicts a 10MHz system where half the bandwidth is given to a numerology with 15KHz subcarrier spacing and the remaining half to a numerology with 30KHz subcarrier spacing. Considering stage-2 transformation as 64-pt IFFT, the stage-1 is designed as 16-pt IFFT for the first 32 subbands and 8-pt for the remaining 32 subbands. Now for every 16-pt IFFT transformation, two 8-pt IFFT transformations will be performed over a given time duration T. Assuming proper channel equalization, the receiver performs the inverse transformation to get back the data without any inter numerology interference.

# Block CP for flexi-OFDMA

Fig 3. Block CP addition

To maintain orthogonality in flexi-OFDMA, the transceiver chain in Fig .1 assumes CP added every stage 2 sub-symbol. This creates high CP overhead and hence a block CP (BCP) scheme is proposed for flexi-OFDMA. Here CP is added over a block of sub-symbols or symbols.

Fig 3. flexi-OFDMA with Block CP

The new transceiver chain with BCP is shown in Fig.4.  In the receiver, for BCP, frequency domain channel equalization is done using a bigger point FFT-IFFT pair followed by the flexi-OFDMA receiver transformation.

# Advantages of Flexi OFDM

The flexi-OFDMA waveform has a lot of advantages over the existing waveform candidates and the baseline waveforms. This has been described in the below table.

| Property | OFDM | w-OFDM /WOLA | f-OFDM | flexi-OFDM | flexi-OFDM with BCP |
| --- | --- | --- | --- | --- | --- |
| Time Orthogonality | Yes | No | No | Yes | Yes |
| Frequency Orthogonality | No | No | Quasi | Yes | Yes |
| Computational Complexity | Medium | Medium | High | Low | Medium |
| Additional CP Overhead | Nil | Low | Nil | High | Nil |
| Additional Guard Band | No | No | Yes | No | No |
| Guard Period | No | Yes | No | No | No |

# Performance Evaluation

Fig 4. Case 1a & 1b in AWGN channel

Fig 5. Case 2 & 4 in AWGN channel with 0KHz guardband

The major advantage of flexi-OFDM is clearly reflected in Fig.5 where two numerologies are simulated with 0KHz guard band in downlink and uplink scenario. As seen in the figure, OFDM has a loss compared to flexi-OFDMA as there is severe inter numerology interference due to lack of guard band/filtering/windowing. Still flexi-OFDMA performs better as orthogonality across numerologies is still maintained.

# Conclusions

Based on the descriptions and analysis above, the following proposals are made

Proposal 1: To consider flexi-OFDMA as a candidate waveform for NR and evaluate.

Proposal 2: To consider block CP based CP addition for flexi-OFDMA.

# References

- RP-161214 - Study on New Radio (NR) Access Technology

- Draft Minutes reportRAN1#85 v010

- R1- 162151 - 5G Waveform: Requirements and Design Principles, Huawei, HiSilicon

- R1-162198 - Waveform Requirements, Qualcomm Incorporated