3GPP TSG RAN WG1 Meeting #86	        		              R1-167905

Gothenburg, Sweden, 22rd – 26th August 2016

Agenda item:	8.1.2

Source: 	National Instruments

Title: 	Generalized Root Raised Cosine (GRRC) Nyquist Filters for Filtered    Modulation

Document for:	Discussion and Decision

# 1 Introduction

The 5G Study Item regarding the “Study on New Radio Access Technology” was approved at the 3GPP TSG RAN#71 meeting. It has the objective to be able to operate from sub 1 GHz to 100 GHz in a large variety of deployment scenarios in a single technical framework [1].  Filtering is expected to be an integral part of New Radio waveforms. For waveforms above 6 GHz, single carrier modulation will require pulse shaping filters. In RAN1#85, such RRC filters were proposed for Nyquist filtering in single carrier modulation [2]. Also time and/or frequency domain filtering has been proposed on a sub-band level to multiplex different numerologies inside one carrier as it can increase the isolation between the sub-bands [3, 4].

RRC filters are often used in communications systems to communicate data with optimal SNR.  Generalized Root Raised Cosine (GRRC) filters are motivated by an understanding that the second derivative of the raised cosine function is discontinuous in the frequency domain and that the first derivative of the square root raised cosine is discontinuous in the frequency domain.  This results in well-known poor adjacent channel performance and the need for subsequent filtering down the processing chain. This paper demonstrates that it is possible to jointly improve time domain and frequency domain performance of the RRC filter while maintaining a low PAPR.

# 2 Generalized Raised Cosine

We generalize the definition of raised-cosine filters starting with additional smoothness conditions in frequency space. Smoothness in frequency space translates into better decay behavior in time.

## 2.1 Frequency Domain

Let T be the symbol period. The well-known frequency behavior H(f) of the raised-cosine filter is defined by:

- Hf=T,  &|f|≤1-β2TT21+cosπTβf-1-β2T,1-β2T<|f|≤1+β2T0, else

The degree of smoothness of H(f) has an impact on the behavior of h(t). Smoothness of H(f) depends entirely on the behavior of H(f) at the critical points f=1±β2T . In the vicinity of any other frequency the function H(f) is analytical in f.

Derivatives of even order starting with the second derivative are not continuous everywhere. Given the structure of H(f), the values of a potentially continuous derivatives of H(f) must vanish at f=1±β2T .

The odd-symmetry of H(f) in the vicinity of f=±12T resulting from the expression

- Hf=T21+cosπTβf-1-β2T

can be maintained by using the following more general model:

- H( f)=T2A0+A1cos1πTβf-1-β2T+A3cos3πTβf-1-β2T+…+A2n+1cos(2n+1)πTβf-1-β2T for 1-β2T<|f|≤1+β2T

Also, Hf=T for f≤1-β2Tand Hf=0 else.

The choice n=1 in conjunction with A0=A1=1  recovers the raised cosine filter.

For the general case the following conditions must be satisfied where 2m is the order of the highest even derivative of interest:

H12T=T2
H1-β2T=T and H1+β2T=0 
H11-β2T=0 and H11+β2T=0  
… 
H(2m)1-β2T=0 and H(2m)1+β2T=0

The first equation translates into

- T2A0=T2 i.e.  A0=1

For f=1-β2T we have  H f=T21+A1+A3+…+A2n+1=T while f=1+β2T results in H f=T21-A1-A3-…-A2n+1=0. This is just one condition only:

- A1+A3+…+A2n+1=1

As in the case of raised-cosine filters, all odd derivatives of H vanish at frequencies f with f=1±β2T. The two conditions for each even derivative 2k are redundant and generate the following equation:

- 12kA1+32kA3+…+(2n+1)2kA2n+1=0

The complete system of conditions can be expressed in matrix notation.

- MnA1A3⋮A2n+1=11…11232…(2n+1)2⋮⋮⋮⋮12n32n…(2n+1)2nA1A3⋮A2n+1=10⋮0

This linear system has a unique solution for any n.

The matrix Mn for any n is a square Vandermonde matrix and both the determinant and the inverse have closed form solutions. This means that the solution vector A1A3…A2n+1Talso has a closed-form solution.

Example 1: n=1:

The matrix in (7) reduces to the identical 1 scalar and the unique solution is A1=1. The raised-cosine filter is recovered again. It is well-known that the factorization

- H( f)=T21+cos1πTβf-1-β2T=Tcos2πT2βf-1-β2T

is one of the most useful features of the raised-cosine filter as it allows the construction of the root raised-cosine matched filter.

Example 2: n=2:

This is the first nontrivial situation. Solving the linear equations (7) result in A1=98 and A3=-18.

- H( f)=T21+98cos1πTβf-1-β2T-18cos3πTβf-1-β2T

Figure 1 demonstrates that the generalized raised cosine function in frequency space has a second continuous derivate while the original raised cosine function has a discontinuous second derivative.

Fig. 1: Frequency behavior of RC (n=1) vs. generalized RC (n=2) for the first two derivatives.

Example 3: n=3:

Solving the linear equations (7) lead to A1=7564, A3=-25128, A5=3128.

- H( f)=T21+7564cos1πTβf-1-β2T-25128cos3πTβf-1-β2T+3128cos5πTβf-1-β2T

The raised cosine functions for n=1, 2, 3 can be regarded as the first members of an infinite family of generalized raised cosine functions and it is a natural idea to employ linear combinations of those functions for their ISI and OOB and PAPR behavior.  To guarantee the desired H(f) behavior only linear combinations with non-negative coefficients sum up to unity are considered.

- Hf=α1H1f+α2H2f+…+αmHmf, αk≥0,α1+α2+…+αm=1

In this way, the GRRC filter can be parameterized and optimized for ISI. The last equation is required as H(0)=1.  Also, we choose positive α1, α2, …, αm to guarantee the non-negativity of H(f). Based on such a model for an individual choice of D/2 and fixed choice of beta an optimal (minimize ISI!) set of parameters α1, α2, …, αm can be determined. Those sets of α1, α2, …, αm will vary with beta.  We choose in this paper to fix the α1, α2, …, αm parameters for each β and let D vary freely.

## 2.2 Time Domain

The expression for the root GRC Filter arises from numerically evaluating the inverse fourier transform of Hf.

- ht=2T01-β2THfcos2πtfdf

# 3 Performance Metrics

The performance of the filters will be evaluated against the following criteria.  The computational complexity must be limited at both the transmitter and receiver, another way of saying, the number of taps must be limited.  Out of band emissions in the factored form must be measured at the transmitter and out of band rejection must be measured at the receiver.  These are not necessarily the same numbers in the generalized case.  Following that, the error in the zero crossings of the composite transmit/receive filter must be measured.

The filter is truncated:

- ht, t ∈-∞,∞≫hDt,t∈-D,D

where D is the filter delay.

The first objective function is the OOB emissions/rejection. It can be measured as follows:

- E=10log101+β2T∞HD2ωdω01+β2THD2ωdω

If we call htt the truncated transmit filter and hrt the truncated receive filter, the composite transmit/receive response, hct, the composite filter response, can be expressed as the convolution of the transmit and receive filters.

- hct=-DDhrt'htt-t'dt'

The second objective function is the intersymbol interference (ISI) introduced by the composite filter.  It can be expressed as follows:

- ISI= n≠0hc2nThc20

Where T is the symbol period.  The results for a given filter are presented as a curve of ISI vs. OOB for a given D with a suite of curves for various D’s.

Figure 2 displays the ISI and OOB behavior for the original raised cosine filter. D varies from 1 to 16 and beta covers the typical range of 0.2 to 0.3. All ISI and OOB values are displayed in dB.

Fig. 2: ISI and OOB for raised cosine.

Figure 3 shows the generalized raised cosine for n=2. The ISI landscape is smoother for n=2 and the ISI gains are significant for most situations as long as D/2>7.

Fig. 3: ISI and OOB for the generalized raised cosine with n=2.

Figure 4 compares ISI and OOB values for the case n=3. The ISI landscape for n=3 is even smoother than the plot for n=2 and the ISI gains are significant for most situations as long as D/2>7.

Fig. 4: ISI and OOB and comparisons against n=1 for the generalized raised cosine with n=3.

Figure 5 demonstrates that optimal sets of α1, α2, …, αm for the case m=3 further reduce the ISI values of Figures 3 and 4.

For linear combinations of smaller numbers of generalized raised cosine functions exhaustive search is applied. For example, for linear combinations of three components this is technically done by listing all discrete α1, α2, α3 such that each parameter is chosen as αk=iN, i=0,…,N. Only combinations that satisfy α1+α2+α3=1 are valid. For larger numbers of components optimization routines must be deployed.

The question arises whether a combination of α1, α2, …, αm can be found that performs well for all D for a given beta. As Figure 6 demonstrates, such combinations do exist.

Fig. 5: ISI and OOB behavior of raised cosine (n=1), generalizations (n=2, n=3), and special linear combination (0.05, 0.65, 0.3) along D/2 for beta=0.25.

Time Domain Peaking Characteristics:

| Waveform | 99% CDF PAR | 99.9% CDF PAR |
| --- | --- | --- |
| RRC, QPSK | 3.22dB | 3.79dB |
| GRRC, QPSK | 3.31dB | 4.01dB |

In the case of GRRC special linear combination (0.05, 0.65, 0.3)  and RRC for beta = 0.25, the PAR increased by 0.09 dB for the 99% CDF PAR level and for the 99.9%, it increased by .22dB.

# 4 Observations and Proposals

For the particular example of (0.05,0.65,0.3) and a delay of 8 symbols, we can see that the ISI is improved by around 12dB, OOB by around 18dB at the expense of around 0.25 dB in baseline PAPR.  It is our expectation that PAPR will be brought inline with RRC PAPR or better with peak limiting in the loop.  OOB emissions will not be affected by peak limiting.

It is too early in the standardization process for the exact coefficients to be determined because there are many unknowns at this point in the above 40GHz regime.  This filter affords much potential for relaxing computational constraints due its short filter impulse responses while optimizing ISI, OOB and PAPR.

Proposal:  Consider the GRRC filter for evaluation in >40GHz NR waveforms.

# 5 References

[1] RP-160671, ”New SID Proposal: Study on New Radio Access Technology”, NTT DoCoMo.

[2] R1-165015, “Waveform proposal for carrier frequencies beyond 40 GHz”,  Nanjing, May 2016, Nokia, Alcatel-Lucent Shanghai Bell.

[3] R1-165011, “OFDM based Waveform for 5G new radio interface”, Nanjing, May 2016, Nokia, Alcatel-Lucent Shanghai Bell.

[4] R1-165261, “Initial downlink performance results for the New Radio waveforms below 6 GHz’’, Nokia, Alcatel-Lucent Shanghai Bell.