3GPP TSG RAN WG1 #86 bis	R1-1609083

# Lisbon, Portugal, 10th – 14th October 2016

Agenda item:	8.1.4.1

Source: 	Samsung

Title: 	Discussions on beam switching

Document for:	Discussion

Introduction

In RAN1#86, the following agreement is captured to define and study beam related procedures, in [1].

RAN1 Agreements: - Beam management = a set of L1/L2 procedures to acquire and maintain a set of TRP(s) and/or UE beams that can be used for DL and UL transmission/reception, which include at least following aspects: - Beam determination= for TRP(s) or UE to select of its own Tx/Rx beam(s). - Beam measurement = for TRP(s) or UE to measure characteristics of received beamformed signals - Beam reporting = for UE to report information of beamformed signal(s) based on beam measurement - Beam sweeping = operation of covering a spatial area, with beams transmitted and/or received during a time interval in a predetermined way.

During RAN1#86, RAN1 also discussed and agreed on the following DL beam management procedures [2].

RAN1 Agreements: - The following DL L1/L2 beam management procedures are supported within one or multiple TRPs: - P-1: is used to enable UE measurement on different TRP Tx beams to support selection of TRP Tx beams/UE Rx beam(s) - For beamforming at TRP, it typically includes a intra/inter-TRP Tx beam sweep from a set of different beams - For beamforming at UE, it typically includes a UE Rx beam sweep from a set of different beams - FFS: TRP Tx beam and UE Rx beam can be determined jointly or sequentially - P-2: is used to enable UE measurement on different TRP Tx beams to possibly change inter/intra-TRP Tx beam(s) - From a possibly smaller set of beams for beam refinement than in P-1 - Note: P-2 can be a special case of P-1 - P-3: is used to enable UE measurement on the same TRP Tx beam to change UE Rx beam in the case UE uses beamforming - Strive for the same procedure design for Intra-TRP and inter-TRP beam management - Note: UE may not know whether it is intra-TRP or inter TRP beam - Note: Procedures P-2&P-3 can be performed jointly and/or multiple times to achieve e.g. TRP Tx/UE Rx beam change simultaneously - Note: Procedures P-3 may or may not have physical layer procedure spec. impact - Support managing multiple Tx/Rx beam pairs for a UE - Note: Assistance information from another carrier can be studied in beam management procedures - Note that above procedure can be applied to any frequency band - Note that above procedure can be used in single/multiple beam(s) per TRP - Note: multi/single beam based initial access and mobility treated within a separate RAN1 agenda item

Regarding use cases of beamforming procedures, RAN1 discussed [3] and captured the following agreements during RAN1#85:

RAN1 Agreements: • Both intra-TRP and inter-TRP beamforming procedures are considered. • Beamforming procedures are considered with/without TRP beamforming/beam sweeping and with/without UE beamforming/beam sweeping, according to the following potential use cases: o UE movement, UE rotation, beam blocking: Change of beam at TRP, same beam at UE Same beam at TRP, change of beam at UE Change of beam at TRP, change of beam at UE o Other cases are not precluded

In RAN2 #95, the following note is captured in [4]. RAN 1 should start a discussion about beam related management if it is necessary.

Including study of RAN2 aspects of 'beam' level mobility, terminology, beam management, measurements for 'beam' level mobility, etc. May need to be treated with lower priority until further progress has been made by RAN1.

In millimetre wave systems, the beam-formed transmission has been considered one of basic principles to overcome severe path loss [2]. For a beam-formed transmission, beam management between gNB and UE should be necessary for a reliable transmission and reception. In this contribution, we suggest to discuss beam management, especially focusing on beam switching in multi-beam systems.

Beam switching

In multi-beam systems, a beam determination based on beam measurement/report is performed to maximize the directional antenna gain. After beam determination, a reliable transmission and reception is supported between TRP(s) and UE. However, if the channel between TRP(s) and UE is changed due to the channel fluctuation and UE rotation/mobility, etc., the received beam quality may be decreased. Conversely, the received beam quality from another beam pair becomes better than the current beam pair. In this case, TRP(s) and UE need to change from the current beam pair to better beam pair.

Additionally, TRP(s) or UE may need to switch its own beam according gNB’s scheduling policy. For example, TRP(s) cannot support all direction for receiving UL small data packets from multiple UEs due to the limited number of antenna ports. In this case, one of solutions may be to use a common TRP(s)’ beam for receiving UL data packets from multiple UEs even if the common TRP(s)’ beam is not the best one for some UEs.

Therefore, TRP(s) and UE should be able to track and switch the beam accordingly to maintain reliable transmission/reception and to support beam scheduling flexibility.

Proposal 1: Beam switching definition should be added like below :

- Beam management …

  - …

  - Beam switching = for TRP(s) or UE to change its own Tx/Rx beam(s), and to deliver beam change information to UE or TRP(s) if necessary, based on beam measurement, reporting and determination.

Regarding an overall beam switching procedure, following mechanism should be considered at least :

- Triggering of a beam switching by gNB and/or UE

- Tx and/or Rx beam ID indication

Proposal 2: The beam switching should be studied considering triggering and beam ID indication mechanism.

Conclusions

In this contribution, Samsung’s view of beam switching is presented. Based on above discussions, following proposals are made:

Proposal 1: Beam switching definition should be added like below :

- Beam management …

  - …

  - Beam switching = for TRP(s) or UE to change its own Tx/Rx beam(s), and to deliver beam change information to UE or TRP(s) if necessary, based on beam measurement, reporting and determination.

Proposal 2: The beam switching should be studied considering triggering and beam ID indication mechanism.

- References

- R1-168468, Definitions supporting beam related procedures, Nokia, Qualcomm, CATT, Intel, NTT DoCoMo, Mediatek, Ericsson, ASB, Samsung, LG

- 3GPP, RAN1#86, Chairman’s Notes

[3] R1-165684, WF on beamforming procedures, Nokia

[4] 3GPP, RAN2 #95, Draft Report