3GPP TSG-RAN WG1 #86bis  													                           R1-1609464

Lisbon, Portugal, 10th – 14th Oct 2016

Source: 	Intel Corporation

Title:	Discussion of advanced CSI feedback

Agenda item:	    7.2.2.1.1

Document for:	Discussion and Decision

# Introduction

In RAN1 #86, the following agreement about advanced CSI reporting to improve the performance of FD-MIMO was achieved [1]:

- Specify CSI feedback enhancement with the following advanced CSI feedback framework:

  - Reduced space (eigenvectors)/W1 is constructed based on one of the following alternatives (TBD RAN1#86bis):

    - Alt1. Orthogonal basis (e.g. orthogonal DFT matrix)

    - Alt2. Non-orthogonal basis (e.g. Rel.13 Class A W1 for rank-1 and/or 2)

  - Reduced space representation/W2 is to further combine selected beams

  - Granularity of weighting(phase and/or amplitude) can be either wideband only or wideband/subband, and is constructed based on one of the following alternatives (TBD RAN1#86bis):

    - Alt1. Phase and amplitude

    - Alt2. Phase-only weighting

  - How the enhanced framework can be applicable for Class A and/or Class B eMIMO-Types is FFS

  - FFS: How to handle the relationship between advanced CSI feedback and legacy CSI feedback framework

In this contribution, more details about the W2 quantization is discussed, and different W1 schemes are compared.

# Explicit CSI report

# 2.1 The description of the proposed explicit CSI feedback

The FD-MIMO system can be described as:

y=HWx+n=HW1W2x+n                                                  (1)

where y, H, W, W1, W2, x, and n represent Nr×1 receiving vector, Nr×Nt channel matrix, Nt×Nl precoding matrix, Nt×Nd vitalization matrix, Nd×Nl precoding matrix, Nl×1 transmitting vector, and Nr×1 noise vector, respectively. The matrix dimension number Nr, Nt, Nd and Nl represent the number of receiving antennas, the number of transmitting antennas, the number of virtualized antenna ports and the number of layers, respectively.

Firstly, the virtualization matrix W1, containing the dominant signal subspaces of all UE signals, is calculated through Eigenvalue decomposition of composite channel covariance matrix. The covariance matrix is calculated according to estimated channel on sounding RS as:

W1=EigenVector(1NUEi=1NUEHi,ULHi,ULH)1:Nd                                                    (2)

where Hi,UL is Nt×Nr the estimated uplink channel of UEi, and NUE denotes number of UEs in the cell served by this given eNB. Then Nl columns of Eigen matrix of the composite channel covariance matrix, corresponding to Nl strongest Eigen values, are adopted as W2. The elements of W2 are quantized and reported by UE to improve the CQI measurement accuracy.

To further reduce the overhead and UE computation complexity, the full spatial dimension of channel can be approximately emulated by halved dimension with same polarization and a co-phasing angle. UE performs Eigenvalue decomposition on the halved-dimension covariance matrix, and utilizes the Nl/2 strongest Eigen vectors w' to construct the W2 as the equation (3). Then it can quantize w' by Givens Rotation or Householder transformation through either even or uneven granularity.

W2=12NdNlw'0ejφw'0    w'0-ejφw'0   w'1ejφw'1   w'1-ejφw'1……Nd×Nl                        (3)

# Discussion on W1 construction

In [4], the channel covariance matrix is projected to the dominant DFT domain with reduced dimension from Nt×Nt to Nt'×Nt', e.g. Nt=32, Nt'=4, and then UE report the indexes of the dominant DFT domain, and the quantized dimension-reduced channel covariance matrix. W1 can be approximately obtained by multiplying the Eigen vectors of dimension-reduced channel covariance matrix with the dominant DFT domain. Here, UE needs to perform the channel estimation of all antenna ports, which increase the UE side complexity, and hundreds of bits need to be reported, e.g. 116 bits for 6 bits quantization granularity.

Compared with the implicit CSI feedback mode supported in LTE system, the reporting overhead of explicit CSI may be higher and, therefore, should be carefully studied. Although the carrier frequency of downlink and uplink is different, they nearly experience same propagation environment, so their second-order statistic has high correlation. By utilizing the uplink sounding reference signal to derive the W1 the UE side complexity can be reduced, and the reporting overhead can be saved.  Additionally, the beam formed CSI-RS ports can be shared by multiple UEs, which can reduced CSI-RS overhead, compared with UE-specific beam formed CSI-RS transmission.

In order to evaluate the performance of the proposed algorithm, the system-level simulation is performed, where the simulation parameters are listed in the appendix. The FTP result is summarized in the Table 1 for λ = 2.8. For benchmarking implicit CSI reporting based 32 ports CLASS A codebook [3] with Config=1, O1=4 and O2=4 is adopted. In Table 1, reported W1 represent one algorithm to obtain W1 which need to report the indexes of the dominant DFT domain and the quantized dimension-reduced channel covariance matrix, instead of deriving by uplink sounding reference signal. Moreover, the halved dimension W2 means a way to construct W2 which can be derived by the Nl/2 strongest Eigen vectors that decomposed on the halved-dimension covariance matrix with same polarization.

From it can be seen that deriving the W1 based on SRS can achieve attractive performance gain, and very few significant performance can be further obtained by reporting the W1. (add explanation ports of W2)

Table 1: Gain over Rel.13 class A codebook feedback with λ = 2.8

| (N1, N2, O1, O2) = (4,4,4,4) | Edge SE | Mean SE |
| --- | --- | --- |
| Class A | 0% | 0% |
| W1 based on SRS, full-dimension W2, Nd=8 | 14.53% | 12.81% |
| Reported W1, halved-dimension W2, Nd=8 | 15.89% | 12.77% |
| Reported W1, full-dimensionW2,Nd=4 | 18.86% | 13.7% |

Proposal 1: To save UE side complexity, reduce uplink overhead and CSI-RS overhead, W1 is calculated based on the estimated channel through SRS.

# Discussion on W2 construction

After virtualization on W1, the larger number antenna elements are virtualized into the smaller antenna ports, e. g. 8 ports, and then UE can derive and report the channel state information based on beamforming the CSI-RS, which can maintain the complexity and overhead in irrespective to the number of antenna elements. Based on beam formed CSI-RS, the channel can be estimated more accurately.

# 2.3.1 Report overhead reduction

Theoretically, the Eigen vector corresponds to the best digital pre-coder and achieves the maximum performance. The antenna ports can be divided into two groups, where each group contains the antenna elements with the same polarization. The eNB can apply the same virtualization matrix on these two antenna port groups. At the UE side the W2 can be constructed based on the Eigen vectors of the halved antenna ports and one additional angle of co-phasing elements to decrease the reporting overhead.

To further reduce the reporting overhead, in [2], two quantization schemes based on Givens Rotation (GR) and Householder (HH) transformation are studied for element quantization. For Nd beam formed CSI-RS antenna ports and rank Nl, total Nl(2Nd-Nl-1) real elements are required to be reported, e.g. 26/14 real elements need to be quantized and reported for rank 2/rank 1 respectively based on 8 beam formed CSI-RS antenna ports. To further reduce the CSI reporting overhead, the uneven quantization can be considered. For example in Type-I approach different range of the elements for quantization can be utilized, as illustrated in the Figure 3 and Figure 4 of [2], where the elements with wider distribution range can be quantized with more bits, e.g. 4 bits, while the elements with smaller distribution range can be quantized with smaller number of bits, e.g. 3 bits. In this case the reporting overhead can be reduced to 97 bits and 52 bits for rank 2 and rank 1. On the other hand, in Type-II approach the Eigen vector corresponding to strong Eigen value are quantized by using more bits, e.g. 4 bits, while the Eigen vector corresponding to weak Eigen value by smaller number of bits, e.g. 3 bits. In this case the reporting overhead can be also reduced to 92 bits for rank 2.

Proposal 2: For beam formed CSI-RS ports, UE should explicitly report precoding matrix corresponding to the strongest Nl Eigen vectors of the beam formed channel covariance matrix, where Nl is the number of layers.

Proposal 3: To reduce the overhead, W2 can be reported by Eigen vectors of halved channel covariance matrix and the angle of co-phasing element. Also, uneven quantization for GR and HH quantization schemes can be utilized to further reduce the CSI reporting overhead.

# 2.3.2 The system-level evaluation results

From the presented result, it can be seen that the proposed explicit CSI reporting scheme can achieve better spectral efficiency comparing to the implicit CLASS A CSI reporting scheme. From Table 1, it can be seen that about 21% gain for 5% UPT for low load scenario, and the proposed uneven quantization can further reduce the overhead with negligible performance loss. The CSI reporting overhead of the proposed scheme is also listed, which is smaller than 100 bits, and can be supported by the existing CSI reporting modes.

Table 1: Gain over Rel.13 class A codebook feedback with λ = 1.8

| (N1, N2, O1, O2) = (4,4,4,4) | Edge SE | Mean SE | Overhead |
| --- | --- | --- | --- |
| Class A | 0% | 0% | 16 bits |
| Non-quantization, Nd=8 | 26.8% | 2.4% | +∞ |
| GR, Nd=8, even quantization, 4bits | 21.5% | 1.9% | ~83bits |
| GR, Nd=8 Type-I uneven quantization, 4 bits+3 bits | 21.1% | 1.7% | ~78bits |
| GR, Nd=8 Type-II uneven quantization, 4 bits+3 bits | 21.5% | 1.2% | ~80bits |
| GR, Nd=8, even quantization, 3bits | 19.3% | -0.7% | ~63bits |
| HH, Nd=8, even quantization, 4bits | 21.9% | 1.9% | ~83bits |
| HH, Nd=8 Type-I uneven quantization, 4 bits+3 bits | 16.0% | 0% | ~78bits |
| HH, Nd=8 Type-II uneven quantization, 4 bits+3 bits | 22.0% | 0.5% | ~80bits |
| HH, Nd=8, even quantization, 3bits | 17.7% | -2.3% | ~62bits |

As the load increase, more UEs becomes active at the same time, then the probability for MU scheduling is increased, owning to abundant MU pairing. The MU transmission mode requires more accurate CSI feedback to suppress the mutual interference. Table 2 illustrates that about 12% gain for 5% UPT and 10% gain can be achieved for average UPT over Class A FD-MIMO.

Table 2: Gain over Rel.13 class A codebook feedback with λ = 2.8

| (N1, N2, O1, O2) = (4,4,4,4) | Edge SE | Mean SE | Overhead |
| --- | --- | --- | --- |
| Class A | 0% | 0% | 16 bits |
| Non-quantization, Nd=8 | 14.53% | 12.81% | +∞ |
| GR, Nd=8, even quantization, 4bits | 12.84% | 10.50% | ~88bits |
| GR, Nd=8 type-I uneven quantization, 4 bits+3 bits | 3.72% | 10.97% | ~69bits |
| GR, Nd=8, even quantization, 3bits | 1.69% | 5.81% | ~62bits |
| GR, Reported W1, full-dimensionW2,Nd=4, even quantization, 4bits | 17.9% | 13.2% | ~111bits (W1 78bits, W2 33bits) |

# Summary

In this contribution, the explicit CSI feedback is proposed. The evaluation results are also provided showing the benefits comparing to implicit Class A schemes. Based on the discussion the following proposals were made:

Proposal 1: To save UE side complexity, reduce uplink overhead and CSI-RS overhead, W1 is calculated based on the estimated channel through SRS.

Proposal 2: For beam formed CSI-RS ports, UE should explicitly report precoding matrix corresponding to the strongest Nl Eigen vectors of the beam formed channel covariance matrix, where Nl is the number of layers.

Proposal 3: To reduce the overhead, W2 can be reported by Eigen vectors of halved channel covariance matrix and the angle of co-phasing element. Also, uneven quantization for GR and HH quantization schemes can be utilized to further reduce the CSI reporting overhead.

# References

[1] R1-165790, WF on the three “evaluate and, if needed, specify” Items	Nokia, Alcatel-Lucent Shanghai Bell, Samsung, CATT

[2] R1-164150	Explicit CSI reporting for FD-MIMO	Intel Corporation

[3] TS 36.213, Technical Specification Group Radio Access Network, Evolved Universal Terrestrial Radio Access (E-UTRA), Physical layer procedures (Release 13)

[4] R1-166274, Discussion on advanced CSI reporting and initial evaluation results, Qualcomm      Incorporated

Simulation assumption

| Parameter | Value |
| --- | --- |
| Scenario | UMa with 500 ISD and 2GHz |
| eNB Antenna Configuration | X-polarized: 45/-45 degrees |
|  | horizontal spacing; vertical spacing |
|  | 2D antenna pattern defined in TR36.873, (4,4,2) |
| UE Antenna Configuration | + polarized: 0/+90 degrees, 2Rxs, |
| UE Configurations | Speed: 3km/h |
|  | UE attachment: Based on RSRP from CRS port 0 |
|  | UE distribution: 80% indoor and 20% outdoor only distributed on floor |
| SRS Configuration | 2Tx at UE |
| System Bandwidth | 10MHz (50RBs) |
| Scheduler | PF |
| Number of UEs per cell | 10 |
| traffic model | FTP, Model 1, 0.5MByte |
| Transmit Mode | Dynamic SU/MU: rank adaptation Max of 2 paired UEs |
| Receiver | Ideal channel estimation |
|  | MMSE-IRC receiver |
| Hybrid ARQ | Maximum 4 transmissions |
| Feedback | PUSCH 3-1 |
|  | CQI and PMI reporting triggered per 10ms |
|  | Feedback delay is 10ms |
| Overhead | 3 symbols for DL CCHs, 2 CRS ports and DM-RS with 12 REs per PRB |