3GPP TSG RAN WG1 Meeting #86bis               	R1-1609575

Lisbon, Portugal 10th - 14th October 2016

Source:	Panasonic

Title: 	Discussion on Single level DCI and two-level DCI

Agenda Item:		7.2.10.2.1

Document for:		Discussion and Decision

Introduction

In RAN1#85, followings were conclusion for study.

Conclusion: - Further discussion during the WI phase (if WI is approved) regarding the single-level DCI vs. two-level DCI considering aspects such as overhead, complexity, potential scheduling restriction, search space design, the corresponding performance, impact of different TTI lengths (if any), etc. - Note: this conclusion is not be included in the TR

In this contribution, we discuss single level DCI /two-level DCI and sPDCCH monitoring configurations. This is update of R1-166966.

Discussion

Single level DCI /two-level DCI

At first, we compare single level DCI and two-level DCI. Within two-level DCI, we identify three alternatives. Following is some detailed analysis on each case,

Single level DCI

For single level DCI, every DCI contains full information and UE should monitor every search space on every sTTI. It is shown in Figure 1. The resource allocation is high flexibility. When multiple sTTIs are allocated to one UE, the DCI contents are duplicated and the overhead of DCI is high. Note that for 2 OFDM symbols, to reduce the overhead of sPDCCH is important for better throughput and latency [1][2].

Figure 1 single level DCI

Two level DCI Alt 1 (Group common slow DCI and UE specific fast DCI)

Group common slow DCI is realized by PDCCH, which is located in legacy PDCCH region. UE specific fast DCI is realized by sPDCCH, which is located in every sTTI. It is shown in Figure 2.

For example, DCI contents can be indicated following manner.

- Group common DCI indicates group common resource block assignment /RV and UE specific TPC command/SRS request CSI request.

- UE specific fast DCI indicates MCS, MIMO related information, HARQ process number and NDI. To realize some scheduling flexibility, it is also possible that UE specific fast DCI includes resource block assignment information but the range or size should be limited.

Figure 2 Two-level DCI Alt1

Two level DCI Alt 2 (UE specific slow DCI and UE specific fast DCI. Slow DCI in PDCCH region)

UE specific slow DCI is realized by PDCCH, which located in legacy PDCCH region. UE specific fast DCI in sPDCCH is realized by sPDCCH, which is located in every sTTI. It is shown in Figure 3.

For example, DCI contents can be indicated following manner.

- Slow DCI indicates Resource block assignment, MCS, SRS request, TPC command, MIMO related information, RV and CSI request. 
If sTTI#0 is allocated by slow DCI, HARQ process number and NDI for  sTTI#0 are also included.

- Fast DCI indicates HARQ process number and NDI.

For RV, if it is better to set different values for different sTTIs in a subframe, it should be indicated by Fast DCI. If a common RV value across all sTTIs in a subframes has no problem, it would be indicated by slow DCI.

In Figure 3, slow DCI for UE A, UE B and UE C in PDCCH indicate PRBs allocation for each UE. For UE A, slow DCI also includes information for fast DCI in sTTI#0. Then UE A receives sPDSCH (red) in sTTI#0. In sTTI#2, UE A and UE B detect fast DCI. Then UE A and UE B receive sPDSCH on PRBs (red for UE A and purple for UE B) which are allocated by slow DCI.

Figure 3 Two-level DCI Alt2

Two level DCI Alt 3 (UE specific slow DCI and UE specific fast DCI. Slow DCI in PDCCH and sPDCCH region)

It is shown in Figure 4. Slow DCI in sTTI#X include full DCI contents for sTTI#X. When UE detect slow DCI in sTTI#X, UE knows sPDSCH in sTTI#X is allocated.

For example, DCI contents can be indicated following manner.

- Slow DCI indicates Resource block assignment, MCS, SRS request, TPC command, MIMO related information, RV, CSI request, HARQ process number of first allocation and NDI for first allocation

- Fast DCI indicates HARQ process number and NDI.

In Figure 4, slow DCI for UE A on PDCCH region indicates PRBs allocation for UEA and it includes information for fast DCI in sTTI#0. Then UE A receives sPDSCH (red) in sTTI. When UE A detects fast DCI in sTTI#2, UE A receives sPDSCH (red) which are allocated by slow DCI. When UE B detects slow DCI in sTTI#2, UE B receives sPDSCH (purple) in sTTI#2.

Figure 4 Two-level DCI Alt3

The comparison of single level DCI and alternatives of two-level DCI is shown in Table 1.

Table 1 The comparison of single level DCI and two-level DCI

|  | Single-level DCI | Two- level Alt 1 | Two- level Alt 2 | Two- level Alt 3 |
| --- | --- | --- | --- | --- |
| Overhead in case multiple sTTIs are assigned to single UE | Large | Low | Larger than Alt1 | Larger than Alt1 |
| Flexibility of resource allocation | Full flexibility | In case of multiple UEs, several sTTI is blocked if there is no additional resource allocation bit in fast DCI. |  | Higher flexibility than Alt 2 |
| Latency of initial DL data transmission | Short delay | Short delay | Larger than the other option because to slow DCI is dedicated and located in PDCCH region. | Short delay |
| Blind decoding for USS | USS in PDCCH for 1st TTI and each sTTI SS | Group common SS and each sTTI SS | USS in PDCCH for slow DCI and each sTTI SS from second TTI | USS in PDCCH for slow DCI and each sTTI SS for slow or fast DCI |
| Power reduction of blind decoding |  | When UE doesn’t detect group common SS, UE can skip to monitor each sTTI SS | When UE doesn’t detect slow DCI in PDCCH, UE can skip to monitor each sTTI SS |  |

For 1 slot sTTI, the overhead reduction of two-level DCI has no gain. Therefore, single level DCI is preferable to support full flexibility of resource allocation.

For 2 OFDM symbols sTTI, two-level DCI has gain to reduce overhead. In Alt 2, latency is larger than the other alternatives because to slow DCI is dedicated and located in PDCCH region. Therefore the alt 1and 3 should be studied.

Proposal 1: Single level DCI should be used for 1slot sTTI. Two-level DCI should be used for 2 symbols sTTI.

sPDCCH monitoring configurations

In above two-level DCI alt1, sPDCCH regions can be allocated by group common DCI.  In other alternatives, for sTTI UE, sPDCCH regions should be indicated by UE specific RRC signalling like a EPDCCH/MPDCCH configuration or indicated by group common DCI like a LAA and eIMTA. If group common DCI indicates sPDCCH region in every subframe, UE can skip sPDCCH monitoring when the group DCI is not detected in PDCCH.

In addition, in order to protect several UL and DL subframes for other purposes (ICIC, CoMP, D2D or eMTC), valid subframe induction to sTTI UE may be considered. It is TDM manner based on a subframe. By so, the effect of latency reduction is limited. However there are several merits. When the valid subframe indication is introduced, the valid DL subframe and valid UL subframe don’t need to be aligned for the timing of sTTI. The timing of sPDCCH to sPUSCH and sPDSCH to sPUCCH can be adjusted without additional indication in DCI. In addition, the false alarm on sPDCCH can be reduced.

Proposal 2: Indication of valid/invalid subframe for sTTI should be introduced.

Conclusion

In this contribution, we discuss single level DCI /two-level DCI and sPDCCH monitoring configurations. We propose

Proposal 1: Single level DCI should be used for 1 slot sTTI. Two-level DCI should be used for 2 symbol sTTI.

Proposal 2: Indication of valid/invalid subframe for sTTI should be introduced.

- Reference

[1] R1-162536 “Evaluation results on shortened TTIs with dynamic overhead of control channel,” Panasonic

[2] R1-166967 “Evaluation results on processing time reduction in physical layer,” Panasonic