3GPP TSG-RAN WG1 #86bis	R1-1609653

Lisbon, Portugal, October 10th-14th, 2016

Agenda item:		8.1.1.2

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Preamble transmission Procedures for the mMTC uplink

Document for:		Discussion and Decision

# 1	Introduction

In the previous two meetings, the following agreements on grant-free multiple access were achieved:

Agreements: [RAN1 86 chairman’s notes]

- NR should target to support UL “autonomous/grant-free/contention based” at least for mMTC

- At least the following options for “autonomous/grant-free/contention based” UL transmission should be studied

  - Opt. 1: a UE performs random resource selection

    - Details FFS

  - Opt. 2: a UE’s resource is pre-configured by eNB or pre-determined

    - Details FFS

  - Other options are not precluded

Agreements: [RAN1 85 chairman’s notes]

- Autonomous/grant-free/contention based UL non-orthogonal multiple access has the following characteristics

  - A transmission from UE does not need the dynamic and explicit scheduling grant from BS

  - Multiple UEs can share the same time and frequency resources

In companion contributions [1][2][3], autonomous frequency hopping with and without code multiplexing of users has been proposed to achieve efficient, reliable, and scalable grant-free access. It has also been proposed that grant-free access should commence with a preamble sequence in order to avoid the complexity associated with blind decoding [4]. In this contribution, we discuss procedures that should be employed for preamble transmission in order to achieve improved detection/false alarm performance in extended coverage scenarios.

# 2	Discussion

In mMTC, it is well understood that in order to support the massive scale of MTC traffic, grant-free access may be required. Grant-free access implies that there is no scheduling grant for either the initial transmission or subsequent re-transmissions of a given packet.

One of the issues governing autonomous, grant-free access is that the resources over which transmissions occur must be clearly identifiable in order to avoid the complexity associated with blind decoding. For this purpose, it is beneficial to commence transmission with a preamble sequence that can be chosen at random by a mMTC user, and associated with a set of frequency hopped resources over which data transmissions occur. When reliably detected, a preamble transmission by a mMTC user has two primary benefits. First, the preamble can be used to further establish timing between the transmitter and receiver. Second, a preamble transmission informs the receiver that a payload is forthcoming (as in single stage approaches) on a specific set of resources where decoding can be carried out. Without the usage of preambles for grant-free access, the receiver complexity is significant as it must consider all possible transmission hypotheses. As a result, we have the following proposal:

Proposal 1: Preamble transmissions should be supported at the start of a grant-free access for mMTC.

For mMTC, it is desirable to provide extremely high service availability in potentially challenging, extended coverage deployment scenarios (e.g., basement, under a bridge). Exacerbating the issue, devices for mMTC may support lower powers than typical LTE terminals. Under these conditions, it is still important to achieve reliable detection with lower false alarm rates. High detection rates for preambles are essential for mMTC as missed detections will result in additional energy-consuming transmit operations by mMTC devices that desire long battery life. Low false alarm rates are essential to minimize BS receiver operations as each false alarm results in the BS needlessly trying to decode a transmission that is not present.

With regards to the use of a preamble, our performance analysis indicates that achieving reliable detection with fewer false alarms requires the transmission of narrowband preambles extended in time and the support of a retry mechanism enabled through the a preamble Ack.

# 3	Preamble Transmission for Grant-free UL mMTC With Extended Coverage

Preamble transmission for mMTC users may occur over random access opportunities within the preamble transmission zone. Each random access opportunity comprises a set of time/frequency resources. Note that different allocations are possible for each random access opportunity ranging from wideband preambles with shorter durations to narrowband preambles, extended in time (see Figure 1). Wideband preambles allow better timing resolution and lower latency but achieve worse detection/false alarm performance than narrowband preambles. Narrowband preambles allow higher significantly improved detection/false alarm performance in extended coverage scenarios at the cost of timing resolution and latency. The number of available preamble sequences with narrowband allocations is slightly less (~5% less) due to the guard band overhead. However, the penalty in terms of timing resolution, guard band overhead and latency are not considered to be significant for a wide range of use cases, and are far outweighed by the improvement in detection/false alarm performance.

- 1x6 allocation (narrowband, extended in time)

- 2x3 allocation

- 6x1 random access allocation (e.g. LTE)

t

t

t

f

f

f

Figure 1: Possible allocations for random access; here mxn allocations may correspond to m PRBs and n sub-frames as an example. The number of available preamble sequences is similar in the 3 cases.

1 PRB x 6 ms

1 PRB x 6 ms

1 PRB x 6 ms

6 PRBs x 1 ms

6 PRBs x 1 ms

Figure 2: Detection/False alarm performance for random access opportunities at different power settings with two different but equally sized allocations: (a) 6 PRBs x 1 ms; and (b) 1 PRB x 6 ms.

Figure 2 shows the detection/false alarm performance for an extended coverage scenario assuming 2 random access opportunities are allocated per 10 ms frame and Poisson arrivals of mMTC transactions giving rise to an average load of 500 attempts per second. Detailed system simulation assumptions are provided in the Appendix. Narrowband preambles, extended in time achieve significantly improved detection/false alarm performance. As a result, we have the following proposals:

Proposal 2: Random access opportunities spanning configurable time/frequency allocations should be supported for preamble transmission in mMTC.

Proposal 3: Narrowband preamble transmissions, extended in time should be supported for extended coverage with mMTC.

## Need for Preamble Ack and Retry Mechanism

With autonomous transmissions, it is important to consider whether the BS should send an acknowledgement (Ack) when a preamble is detected. The primary advantage of not transmitting preamble Ack is that it reduces the need to wait (latency) and decode (energy) the Ack/Nack response. Operating without preamble Acks can also help in reducing overhead on the downlink since the Ack must be transmitted reliably.  If the preamble can be designed to be reliably detected, there can be performance benefits of not having a preamble Ack response.

The key benefits of transmitting a preamble Ack are that missed detections do not result in needless payload transmissions and false alarms can be effectively filtered out by raising the acceptance threshold. If an mMTC UE which previously transmitted a preamble does not receive a preamble Ack, it knows that it needs to re-transmit the preamble, perhaps with some backoff.

Figure 3 illustrates the grant-free access procedure for mMTC UEs with and without a preamble retry mechanism. Without a preamble Ack/retry mechanism, the UE transmits a preamble and then follows it up with data transmission on a set of resources. Detection of the preamble provides an indication to the receiver that data decoding needs to commence on a set of resources over which the transmission occurs. When a preamble Ack/retry mechanism is supported, the UE transmits a preamble and then waits for an Ack. Upon receipt of an Ack, the UE starts data transmission; if an Ack is not received, the UE may retry sending a preamble as long it hasn’t exhausted the maximum number of retries.

(b) Autonomous Tx with Preamble Retry and Ack

- Autonomous Tx without Preamble Retry

BS

mMTC UE

BS

mMTC UE

Preamble

Preamble

Preamble Retry

Data (Initial Tx)

Preamble Ack

...

Retx

Retx

Data (Initial Tx)

Retx

Data Ack

Retx

...

Retx

Data Ack

Figure 3: Autonomous transmission procedure (grant-free access) with and without preamble retry mechanism.

Preamble Ack/Retry

No Preamble Ack/Retry

Preamble Ack/Retry

No Preamble Ack/Retry

Figure 4: Detection/false alarm performance with preamble Ack and Retry mechanism (Load: 500 attempts per second, Random access opportunities: 2 per 10 ms frame, Max number of Retries: 6).

Figure 4 shows that a preamble Ack and retry mechanism can have significant benefits in terms of detection/false alarm performance. With a maximum UE transmit power of 10 dBm, the UE is heavily coverage limited and the number of false alarms can be relatively high. At a detection rate of approximately 84%, the false alarm rate can be reduced by a factor of 3. However, with a maximum UE transmit power of 23 dBm, the number of false alarms can be reduced by a factor of 7 at a detection rate exceeding 96%. The Ack/retry approach has significant benefits for autonomous transmission since the receiver does not need to waste its valuable processing resources on attempting to decode false alarms. While the Ack/retry scheme has the potential to increase latency for mMTC UEs that are unsuccessful in the first attempt, the benefits achieved in detection/false alarm performance far outweigh any increase in latency thus making it attractive for many mMTC applications. As a result, we have the following proposal:

Proposal 4: The support of a Preamble Ack should be studied for grant-free access with mMTC, targeting UEs in extended coverage.

Proposal 5: The support of a Preamble retry mechanism should be studied for mMTC UEs that do not receive a Preamble Ack.

Tradeoffs related to latency with different preamble and data Ack transmission options are discussed in a companion contribution [4].

# 4	Conclusions

We have made the following proposals regarding preamble transmission for UL mMTC:

Proposal 1: Agree that preamble transmissions should be supported at the start of an autonomous transmission for mMTC.

Proposal 2: Agree that random access opportunities spanning configurable time/frequency allocations be supported for preamble transmission in mMTC.

Proposal 3: Agree that narrowband preamble transmissions, extended in time be supported for extended coverage with mMTC.

Proposal 4: The support of a Preamble Ack should be studied for grant-free access with mMTC.

Proposal 5: The support of a Preamble retry mechanism should be studied for mMTC UEs that do not receive a Preamble Ack.

# 5	References

- R1-167248, "Contention based non-orthogonal multiple access in with frequency hopping for the mMTC uplink", Nokia, Alcatel Shanghai Bell.

- R1-1609649, “Grant-free retransmission with diversity and combining for NR,” Nokia, Alcatel Shanghai Bell.

- R1-167252, “Basic principles of contention based access”, Nokia, Alcatel Shanghai Bell.

- R1-1609783, “Transmission Options for the mMTC grant-free uplink, Nokia, Alcatel Shanghai Bell.

Appendix

Simulation assumptions:

| Parameters | Values |
| --- | --- |
| Bandwidth Per PRB | 15 kHz * 12 = 180 kHz |
| TTI | 1ms |
| Cellular topology | 19 sites, 3 cloverleaf cells per site with wraparound, ISD=1732m |
| Carrier frequency | 700 MHz |
| BS antenna | 25 m height, 2 Rx, 17 dBi gain, 3 dB cable loss |
| UE antenna | 1.5m height, 1 Tx, -4 dBi gain |
| BS noise figure | 5 dB |
| Channel model | 3D UMa |
| Channel Model | TU3 |
| Preamble Sequences | ZC |
| Traffic model | Poisson arrivals |
| Load | 500 attempts per sec |
| Random Access Opportunities | 200 per second (spanning 6 PRBs x 1 ms and 1 PRB x 6 ms) |
| Maximum UE Power | mMTC: 23 dBm and 10 dBm |
| Open Loop Power control | mMTC: α=1, optimized P0 |
| UE distribution | Uniformly distributed over cell, 80% indoor |