3GPP TSG RAN WG1 Meeting #86bis 	R1-1609875

Lisbon, Portugal, 10th - 14th October 2016

Source:	Sharp

Title:	Numerology for DL synchronization signal in NR

Agenda Item:	8.1.5.1

Document for:	Discussion and Decision

Introduction

At the RAN1#85 meeting, the following agreements were made for numerology for downlink synchronization signal in NR [1].

Agreements: - For subcarrier spacing of each synchronization signal (e.g. NR PSS,SSS) in a NR carrier, the following alternatives should be studied - Alt 1: Subcarrier spacing is predefined in the specification for a given frequency range - Ex: 15kHz for sub-6GHz, 60kHz for over-6GHz - Note that there are more than one frequency ranges - Alt 2: Subcarrier spacing is selected by NR BS - FFS: Details on the set of possible numerologies - Note: Blind detection of multiple numerologies can be considered - Alt 3: Single subcarrier spacing is predefined in the specification for all frequency ranges - Other alternatives are not precluded - NR synchronization signal is based on CP-OFDM - Note that DFT-spread-OFDM based design is not precluded

In this contribution, we share our views on numerology for DL synchronization signal.

Discussions

Comparison of alternatives

In the last meeting, three alternatives were discussed.

- Alt. 1: Subcarrier spacing is predefined in the specification for a given frequency range

- Alt. 2: Subcarrier spacing is selected by NR BS

- Alt. 3: Single subcarrier spacing is predefined in the specification for all frequency ranges

Figure 1 shows an example of Alt. 1.

Figure 1. An example of Alt. 1.

In this figure, 15 kHz subcarrier spacing is used within frequency range in below 6 GHz and 60 kHz subcarrier spacing is used in above 6 GHz. In Alt. 1, because subcarrier spacing is predefined in the specification for a given frequency range, a UE can search synchronization signal by a predefined subcarrier spacing according to frequency band/range. This alternative means that one candidate on subcarrier spacing is used for cell search in a given frequency range and the UE does not need to blindly detect subcarrier spacing of synchronization signal(s).

Figure 2 shows an example of Alt. 2. This alternative

Figure 2. An example of Alt. 2.

In this figure, each base station can select 60 kHz or 120 kHz as a subcarrier spacing of synchronization signal(s) in above 6 GHz. This alternative means that one subcarrier spacing among candidates of supportable subcarrier spacing is selected by the base station for flexibility of NR operation. However, in this case, a UE should blindly detect applied subcarrier spacing from transmitted synchronization signal(s) including supported subcarrier spacing family. Therefore, if Alt. 2 is supported, the number of candidates of subcarrier spacing should be limited to 2 or 3 to reduce complexity.

Figure 3 shows an example of Alt. 3.

Figure 3. An example of Alt. 3.

In this figure, only 15 kHz subcarrier spacing is applied to synchronization signal(s) as an example. This alternative is the simplest solution and a UE can only search synchronization range with one subcarrier spacing in any frequency band/range. However, generally, wider subcarrier spacing is required to cope with frequency jitter in higher frequency range such as above 6 GHz. Therefore, this solution cannot be realistic. Therefore,

Proposal 1:

- Alt. 1 or Alt. 2 should be studied for numerology for DL synchronization signal

  - If Alt. 2 is applied, the number of candidates of subcarrier spacings should be limited to reduce complexity

Numerology of subcarrier spacing of synchronization signal

At RAN1#85 meeting, the following agreement was made:

Agreements: - For a NR carrier (from network perspective) using multiple numerologies, at least the following is for further study - multiple frequency/time portions using different numerologies share a synchronization signal - Note: The synchronization signal refers to the signal itself and the time-frequency resource used to transmit the synchronization signal

Moreover, reference subcarrier spacing will be defined for NR study as the following agreements in the last meeting:

Agreements: - Subframe duration in ms for a reference numerology with subcarrier spacing (2m*15)kHz is exactly 1/2m ms

These agreements imply that a time unit with a reference subcarrier spacing is the minimum set for time alignment in a given NR carrier in a certain frequency range. Therefore, RAN1 should discuss relationship between a reference subcarrier spacing and subcarrier spacing of synchronization signal(s)

Proposal 2:

- RAN1 should discuss relationship between reference numerology for frame structure and subcarrier spacing of synchronization signal(s)

Conclusion

In this contribution, we have the following proposals:

Proposal 1:

- Alt. 1 or Alt. 2 should be considered for numerology for DL synchronization signal

  - If Alt. 2 is applied, the number of candidates of subcarrier spacings should be limited to reduce complexity in blind detection

Proposal 2:

- RAN1 should discuss relationship between reference numerology for frame structure and subcarrier spacing of synchronization signal(s)

References

RAN1 chairman’s note in RAN1#86, August 2016.