3GPP TSG-RAN WG1 #88                                                                 					R1-1702203

Athens, Greece, 13th – 17th February, 2017

Source: 	Intel Corporation

Title:	On UL Beam Management

Agenda item:	    8.1.2.2.4

Document for:	Discussion and Decision

# Introduction

In RAN1 NR Ad-Hoc meeting, the SRS with beam sweeping has been agreed as follows:

- For NR UL, support transmissions of SRS precoded with same and different UE Tx beams within a time duration

  - Detailed FFS, including the resulting overhead, time duration (e.g., one slot), and configuration, e.g., in the following:

    - Different UE Tx beam: FFS per SRS resource and/or per SRS port

    - Same UE Tx beam across ports: for a given SRS resource and/or a set of SRS resources

    - FFS: The SRS resources can be mapped in TDM/FDM/CDM manner.

    - FFS: overhead reduction schemes such as IFDMA or larger subcarrier spacing

    - FFS gNB can indicate selected SRS port/resource for UE after receiving the SRS.

Further the UL beam management has been agreed to be the working assumption as follows:

- NR supports at least one NW-controlled mechanism for beam management for UL transmission(s)

  - Details are FFS, including at least the following study:

    - Signal(s) for the mechanism(s) if necessary

      - E.g., SRS, PRACH preamble, UL DMRS

      - Additional contents can also be included, e.g., beam reporting

    - Method(s) and content for TRP to indicate selected UE Tx beam and configure UE sweeping

    - Impact of beam correspondence Status

      - E.g., When to use the mechanism(s)

      - E.g., Procedures such as U-1, U-2, U-3, and beam correspondence based procedure

    - UE capability reporting

      - E.g., capability of analog beamforming

    - Consider the cases when UL and DL are from the same TRP and from different TRPs

    - Conditions when the mechanism is particularly useful

For the UL beam management, if reciprocity can be assumed for both TRP and UE side, the DL beam can be a reference to determine UL beam and dedicated UL beam management procedure, if defined, is used to accommodate network topology caused by beam asymmetry, e.g. different downlink and uplink beam. However if the full reciprocity cannot be assumed, the dedicated UL beam management procedures should be necessary. Thus the details UL beam management procedure U-1/U-2/U-3 should be considered to maintain a good UE-TRP beam pair. In this contribution, we provide our view on the UL beam management if the full reciprocity cannot be assumed. Be noted that the PRACH access discussed in this contribution is TRP level access and NR cell level access can be completely different discussion.

# Discussion

Necessity of UL Beam Management

The beam correspondence has been agreed to be the UE’s capability. For the UEs with beam correspondence, the UE could directly reuse the DL Rx beam to be the UL Tx beam. However for those without beam correspondence, some beam sweeping operation should be used to help the UE to determine the best UL Tx beam. Further in the gNB side, without beam correspondence, the gNB cannot easily determine which beam to be used to receive the UL signal. Hence the UL beam management should be supported for the case when partial or no beam correspondence can be guaranteed. Similar to the DL beam management, the UL beam management could also have 3 procedures: U-1, U-2 and U-3, and which procedure can be skipped should be determined by the beam correspondence.

For UL beam management procedure U-1, which is used to enable the TRP measurement on different UE Tx beams to support selection of UE Tx beams/TRP Rx beams, since the PRACH should be the first UL signal to access one TRP, to acquire a good UE-TRP beam pair, it can be done based on the PRACH. The PRACH sequence can be repeatedly transmitted so that the Rx beam sweeping can be supported. Denote those repeated PRACH sequences to be one PRACH group, different Tx beams can be applied to different PRACH groups. Then a good UE-TRP beam pair can be obtained after correct PRACH detection.

For UL beam management procedure U-2/U-3, since the UE has been connected, the SRS to support beam sweeping operation should be used. When current UE-TRP beam pair is not good, the TRP could request the UE to transmit SRS to find out a good UE-TRP beam pair.

Proposal 1: UL beam management should be supported when no full beam correspondence could be confirmed in both gNB and UE side. For U-1, PRACH with beam sweeping operation should be used; for U-2 and U-3, SRS to support beam sweeping can be used.

SRS structure to support beam sweeping

For UL BM U-2 and U-3, to re-use the PRACH structure in P-1 could require large overhead. Hence the SRS with some overhead reduction scheme can be one promising option. The SRS structure for UL beam management has been discussed in [2], where the IFDMA based or larger subcarrier spacing based reference signal can help to create multiple repetitions with limited overhead. The structures are shown in Figure A-1 and Figure A-2 in the Appendix. With the help of the IFDMA based SRS or larger subcarrier spacing based SRS, the UL beam management U-2 and U-3 can be done independently or jointly. To jointly refine the Tx and Rx beam, some repetitions may need to be grouped, which is similar as U-1. One repetition group can be used for the Rx beam refinement for one Tx beam. The number of repetitions within one repetition group should be known for both TRP and UE. Then the TRP can try different Rx beams within one repetition group, and the UE could do the beam sweeping from multiple repetition groups. In [3], some simulation results have shown that the performance of larger subcarrier spacing based or IFDMA based structure is closed to that of normal subcarrier based structure. Therefore the larger subcarrier spacing based or IFDMA based SRS structure should be supported to save the overhead.

For U-2, as it is related to the TRP’s beam, the TRP does not need to inform the UE the decision of the best Rx beam index. For U-3, it should be necessary for the TRP to inform the UE which Tx beam is the best. So if only one repetition group is defined, the SRS can be assumed to be for U-2 only. Then no follow-up signaling on the Rx beam information is needed. If multiple repetition groups are defined, the SRS can be assumed for at least U-3. Then some signaling on the decision of UL Tx beam index should be necessary. Further with the help of hierarchical beam search scheme discussed in [2], the overhead of SRS can be reduced. So the explicit control signaling for U-3 should be necessary.

The U-2 and U-3 can be used for gradual TRP/UE beam change or abrupt TRP/UE beam change. The gradual TRP/UE beam change means the new TRP/UE beam may be around current TRP/UE beam. The abrupt TRP/UE beam change indicates that the new TRP/UE beam could be far from current TRP/UE beam. Then some control signaling may be necessary to inform the UE whether the refined beam should rely on current Tx beam. In this way, the overhead of the SRS can be reduced, especially for the gradual beam refinement case. Therefore the SRS should be configurable.

Proposal 2: with regard to the overhead and latency, at least one of the following options should be supported for the SRS with beam sweeping operation:  the IFDMA based structure; larger subcarrier spacing based structure.

Joint UL/DL beam management when reciprocity is assumed

If the UL/DL channel reciprocity can be assumed, the best TRP Tx and Rx beam can be in the same direction and the best UE Tx and Rx beam can be the same. Then it could be possible to jointly manage the UL/DL beam. For example, the UL U-2 and DL P-3 can be jointly used. Then after receiving the CSI-RS in P-3, the UE could find out the best DL Rx beam and the corresponding UL Tx beam. This UL Tx beam can be used at the UE for SRS transmission in U-3. Then the TRP can find out its best UL Rx beam and corresponding DL Tx beam for the following DL transmission.  A similar fast beam aqusition procedure can be applied to support UL transmission. Therefore with the help of channel reciprocity, joint UL/DL beam management procedure can be achieved within one subframe as illustrated in Figure 1.

Figure 1: Joint DL and UL beam refinement to assist DL and UL transmission

Proposal 3: RAN1 should consider joint UL/DL beam management into account when UL/DL channel reciprocity can be assumed.

# Conclusions

In this contribution we have provided our views on uplink beam management for NR. From the discussion, we have the following proposals.

Proposal 1: UL beam management should be supported when no full beam correspondence could be confirmed in both gNB and UE side. For U-1, PRACH with beam sweeping operation should be used; for U-2 and U-3, SRS to support beam sweeping can be used.

Proposal 2: with regard to the overhead and latency, at least one of the following options should be supported for the SRS with beam sweeping operation:  the IFDMA based structure; larger subcarrier spacing based structure.

Proposal 3: RAN1 should consider joint UL/DL beam management into account when UL/DL channel reciprocity can be assumed.

# References

- Chairman Notes RAN1 NR Ad-Hoc

- R1-1609517, “UL Beam Management”, Intel Corporation

- R1-1700350, “CSI-RS Structure for Beam Management”, Intel Corporation

# Appendix – SRS Structure

Figure A-1: Larger subcarrier spacing based SRS

Figure A-2: IFDMA based SRS