3GPP TSG RAN WG1 #88																					R1-1702205

Athens, Greece, 13th - 17th February 2017

Source: 	Intel Corporation

Title:	On NR Type I codebook

Agenda item:	    8.1.2.3.2

Document for:	Discussion and Decision

# Introduction

In NR Adhoc, Jan. 2017 [1], the following agreement was made for NR:

Agreements:

- For Type I for single panel case with two-stage, i.e. W1W2, codebook-based PMI feedback,

  - Bi in W1 consists of a set of L DFT beams

    - For all ranks: FFS value(s) of L

    - FFS: Orthogonal or non-orthogonal beams

    - Select from following alternatives:

Alt4: W1=BBBB, B as Alt 3

- Other alternatives are not precluded

- Note: the above matrices are constructed with 2D DFT precoders

  - W2 is constructed, by down-selecting from following alternatives:

    - Alt 1: co-phasing only; beam selected wideband (in W1).

    - Alt 2: basis combination coefficient based on L basis based W1

    - Alt 3: beam selection and co-phasing from L-beam based W1

    - Alt 4: LTE-Class-B-type-like CSI feedback (e.g. based on port selection/combination codebook) (NOTE: W1 and W2 are derived from different set of CSI-RS resources)

    - Other alternatives are not precluded

Agreements:

- At least Type I CSI feedback should support multi-panel scenarios by choosing one of the two following alternatives:

  - Alt1: only wideband co-phasing factor across panels

  - Alt2: wideband and subband co-phasing factor across panels

- At least the following criteria should be used:

  - Performance-overhead tradeoff

  - Description of design goal, e.g. for channel compensation or hardware impairments

- FFS: How to capture this feature (co-phasing factor across panels) in codebook design

  - Examples: in W3 with W1W2W3, W1W3W2 or W3W1W2 structure, W1W2 where multi-panel co-phasing is included in either W1 or W2

  - Other examples are not precluded

In this contribution, we share our view on NR type I codebook design.

# Design Consideration

## Single Panel Case

First of all, we provide performance evaluation for different grid of beam configurations, i.e. W1 structure. In the evaluations we have used grid of beams supported by LTE, where configuration 1 containing single beam in W1 was used for the performance reference. As shown in the following tables, the configuration with grid of beam containing more than one beam does not provide significant gains over the baseline configuration. At the same time multiple beams in W1 significantly increasing UE complexity and CSI overhead and therefore should not be considered for NR specification.

Table 1 Performance of codebook configuration, UMa, Low Traffic

| Traffic Load (λ) |  | Low (1.5) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Codebook Configuration |  | 1 | 2 | 3 | 4 |
| UE Average Packet Throughput, Mbps | Average | 30.57 | 30.97 (1%) | 31.19 (2%) | 31.43 (3%) |
|  | 5% of CDF | 10.15 | 10.09 (-1%) | 10.39 (2%) | 10.40 (2%) |
|  | 50% of CDF | 28.13 | 28.84 (3%) | 28.99 (3%) | 29.31 (4%) |
|  | 95% of CDF | 55.62 | 55.63 (0%) | 55.63 (0%) | 55.66 (0%) |
| RU |  | 21% | 21% | 21% | 21% |

Table 2 Performance of codebook configuration, UMa, Medium Traffic

| Traffic Load (λ) |  | Medium (2.1) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Codebook Configuration |  | 1 | 2 | 3 | 4 |
| UE Average Packet Throughput, Mbps | Average | 23.08 | 23.22 (1%) | 23.31 (1%) | 23.49 (2%) |
|  | 5% of CDF | 6.31 | 6.15 (-3%) | 6.16 (-2%) | 6.21 (-2%) |
|  | 50% of CDF | 19.01 | 19.16 (1%) | 19.18 (1%) | 19.21 (1%) |
|  | 95% of CDF | 55.29 | 55.33 (0%) | 55.33 (0%) | 55.35 (0%) |
| RU |  | 39% | 39% | 39% | 39% |

Table 3 Performance of codebook configuration, UMa, High Traffic

| Traffic Load (λ) |  | High (2.6) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Codebook Configuration |  | 1 | 2 | 3 | 4 |
| UE Average Packet Throughput, Mbps | Average | 17.09 | 17.21 (1%) | 17.15 (0%) | 17.23 (1%) |
|  | 5% of CDF | 3.76 | 3.78 (1%) | 3.69 (-2%) | 3.69 (-2%) |
|  | 50% of CDF | 12.94 | 12.96 (0%) | 12.78 (-1%) | 12.87 (-1%) |
|  | 95% of CDF | 46.97 | 47.87 (2%) | 47.93 (2%) | 48.10 (2%) |
| RU |  | 57% | 57% | 57% | 57% |

Table 4 Performance of codebook configuration, UMi, Low Traffic

| Traffic Load (λ) |  | Low (1.8) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Codebook Configuration |  | 1 | 2 | 3 | 4 |
| UE Average Packet Throughput, Mbps | Average | 34.89 | 35.36 (1%) | 35.70 (2%) | 35.89 (3%) |
|  | 5% of CDF | 12.33 | 12.31 (0%) | 12.41 (1%) | 12.53 (2%) |
|  | 50% of CDF | 33.66 | 34.32 (2%) | 34.89 (4%) | 35.07 (4%) |
|  | 95% of CDF | 55.70 | 55.75 (0%) | 55.76 (0%) | 55.77 (0%) |
| RU |  | 21% | 21% | 21% | 21% |

Table 5 Performance of codebook configuration, UMi, Medium Traffic

| Traffic Load (λ) |  | Medium (2.8) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Codebook Configuration |  | 1 | 2 | 3 | 4 |
| UE Average Packet Throughput, Mbps | Average | 25.10 | 25.16 (0%) | 25.56 (2%) | 25.61 (2%) |
|  | 5% of CDF | 6.65 | 6.49 (-2%) | 6.52 (-2%) | 6.55 (-2%) |
|  | 50% of CDF | 21.18 | 21.39 (1%) | 21.69 (2%) | 22.02 (4%) |
|  | 95% of CDF | 55.47 | 55.45 (0%) | 55.48 (0%) | 55.47 (0%) |
| RU |  | 41% | 41% | 40% | 40% |

Table 6 Performance of codebook configuration, UMi, High Traffic

| Traffic Load (λ) |  | High (3.7) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Codebook Configuration |  | 1 | 2 | 3 | 4 |
| UE Average Packet Throughput, Mbps | Average | 16.54 | 16.37 (-1%) | 15.98 (-3%) | 16.12 (-3%) |
|  | 5% of CDF | 3.15 | 3.04 (-3%) | 2.97 (-6%) | 2.97 (-6%) |
|  | 50% of CDF | 12.52 | 12.34 (-1%) | 11.74 (-6%) | 11.81 (-6%) |
|  | 95% of CDF | 44.86 | 44.24 (-1%) | 44.78 (0%) | 45.29 (1%) |
| RU |  | 65% | 66% | 66% | 65% |

Second, since linear combination codebook can be supported by Type II CSI, we propose to adopt Alt 3 for W1 structure with L=1 and W2 should be Alt 1.

Finally, the codebook structure may also support beam patterns with different beam width. For example the narrow beam PMIs may be required for low rank transmission to provide sufficient beamforming gain, while the need of supporting wide beam PMIs arises in the scenarios with higher rank transmission (e.g. 3-4 layers) to increase probability of the higher transmission rank selection without significant loss [2]. The use of widen beam may be also preferable in high mobility scenario for more robust selection of the precoding [3].

Proposal 1. For Type I single panel case, two-stage, i.e. W1W2, codebook-based PMI feedback, where W1 consists of one vector (wideband), i.e. DFT beam or broadened beam (see equation) and W2 is constructed by co-phasing only (subband)

.

## Multi-Panel Support

As agreed in 3GPP RAN1 #87 meeting, uniform, non-uniform array, calibrated and non-calibrated panels, different panels from same or different TRPs, coherent and non-coherent transmission are design considerations in NR CSI feedback or codebook. In order to have a unified codebook structure, we consider general array structure which covers both uniform array as well as non-uniform array as in below figure.

Figure 1 Considered Antenna Array Pattern

First of all, we propose to have co-phasing values among different panels. In some deployment or operational scenario, CSI-RS antenna ports spacing within CSI-RS resource can be non-uniform. For example, realizing large number of antennas using multiple RF modules which contains multiple antenna element is common implementation of large number of antennas. In this case, due to hardware implementation issue, the antenna spacing between two panels may not be the same as spacing between two antenna elements within a panel. In this case, there exists additional phase difference between different panels due to signal path length difference as well as mixer and PLL phase error. While phase difference caused by mixer and PLL phase error as well as non-uniform antenna distance can be compensated by wideband phase correction, the phase difference caused by different signal path delay should be compensated by frequency-selective phase correction. The effect of different signal path delay typically more pronounced for wide bandwidth and high frequency operation.

Proposal 2. Support subband co-phasing across panels in NR codebook design

Second, we propose different beam selection for different panels. The multiple panels deployed at the TRP side can be utilized for the data transmission to one UE. Depending on the beamforming selection on the antenna panels different MIMO transmission schemes can be realized. For example transmission with higher rank can be used to improve the peak data rate of the UE and may require different beam selection on the TRP antenna panels. The high gain / narrow beam (i.e. single narrow beam) transmission can be provided for coverage-limited UEs by using the same beamforming and joint precoding across TRP antenna panels.  Considering various beamforming options using multiple antenna panels, the beamforming on CSI-RS antenna ports as well as the codebook structure should be carefully designed to support all MIMO transmission schemes in the NR system. It should be noted that support of non-coherent precoding may be also beneficial to support some NR scenarios. More specifically, depending on implementation multiple antenna panels at the TRP may not be coherent to support joint precoding discussed above. In addition the panels can be deployed at the different TRPs to perform coordinated multi-point transmission to the UE. To support these scenarios, the codebook can be further enhanced to support independent beam selection on multiple CSI-RS antenna ports. In another approach multiple CSI-RS resources associated with different panels or beams can be used. Similar consideration can be done for CL-MU MIMO where each panel forms different direction beams toward different UEs.

Proposal 3. Support different beam selection for different panel in NR codebook design

Third, in NR system with multiple (analog) beams, beam management and/or CSI feedback should support beam selection. In LTE, beam selection at the TRP is done by CSI feedback or using RRM measurements using DRS while, in NR, separate (analog) beam management protocol (P-1 to P-3) is considered. After beam management procedure, properly beamformed CSI-RS (based on UE feedback) is sent to UE to further calculate and feedback digital beamforming weights. Since one or more beams can be reported by the UE, it would be better to support some level of (analog) beam selection as well as digital beamforming (PMI) selection simultaneously in order to minimize feedback overhead otherwise individual CSI for different analog beamforming should be fed back to gNB. To this end, we can consider a panel selection codebook. UE can select antenna panels to calculate digital precoding weights as well as corresponding RI and CQI. Note that DPS can be done by the extended beam selection functionality as well.

Proposal 4. Consider panel selection functionality in NR codebook design

## Type I Codebook Structure

In this sub-section, we propose a single codebook structure to support aforementioned functionalities, i.e. co-phasing between different panels, different beam selection for different panel, panel selection functionality, and beam broadening functionality. From the result in section 2.1, we propose to have L=1 which is number of DFT beams in W1 in case of single panel. Also, Type I codebook is targeting single user MIMO, we propose similar to LTE Rel. 13 codebook structure which is applying same beam for different polarization. Higher rank selection can be done by different beam selection in different panel or beam broadening function. In this case, we can have following codebook structure, where for simplicity, we show codebook structure without panel selection functionality:

The codebook structure is defined as product of two matrices:

W1W2

where the parameters of the codebook are defined as follows:

- W1 is matrix that contains beams, where beams could correspond to DFT vectors or other vectors e.g. supporting beam broadening.

- W2 is matrix that contains co-phasing between different antenna panels/polarizations and additionally panel selection.

In more detail,

Where the parameters of the codebook are defined as follows

- W1 is 2M∙N x 2N block diagonal matrix

- W2 is 2N x Ns matrix

- is K x L all zero matrix

- M is number of antenna ports per panel per polarization.

- N is number of antenna panel

- Ns is number of spatial streams

- is beam vector for i-th panel

- is co-phasing vector between panels and/or polarizations for s-th spatial stream

Proposal 5. For Type I, two-stage, i.e. W1W2, codebook-based PMI feedback, W1W2, where W1 is matrix that contains beams for each panel and polarization (wideband) and W2 is matrix that contains co-phasing vectors between panels and/or polarizations (subband)

# Conclusion

In this contribution, we discussed consideration on codebook design, and we proposed a general codebook structure to support various desired functionalities for NR.

Proposal 1. For Type I single panel case, two-stage, i.e. W1W2, codebook-based PMI feedback, where W1 consists of one vector (wideband), i.e. DFT beam or broadened beam (see equation) and W2 is constructed by co-phasing only (subband)

.

Proposal 2. Support subband co-phasing across panels in NR codebook design

Proposal 3. Support different beam selection for different panel in NR codebook design

Proposal 4. Consider panel selection functionality in NR codebook design

Proposal 5. For Type I, two-stage, i.e. W1W2, codebook-based PMI feedback, W1W2, where W1 is matrix that contains beams for each panel and polarization (wideband) and W2 is matrix that contains co-phasing vectors between panels and/or polarizations (subband)

# References

- Chairman’s notes, 3GPP TSG RAN WG1 Meeting NR Ad-Hoc, Spokane, USA, 16th – 20th January 2017

- R1-166523, Discussion on Class A codebook extension, Intel Corporation

- R1-1611972, Discussion on NR transmit diversity scheme, Intel Corporation