3GPP TSG RAN WG1 Meeting #88                                       R1-1702841

Athens, Greece, 13th – 17th February 2017

Source:	NTT DOCOMO, INC.

Title:	Evaluation for Beam Management

Agenda Item:	8.1.2.2.4

Document for: 	Discussion and Decision

- Introduction

Beam management is a very new physical layer concept introduced in the NR system. In order to correctly verify and evaluate new algorithms and schemes proposed for the beam management, it is necessary to establish a general evaluation methodology. At the RAN1 NR Ad-Hoc meeting, the issue of evaluation for beam management was proposed in [1] and was decided for further discussion in email discussion [NRAN1-01]. In this contribution, we present our views on evaluation assumption for beam management and provide initial simulation results comparison between no beam management and different management schemes.

- Discussion on Evaluation Methodology and Assumptions

  - Evaluation methodologies

A realistic evaluation model shall be established to evaluate NR systems with beam management functionalities. Compared to the conventional system level evaluation of a cellular system at least the following new features shall be considered.

Antenna array structure:

The latest LTE system level evaluation assumes single panel antenna array. In NR, multi-panel antenna array shall be considered. The latest LTE system level evaluation assumes fixed TXRU to antenna element mapping throughout the simulation. In NR, with the introduction of the hybrid beamforming architecture, analog beamforming, which varies the TXRU to antenna element mapping from time to time, shall be considered. It is noticed that pure analog beamforming is only feasible in wideband. Such impact, e.g., on user scheduling algorithm, interference modelling shall be taken into consideration in system level evaluation.

UE to network association:

In the LTE system level evaluation, UE and the networks are associated with cells. Different types of cells are modeled, e.g., macro/micro-cells with hexagonal deployment and wider coverage, small cells with random location and shorter coverage. Although the parameters used in a real network may differ a lot from the evaluation assumptions, the parameters used for LTE evaluation were unified as much as possible to ensure comparable evaluation results. In NR beam management, UE shall be associated with the network via beams. The characteristics of beams, e.g., beam shape, candidate beam directions, beam number, etc., shall be defined and unified as much as possible, for the purpose of generating comparable results. Note that similar practice was done during the LTE evaluation, e.g., via specifying the antenna gain pattern and bearing/tilting angles.

Proposal 1: Unify the general beam characteristics, e.g., beam shape, beam directions, beam number for each interested evaluation cases for beam management.

Categorization of the schemes:

In the LTE evaluation, transmission schemes are classified in different categories, e.g., SU-MIMO, MU-MIMO, CoMP, etc. Candidate technologies are proposed and compared within each category, specification supported are decided to be made for the most necessary technologies within each category. For the evaluation of beam management, it is necessary to make similar approach. Candidate categories include beam acquisition procedure, beam recovery procedure, transmission procedure. It shall be further discussion, whether categorization shall be made to differentiate the cases of single-panel/TRP single-beam based association, single-panel/TRP multi-beam based association, multi-panel/TRP multi-beam based association etc.

Proposal 2: RAN1 discuss general categorizations of beam management technologies and define evaluation assumptions accordingly.

For beam acquisition and beam recovery, evaluation shall be made to verify the efficiency and effectiveness of beam management procedure via checking the coverage, RS overhead, latency, robustness of the link, etc. For transmission, evaluation shall be made to verify the performance. According to the above mentioned evaluation purposes, performance metrics shall be designed. Coverage can be reflected by SNR/SINR distribution and outage probability. RS overhead can be reflected in the spectrum efficiency or packet throughput. Latency, due to the beam acquisition or beam recovery, shall be reflected in packet throughput. Robustness of the links can be reflected via package loss rate. Based on the discussion, we propose the follow.

Proposal 3: At least the following performance metrics are considered for beam management related evaluation. Different metrics can be selected for different evaluation purposes.

- SNR/SINR CDF;

- Spectrum efficiency;

- Outage probability

- Packet throughput;

- Packet loss rate.

Proposal 4: Latency associated with beam management, e.g., acquisition, recovery, switching, shall be taken into account in calculating the packet throughput.

Proposal 5: RS overhead, caused by beam management, e.g., acquisition, recovery, switching, shall be taken into account in calculating the spectrum efficiency and/or packet throughput.

  - Evaluation assumptions

Based on the discussion about the evaluation methodologies for beam management, we further refine the simulation assumptions for system level evaluation based on [1] as below. Note that the modified parts are highlighted in red.

Table 1 Simulation assumptions for system level evaluation

| Attributes | Values of assumptions |
| --- | --- |
| Scenarios (Carrier Frequency) | Indoor hotspot (carrier frequency 30GHz), Urban macro (carrier frequency 30GHz), Dense Urban (Evaluate micro layer only), carrier frequency 30GHz) - Macro layer: macro-only 10 users per TRP, all UEs are connected to macro layer. Users randomly and uniformly dropped throughout the macro geographical area - Micro layer: micro-only 3 micro BSs per macro BS and 10 users per TRP |
| Mode | DL SU-MIMO / MU-MIMO Note: Companies explain the details of scheduling algorithm if MU-MIMO is adopted. |
| Channel Model | Following related assumption in TR 38.802 |
| TXRU mapping to antenna elements | Companies explain the details of TXRU mapping to antenna elements. (2D DFT based beam as a baseline) |
| TXRU mapping weights | Companies explain the details of TXRU mapping weights. 2D DFT based. Other mapping schemes are not precluded. |
| Criteria for selection for serving TRP | Companies explain the details of criteria for selection for serving TRP. Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming |
| Criteria for Beam Selection for interfering TRP | Companies explain the details of criteria for beam selection for interfering TRP. |
| Constraints for the range of selective beams per TRP sector | Following Phase 2 calibration |
| Scheduling algorithm | PF scheduler |
| Traffic Model | FTP model 1/2/3 with packet size 0.1 and 0.5Mbytes (other value is not precluded). Other traffic models are not precluded. |
| Inter-panel calibration | Ideal |
| Control overhead | 2 symbols |
| UE receiver type | MMSE-IRC |
| BF scheme | Analog BF based on beam selection + Digital BF based on ideal SVD or PMI feedback Note: Reuse LTE Rel. 13 codebook if PMI feedback adopted. |
| Transmission scheme | Multi-antenna port transmission schemes Note: Companies explain details of the using transmission scheme. |
| UE mobility feature | Follow Phase 3 calibration Note: Companies explain whether or which model is used in simulation evaluation. If used, the configuration details should be explained |
| MCS | Use LTE MCS |
| Metric | - SNR/SINR - Spectral efficiency - Outage probability - Packet throughput - Packet loss rate - UE-initiated beam recovery frequency |

For scenario configuration, flexible micro number and UE number should be supported to highlight the advantages of different algorithms. For different evaluation purposes, detailed parameter value shall be discussed.

In multi-beam based system, scheduling becomes more complicated as beam is a new kind of scheduling resource especially for MU-MIMO. In addition scheduling algorithms have great impact on the system performance. Therefore, companies should explain the details of scheduling algorithm if MU-MIMO transmission mode is adopted.

The basic procedure of beam management including the selection of inter/intra TRP Tx beams and UE Rx beams defined as as P-1, P-2 and P-3. There are a various of different beam selection methods like exhaustive search, multi-level search and so on, and the performance of these different beam selection mechanisms are closely related with detailed beam characteristics. Several fixed beam shape/angles were adopted for phased 2 MIMO calibration. However, the evaluation for beam management should encourage variable beam selection schemes for better system performance. In this way, companies shall use 2D DFT beams, specified for phase 2 calibration as baseline beam characteristics. Other beams, if needed for the specific algorithm, shall be reported in detail.

UEs with bad SINR are less probably scheduled by using PF scheduling which would impact the metric of outage defined as the percentage of users that has zero throughput. The prioritization coefficients P can be calculated based on current throughput T and average throughput R with two fairness parametersαandβ: P= TαRβ. The different values of αandβ may affect schedule results. In this way, companies should provide related factors values for fair comparison of the throughput and outage probability.

As to BF scheme, if PMI feedback is needed, it is proposed to reuse Rel. 13 codebook to ensure fair comparison.

Based on the discussion above, we have following proposals:

Proposal 6: General beam management procedure should be provided.

Proposal 7: Following aspects of beam management are considered for evaluation

- Beam selection method, e.g., beam number, beam grouping metrics

- Beam reporting, e.g., beam related ID

- RS patterns and density

- UE mobility related, e.g., frequency of beam pair link blocking

- Initial Evaluation Results

This part posts some initial results of beam management related evaluations. The simulation examines the effect of beam management on SINR performance. Only Analog beamforming is considered in the simulation. Detailed simulation assumption is listed in Table 3 in Appendix. About simulation cases, there are two BS antenna configurations and two sets of beams examined.

BS antenna configuration

- Panel (2,4): (M,N,P,Mg,Ng) = (2,4,2,2,2)

- Panel (4,8): (M,N,P,Mg,Ng) = (4,8,2,2,2)

Beam set

- 32 Beams:

8 horizontal directions: [-52.5° -37.5° -22.5° -7.5° 7.5° 22.5° 37.5° 52.5°]

4 vertical directions:[ 22.5° 67.5° 112.5° 157.5°]

- 8 Beams:

4 horizontal directions: [-45° -15° 15° 45°]

2 vertical directions: [45° 135°]

Figure 1: SINR CDF in UMa without beam management, with different antenna and beam numbers

We also show some initial performance results of spectral efficiency in Urban Macro and Indoor Hotspot scenario. Analog beamforming and digital beamforming based on SVD are applied in the simulation. Some simulation assumptions such as BS and UE antenna configurations and beam management scheme are addressed here.

- UMa scenario: BS antenna configuration is (M,N,P,Mg,Ng) = (4,8,2,2,2) with 12 candidate beams.

- Indoor Hotspot scenario: BS antenna configuration is (M,N,P,Mg,Ng) = (4,8,2,1,1) with 32 candidate beams.

- UE antenna is set as (M,N,P,Mg,Ng) = (2,4,2,1,2) with 8 candidate beams in order to provide analog and digital beamforming gain.

Note: Although UE has two panels, only one panel will be selected for transmission at a time according to the rule: maximizing RSRP.

The detailed simulation assumptions are listed in Table 4 and 5 in Appendix.

Figure 2: Initial spectral efficiency of UMa and Indoor Hotspot scenarios

- Summary

In this contribution, we described our views on evaluation assumption for beam management and provide initial simulation results comparison between no beam management and different management schemes, we made the following proposal.

Proposal 1: Unify the general beam characteristics, e.g., beam shape, beam directions, beam number for each interested evaluation cases for beam management.

Proposal 2: RAN1 discuss general categorizations of beam management technologies and define evaluation assumptions accordingly.

Proposal 3: At least the following performance metrics are considered for beam management related evaluation. Different metrics can be selected for different evaluation purposes.

- SNR/SINR CDF;

- Spectrum efficiency;

- Outage probability

- Packet throughput;

- Packet loss rate.

Proposal 4: Latency associated with beam management, e.g., acquisition, recovery, switching, shall be taken into account in calculating the packet throughput.

Proposal 5: RS overhead, caused by beam management, e.g., acquisition, recovery, switching, shall be taken into account in calculating the spectrum efficiency and/or packet throughput.

Proposal 6: General beam management procedure should be provided.

Proposal 7: Following aspects of beam management are considered for evaluation

- Beam selection method, e.g., beam number, beam grouping metrics

- Beam reporting, e.g., beam related ID

- RS patterns and density

- UE mobility related, e.g., frequency of beam pair link blocking

References

- 3GPP R1-1701514, “Way Forward on Evaluation for Beam Management” Jan. 2017.

- 3GPP R1-1701517, “Summary of email discussion [87-30] on NR-MIMO calibration” Jan. 2017.

Appendix

Table 3: Evaluation Assumptions in UMa Scenario with only analog beamforming

| Parameters | Values |
| --- | --- |
| Layout | Single Layer Macro layer: Hex. Grid |
| Inter-BS distance | 500 m |
| Carrier frequency | 30 GHz |
| Bandwidth | 100 MHz |
| Subcarrier spacing | 15 KHz |
| Channel model | 3D UMa in TR38.900 |
| BS Tx power | 43 dBm |
| BS antenna configurations | (M,N,P,Mg,Ng) = (4,8,2,2,2) / (2,4,2,2,2) (dH, dV, dHg,dVg) = (0.5, 0.5, 4.0, 2.0) |
| BS TXRU mapping | A single TXRU per panel per polarization is used. 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT with oversampling factor=1. Number of TXRUs= Mg x Ng x 2. |
| BS Antenna height | 25 m |
| BS receiver noise figure | 7 dB |
| UE antenna configurations | Onmi-directional |
| UE antenna height | Same as 3D-UMa in TR36.873 |
| UE receiver noise figure | 13 dB |
| UE distribution | 100% Outdoor in cars: 30km/h, 10 users per TRP for full buffer traffic |
| Beamforming scheme | Analog beamforming |
| Beam selection method | Select the best beam pair at the same time based on the criteria of maximizing receive power after beamforming. Companies provide the details of beam selection. BS: Azimuth angle [0 120]; Zenith angle [0 180]; In fixed beamforming case, angles underlined are applied. |
| Interfering beam selection | Non-serving TRPs use its random analog beam to cause interference |
| Scheduling algorithm | Round Robin |

Table 4: Evaluation Assumptions for MIMO Calibration in UMa Scenario with analog and digital beamforming

| Parameters | Values |
| --- | --- |
| Layout | Single Layer Macro layer: Hex. Grid |
| Inter-BS distance | 500 m |
| Carrier frequency | 30 GHz |
| Bandwidth | 40 MHz |
| Subcarrier spacing | 60 KHz |
| Channel model | 3D UMa in TR38.900 |
| BS Tx power | 43 dBm |
| BS antenna configurations | (M,N,P,Mg,Ng) = (4,8,2,2,2) (dH, dV, dHg,dVg) = (0.5, 0.5, 4.0, 2.0) |
| BS TXRU mapping | A single TXRU per panel per polarization is used. 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT with oversampling factor=1. Number of TXRUs= Mg x Ng x 2. |
| BS Antenna height | 25 m |
| BS receiver noise figure | 7 dB |
| UE antenna configurations | (M,N,P,Mg,Ng) = (2,4,2,1,2) (dH, dV) = (0.5, 0.5) Polarization degrees: 0 and 90 |
| UE antenna height | Same as 3D-UMa in TR36.873 |
| UE receiver noise figure | 10 dB |
| UE distribution | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10 users per TRP for full buffer traffic |
| Beamforming strategy | Analog beamforming + digital beamforming based on SVD |
| Beam selection method | Select the best beam pair at the same time based on the criteria of maximizing receive power after beamforming. Companies provide the details of beam selection. BS: Azimuth angle [-5*pi/16 -3*pi/16 -pi/16 pi/16 3*pi/16 5*pi/16]; Zenith angle [5*pi/8 7*pi/8]; UE: Azimuth angle [-3*pi/8 -pi/8 pi/8 3*pi/8]; Zenith angle [pi/4 3*pi/4]; In fixed beamforming case, angles underlined are applied. |
| Interfering beam selection | Non-serving TRPs use its own transmission analog beam and digital precoder to cause interference |
| Scheduling algorithm | Round Robin |

Table 5: Evaluation Assumptions in Indoor Scenario with analog and digital beamforming

| Parameters | Values |
| --- | --- |
| Layout | Single Layer, 12 TRPs |
| Inter-BS distance | 20 m |
| Carrier frequency | 30 GHz |
| Bandwidth | 40 MHz |
| Subcarrier spacing | 60 KHz |
| Channel model | Indoor in TR38.900 |
| BS Tx power | 23 dBm |
| BS antenna configurations | (M,N,P,Mg,Ng) = (4,8,2,1,1) (dH, dV) = (0.5, 0.5) |
| BS TXRU mapping | A single TXRU per panel per polarization is used. 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT with oversampling factor=1. Number of TXRUs= Mg x Ng x 2. |
| BS Antenna height | 3 m |
| BS receiver noise figure | 7 dB |
| UE antenna configurations | (M,N,P,Mg,Ng) = (2,4,2,1,2) (dH, dV, dHg, dVg) = (0.5, 0.5, 0, 0) |
| UE antenna height | 1.5 m |
| UE receiver noise figure | 10 dB |
| UE distribution | 100% Indoor in houses: 3km/h 10 users per TRP for full buffer traffic |
| Beamforming strategy | Analog beamforming + digital beamforming based on SVD |
| Beam selection method | Select the best beam pair at the same time based on the criteria of maximizing receive power after beamforming. Companies provide the details of beam selection. BS: Azimuth angle [pi/16 3*pi/16 5*pi/16 7*pi/16 9*pi/16 11*pi/16 13*pi/16 15*pi/16]; Zenith angle [pi/8 3*pi/8 5*pi/8 7*pi/8]; UE: Azimuth angle [-3*pi/8 -pi/8 pi/8 3*pi/8]; Zenith angle [pi/4 3*pi/4]; In fixed beamforming case, angles underlined are applied. |
| Interfering beam selection | Non-serving TRPs use its own transmission analog beam and digital precoder to cause interference |
| Scheduling algorithm | Round Robin |