3GPP TSG RAN WG1 Meeting #89	R1-1707011

Hangzhou, China, 15-19 May, 2017

Agenda Item:	6.2.5.1

Source:	Huawei, HiSilicon

Title:	Discussion on support of DL 1024QAM

Document for:	Discussion/Decision

# Introduction

In RAN Plenary #75, the new WI of "Enhancements for High Capacity Stationary Wireless Link and Introduction of 1024 QAM for LTE" has been approved [1]. The main objectives on support of DL 1024 QAM are as follows:

- Study the feasibility and performance benefit, and if justified, specify support for 1024QAM for DL channels [RAN1, RAN4]

  - Document the study result of feasibility and benefit in a TR

  - If normative work starts, introduce new MCS table, signalling and CQI feedback to support 1024QAM for PDSCH

In this contribution, we mainly discuss the standard impacts on support of DL 1024QAM.

# CQI/MCS/TBS table for DL 1024QAM

CQI table for 1024QAM

The CQI table is used to support DL adaptive MCS. In order to support 1024QAM transmission, the maximum efficiency of the current 256QAM CQI table needs to be expanded. Since the maximum efficiency supported by current CQI table is 7.4063 bps/Hz, the introduction of 1024QAM can directly increase the efficiency to 7.4063×10/8 =9.2578 bps/Hz, which also increases capacity performance with certain system bandwidth. The design principles for current CQI table are summarized as follows [2]:

- AWGN channel is assumed for generating the CQI table.

- 15-entry CQI table with approximately equal step size in equivalent SNR.

- The modulation order switching points in the CQI table should consider the frequency selectivity.

Following the previous principles, the CQI entries for 1024QAM could be determined using an equal step size in equivalent SNR between 7.4063 and 9.2578.

Table 7.2.3-2: 4-bit CQI Table 2 (256QAM part only)

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 12 | 256QAM | 711 | 5.5547 |
| 13 | 256QAM | 797 | 6.2266 |
| 14 | 256QAM | 885 | 6.9141 |
| 15 | 256QAM | 948 | 7.4063 |

As described in [2], with target BLER 10%, the curve of spectral efficiency and SNR is almost linear, the new entries could be sampled based on approximately equal step size in spectral efficiency. In our companion contribution on 1024QAM performance evaluation, it is observed that 1024QAM performs better than 256QAM with spectral efficiency around 7.45bps/Hz. Therefore, the 256QAM with SE 7.4063 can be kept, and 1 new CQI entry as shown in Table 1 is sampled for 1024QAM between 7.4063 and 9.2578, i.e., with spectral efficiency 8.3321.

Table 1. New entries for 1024QAM

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 13 | 256QAM | 948 | 7.4063 |
| 14 | 1024QAM | 853 | 8.3321 |
| 15 | 1024QAM | 948 | 9.2578 |

Proposal 1: Two new CQI entries are introduced for 1024QAM as shown in Table 2, with the spectral efficiencies corresponding to each entry are 8.3321 and 9.2578. FFS which CQI entries are to be removed.

Table 2. Proposed entries for 1024QAM

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 14 | 1024QAM | 853 | 8.3321 |
| 15 | 1024QAM | 948 | 9.2578 |

MCS table/TBS table

The design principles for current MCS table are summarized [2] as follows:

- A 5-bit MCS table with 4 reserved entries for modulation order signalling.

- CQI values taken as starting point for design.

- The remaining 13 MCS entries chosen as equally spaced between adjacent spectral efficiencies.

With the previous principles, we propose to take new CQI entries for 1024QAM directly as MCS entries. Therefore, we have the following proposal:

Proposal 2: Introduce 2 MCS entries for 1024QAM.

  - 2 MCS entries as shown in Table 3 reflect the corresponding efficiency for the proposed CQI index #14, #15 for 1024QAM.

  - 1 MCS entry for 1024QAM retransmission.

  - FFS which MCS entries are removed.

Table 3. Proposed MCS entries for 1024QAM

| MCS Index | Modulation Order | TBS Index |
| --- | --- | --- |
| 25 | 10 | 35 |
| 26 | 10 | 36 |
| 27 | 2 | reserved |
| 28 | 4 |  |
| 29 | 6 |  |
| 30 | 8 |  |
| 31 | 10 |  |

For TBS table, new TBS values should be aligned with QPP sizes and only used for case that TBS entries would exceed the maximum value in the current TBS_L4 table.

Proposal 3: FFS TBS table design for 1024QAM.

# Conclusion

In the contribution, we discussed the standard impact for DL 1024QAM, with the following proposals:

Proposal 1: Two new CQI entries are introduced for 1024QAM as shown in Table 2, with the spectral efficiencies corresponding to each entry are 8.3321 and 9.2578. FFS which CQI entries are to be removed.

Proposal 2: Introduce 2 MCS entries for 1024QAM.

  - 2 MCS entries as shown in Table 3 reflect the corresponding efficiency for the proposed CQI index #14, #15 for 1024QAM.

  - 1 MCS entry for 1024QAM retransmission.

  - FFS which MCS entries are removed.

Proposal 3: FFS TBS table design for 1024QAM.

# References

RP-170838, “New WI proposal: Enhancements for High Capacity Stationary Wireless Link and Introduction of 1024 QAM for LTE”, Huawei, HiSilicon, Qualcomm Incorporated, China Telecom.

R1-140034, “CQI/MCS table design for 256QAM”, RAN1#76, Prague, February, 2014.