3GPP TSG RAN WG1 Meeting #89	R1-1707491

Hangzhou, China 15th-19th May 2017

Source:	CATT

Title:	Discussion on QCL for NR

Agenda Item:	7.1.2.4.5

Document for: Discussion and decision

Introduction

In the RAN1#86bis & #87, the following agreements on QCL have been reached [1][2]:

- QCL framework in NR is extended with new spatial QCL parameter(s) to support UE side beamforming/receiving procedure

  - FFS details (e.g., receive angle of arrival, transmit angle of departure, spatial correlation of receiver antennas, Rx/Tx beamforming, etc.)

- For DM-RS antenna ports, NR supports:

  - All ports are QCL-ed

  - Not all ports are QCL-ed

    - FFS on details

- FFS, e.g.:

  - QCL between antenna ports transmitted on different CCs

  - QCL between CSI-RS antenna ports within one CSI-RS resource

    - All ports are QCL-ed

    - Not all ports are QCL-ed

  - QCL between CSI-RS antenna ports within two or more CSI-RS resources

    - All ports are QCL-ed

    - Not all ports are QCL-ed

  - QCL of a DM-RS antenna port with other RS types

  - QCL considering channel reciprocity

- Flexible configuration/indication of the QCL assumption should be studied in NR:

  - Possible grouping  of the QCL parameters should be studied:

    - e.g., average gain, average delay

    - e.g., angle of arrival/ departure, delay spread, Doppler spread

- Study the need of supporting UE feedback and contents if needed to assist QCL association between reference signal resources/ports with respect to UE spatial QCL parameter(s) to support UE side beamforming/receiving procedure

- Companies are encouraged to provide details of beamforming/receiving procedures and evaluate performance in terms of at least following metrics:

  - RS overhead

  - Overhead of UE feedback

  - Spectral efficiency

# Agree as working assumption:

- Spatial parameter(s) for QCL in NR describes the spatial channel properties of the RS antenna ports observed at the receiver.

- FFS: Spatial parameter(s) in NR also describes the spatial channel properties of the antenna ports at transmitter(s).

- Support UE reporting for related information, if necessary.

It’s also agreed in RAN1 AH NR that [3]:

- Support DMRS ports grouping, and DMRS ports within one group are QCL-ed, and DMRS ports in different groups are non-QCLed.

  - FFS the grouping principle, e.g. grouping DMRS according to CWs, analog beams, etc.

  - FFS the signalling method of QCL indication, e.g., RRC, MAC CE, DCI, etc.

- Study RS for performing estimation of large scale properties of channel (E.g., Doppler shift/Doppler spread/delay spread)

- QCL supports following functionalities

  - Beam management functionality: at least including spatial parameters

  - Frequency/timing offset estimation functionality: at least including Doppler/delay parameters

  - RRM management functionality: at least including average gain

  - Others are not precluded

Furthermore, in RAN1 #88, the following agreement has been reached [4]:

- By default (i.e., the UE is not indicated), antenna port(s) transmitted on different CCs can’t be assumed to be QCL-ed

  - FFS the case of spatial domain QCL assumptions especially related to carrier frequency

Based on the agreements listed above, we discuss the remaining issues left open for QCL in NR in more details and present our consideration on QCL framework extension.

Discussion on QCL for NR

As described above, with large-scale antenna arrays at both transmitter and receiver side, the large-scale properties of channel seen from different ports depend not only on the geographic location of transmitter but also on the beamforming operations at both ends of the radio link. Therefore, its well accepted in RAN1 to introduce QCL parameters regarding spatial characteristics of antenna ports.

Spatial parameter

Potential extensions of spatial QCL parameters to support UE side beamforming/receiving procedure have been discussed in the past few meetings. Receive angle of arrival, transmit angle of departure, spatial correlation of receiver antennas, Rx/Tx beamforming, etc. are candidate parameters for describing spatial QCL of antenna ports.

- QCL wrt receive angle of arrival

Mean AOA and AOA spread could be estimated at UE side, based on which the preference on selection and grouping of CSI-RS ports can be reported to the network. Then, with QCL indications of CSI-RS port(s) and RS port(s) for physical channel transmission, appropriate beamforming can be performed in DL reception. One potential problem with this approach is the estimation of mean AOA,  especially in rich scattering environment.

- QCL wrt transmit angle of departure

Large-scale properties of channel are determined by beams at both transmitter and receiver sides. As elaborated in [5], even for two transmit beams with slightly different angles of departure, the random reflections/scatterings in the propagation path could result in totally different choices for Rx beams. And then, the ports in the two beam pairs cannot be assumed to be QCL. Another related question is, can it be assumed that the ports transmitted with exactly the same Tx beamforming but from two panels are QCL to each other? The answer seems to be open without knowing the undergoing channel seen by the receive antenna array. That is, QCL may not be assumed based solely on the knowledge from Tx side.

- QCL wrt spatial correlation of receiver antennas

Similar to adding QCL parameter(s) with respect to receive angle of arrival, introducing spatial correlation of receiver antennas as spatial parameter for QCL serves as an intuitional way to capture the spatial channel properties of the RS antenna ports observed at the receiver. Ideally, compared with AOA based description, spatial correlation of receiver antennas gives more insight into the spatial characteristics of the propagation environment. However, with analog or hybrid beamforming, availability of accurate receive side channel correlation estimation at element level is questionable.

- QCL wrt Rx and Tx beamforming

As one of the reasons to introducing spatial QCL parameter(s), beam management is of great importance in hybrid beamforming based large-scale antenna system. Therefore, defining QCL wrt Rx/Tx beamforming directly seems to be a straightforward way of extending QCL framework for NR. To this end, the Tx beam(s) selection and grouping in beam training and tracking procedures could be reported to the network. The RS ports transmitted in Tx beams corresponding to the same group can be assumed to be QCL-ed with respect to Rx beamforming. However, it’s still insufficient to assume that they’re QCL-ed within the same Tx beam group with respect to delay and Doppler parameters from channel estimation and demodulation perspective. To illustrate this, an example is shown in Fig.1.

Figure 1. QCL wrt Rx beamforming

QCL parameter grouping

It’s noted that, due to less accurate beamforming at DL Rx side, two Tx beams corresponding to different clusters, and thus possess different channel properties regarding Doppler spread, Doppler shift, average delay, delay spread, are within the same Tx beam group. Consequently, these antenna ports cannot be assumed to have the same large-scale parameters in channel estimation and demodulation.

In such case, QCL relationship wrt Tx beamforming should also be indicated at least for channel estimation and demodulation purposes. Therefore, QCL parameters could be grouped according to the functionalities, e.g., beam management, channel estimation, demodulation, phase tracking, RRM measurement, and so forth.

Some examples of QCL parameter grouping are listed in Table 1:

Table 1. Examples of QCL grouping

| Type of QCL parameter set | QCL parameters | Example pairs of RSs/RS resource | Application case |
| --- | --- | --- | --- |
| Set 1 | Rx beam | - Among DMRS ports within the same CC - Among CSI-RS ports within the same CC - Between DMRS ports and CSI-RS ports within the same CC | Beam management |
| Set 2 | Delay spread, average delay, Doppler spread, Doppler shift | Between DMRS ports and CSI-RS ports within the same CC | Demodulation |
| Set 3 | average gain | - Among DMRS ports for PBCH within the same CC - Among CSI-RS ports within the same CC | RRM |

For MIMO transmissions based on multi-TRP and multi-panel, the structure of array and the mapping between antenna ports to panel is transparent to UE. However, via DL measurement, the dependency or independency between ports can be obtained at UE side. Consequently, based on the grouping of ports according to the QCL relationships, the layer-to-DMRS port mapping, CSI feedback and control signaling can be defined. It’s worth noting that, depending on the channel experienced by different UE, the grouping could be different. In other words, antenna ports grouping is UE-specific.

QCL across time-frequency resources

In NR, as the QCL relationship relies highly on beamforming operation, with possibly dynamic change in beamformer in both time and frequency domains, it’s not reasonable to assume that it holds over any ranges of time-frequency resource. For example, even the same analog beam could be used on different CCs, different equivalent beams can still be formed at base band for each CC respectively. In such case, without any explicit or implicit indication, antenna ports transmitted on different CCs can’t be assumed to be QCL-ed. For time domain, if time domain bundling is also supported, UE could make channel estimation jointly over a few of adjacent time units. That is, with explicit or implicit indication, DMRS ports on a bunch of bundled time units could be assumed to be QCL-ed at UE.

Conclusions

In this contribution, we discussed the extension of spatial QCL parameter(s) for DL of NR. Based on the discussion above, we propose:

Proposal 1: Antenna ports can be UE-specifically grouped according to their QCL relationship between each other.

Proposal 2: Support at least the following types of QCL parameter set:

| Type of QCL parameter set | QCL parameters | Example pairs of RSs/RS resource | Application case |
| --- | --- | --- | --- |
| Set 1 | Rx beam | - Among DMRS ports within the same CC - Among CSI-RS ports within the same CC - Between DMRS ports and CSI-RS ports within the same CC | Beam management |
| Set 2 | Delay spread, average delay, Doppler spread, Doppler shift | Between DMRS ports and CSI-RS ports within the same CC | Demodulation |
| Set 3 | average gain | - Among DMRS ports for PBCH within the same CC - Among CSI-RS ports within the same CC | RRM |

Proposal 3: Without any explicit or implicit indication, antenna ports transmitted on different CCs can’t be assumed to be QCL-ed w.r.t. spatial domain QCL parameters.

Proposal 4: With explicit or implicit indication, DMRS ports on a bunch of bundled time units could be assumed to be QCL-ed at UE.

- References

- RAN1 #86bis Chairman’s Notes.

- RAN1 #87 Chairman’s Notes.

- RAN1 AH NR Meeting Chairman’s Notes.

- RAN1 #88 Chairman’s Notes.

- R1-1612053, “Views on Quasi Co-Located Antenna Ports”, Qualcomm