3GPP TSG RAN WG1 Meeting #89			      	R1-1707632

Hangzhou, P.R. China 15th – 19th May 2017

Agenda Item:	7.1.3.1.4

Source: 	LG Electronics

Title: 	Discussion on DCI contents

Document for:	Discussion and decision

- Introduction

In this contribution, we summarize basic DCI contents that are mainly from existing LTE DCI contents and present our view on the additional DCI contents that need to be introduced for NR.

- Discussion

There are many fields currently under discussion including starting OFDM symbols for data, data duration, etc. We present our views on DCI contents as in below.

|  |  | DL scheduling | UL scheduling |
| --- | --- | --- | --- |
| Resource allocation | Resource allocation in frequency domain | Resource allocation type 0: RBG based resource allocation Resource allocation type 1: RB based resource allocation Resource allocation type 2: contiguous RB(G) allocation RBG size can be configured per UE | Resource allocation type 0: RBG based resource allocation Resource allocation type 1: RB based resource allocation Resource allocation type 2: contiguous RB(G) allocation RBG size can be configured per UE |
|  | Resource allocation in time domain | start and duration of data transmission can be indicated | start and duration of data transmission can be indicated |
|  | Rate matching pattern | Rate matching for dynamic reserved resource or CSI-RS may be indicated | Rate matching for dynamic reserved resource or UCI or SRS may be indicated |
| Demodulation related parameters | MCS | MCS is used to determine TBS along with resource allocation. When CBG retransmission is used, coarse granularity of MCS may be used (e.g., only modulation is indicated) | MCS is used to determine TBS along with resource allocation. When CBG retransmission is used, coarse granularity of MCS may be used (e.g., only modulation is indicated) |
|  | Beam index for data | Control and data may use different TX-RX pairs | DL control, UL data and UCI/SRS may use different TX-RX pairs |
|  | MU-MIMO related parameters | number of layers, DM-RS parameters, etc. | number of layers, DM-RS parameters, etc. |
| HARQ-related parameters | HARQ process ID | A set of HARQ processes can be configured for DL | A set of HARQ processes can be configured for UL |
|  | NDI |  |  |
|  | HARQ-ACK time/frequency resource offset | Timing and frequency resource for HARQ-ACK is indicated | N/A |
|  | CBG bitmap | When CBG-based retransmission is configured, CBG information can be indicated at retransmission | When CBG-based retransmission is configured, CBG information can be indicated at retransmission |
|  | CBG vs TB based transmission | Whether the scheduling is for CBG based or TB based needs to be indicated | Whether the scheduling is for CBG based or TB based needs to be indicated |
|  | A/N codebook related information (e.g., total DAI) | Support multiple bits of HARQ-ACK transmission | N/A |
|  | RV |  |  |
| Miscellaneous | UL grant/DL scheduling bit | If UL grant and DL scheduling DCI size is aligned, indication is necessary | If UL grant and DL scheduling DCI size is aligned, indication is necessary |
|  | Aperiodic measurement trigger (e.g., CSI, SRS) |  |  |
|  | TPC | For PUCCH | For PUSCH |

- Conclusions

We discussed potential DCI contents in this contribution. We consider LTE DCI contents can be a starting point where additional DCI fields to support CBG based retransmission, various scheduling units, reserved resource, and flexible resource allocation are necessary.

- Reference