3GPP TSG RAN WG1 Meeting #89	R1-1707965

Hangzhou, China 15th – 19th May 2017

Agenda item:	7.1.2.3.5

Source: 	Samsung

Title: 	System-level evaluation for Type II CSI

Document for:	Discussion and Decision

# Introduction

In RAN #75, a new WID related to new radio (NR) has been approved [1]. The objective of the NR WID as captured in [1] includes the following (copied from Section 4.1 of [1]).

According to the outcome of the study item, the NR should specify the followings [TR38.912]: - Physical layer aspects including [RAN1]: - … - Downlink and uplink functionality related to multi-antenna transmission/reception enabling closed loop and open/semi-open loop transmissions, beam management, interference measurement, Type I codebook-based CSI acquisition and Type II CSI acquisition as well as CSI acquisition for reciprocity-based operation, the associated reference signal designs, and related quasi-colocation assumptions.

In RAN1-NR#1 [2] and RAN1#88 [3], the following agreements about Type II CSI acquisition (highlighted text above) were made.

Agreements:

- Support at least one scheme taken from Category 1, 2, and/or 3 for Type II CSI

  - Possible down selection can be performed throughout Phase I WI

    - If more than one schemes is supported, these schemes should be complementary

    - This includes further refinement within each category

    - Note: other schemes within each category are not precluded

  - Descriptions for Category 1 and 2 are given in the following slides

    - For the purpose of summary in TR38.802

- Category 1: precoder feedback based on linear combination codebook

- Dual-stage W = W1W2 codebook

  - W1 consists of a set of L orthogonal beams, e.g. 2D DFT beams

    - The set of L beams is selected out of a basis, e.g. oversampled 2D DFT beams

    - Beam selection is wideband

  - W2: L beams are combined in W2 with common W1

    - Subband reporting of phase quantization of beam combining coefficients

  - Beam amplitude scaling quantization can be configured for wideband or subband reporting

    - …

Agreements:

- Refine the description in 38.802 for Type II CSI Category I as follows

  - Dual-stage W = W1W2 codebook

    - W1 consists of a set of L orthogonal beams taken from 2D DFT beams

      - The set of L beams is selected out of a basis composed of oversampled 2D DFT beams

        - L {2, 3, 4, FFS 6} (L is configurable)

      - Beam selection is wideband

    - W2: L beams are combined in W2 with common W1,

      - Subband reporting of phase quantization of beam combining coefficients

        - Configurable between QPSK and 8-PSK phase related information quantization

- …..

The companion contribution [4] proposes a linear combination based codebook for the Type II CSI reporting for Category 1. This contribution provides additional simulation results in support of the proposed codebook and provides the rationale behind the significant gains shown by the proposed codebook over other Type II codebook proposals. To reduce CSI payload in each CSI reporting instance, a differential CSI based Type II CSI reporting mechanism is proposed in the companion contribution [5].

# Simulation results for Type II CSI Codebook [4]

A linear combination based Type II CSI codebook W = W1W2 is proposed in [4], where

- W1 is used for WB reporting of the first PMI (PMI1), which indicates L beams selected freely from the orthogonal DFT basis, and the index of the strongest beam/coefficient (out of 2L coefficients).

  - L beam selection is common for all layers, and

  - the strongest beam selection is per layer and can correspond to any one of the two polarizations;

- W2 is used for SB reporting of the second PMI (PMI2), which indicates phase (and amplitude if reported SB) of remaining 2L – 1 linear combination coefficients (the strongest coefficient is set to 1).

For amplitude reporting, three types of reporting has been considered: WB only, SB only, or differential WB + SB.

For performance evaluation of the proposed Type II CSI codebook, the non-full-buffer system-level evaluation is carried out for UMi channel model in medium (50% target RU) traffic loading scenario, and dynamic switching between SU-MIMO and MU-MIMO is considered in the simulation. The results are provided for 16 antenna ports with (N1, N2) = (4, 2), where we assume that the first dimension is horizontal and the second dimension is vertical. The relevant simulation parameters are enlisted in Table 1. For comparison, the Rel. 14 advanced CSI codebook is considered as reference, and ideal CSI in which dominant eigenvectors are known at gNB is considered as performance upper bound.

Evaluation 1: W1 basis

In RAN1-NR#1 [2], an agreement was made in which different schemes were proposed to be studied belonging to Category 1. One of the main differences between these schemes is the structure of the W1 basis matrix. The following three types of basis matrix were proposed for further study.

- Basis 0 (2 partitions): W1=B00B where BHB=I.

- Basis 1 (Hadamard-type): W1=B1B2B1-B2 where BiHBi=I.

- Basis 2 (4 partitions): W1=B00B00000000B00B where BHB=I.

The three alternatives for the W1 basis matrix are evaluated. The simulation results for L = 4 beams are shown in Figure 1 (for unquantized amplitude and phase) and Figure 2 (for quantized amplitude and phase; 3 bit quantization is used for both), where phase reporting is SB, and amplitude reporting is either WB or SB. We can make the following observation.

Observation 1:

- Basis 2 (four partitions) is the worst of the three basis matrices;

- Basis 0 (2 partitions) and Basis 1 (Hadamard-type) are comparable in performance, but Basis 1 requires more W1 reporting overhead (because of independent selection of B1 and B2) and has more selection complexity than Basis 0.

- Similar observations can be made with both unquantized and quantized amplitude and phase, and both WB and SB amplitude reporting.

We therefore propose the following.

Proposal 1: For Type II CSI reporting, support Basis 0 (2 partitions) as the W1 basis matrix.

Figure 1: Performance comparison of different W1 basis

Figure 2: Performance comparison of different W1 basis

Evaluation 2: amplitude reporting type

Assuming Basis 0, we next evaluate the following four alternatives for the amplitude reporting types for two layers.

| Type 0: 1a1⋯aL-11a1⋯aL-11a1⋯aL-11a1⋯aL-1T | Type 1: 1a1⋯aL-11a1⋯aL-11a1'⋯aL-1'1a1'⋯aL-1'T |
| --- | --- |
| Type 2: 1a1⋯aL-1aLaL+1⋯a2L-11a1⋯aL-1aLaL+1⋯a2L-1T | Type 3: 1a1⋯aL-1aLaL+1⋯a2L-11a1'⋯aL-1'aL'aL+1'⋯a2L-1'T |

The simulation results for L = 4 beams are shown in Figure 3 (WB amplitude reporting) and Figure 4 (SB amplitude reporting), where amplitude and phase of coefficients are unquantized. The results for 3 bit phase and 3 bit amplitude quantization are shown in Figure 5 (WB amplitude reporting) and Figure 6 (SB amplitude reporting). We can make the following observation.

Observation 2: Between four amplitude reporting types, amplitude type 3 (independent amplitude for two polarizations and two layers) shows significant performance gain over amplitude types 0, 1, and 2 for both WB and SB amplitude reporting, and for both unquantized and quantized amplitude and phase.

- ~10-12% and ~22-25% additional gain in avg. and 5% UPT, respectively with amplitude type 3 when compared with amplitude type 0, 1, and 2

One can argue that for WB amplitude reporting, the performance of the four amplitude types should be close assuming that WB beam power across two polarizations and across two layers are close. We however observe up to more than 12% and more than 25% additional gain in avg. and 5% UPT, respectively with amplitude type 3 when compared with amplitude type 0, 1, and 2. The reason for this phenomenon is significant difference in WB power levels across two polarizations and two layers, which can evident from power ratio distribution plots (in dB) shown in Figure 7 and Figure 8 where unquantized power is assumed.

We therefore propose the following.

Proposal 2: For amplitude reporting, amplitude type 3 (independent amplitude for two polarizations and two layers) is supported.

Figure 3: Performance comparison of four amplitude types

Figure 4: Performance comparison of four amplitude types

Figure 5: Performance comparison of four amplitude types

Figure 6: Performance comparison of four amplitude types

Figure 7: Distribution of WB power ratio across two polarizations and two layers for 16 ports

Figure 8: Distribution of WB power ratio across two polarizations and two layers for 32 ports

Evaluation 3: WB vs. WB+SB amplitude

Assuming Basis 0 and amplitude type 3, we next provide simulation results for amplitude and phase quantization of 2L-1 coefficients, where NP bit 2NP –PSK phase codebook are considered. For amplitude quantization, the following alternatives are evaluated:

- WB only: 3 bits amplitude codebook in [0, 1]

- SB only: 3 bits amplitude codebook in [0, 1]

- WB + SB: 3 bits and 1 bit amplitude codebooks in [0, 1] for WB and SB amplitude respectively.

The simulation results for L = 4 beams are shown in Figure 9. The results for unquantized amplitude and phase (labelled as ‘Unquantized’) are also shown. We make the following observation and proposal.

Observation 3:

- Although SB only amplitude reporting can achieve up to ~10% and ~23% additional gain in avg. and 5% UPT when compared with WB only amplitude reporting, the total overhead increase is significantly large.

- On the other hand, WB+SB amplitude reporting (with 1 bit SB amplitude) can achieve up to ~5% and ~16% additional gain in avg. and 5% UPT when compared with WB only amplitude reporting, while keeping the total overhead increase reasonable.

Proposal 3: WB + SB amplitude reporting (with 1-bit SB amplitude) is supported in Type II CSI codebook proposed in [4].

Figure 9: WB vs. SB vs. WB + SB amplitude reporting

# Conclusions

In this contribution, additional simulation results in support of the proposed Type II CSI codebook [4] are provided. The observations and proposals made are summarized as follows.

Observation 1:

- Basis 2 (four partitions) is the worst of the three basis matrices;

- Basis 0 (2 partitions) and Basis 1 (Hadamard-type) are comparable in performance, but Basis 1 requires more W1 reporting overhead (because of independent selection of B1 and B2) and has more selection complexity than Basis 0.

- Similar observations can be made with both unquantized and quantized amplitude and phase, and both WB and SB amplitude reporting.

Proposal 1: For Type II CSI reporting, support Basis 0 (2 partitions) as the W1 basis matrix.

Observation 2: Between four amplitude reporting types, amplitude type 3 (independent amplitude for two polarizations and two layers) shows significant performance gain over amplitude types 0, 1, and 2 for both WB and SB amplitude reporting, and for both unquantized and quantized amplitude and phase.

- ~10-12% and ~22-25% additional gain in avg. and 5% UPT, respectively with amplitude type 3 when compared with amplitude type 0, 1, and 2

Proposal 2: For amplitude reporting, amplitude type 3 (independent amplitude for two polarizations and two layers) is supported.

Observation 3:

- Although SB only amplitude reporting can achieve up to ~10% and ~23% additional gain in avg. and 5% UPT when compared with WB only amplitude reporting, the total overhead increase is significantly large.

- On the other hand, WB+SB amplitude reporting (with 1 bit SB amplitude) can achieve up to ~5% and ~16% additional gain in avg. and 5% UPT when compared with WB only amplitude reporting, while keeping the total overhead increase reasonable.

Proposal 3: WB + SB amplitude reporting (with 1-bit SB amplitude) is supported in Type II CSI codebook proposed in [4].

# References

- RP-170855, “New WID on New Radio Access Technology,” NTT DOCOMO Inc.

- 3GPP, RAN1-NR#1, Chairman’s Notes

- 3GPP, RAN1#88, Chairman’s Notes

- R1-1707962, “Type II CSI reporting,” Samsung.

- R1-1707964, “Differential Type II CSI reporting,” Samsung.

# Appendix: Simulation Assumptions

Table 1: Simulation Parameters

| Parameters | Values |
| --- | --- |
| Simulation Type | Non-full-buffer (Medium load 50% Target RU, Lambda = 4) |
| Channel model | UMi-2GHz |
| Number of BS (H,V) antenna elements | (8,8), x-polarized, subarray partition |
| (N1,N2, P) | 16 ports: (4,2,2) |
| (O1,O2) | (8,8) |
| BS (H,V) antenna spacing | (0.5, 0.8)λ |
| BS and MS antenna polarizations | BS: (+45°,-45°); MS: (0°, 90°) |
| Number of UE antennas | 2 |
| SU/MU pre-coding | SLNR |
| Scheduling | MU, Proportional fair, up to 4 layers |
| Channel estimation | Non-ideal |
| Transmission rank | 1,2 |
| Receiver | MMSE-IRC |
| CSI feedback schemes | Reference: Rel. 14 advanced CSI codebook Type II CSI codebook: proposed in [4] Ideal: dominant eigenvectors are known to the gNB/TRP |