3GPP TSG RAN WG1#89					          	                 	R1-1708509

Hangzhou, P.R. China, 15th – 19th May 2017

Agenda item:		7.1.3.2.1

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	On the short PUCCH for small UCI payloads

Document for:		Discussion and Decision

# 1	Introduction

Rel-14 NR study item [1] has been closed and a new Rel-15 WI dealing with New Radio Access Technology [2] has been approved. The work item should specify the NR functionalities for enhanced mobile broadband (eMBB) and ultra-reliable low-latency-communication (URLLC) as defined in [3]. The NR under this work item should consider frequency ranges up to 52.6 GHz.

This contribution relates to short PUCCH design in NR, more specifically to a scenario where UCI payload carried via PUCCH consists of just one or two bits. In the previous RAN1 meetings, the following agreements related to short PUCCH were made [4][5]:

Agreements: [4]

For PUCCH in short-duration

- At least following is supported for PUCCH in 1-symbol duration:

  - UCI and RS are multiplexed in the given OFDM symbol in FDM manner if RS is multiplexed.

    - Same SCS between DL/UL data and PUCCH in short-duration in the same slot.

- At least a PUCCH in short-duration spanning 2-symbol duration of a slot is supported.

  - FFS actual structure and waveform.

  - Same SCS between DL/UL data and PUCCH in short-duration in the same slot.

Agreements: [5]

- For 1-symbol PUCCH without SR with 1 or 2 bit(s) UCI payload size, RAN1 will select one from the following options.

  - Option 1: RS and UCI are multiplexed by FDM manner in the OFDM symbol

    - UCI can be sequence

    - FFS: low PAPR design is applied

  - Option 4: Sequence selection with low PAPR

- FFS following cases: If SR only / If with SR + other UCI;

  - This does not imply the necessity of special SR design

  - FFS whether the design may or may not depend on the frequency range

Agreements: [5]

- For 2-symbol NR-PUCCH, following options are considered (including possible down-selection)

  - Option 1: 2-symbol NR-PUCCH is composed of two 1-symbol NR-PUCCHs conveying the same UCI.

    - 1-1: Same UCI is repeated across the symbols using repetition of a 1-symbol NR-PUCCH.

    - 1-2: UCI is encoded and the encoded UCI bits are distributed across the symbols.

  - Option 2: 2-symbol NR-PUCCH is composed of two symbols conveying different UCIs.

    - E.g., time-sensitive UCI (e.g., HARQ-ACK) is in the second symbole

We provide details related to the short PUCCH scenario with UCI payloads from a few bits to at least tens of bits in a companion contribution [6].

# 2	Discussion

## 2.1. 	Multiplexing options for 2-symbol case

In LTE, PUCCH format 1/1a/1b is the solution for conveying small UCI payloads. LTE PUCCH format 1/1a/1b has a high multiplexing capacity providing up-to 36 parallel resources/PRB. This is achieved by means of combination of CAZAC sequence modulation (up-to 12 cyclic shifts per symbol) and orthogonal cover code of length 3.

In addition to FDM between RBs, there is a need to support also orthogonal multiplexing within a PRB allocated to short PUCCH. It can be noted that short PUCCH scenario is not favourable for multiplexing based on orthogonal cover code in time. First of all, it has been agreed in RAN1 #87 that short PUCCH with 1- symbol duration of a slot needs to be supported. Secondly, varying number of symbols available for short PUCCH makes it fairly difficult to utilize orthogonal cover code in time. Hence, multiplexing operation needs to be confined within each symbol of short PUCCH format defined for small UCI payload.

Proposal #1: Multiplexing operation is confined within each symbol of short PUCCH format defined for small UCI payload.

One of the open items related to short PUCCH design is how to construct the transmission in 2-symbol scenario. We think that Option 1-1 where the same UCI is repeated across the symbols using repetition of a 1-symbol NR-PUCCH is the preferred solution for a short PUCCH with small UCI payload. First of all, it represents a modular design, which minimizes the system complexity. It has also inbuilt support for frequency hopping and the solution scales also to scenarios with more than two NR-PUCCH symbols. This may be needed e.g. with mini-slot based operation. Finally, it allows smooth coexistence between short PUCCH of different lengths (1-symbol, 2-symbol).

Proposal #2: Support Option 1-1 for short PUCCH with small UCI payload (the same UCI is repeated across the symbols using repetition of a 1-symbol NR-PUCCH)

## 2.1. 	Multiplexing options for 1-symbol case

Based on learnings from LTE, CDM based on cyclic shifts of a CAZAC sequence provides a straightforward solution for multiplexing within symbol. Figure 1 shows different multiplexing structures for CAZAC sequence based transmission of small UCI payload:

- Single sequence

- Two sequences multiplexed based on IFDM (RPF=2). This corresponds to Option 1 [5].

- Two sequences multiplexed based on TDM. This approach does not support 1-symbol scenario.

Figure 1 Sequence stucture for short PUCCH, a) single sequence b) two sequences with FDM and c) two sequences with TDM.

In the following we consider four different multiplexing options for small UCI payload in the 1-symbol scenario:

- On-off keying: Non-coherent on-off keying supports scheduling request transmission with CM/PAPR comparable to LTE. It can be applied on top of all RS structures shown in Figrue 1. This can be seen as a form of Option 4 defined for 1-symbol PUCCH [5].

- Channel selection: Non-coherent channel selection supports small UCI transmission with CM/PAPR comparable to LTE. One UCI bit can be conveyed using two parallel resources, and two bits using four parallel resources, respectively. It can be applied on top of all RS structures shown in Figrue 1. This can be seen as another form of Option 4 defined for 1-symbol PUCCH [5].

- FDM multiplexing between DMRS and UCI data: This can be based on RS structure shown in Figure 1b. DMRS part utilizes unmodulated Seqence 1, and data part BPSK/QPSK modulated Sequence 2, respectively. This can be seen as a form of Option 1 defined for 1-symbol PUCCH [5]. [7] shows that by carefully selecting cyclic shift pairs for DM-RS and spreading sequence for UCI symbols, CM can be reduced substantially compared to the case when same sequence is applied.

- TDM multiplexing between DMRS and UCI data: This is based on RS structre shown in Figure 1c. It supports up-to two UCI bits with CM/PAPR comparable to LTE. The disadvantage of this approach is that it does not scale to 1-symbol scenario. At the same time, TDM multiplexing between RS and data reduces the opportunities for frequency hopping, which has negative impact to the link performance and control channel coverage.

Based on the discussion above, and performance evaluation in Section 3, we think think that short PUCCH optimized for small UCI payloads should be based on Option 1 and Option 4. This covers both 1-symbol scenario and two symbol scenarios (and scales also to scenarios with more than two symbols).

- Option 1: In order to support high multiplexing capacity for UCI with two bits, it makes sense to consider DMRS –based coherent detection for UCI with small payload. FDM multiplexing between DMRS and UCI data provides a design with reasonably low CM, and it is scalable to any number of symbols allocated to short PUCCH. Furthemore, it’s compatible with any frequency diversity solution discussed in [6].

- Option 4: On the other hand, in order to provide high multiplexing capacity for SR while avoiding the need to transmit signal for every negative SR, Option 4 is an obvious solution.

- In order to flexibly multiplex SR (Option 4) and HARQ-ACK (Option 5) in the same symbol, it makes sense to apply sequence structure shown in Figure 1b as the baseline for SR transmission. In other words on-off keying operates using IFDMA with RPF=2.

It can be noted that the proposal supports CM/PAPR comparable to LTE for scheduling request, and reasonably low CM/PAPR for HARQ-ACK up to 2 bits. We think that this is a sufficient solution taking into account that NR will support also long PUCCH. Regardless of the solution defined for HARQ-ACK transmission on short PUCCH, there will be a significant coverage gap between short PUCCH and long PUCCH. On the other hand, based on the simulation results available, frequency diversity plays a significant role in the link performance of short PUCCH. Low PAPR/CM supported by TDM approach or by channel selection is available only in the case of localized allocation. At the same time, TDM will reduce opportunities for frequency hopping. Channel selection combined with clustered transmission achieves frequency diversity also in the 1-symbol case but at price of lost low PAPR/CM properties. FDM solution combined with frequency hopping or clustered transmission provides reasonable frequency diversity options for both 1-symbol and multi-symbol scenarios.

Based on the discussion above, and simulation results shown in Section 3 we make the following proposals:

Proposal #3: Short PUCCH supports FDM multiplexing between DRMS and UCI with IFDM structure, RPF=2

Proposal #4: Short PUCCH DMRS is a cyclic shift of a CAZAC sequence.

Proposal #5: Short PUCCH UCI is spread by modulating a cyclic shift of a CAZAC sequence.

Proposal #6: Short PUCCH supports on-off keying for scheduling request with CDM based on cyclic shift of a CAZAC sequence (i.e. Option 4)

Proposal #7: Short PUCCH for scheduling request applies IFDM with RPF=2 (i.e. Option 1)

# 3	Performance evaluation

Figure 3 shows six approaches for short PUCCH design assuming one OFDM symbol duration corresponding to 15 kHz subcarrier spacing in the case of symbol splitting [6].

- TDM2: 30 kHz SCS: Localized transmission, TDM between DMRS and UCI, 2 RBs

- TDM4: 30 kHz SCS: Localized transmission, TDM between DMRS and UCI, 4 RBs

- FDM2, Clust.: 30 kHz SCS: Clustered transmission, FDM between DMRS and UCI, 2 RBs (Option 1)

- FDM4, Clust.: 30 kHz SCS: Clustered transmission, FDM between DMRS and UCI, 2 RBs (Option 1)

- FDM2, FH.: 30 kHz SCS: Frequency hopping, FDM between DMRS and UCI, 4 RBs (Option 1)

- FDM4, FH.: 30 kHz SCS: Frequency hopping, FDM between DMRS and UCI, 4 RBs (Option 1)

Figure 2 Six approaches for short PUCCH transmission.

We investigated the performance of these methods by means of link simulations in EPA channel, v=3 km/h. DMRS construction for FDM and TDM is shown in Table 1, and simulation parameters are given in Table 2. The simulated payload corresponds to one UCI bit conveyed via BPSK modulated cyclic shift of a CAZAC sequence. It should be noted that frequency-selective scheduling for short PUCCH was not considered in this simulation. In other words, frequency allocation for short PUCCH (including also localized transmission) is semi-statically configured in all simulation cases.

Simulation results shown in Figure 3 (2RBs) and Figure 4 (4RBs) indicate that FDM outperforms TDM for both 2RB and 4RB allocations. This also indicates that frequency hopping on top of FDM is much better usage for symbol splitting compared to TDM where two symbols are used for time multiplexing between DMRS and UCI. It is also noted that additional frequency diversity provided by clustered transmission is considerable especially with low BER operation points.

Based on the simulation results, it can be noted that FDM (i.e. Option 5) outperforms TDM (i.e. Option 2) in the considered scenario. This holds not only for UL link performance but also for UL coverage, even if the cubic metric difference between FDM and TDM is taken into account (and assuming that maximum regulated PA output power is dimensioned according to DFT-S-OFDM).

Figure 3 Link performance of different S-PUCCH approaches, 2RBs

Figure 4 Link performance of different S-PUCCH approaches, 4 RBs

Table 1 DMRS construction

| FDM | TDM |
| --- | --- |
| Every second RE for DMRS | One symbol for DMRS |

Table 2 Simulation parameters

| Parameter | Value |
| --- | --- |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 2 |
| Modulation | BPSK |
| Channel | EPA, 3 km/h |
| Carrier bandwidth | 20 MHz |
| Carrier frequency | 4 GHz |
| Channel estimation method | MMSE |
| Channel coding | No coding |
| Number of coded Bits | 1 |
| Subcarrier spacing | 30 kHz |
| Number of short PUCCH symbols | 2 |
| Cyclic Prefix length | 2.4 us |

# 4	Conclusions

In this contribution we have investigated short PUCCH design aspects for new radio in the scenario with small UCI payload. Based on the discussion and simulation results, we make the following proposals:

Proposal #1: Multiplexing operation is confied within each symbol for short PUCCH format defined for small UCI payload.

Proposal #2: Support Option 1-1 for short PUCCH with small UCI payload (the same UCI is repeated across the symbols using repetition of a 1-symbol NR-PUCCH)

Proposal #3: Short PUCCH supports FDM multiplexing between DRMS and UCI with IFDM structure, RPF=2

Proposal #4: Short PUCCH DMRS is a cyclic shift of a CAZAC sequence.

Proposal #5: Short PUCCH UCI is spread by modulating a cyclic shift of a CAZAC sequence.

Proposal #6: Short PUCCH supports on-off keying for scheduling request with CDM based on cyclic shift of a CAZAC sequence (i.e. Option 4)

Proposal #7: Short PUCCH for scheduling request applies IFDM with RPF=2 (i.e. Option 1)

# References

- RP-160671, “Study on New Radio Access Technology”, NTT DoCoMo

- RP-170847, “New WID on New Radio Access Technology”, NTT DoCoMo

- TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”, 3GPP

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting NR Ad-Hoc, 3GPP.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #88bis, 3GPP.

- R1-1708508, “On the design of short PUCCH for NR”, Nokia, Alcatel-Lucent Shanghai Bell

- R1-1700365, “CM evaluation for short UL control channel”, Intel Corporation