3GPP TSG-RAN WG1 #89 	R1-1708646

May 15th – 19th, 2017

Hangzhou, P. R. China

Agenda item:	7.1.4.2.1.2

Source: 	Qualcomm Incorporated

Title: 	FRANK polar construction for NR control channel and performance comparison

Document for:	Discussion/Decision

# Introduction

In RAN1#87, Polar codes were adopted as the channel coding scheme for uplink control information and downlink control information (working assumption) for eMBB systems except for very small block length [1]. In RAN1 #88 and #88b, a FRActally eNhanced Kernel (FRANK) construction of polar code is introduced.

FRANK polar is shown to have the following benefits

- A scalable and theoretically justifiable framework of polar code sequence construction

- Enabling efficient and low complexity block based rate matching schemes with superior performance

- Low description complexity, suited for online code construction

In RAN1-88b, sequence design is discussed and the following aspect is agreed to be considered in sequence design:

Conclusions:

At least the factors relevant for evaluating the sequences are:

- Performance

- Information granularity

- Compatibility with rate matching

- Complexity

- Latency

In this contribution, we will further evaluate FRANK polar with respect to the above aspects after a recap of FRANK polar theoretic design principle.

# Information bit allocation for polar code based on reliability metric ordering

Determining the location and distribution of information bits is crucial to polar code design. Most of the existing designs are based on generating a certain reliability metric for each U domain bit (either via SNR dependent numerical density evolution or some formula), sorting those metrics, and selecting the most reliable bits in U domain as information bits.

## Channel polarization

Figure 1 memoryless channel and channel polarization

Let W: XY be a binary-input discrete memoryless channel where channel capacity is C=I(X;Y). (For the example of binary-input, 0 ≤ C ≤ 1).

Channel polarization is a general phenomenon, which can be exploited to create auxiliary channels to achieve coding gain beyond repetition and improve the overall channel coding performance. Consider the following simple example with 1-stage polarization (It can be shown that using polarization to create auxiliary channels via repetition and xor operations while preserving capacity) as shown in the left part of Figure 1:

G2×2=1011

Assume W is a BEC with erasure probability ‘’ and

U1=X1⨁X2=Y1⨁Y2	parity-check (XOR)

U2=X2=X1⨁U1		repetition

It can be readily shown that

Channel W1:U1YN

Erasure probability, ε- = 1-1-ε2=2ε-ε2

Channel W2:U2YN, U1

Erasure probability, + = 2

We denote W+= W2, W- = W1, where  W+ is better than W- as shown about.

The above operation can be performed recursively, increading polarization of the N bits.

The polar sequence design for BEC can be obtained via closed form recursion and sorting of reliabilities of bits in the U domain. For AWGN channels, no closed form expression is available. Most of the existing designs utlize a density-evolution approach with Gaussian mean approximation (GA-DE). In coding literature, mutual information (MI) has been an important metric for DE/EXIT chart design for LDPC/turbo code [3][4]. MI is an important and robust measure of reliability (more robust than the mean sometimes) and we will also show that MI delivers additional information that is not available in mean-based evolution which is critical to the FRANK polar design.

## Discussion on sequence with reliability ordering

There are many discussions regarding reliability ordering, we can look at either BER for U-domain bits or MI output for those bits as a function of the channel input MI. Examples of MI transfer chart for N=32 is shown in Figure 2. It can be shown that, when N is small, total ordering of reliability is possible (for example N = 2, 4, 8, 16 for AWGN channels). However, starting from N = 32, such total ordering does not exist. As can be seen in Figure 4, there are some crossovers of curves in both low MI output and high MI output regions. As a result, there is no total ordering of these channel bits as it will be dependent on which BER/MI output region is concerned. However, for practical purposes, one does not need to care about low MI output region due to the high BER. For short sequence design of length N = 32 or 64, one can simply focus on high MI output region and simply determine the reliability ordering of the short sequence in the region of interest.

Figure 2 MI transfer chart N = 32 polar code

Sequence design with such a procedure is guaranteed to be nested, in the sense that, the ordering to achieve a target MI output value will be fixed, ensuring one nested sequence design to cover the code rate from 1/N to (N-1)/N. An example of MI nested sequence design for N = 64 is given in Appendix C.

One can naturally use this nested MI sequence procedure for long sequences, which performs well. However, one can see that as N increases, there will be more and more bits that have very close reliabilities to each other, reliability based sequence sorting is feasible, but may not be the most efficient approach. However, with the MI metric, more efficient sequence design can be developed.

## Information bit allocation over channels with polarization

One key question of information bit allocation is how the information bits are distributed in the “good” and the “bad” channels after each stage of polarization. To address this question in general, we first consider an exmple based on the BEC. Suppose N coded bits are transmitted through a channel, N/2 of the coded bits will be transmitted via an equivalent channel W+ and N/2 will be transmitted via an equivalent channel W-. As an example in Figure 1, suppose N = 256, N0 = 128 for the W- channel and N1 = 128 for the W+ channel.

Figure 3 memoryless channel and polar code

We make the following observation: for a polar code of rate R to be asymptotically capacity achieving, the information bit distribution over the channels after polarization should be such that the allocated number of infomation bits leads to a rate that is matched to the capacity of the corresponding channel after polarization. Matching the number of information bits to the capacity of the channel after polarization is required to asymptotically achieve overall channel capacity using successive-cancellation (SC) decoding and facilitates SC and SC list (SCL) decoding at finite length. Hence, the information allocation problem is turned into a rate allocation problem at a macro level.

For example, for the BEC channel, consider an (N, K) code of rate R = K/N. The rate allocated to W- should be R0 = R2, the lower part W+ has R1 = 1 - (1 – R)2 = 2R – R2 The information bit allocation follows the following simple linear relationship:

- The proportion of information bits allocated to the W- channel instances: K0 = R/2 * K;

- The proportion of information bits allocated to the W+ channel instances: K1 = (2-R)/2 * K.

This relationship could be applied recursively to get fine-granularity information-bit distribution with further polarized equivalent channel instances.

To generalize the relationship to other channels, we resort to the law of information combining that has been extensively studied in the coding literature (see [2] [3] [4]). The mutual information of channels after polarization (i.e. one channel with repetition coding; one channel with parity check coding (XOR)) can be plotted against the mutual information of channel inputs. One example of the mutual information chart over an AWGN channel is shown in Figure 4. One approximation by ten Brink et al is included in Appendix of [2]. Simple formula is proposed to tightly approximate the MI combining equation over the AWGN channel. One very simple but accurate approximation is also shown in Appendix A, which can be calculated efficiently in a fixed-point implementation. In Appendix B, approximation of MI output with unequal MI channel input values are considered.

Figure 4 channel polarization and mutual information transfer chart

The mutual information output of the channel W- (channel after XOR) is R0 and channel W+ (channel after repetition) is R1. Both can be derived from mutual information transfer chart (see [2][3][4] for derivations and Appendix A and Appendix B). Since polarization preserves capacity, for a code that is capacity achieving, the following have to be satisfied:

- R = (R0+R1)/2.

- The proportion of information bits allocated to the W- channel instances: K0 = R0/R * K/2;

- The proportion of information bits allocated to the W+ channel instances: K1 = R1/R * K/2.

K0/K ratio could be further illustrated as a function of code rate R designed for a specific channel. This  ratio is channel dependent and is plotted in Figure 5 for the AWGN channel, the BEC and the BSC, respectively. According to the information-combining law, the BEC and BSC serve as the combining upper and lower bounds on the information bit ratio. Different channels require different information bit distribution in order to be capacity achieving.

The number of information bits could be allocated recursively for multiple stages of polarization based on the relationship shown in Figure 5 (here it shows the ratio of information bits to be allocated to the W- channel over info bits allocated to W channel) to achieve asymtotically optimal information-bit allocation in order for the overall code to get good performance. It becomes clear that MI based evolution conveys more information than the mean value evolution in GA-DE, that is, the rate-allocation distribution at each stage of polarization.

Figure 5 information bit percentage chart of W- channel

## Nested property of polar codes

We discuss the nested property of multi-stage channel polarization. Based on Figure 3, it can be easily seen that W- has equal or worse capacity than the W+ channel. We further observe that the U reliability ordering within the group remains invariant across different groups (e.g., reliability of bits within the channels of W- and W+ follow the same ordering across MI inputs ranging from [0, 1]), based on the assumption that the inputs are from the same channel (typically assumed to be from an AWGN channel):

Observation 1: For each step of polarization, where N statistically identical channel inputs are polarized into two groups of channels, W- and W+, the U domain reliability ordering of the bits within the channel group of W- and W+ are identical.

The above property could be used to construct a short reference reliability order sequence (a bit group of length Nref) that could be used to determine the locations of Ki information bits for any constituent code comprising a group of Nref bits. The information bit ratio relationship (in Figure 5) could be applied recursively to obtain the number of information bits Ki for each group of length Nref bits of the code, where the short sequence could be derived either numerically or via some formula.

For example, for a polar code of length N = 512, a reference sequence of length Nref = 64 (constructed based on DE or other methods), for any payload size K, the U0~U511 could be partitioned into 8 consecutive groups in the U domain, each of them having the same length as Nref with information bit allocation to each group i as Ki from {K0, …, K7}. With the knowledge of Ki, the information bit location within each group could be determined using the short sequence.

Observation 2: A polar code information bit location mapping could be determined by:

- the number of information bits allocated to each group in the U domain of the polar code based on mutual information recursion

- A short sequence that maps the number of information bits to the information bit location within each group.

## Details of MI transfer (MIT) chart of AWGN channel without and with rate matching

Figure 6 and Figure 7 illustrate examples of the recursion based on the FRANK construction with and without puncturing, where the grey boxes in Figure 7 represent punctured bit groups. Note that, the only difference in these two cases is that the mutual information evolution calculation will account for the puncturing (and similarly for shortening and repetition). As a result, FRANK construction is not only scalable to codes of length N = 2^m, but also works well with block-based puncturing and shortening schemes. For more details on different rate matching schemes, we refer the reader to [9].

Figure 6 recursive information bit allocation

Figure 7 recursive information with block based rate matching

In N=2^m case as in Figure 6, the info bit distribution is based on MI calculation where the input MI to the recursion is set to capacity MI = R = K/N (that is the maximum rate a channel code can support). Recursive calculation the number of information bits for each group, e.g. K0, K1, then subsequently, K00, K01, K10, K11, etc. are carried out.

In the case where N is not equal to 2^m as in Figure 7, the info bit distribution is based on MI calculation accounting for punctured/shortened bits. The mother code length is Nmax, which is the closest power of 2 upper bound of N. In this example, the MI of non-punctured bit groups is also set to capacity MI = R = K/N. In addition, MI of punctured bit groups (total of (Nmax – N) bits) are set to zero to accurately model the MI evolution throughout the polarization procedure. Similar recursive calculation the number of information bits for each group, e.g. K0, K1, then subsequently, K00, K01, K10, K11, etc. are carried out.

Observation 3: FRANK polar construction could easily take into account the impact of puncturing, shortening, repetition, which facilitates the design of block-based rate matching schemes

# Evaluation of FRANK polar design

## Performance

### Numerical simulation Configurations

The simulation setup of the evaluation in this section is summarized in Table 2. FRANK polar with block-based rate-matching schemes is compared with the PW sequence with bit reversed-based rate-matching.

Table 2 Simulation parameters for control channel

| Channel | AWGN |  |
| --- | --- | --- |
| Modulation | QPSK |  |
| Code Construction | CA-SCL |  |
| Sequence Design | FRANK | PW |
| CRC Concatenation | 19-bit CRC with Polar |  |
| Decoding algorithm | CA-SCL with L=8 and other list sizes (pruned to L<=8 based on path metric before CRC based pruning) |  |
| Code rate | 1/6, 1/3, 1/2, 2/3 (and others in N = 2^m sweep) |  |
| Info. block length | {32, 48, 64 80, 120, 200} –16 (and others in N = 2^m sweep) |  |
| Rate matching scheme | Block puncturing/shortening/ repetition | Bit-rev shortening |

We first perform a quick performance sweep of K values in control region with N = 2^m and then evaluate more general cases where N is not a power of 2 and the code length and rate combinations are based on RAN1 control channel performance evaluation agreements.

### Code length N = 2^m

Figure 8 shows the performance of FRANK polar vs. PW polar with N = 256 and N = 512 respectively. As shown, in both cases, the performance is similar, which is understandable as PW is specifically tuned to match good AWGN performance with N = 2^m.

Figure 8 Performance comparison of FRANK polar vs. PW polar with N = 2^m

## Code length N is not power of 2 and comparison of different rate matching schemes

When N is not necessarily power of 2, rate matching plays a role together with sequence design. Here, we evaluate control channel performance of FRANK polar with block puncturing, shortening, and repetition and compare it to PW with bit-reversed shortening. It can be seen that across code rate and list size, FRANK polar performs consistently better. FRANK polar performs better with block shortening at high coding rate (R = 2/3 in this case), and performs better with block puncturing at low coding rate (R <=1/2). Note that, with large list size, the number of CRC bits does not change, path pruning is perfomed based on path metric to get down to L = 8 before the CRC check. It can be seen that, when the information-bit allocation is done as in the FRANK polar case, with L = 32, there are substantial gains for FRANK polar with block rate-matching over PW with bit-reversed shortening (~0.4dB).

To facilitate reproducing these results, the information bit distributions of FRANK with block puncturing/shortening/repetition cases are listed in Appendix D and the associated short sequence is shown in Appendix C, which is based on the nested MI sequence design. The actual number of information bits defined here is (K+19) considering the additional CRC bits for FAR normalization. Code block length for a given code rate is determined based on (K+16) (to enable fair comparisons withith other types of polar construction. For example, PC or distributed-CRC based polar).

Figure 9 Performance comparison of FRANK polar vs. PW polar for control channels (L = 8)

Figure 10 Performance comparison of FRANK polar vs. PW polar for control channels (L = 32)

Figure 11 Performance comparison of FRANK polar vs. PW polar for control channels (L = 2)

Gain as a function of list size

Figure 12 Performance comparison of FRANK polar vs. PW polar sweeping list size

Finally, in Figure 12, it is shown that, with proper sequence design and rate matching (that is FRANK sequence + block based rate matching). The performance continues to improve as list size increases (with normalized FAR). Again, the effect of relatively smaller gain with large list size for PW + bit-rev shortening is likely due to the fact that the bit-reversed shortening and PW (or DE) sequence construction does not take into account rate matching, which results in a suboptimal code. One reason is that bit-rev shortening is deleting high weight rows, degrading the overall code distance spectrum and also after the shortening, the bit reliabilities could have changed or the polarization could have been less effective. More simulation results with list size sweep showing similar trend can be found in Appendix E.

Observation 4: FRANK polar with block rate matching shows superior performance and low complexity than PW polar with bit-rev shortening

Proposal 1: RAN1 to adopt FRANK construction of polar code based on nested extension of short sequence as baseline

Proposal 2: RAN1 to adopt block puncturing/shortening/repetition as baseline rate matching schemes in conjunction with FRANK polar for its superior performance and low complexity

## Information granularity

We perform some information granularity performance sweep in this section. Based on control channel session agreement, 6-REG to CCE mapping is adopted as working assumption, hence factor of 3 based rate matching will be needed. we evaluate a few cases here. It can be easily seen that in DCI region, FRANK based block puncturing based performance is consistently better and more stable than PW with bit-reversed shortening rate matching scheme. Note that here it is PC+CA polar consturction with PW and bit-reversed shortening. The overall performance of PC+CA is similar to that of CA polar only with PW sequence. However, CA polar is significantly better than PC+CA with FRANK polar.

Figure 13 Comparison of FRANK (block puncture) vs. PW sequence (bit-rev shortening) with bit-granularity sweep and list size sweep (N = 216, 432)

Figure 14 Comparison of FRANK (block puncture) vs. PW sequence (bit-rev shortening) with bit-granularity sweep and list size sweep (N = 192, 384)

## Compatibility with rate matching

It was shown in [8] that the FRANK polar construction works with bit-reversed shortening with performance similar to that obtained from DE (with the code rate properly accounting for the shortening of the code). It is further shown in [9] that FRANK polar (without any PC dynamic frozen bits) works even more favourably in the scenario of block-based rate matching and its performance is substantially better than PW with bit-reversed shortening both with and without PC bits across the rate region, payload size, and list size. At the same time, the decoding complexity and latency are also substantially lower with block-based rate-matching schemes [7]. Some simulation results compare PW sequence design with FRANK sequence design to show the robustness of FRANK sequence under block puncturing across a wide rate and block length range. PW sequence on the other hand is not stable with block puncturing for certain rate and payload size combinations.

Figure 15 Comparison of FRANK vs. PW polar under block puncture for control channels (L = 8)

## Complexity and latency

As discussed in previous sections, FRANK polar can work very nicely with different types of rate matching schemes with stable performance. It is also discussed in [10] that  block based rate matching scheme has a decoding latency advantage compared with bit-rev shortening scheme. Hence FRANK polar can work very well with low complexity and high performance raet matching schemes. Detailed analysis of decoding complexity and latency can be found in [10].

# Conclusions

In this contribution, a short sequence with nested extension design of Polar codes for control channel is proposed. The performance of the proposed solution is evaluated and compared in contribution.

Observation 1: For each step of polarization, where N statistically identical channel inputs are polarized into two groups of channels, W- and W+, the U domain reliability ordering of the bits within the channel group of W- and W+ are identical.

Observation 2: A polar code information bit location mapping could be determined by:

- the number of information bits allocated to each group in the U domain of the polar code based on mutual information recursion

- A short sequence that maps the number of information bits to the information bit location within each group.

Observation 3: FRANK polar construction could easily take into account the impact of puncturing, shortening, repetition, which facilitates the design of block-based rate matching schemes

Observation 4: FRANK polar with block rate matching shows superior performance and low complexity than PW polar with bit-rev shortening

Observation 5: FRANK polar is superior in performance, fine information granularity with good performance, and works seamlessly with various rate matching schemes to achieve good complexity/latency.

Proposal 1: RAN1 to adopt FRANK construction of polar code based on nested extension of short sequence as baseline

Proposal 2: RAN1 to adopt block puncturing/shortening/repetition as baseline rate matching schemes in conjunction with FRANK polar for its superior performance and low complexity

# References

- Chairman’s notes RAN1 #87

- S. ten Brink, G. Kramer, and A. Ashikhmin, “Design of low-density parity-check codes for modulation and detection,” IEEE Trans. Commun., vol. 52, no. 4, pp. 670–678, Apr. 2004.

- I. Land, S. Huettinger, P. A. Hoeher, and J. Huber, “Bounds on information combining,” IEEE Trans. Inform. Theory, vol. 51, no. 2, pp. 612–619, Feb. 2005

- I. Sutskover, S. Shamai (Shitz), and J. Ziv, “Extremes of information combining,” IEEE Trans. Inform. Theory, vol. 51, no. 4, pp. 1313–1325, Apr. 2005

- R1-167209, “Polar code design and rate matching”, Huawei, HiSilicon, Gothenburg, Sweden

- R1-1608862, “Polar Code Construction for NR”, Huawei, HiSilicon, Lisbon, Portugal

- R1-1705632 “Polar Coding Construction and Rate Matching Impact on Complexity and Latency”, Qualcomm Inc.

- R1-1700832, “Design of Polar codes for control channel”	Qualcomm Incorporated

- R1-1705634 “A comprehensive Rate-matching scheme for polar codes and performance evaluation”

- R1-1708645 “Polar Coding Construction and Rate Matching Impact on Complexity and Latency”

# Appendix A: simple mutual information transfer chart approximation for AWGN channel from

For BEC channel, we have:

Cbec+=2C-C2

Cbec-=C2

No closed form for AWGN channel is known, however, looking at the BEC and AWGN curves, they are very close and the following simple formula is a good approx.:

Cawgn+=2C-C2-δ

Cawgn-=C2+ δ

where, δ=-C-0.532+164

# Appendix B: simple mutual information transfer chart approximation for AWGN channel with unequal MI input values

In this case, input mutual information values are C and C*alpha. The output mutual information exact calculation and approximations are shown in the following figure. Here the approximation is just an example and further optimization is possible.

Cawgn+=(1+α)C-αC2-δ

Cawgn-=αC2+ δ

where, δ=α-C-0.532+164

# Appendix C: an example of short sequence N = 64 used for FRANK polar code construction

Example of a short sequence of length N = 64 used for simulation evaluations. MI design target threshold set to 0.98. The sequence is ordered in descending order of reliability.

Nested MI Sequence = [  63    62    61    59    55    47    60    31    58    57    54    53    46    51    45    43    30    29    39    56    27    52    23    50    44    49    42    15    41    28    38    26    37    25    35    22    21    14    19    48    13    40    11    36     7    24    34    20    33    18    12    17    10     9     6     5     3    32    16     8     4     2     1     0]

# Appendix D: examples of info bit distribution generated based on FRANK polar construction

Examples of information-bit distributions obtained via the FRANK polar construction.

Table of FRANK polar code combinations used in the simulation results for RAN1 evaluation (Nmax <= 256)

Table of FRANK polar code combinations used in the simulation results for RAN1 evaluation (Nmax = 512)

Table of FRANK polar code combinations used in the simulation results for RAN1 evaluation (Nmax = 1024)

Table of FRANK polar code combinations used in the simulation results for RAN1 evaluation (Nmax = 2048)

# Appendix E: comparison of Frank Polar and PW Polar with different K and list s