3GPP TSG-RAN WG1 #90	R1-1714101

Prague, Czech Republic, 21th – 25th August 2017

Source:	Ericsson

Title:	Interference impact in LTE networks with aerial vehicles

Agenda Item:	5.2.8.1

Document for:	Discussion and Decision

# Introduction

In RAN#75, the study item on enhanced LTE support for aerial vehicles was approved [1]. The objective of the study is to investigate various RAN1 and RAN2 aspects associated with using terrestrial LTE networks to provide connectivity to aerial vehicles. In RAN1#88bis, RAN1#89, and a follow-up email discussion, most evaluation assumptions were agreed for evaluating the performance of using LTE network deployments with base station antennas targeting terrestrial coverage to serve low altitude aerial vehicles (a.k.a., drones). In this contribution, we present some evaluation results to study the interference impact in LTE networks with low altitude aerial vehicles.

# UMa-AV throughput statistics

In this section, we present evaluation results on UMa-AV packet throughput for data traffic including

- DL 5%, 50%, 95% packet throughput statistics for terrestrial UEs Data traffic

- DL 5%, 50%, 95% packet throughput statistics of aerial UEs Data traffic

All the 5 agreed simulation cases are evaluated.

- Case 1: 0 aerial UE per sector for reference

- Case 2: 1 aerial UE per 10 sectors

- Case 3: 1 aerial UE per sector

- Case 4: 3 aerial UEs per sector

- Case 5: 5 aerial UEs per sector

In all cases, there are 15 UEs per sector including aerial UEs.

Two baseline scenarios are considered: Case 1 with 20% resource utilization (RU), Case 1 with 50% RU.

## UMa-AV downlink

Table 1 presents evaluation results on UMa-AV downlink packet throughput for data traffic.  Note that in Table 1, throughput statistics are separately presented for Terrestrial UEs and Aerial UEs.

Table 1: Throughput statistics for UMa-AV downlink: In Terrestrial UEs results, case 1 is chosen as the baseline for comparison. In Aerial UEs case, case 3 is chosen as the baseline for comparison since there are no aerial UEs in case 1 and there are too few aerial UEs in case 2.

### Low offered traffic

We focus on low offered traffic in this section (i.e., 3.12 Mbps per cell in Table 1). As we can see from Table 1, for the same offered traffic across the 5 cases, resource utilization is increased from 20% in case 1 to 28.9% in case 5.  Table 1 further shows that increasing the ratio of aerial UEs in general leads to decreased DL packet throughput for both terrestrial UEs and aerial UEs.  The following are observed from these results:

- Impact on terrestrial UEs

  - 5% user throughput: Impact is low (less than 3% loss) in Cases 2 and 3. Impact is marginal (less than 15% loss) in Cases 4 and 5.

  - 50% user throughput: Impact is low (less than 2% loss) in Cases 2 and 3. Impact is marginal (less than 7% loss) in Cases 4 and 5.

  - 95% user throughput: Impact is low (less than 2% loss) in Cases 2, 3, 4 and 5.

- Throughputs of aerial UEs

  - 5% user throughput: the cell edge throughputs for aerial UEs are much lower than those of terrestrial UEs. This is because aerial UEs experience more interference in the downlink.

  - 50% user throughput: the medium throughputs for aerial UEs are much lower than those of terrestrial UEs. This is because aerial UEs experience more interference in the downlink.

  - 95% user throughput: the 95%ile throughputs for aerial UEs are much lower than those of terrestrial UEs. This is because aerial UEs experience more interference in the downlink.

From the results, we make the following observations.

In UMa-AV downlink, when the offered traffic per cell is low (i.e., 20% RU in Case 1), the impact of aerial traffic on the throughputs of terrestrial UEs is low in Cases 2 and 3.

- less than 3% loss is observed in 5% user throughput

- less than 2% loss is observed in 50% user throughput

In UMa-AV downlink, when the offered traffic per cell is low (i.e., 20% RU in Case 1), the impact of aerial traffic on the throughputs of terrestrial UEs is marginal in Cases 4 and 5.

- less than 15% loss is observed in 5% user throughput

- less than 7% loss is observed in 50% user throughput

In UMa-AV downlink, when the offered traffic per cell is low (i.e., 20% RU in Case 1), the throughputs of aerial UEs are much lower than those of terrestrial UEs. This is because aerial UEs experience more interference in the downlink.

### High offered traffic

We focus on high offered traffic in this section (i.e., 4.78 Mbps per cell in Table 1). As we can see from Table 1, for the same offered traffic across the 5 cases, resource utilization is increased from 50% in case 1 to 72.5% in case 5.  Table 1 further shows that increasing the ratio of aerial UEs in general leads to decreased DL packet throughput for both terrestrial UEs and aerial UEs.  The following are observed from these results:

- Impact on terrestrial UEs

  - 5% user throughput: Impact is marginal (less than 20% loss) in Cases 2 and 3.  Impact is high (more than 58% loss) in Cases 4 and 5.

  - 50% user throughput: Impact is marginal (less than 14% loss) in Cases 2 and 3. Impact is high (more than 34% loss) in Cases 4 and 5.

  - 95% user throughput: Impact is low (less than 6% loss) in Cases 2 and 3. Impact is notable (17%-24% loss) in Cases 4 and 5.

- Throughputs of aerial UEs

  - 5% user throughput: the cell edge throughputs for aerial UEs are much lower than those of terrestrial UEs. This is because aerial UEs experience more interference in the downlink.

  - 50% user throughput: the medium throughputs for aerial UEs are much lower than those of terrestrial UEs. This is because aerial UEs experience more interference in the downlink.

  - 95% user throughput: the 95%ile throughputs for aerial UEs are much lower than those of terrestrial UEs. This is because aerial UEs experience more interference in the downlink.

From the results, we make the following observations.

In UMa-AV downlink, when the offered traffic per cell is high (i.e., 50% RU in Case 1), the impact of aerial traffic on the throughputs of terrestrial UEs is marginal in Cases 2 and 3.

- less than 20% loss is observed in 5% user throughput

- less than 14% loss is observed in 50% user throughput

In UMa-AV downlink, when the offered traffic per cell is high (i.e., 50% RU in Case 1), the impact of aerial traffic on the throughputs of terrestrial UEs is high in Cases 4 and 5.

- more than 58% loss in 5% user throughput

- more than 34% loss in 50% user throughput

In UMa-AV downlink, when the offered traffic per cell is high (i.e., 50% RU in Case 1), the throughputs of aerial UEs are much lower than those of terrestrial UEs. This is because aerial UEs experience more interference in the downlink.

## UMa-AV Uplink

Table 2 presents evaluation results on UMa-AV uplink packet throughput for data traffic.  Note that in Table 1, throughput statistics are separately presented for Terrestrial UEs and Aerial UEs.  For these results, we assume a power control setting of P0 = -83 dBm and alpha = 0.8.

Table 2: Throughput statistics: UMa-AV uplink: In Terrestrial UEs cases, case 1 is chosen as the baseline for comparison. In Aerial UEs case, case 3 is chosen as the baseline for comparison since there are no aerial UEs in case 1 and there are too few aerial UEs in case 2.

### Low offered traffic

We focus on low offered traffic in this section (i.e., 2.04 Mbps per cell in Table 2). As we can see from Table 2, for the same offered traffic across the 5 cases, resource utilization is nearly unchanged in the uplink.  Table 2 further shows that increasing the ratio of aerial UEs in general leads to decreased UL packet throughput statistics for both terrestrial UEs and aerial UEs.

- Impact on terrestrial UEs

  - 5% user throughput: Impact is low (less than 5% loss) in Cases 2 and 3. Impact is marginal (less than 21% loss) in Cases 4 and Case 5.

  - 50% user throughput: Impact is low (less than 3% loss) in Cases 2 and 3. Impact is marginal (less than 12% loss) in Cases 4 and 5.

  - 95% user throughput: Impact is low (less than 3% loss) in Cases 2, 3, 4 and 5.

- Impact on aerial UEs

  - 5% user throughput: the cell edge throughputs for aerial UEs are much higher than those of terrestrial UEs. This is because aerial UEs experience similar interference in the uplink as terrestrial UEs but enjoy higher desired received signal powers than terrestrial UEs.

  - 50% user throughput: the medium throughputs for aerial UEs are much higher than those of terrestrial UEs. This is because aerial UEs experience similar interference in the uplink as terrestrial UEs but enjoy higher desired received signal powers than terrestrial UEs.

  - 95% user throughput: the 95%ile throughputs for aerial UEs are slightly higher than those of terrestrial UEs. This is because aerial UEs experience similar interference in the uplink as terrestrial UEs but enjoy higher desired received signal powers than terrestrial UEs.

From the results, we make the following observations.

In UMa-AV uplink, when the offered traffic per cell is low (i.e., 20% RU in Case 1), the impact of aerial traffic on the throughputs of terrestrial UEs is low in Cases 2 and 3.

- less than 5% loss is observed in 5% user throughput

- less than 3% loss is observed in 50% user throughput

In UMa-AV uplink, when the offered traffic per cell is low (i.e., 20% RU in Case 1), the impact of aerial traffic on the throughputs of terrestrial UEs is marginal in Cases 4 and 5.

- less than 21% loss is observed in 5% user throughput

- less than 12% loss is observed in 50% user throughput

In UMa-AV uplink, when the offered traffic per cell is low  (i.e., 20% RU in Case 1), the throughputs (especially cell edge throughputs and medium throughputs) for aerial UEs are higher than those of terrestrial UEs. This is because aerial UEs experience similar interference in the uplink as terrestrial UEs but enjoy higher desired received signal powers than terrestrial UEs.

### High offered traffic

We focus on high offered traffic in this section (i.e., 4.22 Mbps per cell in Table 2). As we can see from Table 2, for the same offered traffic across the 5 cases, resource utilization is increased from 50% in case 1 to 62.5% in case 2. Table 2 further shows that increasing the ratio of aerial UEs in general leads to decreased UL packet throughput statistics for both terrestrial UEs and aerial UEs.

- Impact on terrestrial UEs

  - 5% user throughput: Impact is low (no loss) in Case 2. Impact is marginal (less than 12% loss) in Case 3.  Impact is high (more than 38% loss) in Cases 4 and 5.

  - 50% user throughput: Impact is low (less than 4% loss) in Case 2. Impact is marginal (less than 9% loss) in Case 3). Impact is high (more than 19%) in Cases 4 and 5.

  - 95% user throughput: Impact is low (less than 3% loss) in Cases 2 and 3.  Impact is marginal (7%-16% loss) in Cases 4 and 5.

- Impact on aerial UEs

  - 5% user throughput: the cell edge throughputs for aerial UEs are much higher than those of terrestrial UEs. This is because aerial UEs experience similar interference in the uplink as terrestrial UEs but enjoy higher desired received signal powers than terrestrial UEs.

  - 50% user throughput: the medium throughputs for aerial UEs are much higher than those of terrestrial UEs. This is because aerial UEs experience similar interference in the uplink as terrestrial UEs but enjoy higher desired received signal powers than terrestrial UEs.

  - 95% user throughput: the 95%ile throughputs for aerial UEs are slightly higher than those of terrestrial UEs. This is because aerial UEs experience similar interference in the uplink as terrestrial UEs but enjoy higher desired received signal powers than terrestrial UEs.

From the results, we make the following observations.

In UMa-AV uplink, when the offered traffic per cell is high (i.e., 50% RU in Case 1), the impact of aerial traffic on the throughputs of terrestrial UEs is low in Case 2.

- no loss is observed in 5% user throughput

- less than 4% loss is observed in 50% user throughput

In UMa-AV uplink, when the offered traffic per cell is high (i.e., 50% RU in Case 1), the impact of aerial traffic on the throughputs of terrestrial UEs is marginal in Case 3.

- less than 12% loss is observed in 5% throughput

- less than 9% loss is observed in 50% throughput

In UMa-AV uplink, when the offered traffic per cell is high, the impact of aerial traffic on the throughputs of terrestrial UEs is high in Cases 4 and 5.

- more than 38% loss is observed in 5% throughput

- more than 19% loss is observed in 50% throughput

In UMa-AV uplink, when the offered traffic per cell is high (i.e., 50% RU in Case 1), the throughputs (especially cell edge throughputs and medium throughputs) for aerial UEs are higher than those of terrestrial UEs. This is because aerial UEs experience similar interference in the uplink as terrestrial UEs but enjoy higher desired received signal powers than terrestrial UEs.

Capture the results presented in this contribution in the TR.

RAN1 studies interference mitigation techniques for the drone density cases that are identified to have impact on throughput.

# Conclusions

In this contribution, we discuss the remaining evaluation assumptions for the study of enhance LTE support for aerial vehicles. We made the following observation:

Observation 1	In UMa-AV downlink, when the offered traffic per cell is low (i.e., 20% RU in Case 1), the impact of aerial traffic on the throughputs of terrestrial UEs is low in Cases 2 and 3.

	less than 3% loss is observed in 5% user throughput

	less than 2% loss is observed in 50% user throughput

Observation 2	In UMa-AV downlink, when the offered traffic per cell is low (i.e., 20% RU in Case 1), the impact of aerial traffic on the throughputs of terrestrial UEs is marginal in Cases 4 and 5.

	less than 15% loss is observed in 5% user throughput

	less than 7% loss is observed in 50% user throughput

Observation 3	In UMa-AV downlink, when the offered traffic per cell is low (i.e., 20% RU in Case 1), the throughputs of aerial UEs are much lower than those of terrestrial UEs. This is because aerial UEs experience more interference in the downlink.

Observation 4	In UMa-AV downlink, when the offered traffic per cell is high (i.e., 50% RU in Case 1), the impact of aerial traffic on the throughputs of terrestrial UEs is marginal in Cases 2 and 3.

	less than 20% loss is observed in 5% user throughput

	less than 14% loss is observed in 50% user throughput

Observation 5	In UMa-AV downlink, when the offered traffic per cell is high (i.e., 50% RU in Case 1), the impact of aerial traffic on the throughputs of terrestrial UEs is high in Cases 4 and 5.

	more than 58% loss in 5% user throughput

	more than 34% loss in 50% user throughput

Observation 6	In UMa-AV downlink, when the offered traffic per cell is high (i.e., 50% RU in Case 1), the throughputs of aerial UEs are much lower than those of terrestrial UEs. This is because aerial UEs experience more interference in the downlink.

Observation 7	In UMa-AV uplink, when the offered traffic per cell is low (i.e., 20% RU in Case 1), the impact of aerial traffic on the throughputs of terrestrial UEs is low in Cases 2 and 3.

	less than 5% loss is observed in 5% user throughput

	less than 3% loss is observed in 50% user throughput

Observation 8	In UMa-AV uplink, when the offered traffic per cell is low (i.e., 20% RU in Case 1), the impact of aerial traffic on the throughputs of terrestrial UEs is marginal in Cases 4 and 5.

	less than 21% loss is observed in 5% user throughput

	less than 12% loss is observed in 50% user throughput

Observation 9	In UMa-AV uplink, when the offered traffic per cell is low  (i.e., 20% RU in Case 1), the throughputs (especially cell edge throughputs and medium throughputs) for aerial UEs are higher than those of terrestrial UEs. This is because aerial UEs experience similar interference in the uplink as terrestrial UEs but enjoy higher desired received signal powers than terrestrial UEs.

Observation 10	In UMa-AV uplink, when the offered traffic per cell is high (i.e., 50% RU in Case 1), the impact of aerial traffic on the throughputs of terrestrial UEs is low in Case 2.

	no loss is observed in 5% user throughput

	less than 4% loss is observed in 50% user throughput

Observation 11	In UMa-AV uplink, when the offered traffic per cell is high (i.e., 50% RU in Case 1), the impact of aerial traffic on the throughputs of terrestrial UEs is marginal in Case 3.

	less than 12% loss is observed in 5% throughput

	less than 9% loss is observed in 50% throughput

Observation 12	In UMa-AV uplink, when the offered traffic per cell is high, the impact of aerial traffic on the throughputs of terrestrial UEs is high in Cases 4 and 5.

	more than 38% loss is observed in 5% throughput

	more than 19% loss is observed in 50% throughput

Observation 13	In UMa-AV uplink, when the offered traffic per cell is high (i.e., 50% RU in Case 1), the throughputs (especially cell edge throughputs and medium throughputs) for aerial UEs are higher than those of terrestrial UEs. This is because aerial UEs experience similar interference in the uplink as terrestrial UEs but enjoy higher desired received signal powers than terrestrial UEs.

Based on the discussion in this contribution, we propose the following:

Proposal 1	Capture the results presented in this contribution in the TR.

Proposal 2	RAN1 studies interference mitigation techniques for the drone density cases that are identified to have impact on throughput.

# References

RP-170779, “New SID on Enhanced Support for Aerial Vehicles,” NTT DOCOMO INC, Ericsson.